System Release Information

System Release 4.01

for OS/2

Copyright 1993 Dialogic Corporation

All Rights Reserved

OS/2 is a trademark of International

Business Machine Corporation

Releases documented: System Release 4.01.

System Release Development Package

This System Release Development Package

contains the following components:

A diskette set that

contains the software for all products supported in the System

Release.

A single

installation routine that selectively installs any of the software

from the diskettes, and a "System Release Software Installation

Reference" that provides detailed instructions.

A release notes

booklet (also provided electronically on the diskettes) that

provides information about changes, bug fixes, compatibility

issues, and restrictions for each product's software.

This installation selectively installs any of

the software product families contained in the

System Release Development Package.

Change Summary

The following features were added to System

Release 4.0 for System Release 4.01:

FAX

- ASCII to FAX support

- Subpage addressing support for FAX

- Selectable receive page length support

for FAX

For more information refer to the Fax Software

Reference for OS/2 and to the Fax Software

Release Note.

Support for D/42-SL, D/42-SX, and DID/120

boards

For more information refer to the D/42-SL

Hardware Reference, D/42-SL Software

Reference, D/42-SX Hardware Reference, D/42-SX

Software Reference, DID/120 Hardware

Reference, D/42- SL Software Release Note, and

D/42-SX Software Release Note.

Addition of new international parameter

files

For more information refer to the

International Software Release Note.

Upgraded voice recognition vocabularies

For more information refer to your hardware-

specific VR Vocabularies Release Note (not

included in the Release Note Booklet).

System Software Installation

New Features

D/42-SL and D/42-SX boards can now be

installed.

Refer to the System Release Software

Installation Reference for OS/2  for detailed

information about the install.

Improvements

Three missing members of the DL_CAP structure

have been added: ca_pvdmxper, ca_pvdszwnd, and

ca_pvddly. They are used to tune Positive

Voice Detection. Users of System Release 4.0

should recompile their application if: A:.

They are using enhanced call analysis. B: They

are using SIT detection. C: They intend to

tune PVD.

sb_rtrcvmt() function has been added to the

library. This function was not available in

System Release 4.0 even though it appeared in

the documentation.

Multiple D/2x boards are now supported

(previously only a single D/2x could be

supported in a system.) (Resolves PTR 1740)

The termination event priority has been

changed as follows: When two or more

termination conditions are satisfied

simultaneously, the library uses a specific

set of rules to select the termination event

that is returned to the application's DL_CSB.

Loop current drop now has the highest

priority. See list below:

Priority  Before       In SR4.01

SR4.01

Highest  T_DTMF       T_LOOP

T_TDIG       T_DTMF

T_TONEID     T_TDIG

T_MXDIG      T_TONEID

T_EOD        T_MXDIG

T_LOOP       T_EOD

T_PATRN      T_PATRN

T_MXSIL      T_MXSIL

T_MXNSIL     T_MXNSIL

T_IDIG       T_IDIG

T_MXSEC      T_MXSEC

T_STOP       T_STOP

Lowest

Restrictions and Limitations

Be sure to remove your previous installation files prior to

installing your new System Release 4.01.

The total number of voice/FAX and DPD channels in the system

should not exceed 64.

Hints

In certain OS/2 systems with single floppy-

disk drives, you should take particular

care to correctly identify the floppy-disk

drive. If you accidentally type the

incorrect name (for example, "b:" instead

of "a:") the system will generate erroneous

error messages.

If you choose to create an example file

(CONFIG.EXM) when generating CONFIG.SYS,

make sure you copy all the required

Dialogic device modifications from

CONFIG.EXM to CONFIG.SYS.

Be particularly careful in the case where

you are installing a new product after

completing an installation of Voice and any

other products; CONFIG.SYS must contain all

the required information for already

installed products.

To ensure proper operation of the system,

make sure that your CONFIG.SYS  file has

only one "device=C:\genload.sys" line and

only one "device=C:\dxxxdrv.sys" line in

that order. Failure to follow this

guideline may cause the system to "hang" or

the OS/2 session window to remain open

after accessing the Dialogic driver

commands.

To install D/21D boards, the configuration

file must be edited after installation has

been completed:

1. Two channel lines (e.g. c2 and c3)

should be deleted for each D/21D   that is

being installed.

2. The board type should be changed

from D/41D to D/21D.

PTRs Addressed

PTR (Problem Tracking Report) numbers are

assigned to software, hardware and

documentation problems reported to Dialogic.

The following table lists all PTRs that have

been addressed by System Release 4.01. For

detailed information about the PTR

resolutions, refer to the release note for the

specified product.

PTR    Description

1679   Installation errors when

creating example

configuration files on a

Micro-Channel machine.

1697   The D/42-NS board

receives inaccurate

called/calling party ID

when multiple calls

arrive at a D/42-NS

channel on a Norstar

Modular Switch running

DR5 firmware.

1740   Lack of support for

multiple 2-channel

boards.

System Release Development Package

The table below lists the supported and system-

tested Dialogic products for System Release

4.01 .

Supported

Products

D/21D     VR/81-MC    D/42-NS

D/41D     VR/160      D/42-SL

D/41E     DID/120     D/42-SX

D/81A     LSI/120     TTS/20

D/81-MC   LSI/80-UK   TTS/40

D/120     LSI/80-MC   TTS/40-MC

D/121     DTI/101     TTS/80

D/121A    DTI/211     TTS/80-MC

D/121B    DTI/212

FAX/40    DMX

FAX/120   MSI

VR/40     MSI/C

VR/121    DPD/120