Table of Contents

Quick Start & RSetup

Optimal Testing and MAKEBOOT.EXE

What is Memory

Why Memory Fails

A Word About Windows & Memory Testing

How RAMexam Tests

Ways to Use RAMexam

Navigating RAMexam

A Memory Error! Now What?

How to Avoid Memory Errors

Command Line Options

Quick Start & RSetup:

Thank you for purchasing RAMexam.  With it, you can use your

PC with the confidence that your system RAM is functioning properly.

Installing RAMexam

Installing RAMexam is a simple task:

1.     Place the RAMexam diskette in the diskette drive.

2.     Type A: and press enter. (If you are installing from a drive

other than A substitute that drive letter.)

3.     Type RSETUP and press enter.  If the Qualitas Memory Tester is

already installed (386MAX v. 7 includes QMT), RSetup replaces

QMT with RAMexam.  RAMexam includes all the features provided

by QMT.

4.     RSetup prompts you for information, including your name, RAMexam

serial number and the directory you want RAMexam installed into.

The serial number can be found on either the inside front cover of

this manual or on the diskette.

5.     If you use MS-DOS 6 MultiConfig, RSetup asks into which configuration

RAMexam should install.  The chapter Ways to Use RAMexam makes

specific suggestions about taking advantage of RAMexam in a

MultiConfig setup. 6. RSetup displays the scheduled testing setup

screen on which you can tell RSetup to install a RAMexam command

into your AUTOEXEC.BAT file for automated testing.   You may choose

to schedule when RAMexam runs from the following options:

Whenever the system starts (or is rebooted)

Daily  (the first time the system is turned on or booted each day)

Weekly  (you choose which day of the week)

Monthly  (you choose which day of the month)

User Defined  (every n days where n is any number)

During installation, you have the opportunity to view any changes (and abort

them) before they are made to your start-up files.  If you choose to allow

RSetup to make the changes then your original files will be backed-up in the

RAMexam directory.

Optimal Testing and MAKEBOOT.EXE:

The Memory Tester includes a program called MAKEBOOT.EXE that

creates a bootable diskette configured to test your system without any

resident software active.  This is optimal for testing effectiveness.  To

use MAKEBOOT you need a formatted floppy disk.  MAKEBOOT deletes all existing

files from this diskette. The diskette needs to be the correct size to fit

in drive A: (unless your system boots from another floppy).  Put the

diskette into the drive and enter:

MAKEBOOT a: [c:]       where a: is the destination drive and c: is the

optional drive letter from which you normally boot (do not include the

brackets).  MAKEBOOT asks you to choose between having a Full or Quick

test run when booting from the floppy.  A Quick test usually takes under

a minute but is not as thorough as a Full test which can take several hours

(for details, see How RAMexam Tests).  MAKEBOOT transfers the necessary

system files to the diskette and creates appropriate start-up files.

To use this diskette, place it in drive A: and restart your system.

After testing completes, remove the diskette and restart your system once

more to return to your original configuration.

What is Memory?:

The term "computer memory" is ambiguous.  It is often used to describe fixed

and floppy disk storage, system and adapter "read only memory" (ROM), video

adapter "random access memory" (RAM) and so on.  For the purpose of this

documentation, the term "computer memory" (or just "memory") refers to the

row or rows of system RAM chips that exist somewhere in your PC.  This is

the memory in which your software runs and it is the memory RAMexam tests.

Few components in your PC are as crucial to its operation as memory and the

hardware which manages memory. Defective memory can cause data loss, prevent

your monitor from displaying properly or even cause your hard disk to crash.

Every element in your system depends on the correct operation of RAM.

The capacity of PC memory to store information is measured in bytes.  A

typical PC comes standard with several megabytes of RAM.  A megabyte equals

one million bytes (actually 1,048,576) - approximately the amount of memory

needed to hold the text for a 600 page paperback novel.  Each byte on an IBM

compatible system contains eight bits.  This means that the average PC with

four megabytes of RAM has over 33 million bits -- each one ready and waiting

for your valuable information.

Let's take a look at just one bit.  Each bit "holds" one of two values -- 0

or 1 -- depending on its electrical state.  All the information in your PC

amounts to nothing more than a series of bits, each one holding a 0 or 1.

For example, the word "memory" has six characters, each represented as a

byte in memory, for a total of 48 bits of information:

01101101 01100101 01101101 01101111 01110010 01111001

m        e        m        o        r       y

Each bit is significant.  What would happen if the last bit was stored in

a defective memory chip and was stuck as zero? The new word would be "memorx".

And if your software program got loaded into the defective bit then there is

a very good chance your system would crash.  Your PC operates on the

assumption that each bit (all 33 million in the above example) functions

perfectly every time you use your PC.

Why Memory Fails:

People expect most machines to break eventually.  We look at all the moving

parts -- hard drives spinning rapidly for hours on end, car engines running

for thousands of miles -- and we know that eventually the machine will stop

working.  We see it happen every day.

Why, then, should we be so surprised to learn that system memory also fails?

RAM itself may not spin, chug or move but it is acted upon in countless ways

-- ways that, given RAM's fragile nature, can have devastating effects.

In addition to faulty RAM, RAMexam may detect errors which occur due to

faults in the  memory subsystem - the hardware that manages the RAM.

Unfortunately, there is no way for software to determine the difference

between defective RAM and a defective memory subsystem.  The causes for

failure, though, are the same.

Here are some of the most common (though by no means all) reasons that

system memory fails:

Static Electricity

If you have opened your personal computer then you know that under the

metal case are relatively vulnerable electronics.  Brushing against system

RAM while carrying even the slightest static charge can destroy chips by

subjecting them to momentary discharges of excessive electricity.

Power Surges (large and small)

We all know about the devastating effect lightning can have on a PC.

While a $20 surge protector may reduce risk, it does not eliminate it.

In fact, power brown-outs -- where power levels dip momentarily (because

somebody upstairs just started their blow dryer) are not handled by most

surge protectors and can be just as damaging over time as surges.

Actually, your PC is hit with a power surge every time it is turned on

and electricity rushes into the cold system.  Even when these brown-outs

and surges do not cause immediate problems, they can stress the semiconductor

chips, which has a cumulative damaging effect.

Dust, Smoke and Pet Hair

Did you ever wonder why the television is usually the dustiest spot in the

house?  Dust and other air impurities are attracted by the electrical charge

of inside appliances like your television and PC where they layer onto

components including system memory.  Over time, these materials can build

up enough to carry electrical charges and "short" the PC's circuitry.  The

dust also forms a blanket, diminishing the electronics' ability to release

heat and stay cool.  This results in heat stress and additional potential

damage.

Changes in Temperature

Put your hand on the back of a PC that has been running for several hours.

It is very warm.  As with all things, PC components expand when their

temperatures increase.  Components of different materials (like the metal

RAM prongs and the plastic material to which it is attached) expand and

contract at different rates.  Listen closely to your PC creak when turned

off after hours of use.  This expansion and contraction can, over time,

stress the system RAM as well as the connections between the RAM and the

other components.

Though we can take precautions (see How to Avoid Memory Errors on page 18),

there are no guaranteed methods for eliminating all chances of memory error.

What we can do, though, is minimize the chances of losing work because of

defective memory by using RAMexam to test system memory regularly.

______________________________________________________________________________

A Word About Windows & Memory Testing:

When Microsoft Windows starts up, one of the first things it does is to

grab almost every available byte of system memory.  Conventional memory,

high DOS memory (UMBs) and extended memory are all allocated.  This means

two important things for RAMexam users:

1)     RAMexam can only test available memory -- which doesn't exist once

Windows has allocated all of it.  RAMexam does not run in a DOS

session under Windows because there is effectively no memory

available for testing.

2)     Windows users should be especially zealous about doing periodic

RAM testing because Windows is one of the few applications to use

the multiple megabytes of RAM on most systems.  Frequently, a systems

run without incident until Windows loads -- at which point Windows

accesses a byte of bad RAM that had never been used before. RAMexam

testing is an effective pre-treatment because it warns you about

defective RAM before Windows (and your work) trip over it.

How RAMexam Tests:

Any time a bit fails to change its value correctly or erroneously changes its

value then a RAM error occurs.   Most (though not all) RAM that fails once

will do so again.  RAMexam uses exhaustive testing to invoke, trap and report

RAM errors before they affect your application software.

Unlike other memory testers, RAMexam uses an underlying consistent fault

model to detect RAM errors efficiently even in complex, multiple fault

situations.  Using sophisticated strategies of writing and reading bit

patterns in memory -- where each strategy targets a specific type of

failure -- RAMexam provides faster and more comprehensive and effective

testing.

Here is a description of the categories of failures for which RAMexam tests:

Stuck-at:  One or more bits retain the value that exists in the memory when

the system boots, regardless of attempts to write new values into the memory.

Transition:  Once changed, the value in the defective bit becomes stuck-at.

Unlinked Inversion:  A bit changes erroneously as the result of a change to

a bit elsewhere in memory.

Linked Inversion:  A bit changes erroneously as the result of changes to two

or more other bits elsewhere in memory.

Unlinked Idempotent:  A bit becomes stuck-at as the result of a change to a

bit elsewhere in memory.

Linked Idempotent:  A bit becomes stuck-at as the result of changes to two

or more other bits elsewhere in memory.

RAMexam detects singular and multiple (coupled) occurrences of the above

errors.

From the basic display, RAMexam provides two testing options: Quick and

Full.  Selecting Quick is the same as choosing the transition fault test

from the advanced display.  Selecting Full is the same as choosing the two

idempotent tests from the advanced display.  All tests can be accessed by

name from the advanced display.  Press ALT-Tab to toggle between basic and

advanced displays.

Ways to Use RAMexam:

The Memory Tester may be used several ways:

You can run RAMexam every time you boot your system by including

RAMexam in your AUTOEXEC.BAT. When running RAMexam this way, you

should use the command line switch for a "Quick" test. Use RSetup

to configure your system this way automatically. Running RAMexam at

system start-up adds between a few seconds and a few minutes to the

time it takes to boot your system, depending on how much memory you

have and the speed of your system.

You can run RAMexam as often as you like from the DOS prompt by

typing RAMexam and pressing enter. Once RAMexam is running, select

the test(s) you wish to run.

RAMexam can test available memory only (you wouldn't want RAMexam

to write over memory in use by another program).  For extensive

testing, you can use RAMexam on a "clean" system without resident

software such as disk caches, RAM disks and memory managers active.

Running RAMexam with no other software active is the most effective

way to test your system. The program MAKEBOOT.EXE, which is included

with RAMexam,  creates a bootable floppy for this purpose. If you

are using MS-DOS 6 MultiConfig or similar utility then you may wish

to create a clean boot configuration which executes the following line:

d:\path\RAMexam FULL=1

where d: is the drive on which RAMexam is installed and path is the

directory.  This provides an easy way to test your system memory

fully.

To start RAMexam, you must be in the RAMexam directory or it must be included

in your DOS search path.

A Note for OS/2 Users...

RAMexam can test the memory on systems that run OS/2.  In fact, RAMexam tests

RAM on any Intel 386, 486 or Pentium-compatible microprocessor or above

regardless of the operating system.  First, you need access to a system

running DOS so you can install RAMexam on that system and use MAKEBOOT.EXE

to create a bootable test diskette.  (MAKEBOOT does not run in an OS/2 DOS

session because OS/2 does not support the necessary DOS SYS.COM command.)

Once you have your bootable diskette use it as described above.  Remember

to observe copyright restrictions and use DOS and RAMexam only on the

systems for which they are licensed.

Navigating RAMexam:

There are two RAMexam displays available, Basic and Advanced. When RAMexam

starts, it displays the Basic screen. Three windows make up the Basic

display. The window in the upper left corner of the screen displays the

program name and copyright. The window in the upper right portion of the

screen displays two buttons labeled Quick and Full. Select the desired test

using the right and left cursor (arrow) keys. Press enter to start the test.

The third window, centered in the bottom half of the screen, displays test

progress.

RAMexam's Advanced screen is for users who want to see the nuts and bolts of

memory testing. Users may switch between the Advanced and Basic screens using

Alt-Tab (even while testing).

There are four windows on the Advanced display, one of which (in the top,

left corner) is the same as in the Basic display.

The Test Selection window, in the top right corner of the screen, displays

the names of the six different memory tests. Each test name is followed by a

field in which you may enter the number of times RAMexam should run the test.

Enter "C" to instruct RAMexam to run the test continually until interrupted

by the user.

The Memory to Test window, in the lower left corner of the screen, displays

the memory regions which RAMexam has identified. Regions displayed in gray

are in use by other software (such as a disk cache) or are not physically

present and cannot be tested.

The Test Status window appears in the lower right corner and includes the

estimated time to test completion. While running, RAMexam reads and writes

(abbreviated in the display as "Rd" and "Wr") various test patterns into

memory. This activity appears in the Status window.

Keyboard Overview

The following keys are available in both Basic and Advanced screens:

F1             displays help

F5             toggles between color and monochrome displays

ESC            if a test is in progress, halts test, otherwise exits RAMexam

The following keys are available while viewing the Basic display:

ALT-Tab        switches to the Advanced display (use F9 on 83-key keyboards)

Cursor alternates between highlighting Quick and Full buttons

Enter          starts testing

The following keys are available while viewing the Advanced display:

ALT-Tab        switches to the Basic display (use F9 on 83-key keyboards)

Cursor selects test

Ctrl-Enter     runs the highlighted test once

H              displays values in hexadecimal format

D              displays values in decimal format

Tab            moves cursor between Test Selection window and Memory to Test

window if there are more memory entries than fit in a single

screen. When in the Memory to Test window the cursor keys

scroll the list up and down.

The following keys are available while viewing the Help display:

F2             prints help topic(s)

F3             displays previous help topic

F4             displays next help topic

ESC            returns to help menu

A Memory Error! Now What?:

If you are fortunate, RAMexam will never find a memory error on your system.

When RAMexam detects defective or damaged RAM then the test halts and a red

warning box appears with all known information about the error.

Unfortunately, RAMexam cannot tell you which chip is the culprit.  Software

accesses memory through special hardware that is responsible for addressing

the individual chips.  There is information within this hardware that maps

the memory address used by software to a physical bit within a chip.  This

information is not accessible to software and varies widely from system to

system.  Should RAMexam detect defective RAM, here are the recommended steps

to take:

See if the error is reproducible.  If a memory error only happens once and

cannot be reproduced it may not be cause for alarm. Occasionally, alpha

particle radiation -- which may not recur -- can cause memory failure.

If the memory error recurs, take your system to a qualified repair facility.

Virtually every system manufacturer uses a unique memory architecture so

that it is impossible for RAMexam to identify specific memory chips to

replace.  DO NOT ATTEMPT TO REPLACE YOUR RAM UNLESS YOU ARE QUALIFIED.

(If you are not sure if you are qualified then you probably are not.)

All error information is saved into the RAMexam log file in the RAMexam

directory.

Diagnosis of memory errors, beyond that offered by RAMexam, requires

hands-on interaction with the problem system.  It also frequently requires

special equipment.  For these reasons, Technical Support cannot assist you

in locating faulty memory chips.  You should speak with a technician only

if you believe that RAMexam itself is not functioning correctly.

How to Avoid Memory Errors:

As with most of life, a byte of prevention is worth a megabyte of cure.

Maintaining a proper environment for your PC can significantly reduce the

chance of memory failure. Here are some tips:

Work in a dust free environment.

Keep pets away from computer work areas.

Do not smoke in the same room as your PC.

Plug your PC into its own electrical circuit.

Use a surge protector or, even better, an uninterruptable power

supply (UPS).

If you use a modem or fax board, make sure that you use a telephone

line surge protector.

Never touch the inside of your PC without first grounding yourself.

Even with these precautions, there is no guarantee that RAM will not fail.

RAM testing, though, significantly reduces the risk of losing work to

defective RAM.

Command Line Options

?                      display help

ADVANCED               start execution in advanced mode

ALL=n                  run all tests n times

ASK[=timeout,response] prompt for action to take, with a default

response to be used after time-out seconds:

Y (Yes)                run RAMexam

N (No)                 skip test and wait until next scheduled occurrence

P (Postpone)           skip test; ask again later

BW or /B               force black & white display

COLOR or /C            force color display

DAILY                  run RAMexam only if it has not been run since midnight

FULL=n                 run Full tests n times

HEX                    display data initially in hexadecimal mode

INTERVAL=n             run RAMexam only if at least n days have expired

since the last time RAMexam was run

K                      avoid spurious keyboard lockups

L                      use larger of BIOS and CMOS sizes if they differ

LNKIDM=n               run Linked Idempotent Coupling Fault Test n times

LNKINV=n               run Linked Inversion Coupling Fault Test n times

LOG[=filename][,o]     append a log entry to RAMEXAM.LOG or other

filename and use o to overwrite any existing file

of the same name

MONTHLY[=date]         run RAMexam only if it has not run in the last month

--  if optional date is specified, RAMexam runs on or

after that day of the month (1-31)

NODPMI                 do not use DOS Protected Mode Interface

NOEXIT                 do not exit after running tests automatically

NOPARITY               avoid spurious parity errors on certain systems

NOXRAM                 do not recover extra memory

QUICK=n                run Quick tests n times

S                      use smaller of BIOS and CMOS sizes if they differ

SAF=n                  run Stuck-At Fault Test n times

TF=n                   run Transition Fault Test n times

TOP384                 recover 384KB COMPAQ-like memory

UNLIDM=n               run Unlinked Idempotent Coupling Fault Test n times

UNLINV=n               run Unlinked Inversion Coupling Fault Test n times

WEEKLY[=day]           run RAMexam only if it has not been run in the last

week --  if optional day is specified, RAMexam runs

on or after that day of the week (Sunday=1, Monday=2,

etc.)