Diamond Computer Systems VIPER SE Power Management vers. 1.0b

06/10/94

Diamond Display Power Management supports three power management states

defined by VESA  (Video Electronics Standards Association, version 1.0)

in both DOS and Windows environments:

STANDBY

SUSPEND

OFF

Power management states in DOS environment can be individually set from 1

to 99 minutes through options of VPROPM (Viper SE power management TSR

intallation program) as following:

Usage:  VPROPM [option]

Options:

h - Displays this help message

u - Unloads utility from memory

bxx - sets Standby timer in minutes (1 to 99) in DOS environment

sxx - sets Suspend timer in minutes (1 to 99) in DOS environment

oxx - sets Off timer in minutes (1 to 99) in DOS environment

For example:

VPROPM b7 s15 o27 <enter>

Viper SE will be in STANDBY state after 7 minutes in DOS

environment.  It will be in SUSPEND state 8 minutes after STANDBY

state in DOS environment.  Then in OFF state 12 minutes after SUSPEND

state in DOS environment.

After Viper SE power management TSR is loaded, you can use VPROPM

to show current power management timer settings in DOS environment.

For example: C:\VPROPM <enter>

While the system switches into Windows enviroment, the power management states

can be individually set from 1 to 75 minutes through DPMS (Diamond Display

Power Management) windows application.

For example:

STANDBY=10 minutes

SUSPEND=25 minutes

OFF=75 minutes

Viper SE will be in STANDBY state after 10 minutes.  It will

be in SUSPEND state 15 minutes after STANDBY state.  Then in OFF state

50 minutes after SUSPEND state.

Display adapter is set to ON state with input from keyboard or mouse (if mouse

device driver is loaded) in both DOS and Windows environments.

DISKETTE CONTAINS:

VPROPM.COM     Viper SE Power Management BIOS TSR 1.0b

INSTALLATION:

1.  Copy VPROPM.COM to root directory C:\.

2.  Modify AUTOEXEC.BAT.  Add C:\VPROPM [option] to set Standby,

Suspend and Off timers in DOS environment.

Viper SE Power Management BIOS TSR will be loaded with every

system boot, you can change those timer settings later by unloading

it and loading again with new settings.

3.  DPMS will be automatically installed with the installation of Viper

SE Windows 3.1 device driver

DPMS CONFIGURATION (setting power management timer in Windows environment):

1.  Double click on DPMS icon to maximize.

2.  All power management states are disabled.

STANDBY=Never

SUSPEND=Never

OFF=Never

3.  Move scroll bar(s) to left to enable a power management state(s).

A number will appear on right of scroll bar to indicate minute(s).

4.  Click on Save! menu to save power management configuration.  It

creates or updates DPMS.INI in C:\WINDOWS.

5.  Select File

6.  Select Properties

7.  Select "Run Minimized"

8.  Click OK

9.  Minimize to start Diamond Display Power Management.

NOTES:

In both DOS and Windows environments:

STANDBY minutes must be <= SUSPEND minutes

SUSPEND minutes must be <= OFF miniutes

LIMITATIONS:

In protected mode application (i.e. AutoCad, Links386 game), DPMS is

not properly supported at this release.

PROGRAMMER'S REFERENCES:

This Viper SE DMPS BIOS TSR supports following VBE/PM services

(VESA Display BIOS Extensions / Power Management, VBE Sub-function 10h)

as defined in VESA VBE/PM version 1.0 (Feb. 4, 1994):

a. Report VBE/PM Capabilities:

Input:   AH = 4fh       VESA Extension

AL = 10h       VBE/PM Services

BL = 00h       Report VBE/PM Capabilities

ES:DI          Null pointer 0000:0000h (reserved for future

use)

Output:  AX =           Status (*)

BH =           Power saving state signals supported

1 = supported, 0 = not supported

bit 0         STANDBY

bit 1         SUSPEND

bit 2         OFF

bit 3         REDUCED ON (intended for flat panel)

bits 7-4      reserved for future power control

BL =           VBE/PM Version number (00010000b - 1.00)

bits 3-0      Minor Version number

bits 7-4      Major Version number

ES:DI          Unchanged

b. Set Display Power State:

Input:   AH = 4fh       VESA Extension

AL = 10h       VBE/PM Services

BL = 01h       Set Display Power State

BH =           Requested Power State

00h       ON

01h       STANDBY

02h       SUSPEND

04h       OFF

08h       REDUCED ON (intended for flat panel)

Output:  AX =           Status (*)

BH =           Unchanged

c. Get Display Power State:

Input:   AH = 4fh       VESA Extension

AL = 10h       VBE/PM Services

BL = 02h       Get Display Power State

Output:  AX =           Status (*)

BH =           Power state currently requested by display

00h       ON

01h       STANDBY

02h       SUSPEND

04h       OFF

08h       REDUCED ON (intended for flat panel)

* Status information:

AL == 4Fh:     Function is supported

AL != 4Fh:     Function is not supported

AH == 00h:     Function call successful

AH != 00h:     Function call failed