cFos ISDN FOSSIL appendage. The functions are to be called  with

AH=0x90 and the function number in AL. See the  FOSSIL  doc  for

more information on FOSSIL programming and user applications.

========================================================================

ISDN appendage, appendage code=0x90:

------- 00 - check for ISDN appendage ----------------------------------

INPUT:

AX = 0x9000

OUTPUT:

AX = 0x1969

BX = maximum function number supported

CX = 'cF'

DX = 'os'

NOTE:

if AX != 0x1969, this FOSSIL doesn't feature any

ISDN appendage.

------- 01 - reset ISDN driver/hardware --------------------------------

This function terminates all connections on all ports and resets

the ISDN driver/hardware. If 'cFos' was loaded  with  -jr,  this

function does nothing.

INPUT:

AX = 0x9001

OUTPUT:

AX = 0x0000 on success

------- 02 - get ISDN connection info ----------------------------------

INPUT:

AX = 0x9002

DX = port number

CX = number of bytes to get from appendage

ES:BX = ptr to ISDN info block in application space

OUTPUT:

AX = 0000, success

CX = number of bytes transfered into application buffer

ISDN info block:

WORD    offset   into   additional    infomation

elements in  bytes,  starting  from  the

beginning of the block

FAR PTR incoming call: incoming caller-id

outgoing call: dialed number

far ptr to ASCIZ string, segment first

BYTE    number of b-channels currently in use by

this port

DWORD   connection characteristics

bit 0 = 1   b2-protocol supports control

flow

bit 1 = 1   b2-protocol  supports  error

correction

bit 2 = 0   incoming call

1   outgoing call

all other bits are reserved  for  future

use

WORD    current CPS rate

the  ISDN  info  block  can  contain  additional

infomation  elements  reflecting  national  ISDN

parameters, e.g. LLC/MSN/Subaddress/EAZ/...

Currently defined:

BYTE    type 1 = 1.TR.6

BYTE    requested EAZ

BYTE    Service Indicator

BYTE    Additional Service Indicator

------- 03 - get charging info -----------------------------------------

INPUT:

AX = 0x9003

DX = port

BX = no of b-channel of this port

(0=main channel, 1=first aux channel...)

ffff = sum of all charges on all channels,

independant of carrier status

OUTPUT:

AX = 0000, success

0001, no valid b-channel

BX = num. of charges used for current connection

CX = num. of seconds remaining in current charging  unit

(valid after  2nd  charging  unit,  otherwise  link

duration in seconds)

Note: If no connection is active this  function  can  be

called with BX = 0 (as well as BX = 0xffff).  Then

it returns CX = 0  and  BX  the  total  number  of

charging  unit  of  the  last  call  (main  +  aux

channels)

------- 04 - add/remove b-channel --------------------------------------

INPUT:

AX = 0x9004

DX = port

BX = 1 -> add, 2 -> remove

OUTPUT:

AX = 0000, success

0001, no active connection

0002, connect/disconnect already in progress

After a  call  to  this  function,  'cFos'  initiates  a

connect/disconnect sequence. During  this  sequence  all

calls  to  this  function  result  in  error  0002.  The

completion can be determined by monitoring the number  B

channels in use.

------- 05 - get ISDN time/date ----------------------------------------

INPUT:

AX = 0x9005

BX = 0000

OUTPUT:

AX = 0000, success

CX:BX = difference between ISDN time and local  time  in

seconds. CX = most significant word.

======= End of Text ====================================================