+----------------------------------------------+

|   MR BIOS (r)  Production BIOS Version 1.65  |

+----------------------------------------------+

|   Copyright (c) 1994,  Microid Research Inc  |

|    2336-D Walsh Ave, Santa Clara, CA 95051   |

|   Tel (408) 727-6991  /  Fax (408) 727-6996  |

+----------------------------------------------+

------------------------------------------------------------------------------

-----------------------------------

CONTAQ  82C596  386/486 WriteBack

-----------------------------------

NOTICE!  Contaq's original definition of REG_17 required b5:0 = 27H.

Versions V080B41x immediately below initialize per this rule.

Some M/Boards do NOT work with this setting.  Some do.

Contaq's later definition of REG_17 requires b5:0 = 22H.

Versions V080B42x further below initialize per this new rule.

All M/Boards we tested DO work with this setting.

>>--->   As a rule:  TRY USING VERSIONS '42x FIRST  !!!

If neither setting works right, and you've figured out some

value for these "magic bits" that works, call us and we'll

cook a bios to your liking!

Filename   Port      Description  '596 Single-Chip WB, REG_17 = 27

--------   -------   ---------------------------------------------------------

V080B410   CNTQ410   386/486/Cx	  REV-A

V080B411   CNTQ411   386/486/Cx   REV-A  Layout: Cyrix #A20M #FLUSH

V080B412   CNTQ412   386/486/Cx   REV-A  Layout: Cyrix #A20M #FLUSH, Fast-A20

V080B417   CNTQ417   386/486/Cx   REV-0

Filename   Port      Description  '596 Single-Chip WB, REG_17 = 22

--------   -------   ---------------------------------------------------------

V080B420   CNTQ420   386/486/Cx	  REV-A

V080B421   CNTQ421   386/486/Cx   REV-A  Layout: Cyrix #A20M #FLUSH

V080B422   CNTQ422   386/486/Cx   REV-A  Layout: Cyrix #A20M #FLUSH, Fast-A20

V080B427   CNTQ427   386/486/Cx   REV-0

------------------------------------------------------------------------------

-----------------------------------

CONTAQ  82C591/2  386/486 WriteBack

-----------------------------------

Filename   Port      Description  '591 WriteBack Chipset

--------   -------   ---------------------------------------------------------

V045B400   CNTQ400   386/486/Cx WriteBack

V045B404   CNTQ404   (TL-)

V045B405   CNTQ405   (TL+)

V045B406   CNTQ406   (CL-)

V045B407   CNTQ407   (CL+)

-----------------------------------------------------------------

8042 Turbo/Turbo-LED control:  (T-) (T+) (TL-) (TL+) (CL-) (CL+)

-----------------------------------------------------------------

Hot-Key TURBO and/or LED control via keystrokes  <CTRL> <ALT> <-/+>  will

simultaneously activate all commonly used 8042 pins: 23,24, 27,28,29,30.

The system board layout must connect one of these pins to an existing

Turbo/LED circuit.  Consideration should be given to the remaining pins

which are also driven.  MR BIOS (r) 8042 or AMI KF keyboard controller

may be used for this purpose.  Award, Phoenix, and Quadtel 8042's cannot

be used with any MR BIOS (r) version designated as having this special

8042 functionality.

Active

Legend  Function  State   Description

------  --------  -----   --------------------------------------------

(T-)    Turbo      Low    8042 turbo/LED circuit ONLY (no chipset register

(T+)    Turbo      High   turbo).  Also, Setup Utility SPEED screen appears.

(TL-)   Turbo-LED  Low    Both chipset turbo and 8042 pins are activated.

(TL+)   Turbo-LED  High

(CL-)   Cache-LED  Low    Used mainly with 486, Cache state indicates "turbo".

(CL+)   Cache-LED  High