+----------------------------------------------+

|   MR BIOS (r)  Production BIOS Version 1.65  |

+----------------------------------------------+

|   Copyright (c) 1994,  Microid Research Inc  |

|    2336-D Walsh Ave, Santa Clara, CA 95051   |

|   Tel (408) 727-6991  /  Fax (408) 727-6996  |

+----------------------------------------------+

-------------------

FOREX Computer Corp

-------------------

Filename   Port      Description  46C521  WriteBack  (486DLC retrofit)

--------   -------   ---------------------------------------------------------

V082B430   FORX420   386/486/Cyrix '521

V082B431   FORX421   x082B430  (T-)

V082B432   FORX422   x082b430  (T+)

V082B433   FORX423   x082b430  (CL-)        <-- demo board

V082B434   FORX424   x082b430  (CL+)

Note - Vers 430-434  fully support Cyrix 486DLC in 386 socket

without any special hardware considerations for it.  (To

maintain cache coherence, Cyrix CCR0 "BARB" is set, &tc).

Filename   Port      Description  46C521  WriteBack (486DLC design-in)

--------   -------   ---------------------------------------------------------

V082B435   FORX425   386/486/Cyrix '521

V082B436   FORX426   x082B435  (T-)

V082B437   FORX427   x082b435  (T+)

V082B438   FORX428   x082b435  (CL-)

V082B439   FORX429   x082b435  (CL+)

Note - Vers 435-439  require specific design-in considerations for

Cyrix 486DLC.  These pins must be connected: #FLUSH, #KEN, #A20M.

These versions give best Cyrix performance (compared with "retrofit"

versions), but will MALFUNCTION unless the board is designed for it.

Filename   Port      Description  46C421A+46C422  WriteBack  (486DLC retrofit)

--------   -------   ---------------------------------------------------------

V067B420   FORX420   386/486/Cyrix '421-A   (NOTE! Rev-0 is NOT supported)

V067B421   FORX421   x067B420  (T-)

V067B422   FORX422   x067b420  (T+)

V067B423   FORX423   x067b420  (CL-)

V067B424   FORX424   x067b420  (CL+)

Note - Vers 420-424  fully support Cyrix 486DLC in 386 socket

without any special hardware considerations for it.  (To

maintain cache coherence, Cyrix CCR0 "BARB" is set, &tc).

Filename   Port      Description  46C421A+46C422  WriteBack (486DLC design-in)

--------   -------   ---------------------------------------------------------

V067B425   FORX425   386/486/Cyrix '421-A   (NOTE! Rev-0 is NOT supported)

V067B426   FORX426   x067B425  (T-)

V067B427   FORX427   x067b425  (T+)

V067B428   FORX428   x067b425  (CL-)

V067B429   FORX429   x067b425  (CL+)

Note - Vers 425-429  require specific design-in considerations for

Cyrix 486DLC.  These pins must be connected: #FLUSH, #KEN, #A20M.

These versions give best Cyrix performance (compared with "retrofit"

versions), but will MALFUNCTION unless the board is designed for it.

Filename   Port      Description   36C311 Single Chip 386SX with Cache

--------   -------   ---------------------------------------------------------

V052B320   FORX320   386SX/486SLC with cache

V052B323   FORX323   x052B320  (T+)       <-- eval board

Filename   Port      Description   46C411 + 46C402  WriteThru

--------   -------   ---------------------------------------------------------

V050B410   FORX410   386/486/Cyrix - 2nd generation chipset

V050B413   FORX413   x050B410  (T+)

V050B418   FORX418   x050B410  (CL-)

V050B419   FORX419   x050B410  (CL+)

Filename   Port      Description   36C300 + 36C200/46C402  WriteThru

--------   -------   ---------------------------------------------------------

V049B300   FORX300   386 chipset - 1st generation

V049B303   FORX303   x049B300  (T+)

-----------------------------------------------------------------

8042 Turbo/Turbo-LED control:  (T-) (T+) (TL-) (TL+) (CL-) (CL+)

-----------------------------------------------------------------

Hot-Key TURBO and/or LED control via keystrokes  <CTRL> <ALT> <-/+>  will

simultaneously activate all commonly used 8042 pins: 23,24, 27,28,29,30.

The system board layout must connect one of these pins to an existing

Turbo/LED circuit.  Consideration should be given to the remaining pins

which are also driven.  MR BIOS (r) 8042 or AMI KF keyboard controller

may be used for this purpose.  Award, Phoenix, and Quadtel 8042's cannot

be used with any MR BIOS (r) version designated as having this special

8042 functionality.

Active

Legend  Function  State   Description

------  --------  -----   --------------------------------------------

(T-)    Turbo      Low    8042 turbo/LED circuit ONLY (no chipset register

(T+)    Turbo      High   turbo).  Also, Setup Utility SPEED screen appears.

(TL-)   Turbo-LED  Low    Both chipset turbo and 8042 pins are activated.

(TL+)   Turbo-LED  High

(CL-)   Cache-LED  Low    Used mainly with 486, Cache state indicates "turbo".

(CL+)   Cache-LED  High