+----------------------------------------------+

|   MR BIOS (r)  Production BIOS Version 1.65  |

+----------------------------------------------+

|   Copyright (c) 1994,  Microid Research Inc  |

|    2336-D Walsh Ave, Santa Clara, CA 95051   |

|   Tel (408) 727-6991  /  Fax (408) 727-6996  |

+----------------------------------------------+

-----------------------

VLSI TOPCAT / Intel-340

-----------------------

Filename   Port      Description

--------   -------   ---------------------------------------------------------

V019B301   VLSI301   386 SX/DX, non-cache

V019B302   VLSI302   386 SX/DX, non-cache, with 82C106 combo chip

V019B312   VLSI312   386 SX/DX, 385 cache, with 82C106 combo chip

V019B401   VLSI401   486

V019B402   VLSI402   486, with 82C106 combo chip

V019B404   VLSI404   486, with 82C106, on-board COM2 inhibited for SCO

--------------------------

ZYMOS POACH-SX / Intel-335

--------------------------

Filename   Port      Description

--------   -------   ---------------------------------------------------------

V004B300   ZYMS300   386SX

V004B301   ZYMS301   Turbo: 8042 Pin 23 (only)  Active Low

V004B302   ZYMS302   (T-)

V004B303   ZYMS303   (T+)

------------------------------------------------------------------------------

-----------------------------------------------------------------

8042 Turbo/Turbo-LED control:  (T-) (T+) (TL-) (TL+) (CL-) (CL+)

-----------------------------------------------------------------

Hot-Key TURBO and/or LED control via keystrokes  <CTRL> <ALT> <-/+>  will

simultaneously activate all commonly used 8042 pins: 23,24, 27,28,29,30.

The system board layout must connect one of these pins to an existing

Turbo/LED circuit.  Consideration should be given to the remaining pins

which are also driven.  MR BIOS (r) 8042 or AMI KF keyboard controller

may be used for this purpose.  Award, Phoenix, and Quadtel 8042's cannot

be used with any MR BIOS (r) version designated as having this special

8042 functionality.

Active

Legend  Function  State   Description

------  --------  -----   --------------------------------------------

(T-)    Turbo      Low    8042 turbo/LED circuit ONLY (no chipset register

(T+)    Turbo      High   turbo).  Also, Setup Utility SPEED screen appears.

(TL-)   Turbo-LED  Low    Both chipset turbo and 8042 pins are activated.

(TL+)   Turbo-LED  High

(CL-)   Cache-LED  Low    Used mainly with 486, Cache state indicates "turbo".

(CL+)   Cache-LED  High