+----------------------------------------------+

|   MR BIOS (r)  Production BIOS Version 1.65  |

+----------------------------------------------+

|   Copyright (c) 1994,  Microid Research Inc  |

|    2336-D Walsh Ave, Santa Clara, CA 95051   |

|   Tel (408) 727-6991  /  Fax (408) 727-6996  |

+----------------------------------------------+

---------------------------

SARC 386SX  RC2016A (A4/A5)

---------------------------

Filename   Port      Description  SARC 386SX RC2016A  Rev A4/A5

--------   -------   ---------------------------------------------------------

V064B312   SARC312   386SX standard bios

V064B316   SARC316   built-in EMS

V064B31A   SARC31A   (*) for Cyrix layout

V064B31E   SARC31E   (*) for Cyrix layout, built-in EMS

(*) Cyrix 486SLC layout with #FLUSH,#A20M connected.

Note - BIOS "Fast-A20" feature is disabled.

Note - The Rev A4/A5 bios disables some of the shadow-RAM

features and memory maps available in the earlier

Rev A3 chipset/bios (see below).  With this exception

of reduced capabilities, Rev A4/A5 bios will work OK

with older Rev A3 chipset.

------------------------

SARC 386SX  RC2016A (A3)

------------------------

Filename   Port      Description  SARC 386SX RC2016A  Rev A3

--------   -------   ---------------------------------------------------------

V064B302   SARC302   386SX standard bios

V064B306   SARC306   built-in EMS

V064B30A   SARC30A   (*) for Cyrix layout

V064B30E   SARC30E   (*) for Cyrix layout, built-in EMS

(*) Cyrix 486SLC layout with #FLUSH,#A20M connected.

Note - BIOS "Fast-A20" feature is disabled.

------------------------------------------------------------------------------

-----------------------------------------------------------------

8042 Turbo/Turbo-LED control:  (T-) (T+) (TL-) (TL+) (CL-) (CL+)

-----------------------------------------------------------------

Hot-Key TURBO and/or LED control via keystrokes  <CTRL> <ALT> <-/+>  will

simultaneously activate all commonly used 8042 pins: 23,24, 27,28,29,30.

The system board layout must connect one of these pins to an existing

Turbo/LED circuit.  Consideration should be given to the remaining pins

which are also driven.  MR BIOS (r) 8042 or AMI KF keyboard controller

may be used for this purpose.  Award, Phoenix, and Quadtel 8042's cannot

be used with any MR BIOS (r) version designated as having this special

8042 functionality.

Active

Legend  Function  State   Description

------  --------  -----   --------------------------------------------

(T-)    Turbo      Low    8042 turbo/LED circuit ONLY (no chipset register

(T+)    Turbo      High   turbo).  Also, Setup Utility SPEED screen appears.

(TL-)   Turbo-LED  Low    Both chipset turbo and 8042 pins are activated.

(TL+)   Turbo-LED  High

(CL-)   Cache-LED  Low    Used mainly with 486, Cache state indicates "turbo".

(CL+)   Cache-LED  High