+----------------------------------------------+

|   MR BIOS (r)  Production BIOS Version 1.65  |

+----------------------------------------------+

|   Copyright (c) 1994,  Microid Research Inc  |

|    2336-D Walsh Ave, Santa Clara, CA 95051   |

|   Tel (408) 727-6991  /  Fax (408) 727-6996  |

+----------------------------------------------+

-----------------------------

Syslogic - Solutions - PerCom

-----------------------------

Filename   Port      Description

--------   -------   ---------------------------------------------------------

V028B301   SLGC301   386, non-cache

V028B304   SLGC304   (TL-)

V028B305   SLGC305   (TL+)

V028B302   SLGC302   386, with cache

V028B306   SLGC306   (TL-)

V028B307   SLGC307   (TL+)

V028B401   SLGC401   486, no external cache

V028B404   SLGC404   (TL-)

V028B405   SLGC405   (TL+)

-----------------------------------------------------------------

8042 Turbo/Turbo-LED control:  (T-) (T+) (TL-) (TL+) (CL-) (CL+)

-----------------------------------------------------------------

Hot-Key TURBO and/or LED control via keystrokes  <CTRL> <ALT> <-/+>  will

simultaneously activate all commonly used 8042 pins: 23,24, 27,28,29,30.

The system board layout must connect one of these pins to an existing

Turbo/LED circuit.  Consideration should be given to the remaining pins

which are also driven.  MR BIOS (r) 8042 or AMI KF keyboard controller

may be used for this purpose.  Award, Phoenix, and Quadtel 8042's cannot

be used with any MR BIOS (r) version designated as having this special

8042 functionality.

Active

Legend  Function  State   Description

------  --------  -----   --------------------------------------------

(T-)    Turbo      Low    8042 turbo/LED circuit ONLY (no chipset register

(T+)    Turbo      High   turbo).  Also, Setup Utility SPEED screen appears.

(TL-)   Turbo-LED  Low    Both chipset turbo and 8042 pins are activated.

(TL+)   Turbo-LED  High

(CL-)   Cache-LED  Low    Used mainly with 486, Cache state indicates "turbo".

(CL+)   Cache-LED  High