The following video drivers are available for EGA/VGA cards:

----------------------------------------------------------------------------

Drivername Resolution  Mode Note Graphics Adapter

----------------------------------------------------------------------------

EGA        640 x 350   10h       standard EGA

VGA        640 x 480   12h       standard VGA

EWOND410   752 x 410   52h       ATI EGA-Wonder

PEGA480    640 x 480   50h       Paradise AutoSwitch EGA 480

EVA480     640 x 480   25h       Tseng Labs. Eva/480

EMAX480    640 x 480   73h       FullHouse EGAMAX

LEGA480    640 x 480   33h       ???

EWOND560   800 x 560   53h       ATI EGA-Wonder

EMAX600    800 x 600   79h       FullHouse EGAMAX

Genoa SuperEGA 800 x 600 (with BIOS >= 3.0)

TVGA600    800 x 600   5Bh       TRIDENT TVGA

MORSE KP 1000 EVGA

Informtech Industrial Ltd. IT-VGA

TVM600     800 x 600   58h       TVM VGA 860

EIZO MD-B07

Paradise VGA

EIZO600    800 x 600   29h       EIZO MD-B10

Puretek VGA-Ultra

Tseng Labs. Eva/1024

Interquadram VGA QC GTi 16 b 800 x 600

Optima 1024

Genoa SuperVGA 16-Bit

V7VR600    800 x 600   62h  (1)  Video7 V-RAM VGA

MAXE600    800 x 600   10h  (2)  MaxLogic MaxEGA

MAXV600    800 x 600   10h  (2)  MaxLogic MaxVGA MX-675

CIRR600    800 x 600   64h  (2)  Cirrus Logic 800x600

MORSE600   800 x 600   64h  (5)  MORSE KP 800 VGA

VWOND600   800 x 600   54h       ATI VGA-Wonder

VWIZ600    800 x 600   6Ah       VGA Wizard/Deluxe, AHEAD Systems Inc.

MINV600    800 x 600   52h       MINTEK VGA-210, Lyicheng Enterpr. Co. Ltd.

Orchid's Designer VGA

EIZO768    1024x 768   37h  (3)  EIZO MD-B10 (Chip: ET3000)

Puretek VGA-Ultra

Tseng Labs. Eva/1024

Orchid's Designer VGA

V7VR768    1024x 768   65h (1,4) Video7 V-RAM VGA

TVGA768    1024x 768   5Fh  (3)  TRIDENT TVGA

MORSE KP 1000 EVGA

Informtech Industrial Ltd. IT-VGA

TVGA768B   1024x 768   5Fh  (4)  TRIDENT TVGA (Chip: TVGA8900)

VWIZ768    1024x 768   74h       AHEAD Systems Inc., VGA Wizard/Deluxe

GVGA768    1024x 768   5Fh  (3)  Genoa SuperVGA 6400 16-Bit (BIOS 2.30)

OMEGA768   1024x 768   37h  (4)  OMEGA Super VGA 16-Bit (Chip: ET3000)

BOCA768    1024x 768   3Fh  (4)  BOCA Super VGA 16-Bit (Chip: ET4000)

VWOND768   1024x 768   55h  (4)  ATI VGA-Wonder (Chip: ATI28800)

VESA16A     800x 600  102h       VESA compatible

VESA16B    1024x 768  104h       VESA compatible

----------------------------------------------------------------------------

Notes:

(1) video mode set with the following commands:

MOV    AX,06F05h

MOV    BL,Mode

INT    10h

(2) video mode set with special patch code

(3) banks A000 and B000 are used, therefore EMS might cause problems if

the EMS-driver uses these addresses, too

(4) only segment A000 (bank switching) is used

(5) use NEC_MS.COM or NECPARMS.COM from the Morse-Utilities-Disk

----------------------------------------------------------------------------

If your video adapter doesn't show up in the table you can try to patch an

existing one with the desired resolution. Using a binary file editor take

the driver file name.BGI, locate the byte sequence

08h,08h,90h,90h,Mode

and replace the mode byte with the one needed by your graphics adapter.

See the file VQUERY.DOC for information on how to determine the mode byte

for your graphics adapter.

============================================================================

The following video drivers are available for non-EGA/VGA cards:

----------------------------------------------------------------------------

Drivername Resolution            Graphics Adapter

----------------------------------------------------------------------------

IBM8514    1024x 786             IBM 8514

ELSA       set by XHR driver     all ELSA XHR adapters

TIGA       set by TIGA driver    all TIGA adapters (except Hercules Graphics

Station!)

HGSCA      800 x 600             Hercules Graphics Station (TIGA mode)

----------------------------------------------------------------------------

ELSA.BGI:

This driver has been developed for the graphics card ELSA XHR which

contains the Hitachi graphics controller ACRTC HD63484. It has been

tested with ELSA XHR Alpha644 up to a resolution of 1024x768.

Preconditions:

The XHR driver (XHRDRV.SYS) must be loaded and the card must be adjusted

for the desired resolution (see ELSA manual).

The following DOS variable must be set:

SET BGIDRIVER=ELSA

----------------------------------------------------------------------------

HGSCA.BGI:

This driver has been developed for the Hercules Graphics Station Card

(in TIGA mode).

Preconditions:

- the Hercules operation system (HBOOT.SYS) must be installed;

- the TIGA Communication Driver (TIGACD) must be installed;

- the TIGA DOS variable must have been set

(e.g. SET TIGA=-MC:\TIGA -LC:\TIGA);

you will find a description of the above items in the Hercules manual

delivered with your graphics card;

The following DOS variable must be set:

SET BGIDRIVER=HGSCA

After the desired resolution is adjusted you must generate AL modules

from the RL modules EXTPRIMS.RLM (delivered with the graphics card)

and BGIPRIMS.RLM (delivered with EAGLE). You can use the batch file

ALMGEN.BAT for this purpose which possibly has to be modified.

The generated modules EXTPRIMS.ALM and BGIPRIMS.ALM must either be stored

in the actual directory or in the TIGA directory.

For copyright reasons you may use the HGSCA BGI driver ONLY with EAGLE

and may not give it away to any third party users.

----------------------------------------------------------------------------

HGSCR.BGI:

This driver has been developed for the Hercules Graphics Station Card

(using TIGA 2.0 or above).

Preconditions:

- the Hercules operation system (HBOOT.SYS) must be installed;

- the TIGA Communication Driver (TIGACD) must be installed;

- the TIGA DOS variable must have been set

(e.g. SET TIGA=-MC:\TIGA -LC:\TIGA);

- the predifined resolution of the TIGA card will be used;

you will find a description of the above items in the Hercules manual

delivered with your graphics card;

- the file BGIPRIMS.RLM must be stored in the actual or in the TIGA

directory;

The following DOS variable must be set:

SET BGIDRIVER=HGSCR

For copyright reasons you may use the HGSCR BGI driver ONLY with EAGLE

and may not give it away to any third party users.

----------------------------------------------------------------------------

TIGA.BGI:

This driver has been developed for TIGA standard cards and has been

tested with ELSA XHR Gemini644 (up to 1024x768).

Preconditions:

- the Gemini operation system (GMIBOOT.SYS) must be installed;

- the TIGA Communication Driver (TIGACD) must be installed;

- the TIGA DOS variable must have been set

(e.g. SET TIGA=-MC:\TIGA -LC:\TIGA);

- the predifined resolution of the TIGA card will be used;

you will find a description of the above items in the Gemini manual

delivered with your graphics card;

- the file BGIPRIMS.RLM must be stored in the actual or in the TIGA

directory;

The following DOS variable must be set:

SET BGIDRIVER=TIGA

For copyright reasons you may use the HGSCA BGI driver ONLY with EAGLE

and may not give it away to any third party users.

----------------------------------------------------------------------------