Notes for POWERLan 3.01

========================

This file contains changes from the documentation and various other useful

bits of information which may not be documented elsewhere.

Contacting Performance Technology

---------------------------------

Detailed information regarding Performance Technology's end user support

policy is included in your POWERLan package.

Sales:       800-FAST-LAN (800-327-8526)

Main:        210-979-2000

Main FAX:    210-979-2002

Support:     210-979-2010 (9AM-5PM Central time)

Support FAX: 210-979-2011

BBS:         210-979-2012

Internet:    support@perftech.com

Compuserve:  GO PERFTECH (PCVENH, section 13)

Printing Speed

--------------

By default, unspoolers operate in "interrupt" mode whenever possible.  This

provides the fastest possible print speed, but in some cases may consume

excessive CPU time, making other applications (such as Windows) run too

slowly.  If you experience this problem, we recommend increasing the unspooler

buffer size or setting your unspooler to "polled" mode by configuring an

interrupt level of zero.

Automatically Starting Windows Alert

------------------------------------

The POWERLan Setup program automatically starts the Windows Alert

program (WALERT.EXE) whenever a machine is configured to receive alerts.

It does this by modifying the "LOAD=" line in WIN.INI.  The POWERLan for

Windows User's Guide describes a different method for automatically starting

the Windows Alert program.  That method should be used only if you require

alerts while in Windows, but not while in DOS (e.g. a machine that uses

Windows exclusively).

Serial Printer Cabling

----------------------

The POWERLan print unspooler can use software or hardware handshaking,

sometimes called flow control, or pacing.  This is how the printer tells

the unspooler when it can accept more data.  Software flow control is

commonly called XON/XOFF.  In this mode, the printer sends characters back

through the serial port to the unspooler.  When an XOFF is sent from the

printer, the unspooler stops sending data until an XON is received.  Hardware

flow control uses signals on the RS-232 cable to control this same function.

Two common methods are called CTS/RTS and DSR/DTR.  The only difference

between these methods is the particular signal used.  In both cases, when the

appropriate signal (either CTS or DSR) from the printer is "true," then

the unspooler is allowed to send data to the printer.  Some printers use the

CTS signal, while others use DSR.  The POWERLan unspooler automatically

supports all three types of flow control.  By monitoring data received from

the serial port, it always supports software flow control.  In addition,

the unspooler will not send data to the printer unless BOTH CTS and DSR are

"true."  This generally works because printers which use the CTS signal

for flow control typically always assert the DSR signal, and printers which

use the DSR signal for flow control typically assert the CTS signal.  In

addition, the print unspooler will consider the printer to be "on line"

whenever EITHER the CTS or DSR signal is asserted.  Most standard serial

printer cables and printers should use one of these methods, and there is

no particular performance or reliability advantage to any of them.