FFFFFFFF

FF                          an ISDN CAPI FOSSIL driver

ccc FF    ooooo   sssss

cc   FFFF oo   oo ss                by Martin Winkler &

cc   FF   oo   oo  ssss              Christoph Lueders

ccc FF    ooooo      ss

ssssssssssssssssssssssss          from Zaphods BBS, Bonn/FRG

--- Documentation ---

for

cFos, cFos/Plus and cFos/Pro

Version 1.05, Release date 15-Dec-1994

0. Distribution Files

README          Introduction

ORDER.DOC       How to obtain the professional version (German)

ORDER.ENG       How to obtain the professional version (English)

XMAS.DOC        'cFos' Special Christmas Edition! (German)

XMAS.ENG        'cFos' Special Christmas Edition!

COPYING.CF      License Agreement

WHATSNEW        Revision history - What has changed ?

CFOS.DOC        German documentation

CFOS.ENG        English documentation

MODEM.DOC       Survey of the modem commands

APPEND.DOC      'cFos' ISDN appendage documentation

CFOS.FAQ        Frequently asked questions about 'cFos'

CFOS.EXE        'cFos' executable (resp. 'cFos/Plus or 'cFos/Pro')

FILE_ID.DIZ     File description for mailboxes

'cFos' is distributed as a ZIP-Archive including a  AV.  PKZIP's

AV is a kind of seal that will break when files of  the  archive

are deleted, changed or new files are added. The 'cFos'  archive

is the authors' original version only iff the following  message

appears after unpacking

��������������������������������������������������������������������ķ

�  This is an original cFos distribution, iff the above lines read:  �

�                                                                    �

�     Authentic files Verified!   # MFA364                           �

�     Zaphods BBS                                                    �

�                                                                    �

�  Thanx for using cFos!                                             �

��������������������������������������������������������������������ͼ

If you see anything else the archive is not the same as released

by the authors. If you get a warning like

PKUNZIP: (W16) Warning!

Authenticity Verification Failed!

One or more of these files has most likely been tampered with.

be careful, the contends may be corrupted.

'cFos/Plus' and 'cFos/Pro' (see below for  further  explanation)

may come in different flavours and may not have this AV-seal. So

don't be alarmed.

Contents:

Introduction  . . . . . . . . . . . . . . . . . . . . . . . .  1

Features at glance  . . . . . . . . . . . . . . . . . . . . .  2

'cFos' Command Line Parameters  . . . . . . . . . . . . . . .  3

AT Command Emulator . . . . . . . . . . . . . . . . . . . . .  4

Outgoing calls  . . . . . . . . . . . . . . . . . . . . . . .  5

Incoming calls and automatic protocol selection . . . . . . .  6

'cFos' Status Line  . . . . . . . . . . . . . . . . . . . . .  7

Safety/Debug features . . . . . . . . . . . . . . . . . . . .  8

Blocksizes and memory requirements  . . . . . . . . . . . . .  9

Windowsizes, the ultimate speed . . . . . . . . . . . . . . . 10

'cFos' as a multiport fossil  . . . . . . . . . . . . . . . . 11

'cFos' channel bundling (CCB) . . . . . . . . . . . . . . . . 12

'cFos' compatibility with current software  . . . . . . . . . 13

ISDN hardware/software  . . . . . . . . . . . . . . . . . . . 14

Miscellaneous things  . . . . . . . . . . . . . . . . . . . . 15

Adresses, authors, support, availability  . . . . . . . . . . 16

Credits . . . . . . . . . . . . . . . . . . . . . . . . . . . 17

End of Documentation, Thanks for using 'cFos' . . . . . . . . 18

Appendix

Capi error messages . . . . . . . . . . . . . . . . . . . . .  A

Format of V.110 user rate (ATS27) . . . . . . . . . . . . . .  B

B2 frames and windowsizes X.75  . . . . . . . . . . . . . . .  C

Connect problems with ISDN Blaster (X.75) . . . . . . . . . .  D

Connect problems with ELINK Modems  . . . . . . . . . . . . .  E

CCB with several S0-busses of different phone numbers . . . .  F

'cFos' modem error messages . . . . . . . . . . . . . . . . .  G

Software compatibility list . . . . . . . . . . . . . . . . .  H

Availability  . . . . . . . . . . . . . . . . . . . . . . . .  I

'cFos' and CompuServe with DOSCIM/WINCIM  . . . . . . . . . .  J

1. Introduction

'cFos' is a FOSSIL driver / BIOS INT 14h Emulator which works on

a basis of a modem emulator and a CAPI driver  V1.1,  Profil  A.

'cFos' allows you to make use of a variety of existing  software

even for ISDN.

As ISDN is more than a net for high  speed  data  transfer,  our

goal is to make most of the features of ISDN accessable  by  the

use of 'cFos'.

'cFos' comes in three flavours:

- 'cFos' (without any extension): This is the shareware version

of our driver. You may use this  version  for  an  evaluation

period of up to 90 days, in order to  determine  whether  the

program meets your  needs  before  purchasing  it.  Once  the

evaluation period ends, you agree to either stop using it, or

to purchase the Professional Version of cFos ('cFos/Pro').

- 'cFos/Plus': This Version of  'cFos' has serveral  additional

features and comes with  an  installation  and  configuration

program. The additional features available in 'cFos/Plus' are

marked with a '{+}'. You may use  this  version  without  any

time limitation.

- 'cFos/Pro': This is the Professional Version  of  'cFos'.  It

has ALL features discussed in this  documentation  and  comes

with  an  interactive  configuration  utility  (CFOSCFG.EXE).

Features that are only available in  the  'cFos/Pro'  version

are marked with a '{++}'. Of course, all features marked with

a '{+}' are also available.

Features of the different 'cFos' flavours:

| Feature                     | cFos | cFos/Plus | cFos/Pro |

+-----------------------------+------+-----------+----------+

| FOSSIL rev. 5               | yes  |   yes     |    yes   |

| BIOS INT 14h Emulator       | yes  |   yes     |    yes   |

| Modem Emulator              | yes  |   yes     |    yes   |

| Configuration program       |      |   yes     | yes/ext. |

| MultiPort Mode              |      |   yes     |    yes   |

| Time-synchronization        |      |   yes     |    yes   |

| +++ Hangup-sequence         |      |   yes     |    yes   |

| SPV-Support (1.TR.6 only)   |      |   yes     |    yes   |

| "Knocking" (*)              |      |   yes     |    yes   |

| cFos Channel Bundling       |      |           |    yes   |

| dyn. cFos Channel Bundling  |      |           |    yes   |

| No-debug version            |      |           |    yes   |

| NetWare Time-sync.          |      |           |    yes   |

| Charge-timer                |      |           |    yes   |

| FOSSIL Dump                 |      |           |    yes   |

| Logging (*)                 |      |           |    yes   |

| Baudrate evaluation         |      |           |    yes   |

| Hangup per command          |      |           |    yes   |

| CallerId evaluation         |      |           |    yes   |

| phone support               |      |  vendor   |  vendor  |

| printed documentation       |      |           |    yes   |

| unlimited usage             |90days|    yes    |    yes   |

+-----------------------------+------+-----------+----------+

* not yet implemented

To obtain 'cFos/Pro' read and fill out the order form  ORDER.ENG

and send it to the address mentioned in the form.

To run 'cFos' an 286 AT class computer is at least required.

'cFos' quick installation:

In most cases you can simply load 'cFos' by "CFOS i" and  use

it with the modem string AT&F.

{+}        'cFos/Pro'  and  'cFos/Plus'  come   with   the   interactive

configuration utility CFOSCFG.EXE to  ease  the  installation

and to change the PROFILE file from outside  'cFos'.  It  may

also be used for quick configuration.

Just get  into  the  Hardware  setup  and  select  all  those

ISDN-hardware you use. On exit, save the profile. After  that

select the Software setup and tag all software  products  you

plan to use. The CFOSCFG utility will modify the profile in a

way that all of them  will  run  well.  At  the  end  of  the

configuration CFOSCFG will ask if it shall generate a  report

file.

CFOSCFG will also create a STARTFOS.BAT and STOPFOS.BAT  file

for quick  startup  (and  quick  unload)  of  the  configured

'cFos'. They will reflect your Hard- and Software setup.  You

should  use  ATZ  as  the  modem  init  string  so  that  the

parameters stored in PROFILE get effective.

{++}       Users of 'cFos/Pro' can also select "extended settings"  that

allows you to configure 'cFos' Channel  Bundling  and  preset

the ISDN protocol,  EAZs,  charge-/time  limited  calls,  the

outdial prefix for telephone equipment and 'cFos' status line

easily.

For every selection a help text and some hints are displayed.

'cFos' and the different national ISDN networks:

The manufacturer of ISDN hardware supplies  drivers  for  his

ISDN equipment, including a CAPI driver, Version 1.1,  Profil

A. Among others these drivers are  used  to  do  the  ISDN  D

channel protocol. In Germany the D channel protocol is either

1.TR.6  or  DSS1  (EuroISDN).  The  CAPI  V1.1  standard  was

designed for the 1.TR.6 protocol. Therefore there exist  such

values as "Additional Service  Indicator"  and  "EAZ",  which

normally do not occur in other ISDN networks. In addition  to

the 1.TR.6 drivers manufacturers of  ISDN  hardware  normally

offer drivers for other different D channel protocolls. These

drivers then translate information  from  the  national  ISDN

network to values of the 1.TR.6 specification. As an  example

if you use a DSS1 (EuroISDN) D channel driver  the  "Multiple

Subscriber Numbers", MSNs are mapped to EAZs (the last  digit

of a phone number). The same is done for LLC values which are

translated into values for the Additional  Service  Indicator

and vice versa and BA values are  mapped  to  values  of  the

Service Indicator.

'cFos' can be used  in  different  ISDN  networks  under  the

assumption  that   the   manufacturer   has   realized   this

translation from/to national D  channel  information  to/from

1.TR.6. In this documentation we therefore talk about  "EAZ",

"Service  Indicator"  and  "Additional  Service   Indicator",

independant of what is actually transmitted in the respective

ISDN network.

At this time 'cFos' is used successfully with 1.TR.6 and DSS1

(EuroISDN) in many different countries.

The CAPI 2.0 standard overcomes the limitation to 1.TR.6.  We

will be able to offer a 'cFos' version for CAPI 2.0 soon.

2. Features at a Glance

* 'cFos' runs with any card that has an ISDN CAPI V1.1. Profil A

* 'cFos'supports FOSSIL spec. Revision 5 (according to FSC-0015)

with several extensions.

{+}     * 'cFos' supports several B-channels simultaneously, for  choice

with automatic ringdown in MultiPort mode.

* 'cFos' runs  stand-alone  or  in  co-operation  with  standard

RS-232 FOSSILs.

* 'cFos' has a built in BIOS-Emulator so that you can  also  run

programs that only support INT 14h.

* 'cFos' is fast. Having windowsizes >= 2 you  achieve  transfer

rates with X.75 of 7900 cps per channel and even more.

* 'cFos' has a  built  in  Modem-Emulator.  With  this  you  can

imitate all modem messages. You can also set/adjust  all  ISDN

specific parameters and save them. By the support of our 'cFos

built-in  modem  emulator'  you  can  make  use  of  all  ISDN

paramters.

* 'cFos' can display a status  line  that  shows  you  all  ISDN

connecting parameters plus Modem LEDs for carrier detect,  Off

Hook,  transmitted  and  received  data.

{++}      The professional version also displays the  remaining  seconds

per charging unit (toll saver) and a CPS-O-METER, which  shows

the thruput of the last seconds.

{+}     * 'cFos' supports  SPV's  (Germany,  1.TR.6:  semipermanent con-

nections)

* Support of almost every B2 and  B3  protocol,  e.g.  X.75  and

V.110.

* Support   of  TELES   channel   bundling   (128kbps   with   2

B-channels).

* Savety, inactivity and connect timers to avoid toll explosion.

'cFos' has a special code to deal with  possible  malfunctions

of the CAPI/ISDN net.

{+}     * You can evaluate the Time/Date information of the ISDN network

and set the local computer time.

{++}      You can also set the time of a NetWare Server.

* 'cFos' can hold its buffers in DOS and XMS UMBs.

{++}    * 'cFos' has its own built in CHANNEL  BUNLDING! 'cFos'  Channel

Bundling (CCB) operates independently of your  ISDN  card.  It

supports up to 4 B-channels - also with  several  ISDN  boards

simultaneously.  Besides  that  you  can  arbitrarily  combine

'cFos' MutliPort Mode with CCB. E.g. you can  bundle  2  ports

simultaneously with 2 channels each. 'cFos' achieves 15800 CPS

with 2 B-channels, 22500 CPS with 3 B-Channels and  more  than

30000 CPS with 4 channels.

3. 'cFos' Command line parameters

'cFos' offers the following commands:

i Install

Installs 'cFos' as TSR in PC memory. It requires that  a  CAPI

driver is already loaded (warning message if not loaded).  All

switches have to be set. Later setting will only  be  possible

with specific commands.

d Deinstall

Deinstalls 'cFos' and frees all memory used by 'cFos'.  Please

deinstall  'cFos'  first,   then   the   CAPI   driver.   When

deinstalling 'cFos' all existing  connections  run  by  'cFos'

will be disconnected.

r Reregister

Re-register 'cFos' with CAPI. Thereby the CAPI driver  can  be

initalized anew. All existing connections will be disconnected

and the internal CAPI structures will be initalized.

{+}     t Tranx

Syncronize the computer's clock with the  time  that  is  made

available by ISDN. For further information see the explanation

below. If no -jn is set on the commandline a possible  Network

server is synchronized as well.

{++}    bps:<portnb> (bits per second)

The Environment Variable BPSRATE is set to the  value  of  the

baudrate of the current connection. This  can  be  useful  for

later evalution in batch files (for  example  in  calling  BBS

software). You can specify a port number  with  this  command,

for example "cfos  bps:2",  if  none  is  specified,  zero  is

assumed.

{++}    callerid:<portnb>

The environment variable CALLERID is set to the caller  ID  of

the current or last caller  on  port  <portnb>.  This  may  be

useful for callback, logging or verification of the caller.

{++}    eaz:<portnb>

The requested EAZ of the last or current call  is  written  to

the environment variable EAZ, so it can be checked within e.g.

batch  files.  This  may  be  useful   for   selecting   which

application to spawn from a mailer  system,  e.g.  Mailbox  or

answering machine.

reboot

The FOSSIL Definition requires a  reboot  function,  which  is

also available  on  the  command  line.  Before  the  computer

reboots all open files of DOS  applications  will  be  closed,

then the following disk caches will be flushed: QCache,  Super

PC Kwik, PC Tools PC-Cache 5.x & 6.x,  Qualitas  Qcache  4.00,

Norton Utilities NCACHE, SMARTDRV v4.00+ and Hyperdisk  4.50+.

After that the computer is booted.

init:<portnb>

This  initializes  the  COM-Port  <portnb>  for  FOSSIL   use.

Sometimes (e.g. after  a  HS/Link  session)  the  COM-port  is

de-init'ed and some programs may have problems with  this,  so

you can init it manually.

and the follwoing switches

-b maximum B2-Framelength

maximum   B2-Framelength   you   can    choose    with    the

modem-emulator, default = 2048 bytes

-w maximum windowsize

Maximum B2-windowsize you can choose in modem-emulator

These two functions also reserve the amount of memory  necessary

for registering at the CAPI. By using ATS22=xxxx and ATS26=x you

can certainly set a smaller value for the next  connection,  but

not a bigger one.

-c Portnumber (default all ports)

0=COM1, 1=COM2, ...

-c can be set several times. The setting of the ports  allows

the co-operation of 'cFos' with an RS232 FOSSIL on  a  single

computer. 'cFos' checks every INT 14h call, whether it should

support this port number.  If  not,  'cFos'  will  issue  the

original INT  14h,  so  that  an  underlying  driver  or  the

original BIOS code is executed.

-e Enable BIOS emulator

0 = off, 'cFos' works as a FOSSIL only

1 = on,  'cFos' works as a FOSSIL as well  as  BIOS  INT 14h.

Thereby you can run programs for ISDN  that  do  not

use a FOSSIL but BIOS INT 14h

2 = Force on, 'cFos' operates only as a BIOS emulator.

Recommendations for programs that use BIOS  INT  14h

on PS/2 computers or other INT  14h  functions.  The

INT 14h functions 4-21h are de-activated.

Note:  The  INT  14h  function  'set  baudrate'  is  used  to

terminate a call when the BIOS emulator is  not  turned  off,

you specified the -jk switch and the port is NOT init'ed  for

FOSSIL use (ie. no  -ji  was  set  on  commandline).  Besides

FOSSIL function 0x06 (raise/lower DTR) and the  "+++ATH0<cr>"

sequence  it  is  the  only  non-interactive  possibility  to

hangup.

-r rxbufsize, size of the FOSSIL's receiver buffer.

Default size is 1024 bytes. Increasing this  size  is  mainly

only useful  if  connecting  with  machines  that  send  with

windowsize 1.

-t txbufsize, size of the FOSSIL's transmitter buffer.

Default is B2-framelength * (windowsize + 1) bytes.

-v CAPI interrupt number, default 0xf1 = 241.

{++}    -a auxiliary port including controller number (e.g. -a1).

The professional 'cFos' version offers you the possibility of

activating so  called  auxiliary  ports  for  'cFos'  channel

bundling (CCB), that means data  transfer  on  serveral  ISDN

B-channels simultaneously. For details see chapter 12.

Postfixing a number with a "k" has  the  effect  of  multiplying

this number  by  1024.  "-r4k"  means  for  example  4096  bytes

receiver buffer. In addition to that all values can be specified

in hexadecimal by prefixing them with 0x, for  example  0x800  =

2048.

Additional -j flags:

-j3  disable 386 CPU support.

-jb  Reboot on an emergency_stop() condition. This is  a  severe

error condition and normally  'cFos'  only  loggs  out  all

active connections, but setting this option  will  increase

safety.

-jc  enable carrier detect LED. The Scroll-Lock LED is used as a

carrier detect LED. During a connection the LED in  on.  As

long as an incoming call is neither accepted  nor  rejected

the LED flashes. Warning for programs using the Scroll-Lock

LED for  different  purposes  (for  example  the  FrontDoor

terminal).  They  will  react  to  the  activity   of   the

Scroll-Lock LED. You may specify -jc1 to use the NumLock of

-jc2 to use the CapsLock LED.

{++}    -jd  Data dump. When this flag is set 'cFos' logs  all  received

and transmitted data into a file called FOSSDUMP.  FOSSDUMP

is located in the same directory as CFOS.EXE. Teh data dump

can only be stopped by de-installing the the FOSSIL driver.

The  data  dump  function  requires  10k  Bytes  additional

memory. This Option is not included in the No-Debug Version

of 'cFos'.

-je  Disable environment deallocation. This switch may  help  in

case of a system crash or a severe  DOS  error  immediately

after loading 'cFos'.

-jg  Disable all XMS functioncalls.

-ji  Loads  'cFos'  with  initalized  COM  ports.  This  is  for

software which neglects to initialize COM ports before use.

This initializes the ports for FOSSIL  use.  So  some  BIOS

functions won't run, because 'cFos' thinks you're using the

FOSSIL interface. So only use this  parameter  with  FOSSIL

applications.

-jk  Enable Set_Baudrate() carrier drop. If 'cFos' is using  the

BIOS emulator, the carrier is high,  the  COM-port  is  not

used via FOSSIL (i.e. not FOSSIL init'ed) and  this  switch

has been set, the current connection will be disconnected.

-jm  Disable alternate monitor support.  Only  possible  in  the

debug version.

{++}    -jn  Disable NetWare Support. If this switch is set 'cFos'  does

not attempt to find or to support NetWare.

-jo  Disable far CALLs and use "real INTs" instead to call CAPI.

This is only useful in  Protected  mode  environments  that

catch interrupts. OS/2 is deteced automatically.

-jp  Activates passive layer B3 setup in order  to  fulfill  the

specific requirements of ISDN Blaster  FOSSIL  PCIF  V5.78.

(Recommendation for AVM only - not for Teles). It might not

be neccessary any more if the remote site uses  PCIF  V5.81

or higher.

-jr  Disable CAPI re-register  function  (neccessary  for  SOLIS

boards). If your CAPI crashes after some  calls,  try  this

switch (but only use it, if it helps).

{+}     -js  Ignore seconds in ISDN date/time info. For further informa-

tion see Tranx.

-ju  Do not use UMB memory. Otherwise 'cFos' tries  to  allocate

UMB memory first before using conventional memory.

-jv  Disable V.110. By setting -jv 'cFos' knows that the respec-

tive CAPI does not support V.110 and upon the ATB1 and ATB2

commands 'cFos' issues the "ERROR" message.  Also  incoming

calls  which  indicate  V.110  by  the  additional  service

indicator are rejected. The purpose of this feature  is  to

save the caller phone charges. For further information  see

chapter 6.

-jx  pretend to be the X00 FOSSIL. This switch forces to  return

the same values in CX and DX after having executed function

0x1b as X00 does. This is required for  older  versions  of

XBTX.

-d[d] debugging trace:

If this switch is set at load time 'cFos' will log  all  CAPI

messages into a trace file, called CTRACE located in the same

directory as CFOS.EXE. This  requires  10k  bytes  of  buffer

space  in  addition.  That  file  can  easily  consume  large

quantities of disk space. Therefore this function should only

be used in rare cases. -d turns on the logging function.  -dd

results in a hex dump of CAPI  messages.

In case 'cFos' does not properly run on your computer or does

not accept incoming calls, etc... you can send us a CTRACE of

your problem. You should load 'cFos' with CFOS i -dd and  you

should in any case deinstall 'cFos' with CFOS  d  before  you

pack or mail the CTRACE file.

The debug trace functionality is not included in the No-Debug

version of 'cFos'.

{+}     Tranx:

The ISDN network provides the actual time/date to the calling

party during connection setup and clearence. 'cFos'  compares

the ISDN time with the clock of the computer  and  calculates

the difference. 'cFos' does  not  change  the  clock  of  the

computer immediately, only on request: By  calling  'cFos  t'

the  computer's  clock  will  be  corrected  by  the  average

difference calculated before. This can also  be  accomplished

by the modem command AT&T. This way of setting  the  time  is

much saver than setting up a connection.  If  you  constantly

want to set the time place AT&T into the modem init string.

Note: In some  areas  the  ISDN  network  provides  the  time

without  seconds.  'cFos'  is  designed  in  a  way  that  it

automatically approximates the correct  time  within  several

connections. This mode can be enforced (-js  switch).  If  so

the seconds are ignored, even if they are provided. This  can

be  useful  when  using  local  telephone   equipment   which

simulates the seconds.

You may set the maximum minutes the ISDN time may be off from

your systems time  with  the  -m  switch.  So,  setting  -m90

(default)  will  only  accept  and  set  your  systems   (and

networks) time, if the difference  between  actual  and  isdn

time is less or equal 90 minutes.

{++}       If a NetWare server is detected, its time will be set  unless

-jn switch is set. The user has to be a file  server  console

operator, however.

4. AT command emulator:

As this driver is designed to enable the user  to  use  existing

modem software with ISDN, 'cFos'  emulates  a  modem  that  uses

commands such as  ATD  for  dialing  and  AT&V  to  display  the

configuration. The entire control of the  CAPI  driver  and  the

behaviour of 'cFos' is operated by the modem emulator.

The emulator offers help which can be  displayed  with  the  AT?

command. You can find a complete list of all modem commands  and

registers in MODEM.DOC. There is  a  listing  of  the  numerical

result codes as well (we designed them as like the USR HST modem

gives them back (codes 0 to  11)  and  extended  them  for  ISDN

requirements).

The receiver buffer has to be big enough to store the respective

output of the modem emulator. For example the buffer should have

the size of 2k bytes to store a complete AT? screen.

If the receiver buffer is too small  (caused  by  -rxxx  switch)

then the error message "rx-buffer too small, output  lost"  will

be displayed.

It is sometimes difficult to  relate  a  recent  and  much  more

complex technology as ISDN to  the  terminology  of  a  standard

analogous modem. Therefore a brief explanation  what  the  modem

messages mean:

- NO ANSWER

The call came through but the called party does not answer the

phone. CAPI cause:

0x34ba: No user responding

- NO DIALTONE

'cFos' could not come through the ISDN network. CAPI causes:

0x3301: D channel layer 1 setup error

0x3302: D channel layer 2 setup error

0x3305: D channel layer 1 shutdown

0x3307: D channel layer 3 shutdown

0x348a: No channel available

0x34a0: Outgoing calls barred

0x34d9: Network congestion

0x34f0: Local procedure error

0x34a3: Nonexistent CUG

- BUSY

The call reached the destination but it could  not  answer  or

rejected the call. CAPI causes:

0x34a1: User access busy

0x34bb: User busy

0x34bd: Incoming calls barred

0x34be: Call rejected

- NO CARRIER

All other causes result in a NO CARRIER message from the modem

emulator. If you think that a certain  cause  belongs  to  the

messages above please mail us.

0x3483: This network error is issued if you try  to  call  a

party  with  a  non  supported  service,   e.g.   an

analogous phone. We could issue a VOICE message, but

this is not yet implemented.

0x34b5: Destination not obtainable; you placed a  call  with

an invalid number or an invalid service. This may be

due to bad connection parameters. See CFOS.FAQ.

0x34f1: Remote procedure error; the calling  party  provided

unsupported   parameters,   such   as   an   invalid

Additional Servive Indicator.

In addition to that the modem emulator is  capable  of  serveral

other commands. For  an  explanation  please  refer  to  the  AT

command chart in MODEM.DOC. Apart  from  that  here  a  detailed

explanation of the ATIn  commands.  They  are  used  to  display

information about the last connection.

- ATI0   product info

- ATI1   status line (see chapter 7)

- ATI2   link information:

"Last inbound call":

contains  caller  id,  requested  service   indicator   and

additional service indicator of the last caller as well  as

his EAZ.

"Last outbound call":

contains the charges of the last outbound call.

"Last disconnect":

contains the  reason  and  the  CAPI  cause  for  the  last

disconnect. The reason can assume the following values:

Active:

the  application  requested  'cFos'  to  terminate   the

connection.

Passive:

The remote end or the CAPI driver disconnected.

Disconnect B3 timeout:

The respective safety timer has been timed out.

Disconnect D timeout:

The respective safety timer has been timed out.

CAPI reset:

All timers timed out and  'cFos'  re-registered  at  the

CAPI driver. This is an emergency stop condition.

Connect timeout:

the time needed to establish a connection  exceeded  the

value specified in register S7.

Inactivity timeout:

There was no data to be transmitted/received longer than

the time specified in register S19.

Appendage reset:

The connection was terminated due to a reset function of

the 'cFos' ISDN appendage.

- ATI4: Message dump:

Dump of  the  last  10  CAPI  messages  containing  w-elements

(1.TR.6), except charging  info  and  time/date  info.  As  an

example this can be used to display  the  last  caller-IDs  of

users who called during the last connection.

Profile saving:

In an ordinary modem  all  settings  can  be  written  into  a

battery buffered RAM by the AT&W command. Since 'cFos' has  no

such RAM the settings are stored on  disk  in  a  file  called

PROFILE. With DOS 3.1+ this file is located in  the  directory

of CFOS.EXE, otherwise in the  directory,  which  was  default

when 'cFos' was loaded. The file contains all modem  profiles,

stored by an  AT&Wn  command  (n=0..9).  The  respective  ATZn

commands restore and activate a profile. If no number is given

the AT&W and ATZ commands operate on profile 0.

The profile 0  is  loaded  at  'cFos'  startup  and  therefore

contains 'power on' defaults. You should  save  a  simple  and

working configuration to profile 0.

As DOS sometimes does not allow reading and writing  of  files

'cFos' checks  DOS's  critical  error  flag  to  determine  if

read/write operations are allowed. If you try to read/write  a

profile at a time  these  file  operations  are  not  allowed,

'cFos' returns ERROR.

ERROR is also returned if the file PROFILE is not valid  (e.g.

if it was created by a quite old version of 'cFos').

If 'cFos' cannot find a PROFILE, an ATZ command is  equivalent

to an AT&F command and is answered by OK.

Note: If 'cFos' is used for multiple ports, all ports  can  be

easily used with the same profile (for example by using ATZ on

all ports). To  avoid  that  you  can  use  different  profile

numbers for different ports (ATZ0..ATZ9).

In MultiPort Mode the registers

S13   serviced EAZ mask

S14   serviced SI  mask

S41   info-mask low

S42   info-mask high

are "mirrored". That means a change of one of these  registers

for one port automatically changes the value  of  the  profile

for all other ports. This is imposed by the design of CAPI.

5. Placing outbound calls:

Besides the phone number ISDN enables you to set up a variety of

parameters,  such  as  the  kind  of  service  and  transmission

protocols. For 'cFos' the default service is 'data transmission'

using  X.75  as  B2  protocol  and   transparent   (that   means

non-existent) B3 protocol. All  neccessary  parameters  for  the

different protocols, the respective CAPI supports, can be set by

the registers S16-S18, S20-S36, S39 and S40.

To ease the choice of the most important transmission  protocols

you  can  use  simple  AT  commands  to  select  the   following

protocols:

ATB0

Data transmission using X.75 and transparent B3  protocol.  In

contrast to the CAPI default, the X.75 window- and  frame-size

are set to the values specified when  'cFos'  was  loaded  (by

switches -b and -w). An Additional Service Indicator of  0  is

used for outgoing calls.

ATB1

Data transmission using V.110 with 38400,8,n,1,  ansynchronous

and  transparent  B2/B3  protocol.   An   Additional   Service

Indicator of 64 (40 hex) is used.

ATB2

Data transmission using V.110 with 19200,8,n,1,  ansynchronous

and  transparent  B2/B3  protocol.   An   Additional   Service

Indicator of 199 (C7 hex) is used.

ATB3

Like ATB0, but with channel bundling (128k bps). Only  TELES's

channel bundling is currently supported. This is only possible

if BUNDLE.EXE is loaded by STARTS0. Note that TELES's  channel

bundling is not compatible with 'cFos' Channel Bundling (CCB),

which is actived by AT&Bn (explained below).

ATB4

Elink compatiblity mode,  data  transmission  using  X.75  and

transparent B3 protocol. In contrast to the CAPI  default,  an

X.75 windowsize of 7 and a framesize of  256  bytes  is  used.

This mode is only available if 'cFos' was loaded at least with

-b256. Otherwise ATB4  results  in  an  ERROR.  An  Additional

Service Indicator of 146 is used.

We recommend that you use  these  commands  for  placing  calls.

However you can also set the individual registers  manually  for

special applications. For example some  called  parties  do  not

accept calls if the Additional Service Indicator is not 0  (e.g.

CompuServe).

In addition here are some settings for special calls, e.g.  with

TELIX-Int14 or Terminate:

for CompuServe und Datex-P Gateways

V110, 9600,async,7,E,1                ATB0S20=8S27=237

but for DOSCIM or WINCIM              ATB0S20=8S27=197S10.4=1

V110, 9600,async,8,N,1

6. Handling incoming calls / protocol selection:

First you have to tell the CAPI driver which ISDN  services  and

EAZs it should listen to. This is done by setting the  "Serviced

SI Mask" and the "EAZ  Mask".  The  services  are  specified  by

setting  register   S14.   For   example   the   service   "data

transmission" is selected by setting bit  7  =  1.  Bit  1  =  1

enables incoming calls with service "telephony/voice calls". The

active EAZs are specified by setting  the  appropriate  bits  of

register S13 or by the AT&Lxxxx  command.  AT&L*  activates  all

EAZs. AT&123 activates EAZs 1, 2 and 3.

'cFos' currently supports the services "data  transmission"  and

"telephony". If other services are activated,  'cFos'  uses  the

protocols and parameters given by registers S20-S36 to answer  a

call.

On incoming calls with service "telephony" 'cFos'  uses  the  B2

protocol 'bittransparent' and  the  b3  protocol  'transparent'.

This results in a constant data stream of 8000 cps of  digitized

analog-samples to the application. However the service activated

by default is "data transmission" only.

Due to the design of  CAPI  it  is  not  possible  to  select  a

protocol on the basis of a  negotiation  between  both  parties.

Thus 'cFos' selects the protocols for incoming  calls  with  the

service "data transmission" according to the "Additional Service

Indicator". A later change of  the  protocol  is  not  possible,

since there is no way the CAPI driver can indicate the protocols

available to the remote party nor its parameters.

The B3 protocol is  always  chosen  due  to  registers  S21  and

S30-36. We  recommend  to  use  'transparent',  since  the  CAPI

default of 1 (= T70 NL) is very rarely used by any system.

The B2 protocol is selected according to the "Additional Service

Indicator" regarding 1.TR.6.

For V.110, 38400 bps, asynch. no value  for  additional  service

indicator is specified by 1.TR.6. However callers  can  activate

this protocol by using a value of 64 or 128.

Specification for the Additional Service Indicator :

0000 0000    caller requests  X.75, 64000 bps

1000 0000    caller requests V.110, 38400,8,n,1,asynch.

01-- -000    caller requests V.110, 38400, asynch.

1010 ----    caller requests V.110, X.30 (ECMA 102), synch.

1011 ----    caller requests V.120, synch.

11-- ----    caller requests V.110, X.30 (ECMA 102), asynch.

the most important :

1100 0111    caller requests V.110, 19200,8,n,1,asynch.

0100 0000    caller requests V.110, 38400,8,n,1,asynch.

Whether these protocols are supported  by  the  respective  CAPI

driver depends on the manufacturer of the CAPI driver. For V.110

'cFos' uses "V.110 with transparent B2-protocol" (8).

Callers who do not support this specification or  use  telephone

equipment, which filters the Additional Service Indicator, might

activate another protocol for the called 'cFos' than expected.

To  overcome  the  trouble  with  the  protocol  selection,   we

implemented a few "specials" in 'cFos':

If the additional service  indicator  is  unknown,  'cFos'  uses

register S43 to select the protocol. The high byte specifies the

protocol and the low byte the V.110 user  rate,  if  applicable.

Default for S43 is V.110, 38400,8,n,1,asynch., because this mode

seems to cause often most of the trouble.

In addition each individual  EAZ  can  be  fixed  to  a  certain

protocol, used if the additional  service  indicator  equals  0.

This is done by registers S50-S59 for EAZ 0 to EAZ 9. If the low

byte of the  respective  register  is  nonzero  the  feature  is

activated for this EAZ and the value is used  as  a  V.110  user

rate. The  high  byte  then  specifies  the  protocol  selected.

However an additional service indicator of nonzero activates the

above protocol selection according to 1.TR.6.  By  default  this

feature is disabled.

Example:

Let S51 = 0x0 and S52 = 0x0840 and  S53  =  0x08C7  and  the

Serviced EAZ Mask = 14 (you thus listen to  EAZ  1-3);  then

the following protocols are selected:

EAZ        AddSi = 0      AddSi <> 0

1        X.75           according to 1.TR.6

2        V.110,38400    according to 1.TR.6

3        V.110,19200    according to 1.TR.6

Thus:

Callers, requesting X.75, can call EAZ 1.

Callers, using 1.TR.6 conform  hardware/software,  reques-

ting V.110 can call all EAZs.

Callers using hardware/software nonconform to  1.TR.6  can

call EAZ 2 for V.110, 38400 and EAZ 3 for V.110, 19200.

See Appendix B for an  exact  specification  of  the  Additional

Service Indicator.

7. Status Line:

For the purpose of a look and feel of a modem or  for  debugging

purposes, 'cFos' can display a status line on the screen.

AT&D0     status line off

AT&D1     status line on, when the COM port is initialized

AT&D2     status line on, when a connection is active

ATS11=xx  display row of the status line (0 = top of screen)

Its layout is as follows:

|

|      # of frames 'cFos' is sending

|      |

{++}                      |      |   remaining sec's of current charging unit

|      |                                          |

{++}                      |      |            CPS-O-METER of last 8 sec's   |

|      |            |                             |

cFos> C-B3  ACOD  0R:64  0T:1024 C:12 7680 V110 19200  9111041     39

| |   ||||     |      |    |         |    |      |

connect/ |   ||||     |      |    Charging  |    |      Caller ID/

disconnect |   ||||     |      |    Units     |    |      dialed number

indicator |   ||||     |      |              |    |

(*)       |   ||||     |      |              |    bps rate bei V110

level   ||||     |      |              B2 protocol (e.g. X75)

||||     |      last transmitted block length, TX-"LED"

||||     last received block length, with RX-"LED"

Auto-Ans|||

||DTR-"LED"

Carrier Detect|

Offhook-"LED", on = reject calls (different to a modem)

(*) during an active connection the number of active B  channels

used is displayed here ("B3-1")

{++}    with 'cFos' Channel Bundling (CCB) "B3-2" can appear for example.

8. Safety/Debug features:

Starting out with ISDN it might appear unusual to you  that  you

don't always know if you still hold the connection or  hopefully

hung up.

'cFos' can prevent a charge explosion in several ways:

1. 'cFos' modem emulator has an inactivity timer  which  can  be

activated by setting register S19  to  the  time  in  minutes

which may pass without  transmission  or  reception  of  data

before 'cFos' disconnects.

2. In the process of disconnecting watchdog timers are  started.

For  example  'cFos'  waits  5  sec.   for   the   respective

DISCONNECT_B3_INDs and DISCONNECT_INDs before it re-tries the

disconnect request. It tries first to disconnect the B3 level

twice, then the D channel twice and  as  a  last  attempt  to

disconnect, it re-registers with CAPI. Then  all  connections

should be terminated.

3. In the connection phase 'cFos' uses a timer  to  wait  for  a

"carrier". If the connection cannot be established within the

time specified by register S7 it will be  disconnected.  This

avoids a "hang" while connecting the B3 layer.

4. In the debug version 'cFos' supports an alternate screen. You

can monitor the flow of messages to/from CAPI and other debug

information on this screen.

9. Blocksizes and Memory Requirements:

This FOSSIL driver offers a  confusing  variety  of  blocks  and

buffers. Therefore we try explain all of them.

B2-Framelength:

X.75 sends  data  on  layer  2  in  frames.  These  frames  (=

packages) are limited to a maximum length.  That  is  what  we

call  B2-Framelength.  The  CAPI   specifactions   limit   the

B2-framelength to a maximum of 2048  bytes.  Receiving  bigger

lengths may cause loss of data  and  disconnection.  Therefore

ISDN boards that send bigger B2 framelengths  are  incomatible

with CAPI applications !!!

B3-Framelength:

Data are sent in frames on layer 3 as  well.  If  layer  3  is

transparent (it has no protocol) B3 frames have the same  size

as B2 frames.

But if a protocol is in use on level 3 ,  for  example  T70.NL

(CAPI default but  not  'cFos'  default)  then  this  protocol

requires several bytes overhead. The B2 layer considers  these

bytes  as  raw  data  and  therefore  they  are  stored  in  a

corresponding buffer.

B3-Framelength  is  also  the  maximum  amount  of   bytes   a

DATA_B3-IND can signal on reception.

B2-Windowsize:

B2-windowsize is the maximum amount of B2 data blocks that the

CAPI can send without the necessity to confirm  the  reception

of data by the counterpart. To  enable  a  full-streamed  data

transfer (data blocks will be  continously  sent  without  the

delay  of  waiting  for  confirmation  of  data  receive)   B2

windowsize should at least be set on 2 unless the  counterpart

is not able to choose at least the same value (if  it  is  not

possible you may overrun your counterpart).

Buffer for API_REGISTER:

CAPI requires at least a buffer for  incoming  B3  datablocks.

This buffer has to be able to accept  at  least  one  B2  data

block including the B3 overhead. If B2 is set to X.75  and  B3

is to T.70.NL and a requested maximum B3 framelength  of  128,

CAPI requires a B2 framelength of 130 bytes,  thus  130  bytes

per block are necessary for this buffer. A buffer must be made

available to CAPI to store the maximum  B3  frames,  which  in

turn  depends  of  the  B2  windowsize  and  the   number   of

simultaneous connections. CAPI also requires  memory  for  its

message queues. Our example  leads  to  the  following  memory

requirements used for API_REGISTER (we assume that 10 messages

can be queued) :

(10 * 180) + (# of B3 connections * B2-windowsizes * 130)

Buffer for FOSSIL Functions:

A FOSSIL also requires memory for its ringbuffers.

The receiver buffer should at least have the size  of  the  B3

framelength.  Otherwise  'cFos'  can  not  fully  accept   the

received data block from CAPI. The data  block  will  then  be

splitted into several parts. These parts are written into  the

FOSSIL receiver buffer. This procedure may cause slight delays

of the handling of the received data.

'cFos' can also queue up to 8 messages received from CAPI  and

copy them into the receiver buffer,  if  the  receiver  buffer

provides enough memory. This is  more  than  the  maximum  B2-

windowsize by CAPI standards. But it requires  that  CAPI  has

sufficient receiver memory.

While sending 'cFos' tries to send  data  immediately  and  in

biggest blocks possible (and also as many as possible :-) only

limited by the maximum B2-windowsize). The way CAPI  transmits

data imposes that data located near the limits of  the  FOSSIL

transmitter buffer can not always be grouped to  blocks  of  a

maximum B3 framelength. The size of  the  transmitter  buffers

should be around B2-Framelength * (windowsize +  1)  to  avoid

the effect described above.

{++}      Using Channel Bundling the size/value should  be multiplied as

well by the amount of B channels.

10. Windowsize, the ultimate speed

You can try to increase the speed by using B3 datablocks as  big

as possible. This method might not be succesful, due to the  way

data is transmitted using X.75.

As it was explained above B2-windowsize is the amount of  frames

that are allowed to be on transmission without the necessity  of

confirming them. For example: If windowsize is 3,  three  blocks

can be sent one by one (without any delay) without  waiting  for

the receiver's confirmation.

Therefore 'cFos' has to wait for a confirmation for  each  block

if  windowsize  is  1  before  it  can  send  a  new  one.   The

confirmation can not be signaled before the application  of  the

ISDN board of the counterpart has accepted the block. (*)  (This

is very useful because it enables a way of flow-control  between

the counterparts. ISDN does not offer a RTS/CTS handshake :-)).

With a windowsize of 1 the result is that only one block  is  on

the way and ISDN transfer gains exactly the same time  behaviour

as X-Modem has.

This is stone age telecommunication !!!

But if windowsize is 2 'cFos' can already send  a  second  block

while the first one is on the way. The counterpart  can  confirm

the acceptance of the first block  while  accepting  the  second

one. By this way data blocks can be  continuously  sent  without

any delays.

Therefore you actually gain quite some speed. Our tests achieved

more than 7900 cps (theoretical maximum of 8000  cps)  using  an

average blocksize of 2048 bytes.  We  used  Z-Modem  for  mailer

protocol.  We  increased  the  windowsize  to  3   without   any

improvements.

Our Recommendation:

Even though 'cFos' transfer routines are designed for high speed

we do not care much about the CPS war.  By  a  closer  look  the

actual transfer time is what really matters. That is what  costs

your money. Transfering a 2 MB file with  7500  cps  instead  of

7900 cps makes a difference of 15 seconds. In the worst possible

case it costs 1 charging unit for a domestic call.

Another  important  factor  concerning  transfer  rates  is  the

protocol overhead (you should not overestimate it - more crucial

is the timing discribed above).  To  minimize  the  overhead  we

recommend B3 protocol = 4, that means transparent. Therefore the

overhead will be 0. To minimize the B2 protocol overhead as well

we recommend a B2 framelength of 2048  bytes.  All  ISDN  boards

should be able to handle this framelength  while  receiving;  at

least because of the reason to be compatible to CAPI.

Setting  ISDN   boards   to   bigger   framelengths   leads   to

imcompatibility to other cards. ISDN-Blaster unfortunately sends

16k byte frames by default. That's the reason why  there  is  no

transfer possible with this card without changes. But there is a

simple trick: As an owner of a Blaster just set  an  ATS75=0x800

in the init-string and Blaster will send a maximum  of  2k  byte

frames.

Anyone who cannot accomplish  a  connect  with  an  ISDN-Blaster

should try V.110. He should then write to the owner to  set  his

B2 framelength on sensible values via ATS75=0x800, otherwise  he

is only compatible to other ISDN Blaster  boards  -  but  incom-

patible to the rest of the world.

If CRC errors often occur while  sending  the  reason  for  this

might be that the counterpart receives with a windowsize of only

1. In this case you can also decrease the windowsize to  1  with

'cFos' by ATS26 = 1. But most boards can handle a windowsize  of

2 or more.

Therefore we recommend a windowsize of 2.  Stonage  Telecommuni-

cation hopefully does not reconquer FidoNet.

For the  present  time  ISDN  does  anything  else  but  working

properly with FidoNet. One reason is that a mutual ISDN  B2  and

B3 protocol standard has not been  agreed  upon  yet.  When  two

boards with different  protocol  parameters  connect  sooner  or

later transfer problems and aborted connections occur.

Sooner or later an agreement is necessary what  the  ISDN  flags

in the nodelist should exactly mean.

Here is our proposal (and 'cFos' default setup)

B2-Protokoll    : X.75      (of course :-) )

B2-Framelength  : 2048      (see explanation above)

Link-Address A  : 3         (CAPI default)

Link-Address B  : 1         (CAPI default)

Modulo Mode     : 8         (CAPI default)

Windowsize      : 2         (see explanation above)

higher values consume too much

memory at 2048 framesize

B3-Protokoll    : transparent, this means none.

We think that this might be the best chance for a good transfer.

(*) With active ISDN boards the  confirmation  can  be  signaled

sooner.

{+} 11. 'cFos' as a Multiport FOSSIL:

Theoretically 'cFos' can support up to 255 different  ports.  We

would like see a  computer  with  such  a  performance  and  the

adequate ISDN equipment (8 S2M-Busses :-).  This  'cFos'-Version

is compiled for up to  4  COM  ports.  Buffer  memory  and  data

storage  memory  will  be  reserved  for  each  COM  port   when

installing. On installation  you  can  select  -c0  and  -c1  to

support COM1 and COM2, for  example.  There  are  then  2  modem

emulators accordingly. At the same time you can either call  two

different systems or you can be called by  one  or  two  systems

and/or answer a  ring  on  the  other  port.  But  this  feature

requires either software that can operate multiple ports,  or  a

multitasker.

In multiport mode all "modems" listen to a RING  with  automatic

ringdown from the first port specified by the  -c  parameter  to

the next ports specified.

{++}12. 'cFos' Channel Bundling (CCB):

Static Channel Bundling:

'cFos' channel bundling was designed to operate independantly of

any respective CAPI producer/manufacturer. Every 'cFos' user can

do channel bundling with every other user of 'cFos', even though

they use different ISDN hardware. 'cFos' channel bundling is not

a protocol, it is an operation mode. You can either bundle  with

X.75 or with any other packet oriented protocol.

CCB requires that the  ISDN  equipment  can  operate  several  B

channels simultaneously with the  service  "data  transmission".

Following cards fulfil the requirement: TELES, AVM (but not  the

old A1 board, ELSA (starting with CAPI 1.43).  Stollmann  boards

TINA DS and TINA D (with the exeption of TINA DD)  are  not  fit

for CCB because one of the two B channels is used  for  the  A/B

Adapter on a hardware basis.

A further requirement is  that  'cFos'  is  loaded  for  several

ports. That is the  case  for  example  when  it  is  loaded  in

multiport mode, that means when several ports are  activated  by

using the parameter -c.

If 'cFos'is supposed to  support  only  one  COM  port  -aX  can

activate so called auxiliary  ports.  'cFos'  uses  these  ports

internally, but they cannot be accessed from outside for example

by INT 14h calls. Parameter  X  indicates  on  which  controller

(ISDN board) the according B channel ought  to  run.  'cFos'  is

able to carry  out  channel  bundling  on  several  ISDN  boards

simultaneously provided that CAPI supports this.

Examples:

cfos i -c0 -c1      'cFos' is loaded in multiport  mode.  'cFos'

supports the ports COM1  and  COM2.  CCB  is

possible with 2 channels.

cfos i -c2 -a0      'cFos' only  supports  COM3.  CCB  is  still

possible with 2 channels by using  a  second

(auxiliary) channel on  the  ISDN  board  0.

This will be most often the case.

cfos i -c0 -c2 -a1  'cFos'  supports  COM1  and  COM3.  CCB   is

possible with 2 or 3 channels. If  you  want

to bundle with only 2 channels  'cFos'  uses

the "main  port"  (COM1  or  COM3)  and  the

auxiliary port. Only in case that 3 channels

ought to be bundled the second main port  is

needed provided it is not used at  the  time

the connection is established.

Unfortunately things have became a little complicated  but  this

enables that channel bundling and multiport mode do not  exclude

each other. Any combination of both of them is possible.

Basically 'cFos' prefers to select auxiliary ports  rather  than

main ports. Only in case all auxiliary  ports  are  busy  'cFos'

uses additional main ports for channel bundling. If 'cFos'  uses

an additional main port for channel bundling the modem  emulater

will always send the message 'OK' on all  modem  commands.  This

could be the case with 2 mailer tasks  run  under  DesQView.  No

modem command will be executed but the mailer still  knows  that

its "modem" is still there.

The establishing of a connection for outgoing or incoming  calls

while doing channel bundling, especially the choice of  transfer

protocols, does not differ from the  establishing  of  only  one

channel.  Auxiliary  ports  share  the  same  profile   as   the

corresponding main channel with the exeption of  the  controller

bytes (register S40). This will be determined by  the  value  of

the -a parameter.

The modem command AT&Bn determines how many channels  should  be

used for CCB. The ATD <nummer> command builds up  the  channels.

If you try to use more channels than made  available  at  'cFos'

load time 'cFos' responds with the message 'ERROR'. If there are

enough channels but some are in use for  other  purposes  'cFos'

will execute CCB with the channels that are available. The  same

procedure applies to incoming calls. When calls  are  coming  in

'cFos' checks whether there is  a  connect  for  the  respective

caller id and EAZ/SI/AddSI, and, if possible, switches to bundle

mode.  If  you  call  'cFos'  with  the   same   caller   ID/EAZ

simultaneously and twice 'cFos' will asume CCB. Therefore 'cFos'

requires that the caller transfers his Caller ID.

If you'd like to test now, here is a quicky:

cfos i -a0                     ; load cfos with defaults for

; COM1 + 1 Aux.Port

start you terminal software for COM1

AT &F &B2 D +49-228-9111041    ; call Zaphods BBS

^

Insert your int'l prefix here

There is no specific CONNECT string for channel bundling because

'cFos' does not have any information about  the  amount  of  the

bundled channels at the moment it issues the connect message.

CCB is possible with up to 4 channels. On customers  request  we

can compile special versions with more than 4 channels.

CCB is  also  possible  with  several  S0  busses  of  different

telephone numbers. As this feature might be of any interest only

for a few users see for further informatin appendix F.

Dynamic Channel Bundling:

With 'cFos' dynamic channel bundling it is possible  to  connect

or disconnect B-channels with respect the the current thoughput.

If the throughput is below a certain  limit  you  can  save  the

additional costs of a further  B-channels  not  necessary  then.

Otherwise, if you call with only one B-channel  another  channel

will be automatically  connected  if  the  thoughput  reaches  a

certain  limit.  In  oder  to  use  the  current  charging  unit

optimally channels are disconnected  a  few  seconds  before  it

ends. As with static channel  bundling  the  maximum  number  of

channels to use is selected by the AT&Bn modem  command.  Static

channel bundling is selected by AT&M0 (which  is  the  default),

while dynamic channel bundling is selected by AT&M1. In  dynamic

mode 'cFos' connects one B-channel only on an ATD... command and

estabishes further channels during the connection. The following

modem S registers allow you to  control  the  behaviour  of  the

dynamic channel bundling for  maximum  throughput  with  minimal

phone costs:

S64 = Lower limit in cps per channel. If the throughput  of  a

connection falls below this  limit  a  channel  will  be

disconnected with the end of the current charging unit.

S65 = Upper limit in cps per channel. If the throughput  of  a

connection exceeds this limit a further channel will  be

connected.

We recommend a difference of at least 1000 cps in  the  values

of both registers. 'cFos' has a 'CPS-O-METER'  in  its  status

line, so that you can monitor the throughput  of  the  last  8

seconds.

The following modem registers  are  provided  for  further  fine

tuning purposes:

S67 = Pre-off time in seconds. A channel is only disconnected,

when the remaining seconds of the current charging  unit

of this channel  are  less  than  the  seconds  in  this

register. The default value of 5 for example means  that

a channel will be disconnected 5 seconds before  end  of

the current charging unit. 5 seconds is also the minimum

time.

S66 = Charging unit simulation time  in  seconds.  Some  users

don't have the ISDN network provide charging information

during a connection. In  this  case  no  B-channels  are

disconnected, since  the  current  charging  unit  never

ends.  The  users  can  let  'cFos'  generate   charging

information every n seconds, where n can be specified in

this register. 0 disables this feature.

S68 = Dynamic bundling tries. This  registers  determines  the

number of times 'cFos' tries to  connect  an  additional

B-channel. Default is 1.

S69 = Delay  (in  seconds)  before  'cFos'  tries  to  connect

another channel. Default is 0.

In addition to that an outgoing call can be configured  to  be

time- or charge limited. This can be selected for  main  ports

and auxiliary ports independantly. Registers S60-S63 are  used

for this purpose.

S60 = charge limit for main port.

S61 = charge limit for aux. ports.

S62 = time limit in seconds for main port.

S63 = time limit in seconds for aux. ports.

Time and charge limits can also be configured when no  channel

bundling is used. They operate independant of each  other,  so

that you can specify a time and a charge  limit  at  the  same

time. Default is 0 for these registers,  which  disables  this

feature.  Clearly  disconnection  of  the  main  port/channels

results in disconnecting all respective auxiliary ports

'cFos' Channel Bundling example:

You are a mailbox user and after log in you first check all

new files and tag those, you want to download. You then can

use the following 'cFos' initialization:

AT&B2 &M1 S64=5000 S65=7000

Which results in tagging the new files with only one channel

but, when the bandwith is needed, downloading them with two,

getting more than 15000 cps!

WARNING: 'cFos' Channel Bundling (CCB) of Version 1.01a is  not

compatible with that of  earlier  versions,  since  we  had  to

change a few things to get dynamic channel bundling to work. The

newer version can deal with  CCB  of  the  older  versions,  but

dynamic CCB works only if you call a newer 'cFos' version.

13. 'cFos' Compatibility with Current Software

We tested  'cFos'  with  a  variety  of  applications  and  ISDN

hardware and software. A few programs could lead to difficulties

with the high baud rates (most programs offer a  signed  int,  a

value of 32767 reaches the upper end of the scale.)

FOSSIL spec. suggests to test the  ability  of  your  FOSSIL  by

running it with current software.

Some programs have no fast FOSSIL support,  especially  if  they

receive data by using the get_status and receive_char  functions

instead of using the receive_block function. This way of dealing

with FOSSIL produces at least 2 INTs and 2 IRETs  per  character

(sums up to 2000 clocks working with a 386 class  computer  with

QEMM or EMM386) neglecting all other  call  overhead.  We  tried

hard to enable a data transfer with a  decent  speed  even  with

such an application. But such a FOSSIL support does not allow  a

data transmission with a good transfer rate  on  slow  computers

(especially 386 class computers with memory managers).

It is task of the programmers of the terminal software  to  look

for better ways to support FOSSIL. In other  words  to  transfer

datas in bigger blocks  with  receive_block  and  transmit_block

instead of sending byte by byte.

Many FOSSIL supporting software needs to know a baud  rate  with

which the software is able to communicate  with  the  modem  via

RS232 serial port. This baud rate is also required to lock  this

port on. This baud rate is very import  for  communicatiom  with

the modem but for our solution  of  no  importance  at  all  (as

without RS232 and external modem). You should write a baud  rate

of 38400 or 19200 in those fields to stay on the save side.

FrontDoor

FidoNet-Mailer written by Absolute  Solutions  (JoHo).  Mailer

and terminal programm run well with FOSSIL. That is the reason

why this programm still achieves transfer rates of  more  than

7300 cps even with slow computers (386  DX  with  6  mhz)  and

passive ISDN boards. FD on fast computers is one of  the  best

terminal programs we could find (at least concerning  transfer

speed).

Problem 1: Peculiar about FD is that  this  programm  is  only

able to recognize "canned" CONNECT strings. (In FD versions  <

2.11 there is no connect string for  64000,  but  for  38400.)

Therefore you should simply use another unused connect string.

That leads to the result that FD  miscalculates  the  transfer

time estimations, but at least it works.

Solution 1: a) Using FD 2.20/c or FD 2.11/nc the problem  does

not arise. b) You can use other CONNECT messages for  the  new

bps rates or c) If you do not  use  a  FD  >=  2.11  then  set

ATS9.1 = 0 so that the "CONNECT 9600" message always  appears.

For logging purposes 'cFos' appends the  B2  protocol  to  the

connect string, thus you get CONNECT 9600/X75. This works with

FD.

Problem 2: The Caller ID delivered by ISDN. 'cFos' can display

the caller id behind the "RING" message.

Solution 2: If you configured the "RING" message  with  FD  to

"RING|" to avoid the  "RING"/"RINGING"  confusion  you  should

disable this feature by ATS9.2=1. Otherwise FD cannot detect a

"RING". You  may  also  enable  a  2-line  RING  message  with

ATS10.3=1.

FrontDoor requires about 250 kb memory. So you should not  run

into memory problems (even though using DesQView).

InterMail

As with FrontDoor, 'cFos' runs well with InterMail. The  usage

of memory is similar, too.

BinkleyTerm

Binkley uses very fast  and  skillfully  written  routines  to

access a FOSSIL. Even on slow computers  high  transfer  rates

can be achieved.

Problem: Binkley 2.50 (and 2.50 EE up to Beta  D  incl.)  uses

(as far as we know) a signed int for baud rates.  That  causes

errors with a CONNECT of 64000 bps.

Solution: An ATS9.1 = 1 always leads to a report of  the  baud

rate of 9600.  Then  they  work  properly  together.  But  the

timings are wrong.

Binkley requires about 300 kb of memory.  No  memory  problems

should occur either.

D'Bridge

D'Bridge (DB) is a mailer written  by  Chris  Irwin  from  the

sunny Miami. To get 'cFos'  to  run  with  DB  select  CONFIG,

Comm/Modem Setup in the menu. You find there a  setup  screen.

You have to use one of the DATA/1 ... DATA/3  lines  to  enter

"CFOS" in the column "MCF name".

Then you also have to create a file called  CFOS.MCF,  located

in  the  DB  system-directory  with  at  least  the  following

contents:

MCF CFOS ISDN-board + cFos

BAUD 64000

LOCKED

DELAY 0

INIT ATZ

OFFHOOK ATH1

ANSWER ATA

DIAL 300 ATD

DIAL 19200 ATD

DIAL 38400 ATD

DIAL 64000 ATD

TRANSLATE 9600 CONNECT 9600

TRANSLATE 38400 CONNECT 38400

TRANSLATE 64000 CONNECT 64000

The CONNECT strings of 'cFos' must exactly match with  one  of

the above strings. Otherwise DB reports  a  baud  rate  of  0.

Therefore you should use ATS9.4=0 to suppress  the  "/X75"  in

the connect string. If you know another solution for DB please

mail us.

We recommend to use DB  version  1.54  or  higher,  since  the

author improved the FOSSIL support in that version.

Yuppie!

Yuppie is a 3d  pointprogram.  We  tested  version  2.10.  Its

transfer routines are based on  Binkley.  Therefore  it  works

fairly well. But it  cannot  handle  a  'CONNECT  64000'.  Set

"ATS9.1=0" so that it recognizes that a  call  was  succesful.

Yuppie requires much memory  (it's  written  in  CLIPPER),  if

possible load CAPI and/or FOSSIL into UMBs.

Portal of Power

PoP when sending, stops for a short amount of time  after  the

second file out of  unknown  reasons.  It  is  not  caused  by

'cFos', and is no reason to worry.

You have to watch not to load X00  automatically.  In  case  a

change of POP.BAT file may help.

At least version 0.61 cannot handle incoming calls. But is not

due to 'cFos' because PoP has the  same  problems  with  other

hardware and software. As far as we know,  newer  versions  of

PoP run quite well with 'cFos'.

McMail

A new Mailer from Germany with some  nice  features  and  good

'cFos' ISDN appendage support (e.g.  it  gets  the  real  ISDN

charges from the FOSSIL  and  you  may  interactively  add  or

remove channels in CCB mode).

CrossPoint

Starting with version  2.92, XP  fully  supports  FOSSIL,  for

example the termial program etc.  Just  switch  on  the  topic

FOSSIL in the communication/modem menu. Earlier versions of XP

had only a partial FOSSIL support.

Maximus

Maximus is a BBS program written by Scott J. Dudley. As we use

this program we tested it thoroughly.

It is possible that the Maximus WFC Interface and SpawnBBS  do

not like baudrates higher that 32767.

Menus and textfiles are sent character by character. Therefore

ISDN cannot reach its highest speed. But that does not  matter

much. When downloading Maximus sends datas in 128 byte  blocks

to the FOSSIL. This way it achieves good transfer rates.

Unfortunately  Maximus  does  not  use   the   receive_block()

function of FOSSIL for receiving data. Maximus rather  accepts

byte by byte with the function receive_char(). That leads to a

huge  overhead.  Slow  computers  cannot  reach  the   highest

transfer rates possible.

As we wrote the functions receive_char() and  get_status()  in

assembler we obtained an acceptable throughput.  Maximus  will

always be faster downloading than uploading.

RemoteAcces

RemoteAcces is BBS software  written  by  Andrew  Milner.  Our

tests proved that RemoteAcces runs well without any  problems.

RemoteAcces can handle transfer rates of more than  7300  cps.

It is able to accept calls  independently.  At  least  version

2.00 should not have any problems handling baud rates of 38400

or 64000.

Unfortunately RemoteAcces can  only  accept  "canned"  CONNECT

strings. They need to be set in RACONFIG. Version 2.00  offers

all strings necessary for ISDN. Only the RING string has to be

set to "RING" so that "RING CallerID" will  not  be  mixed  up

with "RINGING".

PCBoard

A PCBoard/M version is required to support FOSSIL. 'cFos' runs

without any bigger problems with this version. But you have to

set "ATS9.4=0" to limit the CONNECT message to 'CONNECT <bps>'

because PCBoard uses the last number of the CONNECT string  as

a baud rate. In addition you  can  not  use  the  'cFos'  BIOS

emulator (-e2) with 'cFos'. In case of problems try using  the

-ji switch.

Wildcat 4.0

Wildcat works with 'cFos' if FOSSIL support is  activated  and

the baudrate is set to 38400. Unfortunately the  interal  file

transfer protocols are not the fastest. We  recommend  to  use

external protocols.

Terminate

Terminate is a fairly recent programm written by  Bo  Bendtsen

from Denmark. It  offers  a  great  variety  of  features  and

functions, for example FOSSIL support  (as  well).  We  tested

version 1.41 to 1.50.

Unfortunately  Terminate  uses  only  the  FOSSIL's  functions

receive_char and transmit_char. Therefore  transfer  rates  on

slow computers could be much below the maximum of 8000 cps. In

version 1.50 they improved the use of FOSSIL functions, so  we

recommend version 1.50.

Terminate requires 300 kb memory. The author  worked  hard  to

reduce memory consumption.

TeleMate

TeleMate is one of the most  famous  and  widespread  terminal

programs on the PC market. TeleMate supports FOSSIL for  quite

some  time.  TeleMate  offers  the  very  special  feature  of

internal multitasking, that means you can download a file  and

edit a text at the same time. But multitasking consumes a vast

amount of transfer speed if your computer is not fast  enough.

A 386 DX is  at  least  required  to  keep  up  an  acceptable

transfer speed.

Telemate  requires  430  kb  memory  which  is  quite  a  lot.

Therefore try to do  without  as  many  drivers  and  resident

programs as possible.

Important: If you configure TeleMate  to  support  FOSSIL  the

baud rate *must not* exceed 38400.  Otherwise  TeleMate  would

not accept FOSSIL support. If 'cFos' does not  respond  to  AT

commands, please check if FOSSIL support is still enabled.

HS/Link

HS/Link deinitializes the COM-port after use and some programs

will barf at that (e.g. the FrontDoor Terminal).  Load  'cFos'

with the -ji switch or issue a 'CFOS INIT:<portnb>' command to

re-init the port. Warning: HS/Link will eventually  disconnect

your connection if 'cFos' is loaded with the -jk switch.

XBTX

XBTX is a BTX decoder written  by  Juergen  Buchmueller.  XBTX

supports FOSSIL starting with version 1.50. As  you  can  also

run BTX via ISDN, and the Datex-J ports of German TELEKOM  are

capable of ISDN, we added BTX support to 'cFos' (ATB5).

XBTX checks the ports 0-7 for a the presence of the X00 FOSSIL

only. Therefore you need the the  -jx  switch  to  let  'cFos'

pretend it were X00.

XBTX requires 490 kb. Therefore all TSRs should be reduced  to

an absolute minimum necessary.

DoorWay

It works. But it is strongly recommended  to  switch  off  the

'cFos' status line. Otherwise DoorWay will always transmit the

changes of the status line (it does not cause any problems but

it bothers).

MS-Kermit

You have to switch Kermit in INT 14  mode  (for  example  with

"SET PORT  BIOS1").  Then  use  "C"  to  access  'cFos'.  When

problems arise a "SET BELL OFF"  often  helped  (nobody  knows

why).

pcANYWHERE for DOS

Configure pcANYWHERE for  DOS  to  INT14,  set  the  Modem  to

{+}       "manual settings" and set the Initstring to "ATZS10.4=1".  The

hangup string MUST be exactly "+++ATH0". If you  do  this,  AW

will run fine and Zmodem  transfers  will  run  at  7500  cps.

'cFos' can be  loaded  with  "cfos  i".  If  pcANYWHERE  modem

settings requires some other string use the  AT&H  command  to

change the string to the desired form (see MODEM.DOC).

PROCOMM PLUS for Windows

Set the Port to INT14 in the "Setup" Menu (ALT-S), "Advanced",

"Port".

DesQView

We tested 'cFos' working with DesQView as a MultiPort  FOSSIL.

You have to load  'cFos'  BEFORE  DesQView  with  serveral  -c

parameters  so  that  several  tasks  can   access   different

"modems".

Running  DesQView  we  called  one  FrontDoor   from   another

FrontDoor. We achieved transfer rates of more  than  7500  cps

(386DX-40).

MS-Windows

If you want to run 'cFos' with MS-Windows  you  have  to  load

'cFos' prior to  MS-Windows.  Some  Windows  programs  do  not

support FOSSIL but they can use INT  14h.  If  so  'cFos'  can

operate with them.

OS/2 DOS-Box

As far as we tested, 'cFos' should now run OK in the OS/2  DOS

Box. The allocation of DOS UMB's is turned off,  because  OS/2

behaves  strangely  in  this  matter  ('cFos'  loads  with  an

effective commandline of  cfos  i  -jo  -jg  -ju  if  OS/2  is

detected).

See Appendix H for a compatibility list.

14. ISDN Hardware/Software

'cFos'is based on CAPI which is an official German standard ISDN

interface.  But  there  are  still  some  questions  unanswered.

Therefore it is necessary to test an application with the  given

hardware.

Here we present a report of our experience (and other  users' on

the subject of ISDN boards):

TELES

Creatix S0

'cFos' was developed and tested  on  a  TELES.S0  ISDN  board.

TELES GmbH, Berlin kindly supported  and  still  supports  the

development 'cFos' by their developers of their CAPI.

We run our own mailbox (Zaphods BBS) on a TELES.S0 board  with

'cFos'. We always try to supply the latest  version  of  TELES

CAPI.

'cFos' recognizes TELES CAPI and checks  whether  the  modules

are loaded for V.110 or bundle protocol. 'cFos'  then  enables

the use of either  V110  or  channel  bundling  and  therefore

ATB1...ATB3.

AVM

'cFos' runs well on both the passive AVM A1+  and  the  active

AVM B1. The rather abnormal behaviout of the X.75 ISDN Blaster

board could cause problems. Meanwhile AVM has a new version of

their drivers to solve the problems  when  calling  a  Blaster

with an AVM board (version 2.07-10 or higher). If you want  to

call a Blaster that uses FOSSIL PCIF V5.78 it  is  recommended

to set 'cFos' -jp flag (for further details see chapter 3).

In parts AVM CAPI does not include V.110. If it  is  the  case

the error message "ERROR/B2" will be displayed after a connect

message when you try to call. A new version of AVM CAPI  fixed

the problem. But there are still old A1  boards  that  lack  a

V.110 driver.

The AVM B1 (with an additional module by AVM) can be  used  to

plcae outbound  analogue  2400bps,  V.22bis  calls  using  the

following  init-string:  ATB0S20=0x81S27=0xa3S16=1S17=1.   You

should run at least CAPI 2.09 to do this.

CPV Stollmann

'cFos' runs well on the active boards Tina D and Tina DS. Even

on slow computers 'cFos' is fast.

Stollmann CAPI cannot execute V.110 until  now  but  Stollmann

anounced to work it out soon.

Our  mailbox  offers  the  latest  versions  of  drivers   for

Stollmann boards (TINACAPI.ZIP and TINAETSI.ZIP). We recommend

to use them by all means.  And  we  recommend  to  follow  the

instructions for executing a call  with  TICAPI  -b  to  avoid

problems with ISDN Blaster when placing calls.

There is a COM-port emulation that unfortunateley when  loaded

prevents that 'cFos' succesfully registers in CAPI.  When  you

want to use 'cFos' make sure that this module is  not  loaded.

CPV Stollmann kindly made us a TINA DS available for  testing.

Therefore 'cFos' should  work  with  this  board  without  any

problems.

{++}      Unfortunately channel bundling is not possible with a  TINA  D

or a TINA DS board (but it functions with a TINA  DD)  because

the second  B-channel  is  connected  to  the  integrated  A/B

adapter on a hardware basis and therefore is not available for

CAPI.

mbp SOLIS

SOLIS boards produced by the company mbp work well with 'cFos'

when you follow certain instructions:

Set switch -jr when loading 'cFos'.

Solis supplies a COM port emulation. We recommend to switch it

off. This COM emulator listens to EAZ 9 by default.  This  can

be changed with Util ICOMONF. The COM emulator and 'cFos'  may

not listen to the same EAZs at the same time.

We suggest to use the latest mbp  CAPI  version  in  any  case

because older ones might cause problems.

ITK iX1

iX1 does not allow a registration with  a  windowsize  smaller

than 2 ('cFos' default = 2). 'cFos' works well with iX1.

The iX1 may be switched to do outgoing  analogue  modem  calls

with up to 14.400 bps, but without error correction using  the

following init-string: ATB0S20=12S16=1S17=1.

ELSA Microlink ISDN/PC, ISDN/PCC

ELSA  kindly  made  us  available  an  ISDN/PC   plug-in-board

including CAPI 1.43 for testing.

'cFos' runs well with this configuration. ELSA CAPI needs only

few memory and is fast. Unfortunately it is not (yet) able  to

execute V.110. But ISDN/PC offers a COM-port with a 16550 "for

compensation".

{++}      'cFos' can execute channel bundling starting  with  ELSA  CAPI

version 1.43 .

Sedlbauer S0-BOX and S0-plug-in board

The S0-BOX is an external equipment that needs to  be  plugged

on the printer port. All data between ISDN  and  the  computer

are exchanged via printer port. For the  presence  the  S0-BOX

can operate only one channel. Sedlbauer  announced  to  change

that. Therefore channel bundling will be possible in the  near

future.

The S0-BOX only supports windowsize 7 so far.  Therefore  load

'cFos' with "cfos i -w7" and set ATS26=7.

When you observe our recommendations the S0-BOX will run  well

with 'cFos'. All said about the S0-Box also applies to the  S0

plug-in board. We thank  Sedlbaur  for  making  us  an  S0-BOX

available for testing.

Diehl Diva

Diva is an active ISDN board. 'cFos' runs fast on Diva  boards

especially on slow computers. In the near future the  "at  run

time" loadable firmware will  also  support  V.110.  We  thank

Diehl for making as a Diva board available.

According to the information of other 'cFos' users 'cFos' also

works well with the Diehl SX, SY and Diehl SCOM board.

Dr. Neuhaus

Dr. Neuhaus kindly tested 'cFos' with their  ISDN  Boards  and

reported that is runs at least with their passive NICCY  1000,

1016 and active 3008, 3009 and  5000  CTX  boards.  Since  all

these boards have two b-channels, you can run CCB with them.

KSI SA-1

Digiteam Digiboard

As far as we tested it,  'cFos'  is  running  with  the  SA-1.

Thanks to KSI for supplying us with a  test  board.  The  SA-1

CAPI uses 180kb of memory,  so  you  should  run  'cFos'  with

smallest buffers possible. It seems not to support V.110.

NCP ISDN P-8 / P-16

'cFos' runs well on NCP cards with the HST CAPI. Thanks to HST

for sending us a baord for testing purposes. The card features

an extremely small CAPI with optional V.110 and ISO  8208  and

is able to run multiple channels at the same time.

IBM PCMCIA Active 2000

IBM Employees told us that this board runs well with 'cFos'.

Other Boards

If problems arise  with  ISDN  hardware/boards  not  mentioned

above check first if there  is  a  more  recent  CAPI  version

available. If so try it first. Otherwise send mail to us.

We  would  like  to  take  the  chance  to  say  thanks  to  the

manufacturers of  ISDN  boards  for  the  good  cooperation,  we

especially like to mention TELES, but also CVP/Stollmann,  ELSA,

Diehl and Sedlbauer.

15. Miscellaneous:

- Disconnect Reasons 'cFos' reports to the remote party:

0x00 : normal disconnect (also 0x80)

0xbe : call rejected

0xbb : user busy

0xb9 : out of order

The disconnect reason arrives at the remote  party  as  0x34??

cause. However  some  causes  are  filtert/translated  by  the

telephone switching equipment.

- Service Indicator (SI) not equal 7

If you enable  other  services  than  "data  transmission"  by

register S14, 'cFos' issues a "CONNECT VOICE" message, if  the

service  indicator  =  1   and   selects   the   B2   protocol

"bittransparent".  This  can  be  used  by  applications   for

"telephony".

If SI <> 7 and SI <> 1, 'cFos' issues a  "CONNECT  ?"  message

and selects the protocols given by registers S20-S36.

If you place a call, you can choose the service  indicator  by

setting register S16 in the same way you choose the additional

service indicator as  well  as  the  B2  and  B3  protocol  by

registers S17, S20 and S21.

- For further study

Here is some literature  (including  programs)  references  to

anyone who is further interested in the subject of ISDN:

CAPI Dokumentation

COMMON-ISDN-API,   Einheitliche    Schnittstelle    zwischen

Applikationsprogrammen  und  ISDN-Adaptern,   Spezifikation,

Version 1.1, Profil A, Editorisches Datum: 07.09.90

e.g. at Zaphods BBS as ISDNAPI.ZIP, 24k (german)

or CAPI_ENG.ZIP, 20k (english)

or CAPI11.ZIP, 62k (german, WORD-Format)

1.TR.6 Dokumentation

The 1.TR.6  documentation  is  available  at  the  following

address:

DBP Telekom

FA Bad Kreuznach

Projekt Roland

Arbeitskreis CAPI/PCI

z. Hd. Herrn Kreuzer

Postfach 9100

W-6550 Bad Kreuznach

Germany

PAPI Source

PAPI is an ISDN paket driver for TCP/IP, which uses CAPI.

e.g. at Zaphods BBS as PAPI*.ZIP, about 38k each

If you are interested in the FOSSIL specs, there is

FSC-0015 available.

THE original FOSSIL Doc by Rick Moore.  As  FSC-0015.A??  in

every good FIDO BBS available. 25k.

X00REF.DOC

The function refence  by  Ray  Gwinn  for  FOSSIL  developer

contains many good and important comments.

e.g. at Zaphods BBS as X00150.ZIP, 105k.

- If you should ever use the "V.110 inband negotiation", you get

a "CONNECT 9600" message from 'cFos',  since  'cFos'  can  not

know which baudrate was actually negotiated.

16. Addresses, Authors, Availability:

- License Terms

see our  license in COPYING.CF.

- Authors

Christoph Lueders      Martin Winkler

Fidonet:        2:2453/30.1            2:2453/30.6

Internet:       chris@rhein.de         winkler@zaphod.rhein.de

Surface Mail:   Reuterstr. 133         Dorotheenstr. 38

53113 Bonn             53111 Bonn

Germany                Germany

.--  Voice:                                 +49-228-650389

|

|

|    Voice phone calls:

|

|--  Phone us for commercial purposes only, please.

|

{++} |    Users of  'cFos/Pro'  are  provided  phone  support  by  their

|    respective  suppliers/vendors.  Please  don't  ask   questions

|    treated in this documentation.

|

|    We also started a Fido echomail conference, called  CFOS_HELP.

|    In addition you can write a mail to SysOp at Zaphods BBS (ISDN

|    0228-9111041, V32b  0228-262894).  This  can  often  help  you

|    faster than calling somebody voice.

|

`->  Before support can be given, we'd appreciate  that the  caller

has carefully read

CFOS.ENG, WHATSNEW, MODEM.DOC, CFOS.FAQ

and already experimented with 'cFos'.

You can call us, if you use ISDN equipment  not  mentioned  in

this documentation.

If you have problems with the configuration of  your  applica-

tion software, it  is  always  a  good  idea  to  contact  the

respective "Help-Sites".

Mail/Bug-Reports:

We are always interested in

- obvious bugs in 'cFos' or the documentation.

- experiences with software or ISDN hardware unkown to us.

- suggestions of features to include in 'cFos'.

- What else can be done with 'cFos'.

Interesting ISDN phone  numbers,  for  example,  are  always

welcome.

A Mail to us should at least contain:

- your name and (if it's a bug report  or  you  have  an  ISDN

board not mentioned in the documentation) your voice  number

so that we can call back (and  the  hours  you're  reachable

under that number).

- The version number of 'cFos'.

- Used Computer, Software, CAPI driver and ISDN board,  please

always include their version numbers.

- New Versions:

The most recent version of 'cFos' is always available  at  our

own BBS, called Zaphods BBS, in Bonn, Germany. In addition  we

try to spread the 'cFos' archive as fast as possible via  Fido

BBSes, Internet servers and MAUS BBSes.

For future versions we plan

- to improve the readability of the ATIn displays.

- to allow AT&E commands for each individual port.

- to support Ralf Brown's AMISL Alternate Multiplex  Interrupt

Specification for resident programs.

{++}      - to include serveral  logging  features  to  evalutate  phone

costs, incoming calls, etc.

17. Credits:

The order does not imply any preference ;-)

Andreas Illg, Alexander Bell, Eberhard Mattes,  Dietmar  Friede,

Uwe  Engelmann,  Scott  J.  Dudley,  Robert   Bergermann,   Jens

Osterwohldt, Markus Kessler, Olaf Droege, Tobias  Erichsen,  Jan

Ceuleers, Kalle Braun, Roland  Steinmeyer,  Oliver  von  Bueren,

Rainer Schuetze, Thorsten Winkler, Manfred Recla.

18. End of Documentation;  Thanx for using 'cFos'.

Practice random kindness and senseless acts of beauty!

----------------------------------------------------------------

APPENDICES

----------------------------------------------------------------

A. CAPI Error messages:

Here is a list of the CAPI error messages of  the  "Arbeitskreis

CAPI", extended by  V.110  error  messages,  1.TR.6  causes  and

manufacturer specific messages:

0x0000: No error

0x1001: Error on API_REGISTER

0x1002: Illegal application-id

0x1003: Illegal message

0x1004: Illegal command or subcommand

0x1005: Queue is full

0x1006: Queue is empty

0x1007: Queue overflow

0x1008: Deinstall error

0x1009: Windows address error

0x2001: Illegal Controller

0x2002: Illegal PLCI

0x2003: Illegal NCCI

0x2004: Illegal type

0x3101: B-channel erroneous

0x3102: Infomask erroneous

0x3103: Serviced-EAZ-mask erroneous

0x3104: Serviced-SI-mask erroneous

0x3105: Illegal B2 protocol

0x3106: Illegal DLPD

0x3107: Illegal B3 protocol

0x3108: Illegal NCPD

0x3109: Illegal NCPI

0x310A: Illegal flags

0x3201: General controller error

0x3202: non-unique LISTEN_REQs

0x3203: function not supported

0x3204: PLCI inactive

0x3205: NCCI inactive

0x3206: B2 protocol not supported

0x3207: can't select B2 protocol now

0x3208: B3 protocol not supported

0x3209: can't select B3 protocol now

0x320A: illegal DLPD parameters

0x320B: illegal NCPD parameters

0x320C: illegal NCPI parameters

0x320D: data length not supported

0x3301: D channel layer 1 setup error

0x3302: D channel layer 2 setup error

0x3303: B channel layer 1 setup error

0x3304: B channel layer 2 setup error

0x3305: D channel layer 1 shutdown

0x3306: D channel layer 2 shutdown

0x3307: D channel layer 3 shutdown

0x3308: B channel layer 1 shutdown

0x3309: B channel layer 2 shutdown

0x330A: B channel layer 3 shutdown

0x330B: B channel layer 2 reestablished

0x330C: B channel layer 3 reestablished

0x3400: Normal disconnect, no cause given by network

0x3480: Normal disconnect, no cause given by network

0x3481: Invalid CR value

0x3483: Bearer service not implemented

0x3487: Unknown caller identity

0x3488: Call Identity already suspended

0x3489: No channel available

0x348a: No channel available

0x3490: FAC Code unknown in this network

0x3491: requested service rejected

0x34a0: Outgoing calls barred

0x34a1: User access busy

0x34a2: Nonexistent CUG

0x34a3: Nonexistent CUG

0x34A5: Invalid or unknown destination

0x34b5: Destination not obtainable

0x34b8: Number changed

0x34b9: Out of order

0x34ba: No user responding

0x34bb: User busy

0x34bd: Incoming calls barred

0x34be: Call rejected

0x34d8: Invalid destination address

0x34d9: Network congestion

0x34da: Remote user initiated

0x34f0: Local procedure error

0x34f1: Remote procedure error

0x34f2: Remote suspended

0x34f3: Remote not suspended

0x34ff: Local reject of User to User info

0x4001: Stollmann: too many applications

0x4002: Stollmann: block size too large

0x4003: Stollmann: error on init of message queue

0x4004: Stollmann: no PLCI cntl block available

0x40ff: Stollmann: function not allowed in current context

0x4101: Loss of frame-synchronisation

0x4201: Stollmann: can't deinstall, not on top of int chain

0x4202: Stollmann: can't deinstall, application still active

B. Format of the V.110 user rate (S27) and

Additional Service Indicator (S17):

Bit 76

01  Extension for asynchr. transmission

Bit 5                Bit 4               Bit 3

0     8 data bits    0     1 stopbit     0     no parity

1     7 data bits    1     2 stopbits    1     even parity

Bit 210

000   38400 bit/s

11  Asynchr. transmission with bit rate adaption

acc. CCITT V.110

Bit 5                Bit 4               Bit 3

0     8 data bits    0     1 Stopbit     0     no parity

1     7 data bits    1     2 Stopbits    1     even parity

Bit 210   user rate in bit/s

000   1200

001   1200/75

010   75/1200

011   2400

100   4800

101   9600

110   14400

111   19200

10  Synchronous transmission with bit rate adaption acc. CCITT

V.110

Bit 54

10     never change

Bit 3210   User Rate in bit/s

0000   1200

0001   1200/75

0010   75/1200

0011   2400

0100   4800

0101   9600

0110   14400        V.32bis

0111   19200

1000   48000

1001   56000

1010   56000        USA

1111   in band negatiation

C. B2-Frames and windowsizes with X.75:

For all, who still believe 16k byte frames and windowsize 1  are

good for high transfer speeds, we include a few  diagrams  here,

which show the different timing at different frames and  window-

sizes:

The windowsize specifies how many frames may be sent  until  the

sender needs a confirmation before sending further frames.

Windowsize 1, 2k Byte Frames:

sender    -<2kb daten>------<2kb daten>------<2kb ...

receiver  ------------<conf>-----------<conf>---- ...

delay     ------------!!!!!!-----------!!!!!!---- ...

time ----------------------------------------------->

With 1 MB transmitted data  you  thus  have  512  times  a  time

consuming delay. With 16k bytes frames you still have 64 delays.

We see that we cannot achieve the  maximum  CPS  rates  by  this

transmission mode.

Now Windowsize 2 und 2kb Frames:

sender    --<2kb daten1><2kb daten2><2kb daten3><2kb daten4>--

receiver  --------------<conf1>-----<conf2>-----<conf3>-------

delay  ------------------------------------------------------>

delay: None!

A Windowsize of 1 can be  compared  with  data  transmission  of

X-Modem, whereas windowsizes greater 1 are comparable  with  the

full  streamed  data  transmission  of  Z-Modem.  Allthough  the

overhead of Z-Modem is bigger than that of X-Modem, it is  still

faster.

CRC errors during transmission:

In Germany the Telekom garanties  an  error  rate  smaller  than

1:1,000,000 on ISDN phone lines. Thus  the  worst  case  is  one

error per transmitted MB, resulting in one  frame  resend.  With

16k frames the resend costs 2 seconds (at 64000 bps). In case of

an error you therefore lose about 300 CPS per MB - but  with  2k

byte frame only 40 CPS !

D. Connect problems with ISDN Blaster (X.75):

Even if the Blaster is set up to 2k bytes  frames  it  is  still

possible that problems occur  with  Stollmann  and  AVM  boards.

These problems always occur if you place a  call  to  a  Blaster

board - not if you are called by it. The reason is a bug in  the

implementation of the Blaster FOSSIL PCIF 5.78 and 5.80. The bug

however is fixed in PCIF 5.82.

To solve  the  problem  users  of  CPV/Stollmann  boards  should

obtain the most recent CAPI driver from Stollmann  or  from  our

BBS. This driver has the ability to run in LAP B mode (TICAP -b)

and thereby avoiding the problems.

AVM now also provides a driver for all of their ISDN boards with

a work-around for this problem. For PCIF 5.78 you can also  load

'cFos' with the -jp switch. But this does  not  work  with  PCIF

5.80.

In any case you can place calls to a Blaster using V.110, 38400,

async. and then tell the Blaster user to set up the  frame  size

to 2k bytes (by placing ATS75=0x0800 in  the  init  string)  and

install PCIF 5.82.

E. Connect problems to ELINK "Modems":

Some Elink user set up their "modems" in a way  that  can  cause

connect problems or the Elink does not answer calls at  all.  To

solve this problem here we include the Elink settings which make

it compatible with other ISDN hardware/software.

V.110-Subkanal  (S18): 000

Betriebsart   &B16   : 64kbps,sync

Protokollmodus \N6   : X.75

Extended mode  \X0   : aus

SIN unbekannt    \S1 : Ruf annehmen

SIN <> abgehend  \S3 : Ruf annehmen

SIN abgehend  \S5\S7 : herst.spez.

Rate adjust      \J1 : ein

Blocklaenge      \A3 : 256 Bytes

XID-Prozedur         : aus

Please let the owners of  Elinks  know  of  these  settings  and

inform them to use EPROM release 2.12 or higher, since some bugs

were fixed since then.

F. CCB with several S0-Busses of different phone numbers:

outgoing calls:

In order  to  call  several  S0-Busses  with  different  phone

numbers you can specify serveral phone numbers,  seperated  by

':' or '|' in the ATD command. For example ATD 123456:234567.

They are used in the following way:

Channels  Dial         Channel 1 Channel 2 Channel 3 Channel 4

AT&B2     ATD A        A         A         -         -

ATD A:B      A         B         -         -

AT&B3     ATD A        A         A         A         -

ATD A:B      A         A         B         -

ATD A:B:C    A         B         C         -

AT&B4     ATD A        A         A         A         A

ATD A:B      A         A         B         B

ATD A:B:C    A         A         B         C

ATD A:B:C:D  A         B         C         D

Of cause 'A', 'B', 'C', 'D' name phone numbers.

We admit that dialing several number at the  same  time  is  a

rare case. But this provides a way of calling  a  person,  who

offers 4 channels for  CCB  on  two  different  S0-busses,  by

operating two ISDN boards in one computer. Sometimes these two

S0-Busses have different phone numbers.

Incoming calls:

To allow CCB with caller ids which are  similiar,  'cFos'  has

register S49. The value in this registers specifies  how  many

digits in the caller id are ignored (from right to left), when

deciding if this is a CCB call. Default is 0.

Example:

You are called by a user  with  the  S0-busses  of  the  phone

number 12345-01 and 12345-67. If you set S49 to the value of 2

then 'cFos' ignores the digits '01' and  '67'  when  comparing

the caller ids. Notice that the lengths of the caller ids have

to be the same.

However the normal case is that both  S0-busses  have  the  same

phone number, because they  are  connected  by  a  ringdown  (or

similar) circuit.

G. 'cFos' modem error message:

ERROR

The last modem command has an incorrect format or the values

given were out of the allowed range.

ERROR/B2:

The selection  of  the  B2-protocol  resulted  in  an  error

generated by the CAPI driver. In most cases this means  that

the respective CAPI driver does  not  support  the  selected

protocol. (Some older German CAPIs  do  not  support  V.110.

'cFos' then disables V.110 as if a -jv switch was given).

ERROR/B3:

The selection  of  the  B2-protocol  resulted  in  an  error

generated by the CAPI driver. In most cases this means  that

the respective CAPI driver does  not  support  the  selected

protocol.

ERROR/LISTEN:

The selection of the kind  of  calls  the  CAPI/cFos  should

listen was invalid. In this case you should check the values

specified in the S registers for Serviced SI Mask, Info Mask

and Serviced EAZ Mask (Registers S13, S14, S41, S42).

ERROR/CAUSE=xxxx

During call setup the CAPI indicated an error. For a list of

errors refer to appendix A. In many cases this  problem  can

be solved by checking the modem S registers.

H. Software compatibility list:

Among others 'cFos' runs with the following software:

Product               Manufacturer                  Program-Type

-------               ----------                    ------------

FrontDoor             Absolute Solutions,           Fido-Mailer

Joaquim H. Homrighausen

Intermail             Peter Stewart                 Fido-Mailer

BinkleyTerm           Vince Perriello/Bob Hartman   Fido-Mailer

D'Bridge              Chris Irwin                   Fido-Mailer

(*) McMail                Gordian Schuermann/           Fido-Mailer

Albert Freriks

Portal of Power       Soren Ager & The Portal Team  Fido-Mailer

Xenia                 Lentz Software Development    Fido-Mailer

Yuppie! V2.10+        YEAsoft, Aachen               Fido-Point

System

CrossPoint V2.14+     Peter Mandrella               Fido-Point

System

Maximus               Scott J. Dudley               BBS Software

RemoteAccess          Andrew Milner                 BBS Software

PCBoard/M             Clark Development             BBS Software

SuperBBS              Risto Virkkala & Aki Antman   BBS Software

Wildcat 4.0           Mustang Software, Inc.        BBS Software

TeleMate V4.12+       White River Software          Terminal Prg

Terminate V1.40+      Bo Bendtsen, Daenemark        Terminal Prg

Telix INT14           EXIS Inc. / ELSA              Terminal Prg

Unicom                Data Graphics                 Terminal Prg

PCPLUS/Procomm Plus   Datastorm Technologies, Inc   Terminal Prg

MS-Kermit             Trustees of Columbia Univers. Terminal Prg

DN 3270               Lange Communication           Terminal Prg

COMM90                Bausch Datacom                Terminal Prg

VipCOM                Telesoft GmbH                 Terminal Prg

HS/Link               Samuel H. Smith               Transmission

protocol

Engine

CEXYZ                 Cutting Edge Computing        Transmission

protocol

Engine

SDPFL                 Streamline Design             Transmission

protocol

Engine

PC-Anywhere 5.0       Symantec, Inc.                Remotecontrol

Software

DoorWay               Marshall Dudley               Remotecontrol

Software

FernTerm                                            Remotecontrol

Software

OzCIS 2.x             Steve Sneed                   CompuServe

Navigator

OzWIN                 Steve Sneed                   CompuServe

Navigator

DOS-CIM               Compuserve, Inc.              CompuServe

Info.Manager

WIN-CIM               Compuserve, Inc.              CompuServe

Info.Manager

XBTX                  Juergen Buchmueller,          BTX Decoder

Bonn, Germany

(*) CFOSTALK              Michael Grube                 Answering

Machine

Waffle                Darkside International        UUCP BBS Prg

'cFos' is compatible with DesQView, MS-Windows,  PAPI 0.16,  X00,

ISDN-Talk and OS/2.

Thanks to the people who helped us testing the software.

Products marked with an asterik  (*)  support  the  'cFos'  ISDN

appendage (see APPEND.DOC).

I. Availability:

We release 'cFos' as a ZIP archive, but beware that  it  may  be

repacked to some other archive style by other sites.

The original name of the archive is CFOSnnnn.ZIP,  where  'nnnn'

is some kind of version numbering, like CFOS095.ZIP for  version

0.95.

The most recent version of cFos may be obtained at...

Our home BBS :

Zaphods BBS, Bonn/Germany, Sysop Chris Lueders

+49-228-262894   14400bps, HST/V32b   (fido 2:2453/30)

+49-228-9111041  64000bps, X75/V110   (fido 2:2453/33)

German Internet :                             Directory:

FTP.DFV.RWTH-AACHEN.DE [137.226.4.111]    /pub/network/isdn/cfos

We try to keep the following sites up to date:

FIDO.Classic:

Freak Out!, Aachen/Germany, Sysop Alexander Bell

+49-241-604825   19200bps, ZYX/V32b   (fido 2:242/9)

+49-241-6080060  64000bps, X75/V110   (fido 2:242/49)

German CompuServe:

See the TELECOM forum, area Data/ISDN or,

see the GERNET forum, area Telecommunication.

Internet fileservers (Europe):                Directory :

FTP.FUNET.FI                              ?

Europeans: please avoid calling over the Atlantic. Get  your

files from one of the  above  or  some  SIMTEL/Garbo  mirror

site. The network will appreciate that ;-)

Internet fileservers (North America):         Directory :

OAK.OAKLAND.EDU        [141.210.10.117]   ?

J. 'cFos' und CompuServe mit DOSCIM/WINCIM

CompuServe Information Service (CIS) and  the  ISDN  Access  with

'cFos', (Thanks to Manfred Recla for this description):

Before you start DosCIM (or WinCIM), you have to check  if  V.110

support was loaded with the CAPI driver. Please check  your  CAPI

manual for V.110 support in your CAPI.

With TELES (or CREATIX) S0 boards the module V110.EXE  is  loaded

in STARTS0.BAT batch. Then you are ready to use the  nost  recent

release of DosCIM  V2.2.3  or  WinCIM  V1.3.1  (with  Int-14h/LAN

support!) with 'cFos'. 'cFos' should simply be loaded  after  the

CAPI driver with  parameter  "i".  It  should  be  loaded  before

starting Windows.

CompuServe's WinCIM is  available  in  an  English  or  a  German

Version. If you don't have a  version,  please  download  it  for

example from ZAPHODS BBS.  In  addition  to  that  you  can  also

download  an  updated  version  of   the   modem   database   for

DosCIM/WinCIM with a modem init string  for  "cFos  -  ISDN  CAPI

Modem Emulation". This can be used to configure  'cFos'  for  the

CompuServe requuirements (e.g. calling with an Additional Service

Indicator of 0, since CompuServe  uses  Dr.  Neuhaus  NICCY  3000

terminal apadptors, which can not evaluate the Additional Service

Indicator.

Replace  the  pre-installed  file  "MODEM.DB"  with  the  updated

version from Zaphods BBS. If you installed WinCIM in the  default

directories, the file is located in \CSERVE\SCRIPTS.  Now  follow

the configuration defaults for the  English  or  German  version.

After that you are ready for your first online-session.

Now follows an installation example for DosCIM v2.2.3:

������������������������������������������������������������������������������

�                         ����������������������Ŀ                           �

�                Session: �         cFOS         �               ���������Ŀ �

�                         ������������������������               �   New   � �

� �����������������������������������������������������������Ŀ  ����������� �

� � Your Name: Manfred Recla___________                       �              �

� �   User ID: 100042,1170___                                 �  ���������Ŀ �

� �  Password: ��������������         Phone: 0049-89-66530130 �  � Delete  � �

� �                                                           �  ����������� �

� ��Baud Rate�Ŀ �Comm Port��Ŀ �Network����Ŀ �Dial Type��Ŀ �              �

� ��    9600   � �   INT 14   � � CompuServe � �    Tone    � �  ���������ͻ �

� �������������� �������������� �������������� �������������� �  �   OK    � �

� �                                                           �  ���������ͼ �

� ��Alternate Session����Ŀ ����������Ŀ���������Ŀ���������Ŀ�              �

� ��         cFOS         � � Modem... �� LAN...  �� More... ��  ���������Ŀ �

� ������������������������� ���������������������������������ٳ  � Cancel  � �

� �������������������������������������������������������������  ����������� �

������������������������������������������������������������������������������

��������������������������������������������������������������

�                   Modem Control Strings                    �

�                                                            �

�         � Modem Type �����������������������������Ŀ       �

�         � cFOS - ISDN CAPI Modem Emulation         �       �

�         ��������������������������������������������       �

�           Initialize:   ATB0S20=8S27=197S10.4=1^M          �

�               Prefix:   AT_______________                  �

�                Reset:     &F_____________  [ ] Speaker Off �

�            Dial Tone:     DT_____________                  �

�           Dial Pulse:     DP_____________    ����������ͻ  �

�              Hang Up:     H0_____________    �    OK    �  �

�               Suffix:   ^M_______________    ����������ͼ  �

�               Escape:   +++______________    ����������Ŀ  �

�    Modem Acknowledge:   OK_______________    � Security �  �

�      Connect Message:   CONNECT__________    ������������  �

�      Connect Failure:   NO CARRIER_______    ����������Ŀ  �

� [ ] Error Correction:   _________________    �  Cancel  �  �

� [ ] Data Compression:   _________________    ������������  �

�                                                            �

�                  * Modified from Default                   �

��������������������������������������������������������������

If you have installed DosCIM (v2.2.3) or WinCIM (v1.3.1) already,

but you don't have an updated "MODEM.DB", you can  change  it  as

follows:

[Version]

Version=3.5.2 (10/23/94)

[Modems]

cFos - ISDN CAPI Modem Emulation=cFOS

[cFos]

Init=AT B0 S20=8 S27=197 S10.4=1 &D2 S11=24^M

an updated "MODEM.DB" file with changes for 'cFos'  is  available

at ZAPHODS BBS.

CompuServe new user login:

If you are not already a user of  CompuServe  and  don't  have  a

CompuServe user ID you have to  edit  your  Session  Settings  as

illustrated above. Please fill in your name and as your user ID a

0.  After  that  use  menu  "Special"  to  choose  the   terminal

emulation. Klick the button "Manual Connect" and then  "OK".  You

are now in the terminal emulation. Now supply the following  dial

string:

AT B0 S20=8 S27=197 S10.4=1

AT D 0049-89-66530-130

After a few seconds a "CONNECT 9600" message will  be  displayed.

Now press RETURN (or ENTER) once which initiates transfer of your

login data to the CompuServe host. The prompts  are  answered  as

follows:

Host-Name:        CIS

USER ID:          177000,5603

Password:         NEUE/WELT

Agreement-Number: BTX92D

Serial-Number:    92283

You are also asked for name, address, credit  card  no.  or  bank

account. CompuServe then assigns you  your  own  user  ID  and  a

temporary  password,  valid  for  2  weeks.  Withing  that   time

CompuServe informs you about your final password via mail.

For now write down your user ID and the  temporary  password  and

answer the CompuServe command prompt with the  word  "OFF",  thus

terminating the  connection  to  the  host.  Close  the  terminal

emulation window and enter your user ID and password in the  menu

"Session Settings".  Now  DosCIM/WinCIM  is  ready  to  login  at

CompuServe automatically. You only need to "klick" on  a  desired

service.

-------------------- End of Documentation -----------------------