AIOCOMX.NLM Version 2.00

Novell Inc.

(July 8, 1992)

This driver can allow AIO applications to use the built-in

serial ports on the server or those of an add-in card that

emulates standard serial ports.  Up to four ports can be

supported.  Each port requires it's own interrupt.  Speed

of up to 19,200 bps can be supported if the ports use 16550

UARTS (2400 bps with 8250/16450 devices).

In case of problems please call Novell technical support at

800 NETWARE (800-638-9273).

Load directives:

1)  Entering 'load aiocomx' will cause COM1 (port=3f8, int=4) to

be loaded if the hardware is available.  Entering

'load aiocomx', again, will cause COM2 (port=2f8, int=3) to

be loaded if hardware is available.

OR

2)  Entering 'load aiocomx {optional parameters ...}' will allow

the user to customize the driver according to the options

described in the help screen.

Most users will deem these options as unnecessary for the

majority of cases.

The optional load parameters and their descriptions are listed

below:

PORT=W,	        where W is the hexadecimal port address at

which the device resides.

INT=X,	        where X is the interrupt number assigned to the

particular device port.

NAME=Y,         where Y is the user-defined name which

correlates to the board loaded.

NODE=Z,         where Z is the decimal number which will override

the hardware-defined board number which the driver

determines.  Ordinarily, COM1 is assigned board 0,

and COM2 is assigned board 1.

FORCE           where FORCE overrides hardware device validation,

resulting in the driver using the specified

hardware resources regardless of whether they exist.

NOFIFO,         where NOFIFO overrides the automatic selection of

16550 FIFO mode, regardless of whether 16550 FIFO

mode capability has been detected.

RXT=A,          where A is the default override value to use for

receiver data interrupt threshold, when 16550 FIFO

mode has been selected.

TXQ=B,          where B is the default override value to use for

transmitter data queue, when 16550 FIFO mode has

been selected.

Performance Issues:

If your system has a 16550 UART device present communications

performance will be significantly enhanced.  The driver

automatically configures itself to run in enhanced (fifo) mode

if it detects the device.  This is one of the major features of

this latest version.  The receiver data threshold and transmit

queues for the fifos can be changed by the user if this mode is

allowed, but their default values are probably optimal.  If a

16550 device is not present in your system the user is prohibited from attempting to place the driver in

enhanced mode.

Differences between this version and earlier version(s):

1)  Only ONE board is loaded for each instantiation of the

driver (to be consistent with ALL other drivers).

2)  Driver will now operate in a NetWare 3.2 (4.0) environment,

which was not previously possible.

3)  Driver now offers enhanced performance where 16550

hardware available.

4)  Driver rated at 2400bps for 8250/16450 hardware.

5)  Driver rated at 19200bps for 16550 hardware.

6)  More command line options available at driver load time

7)  Board numbering is directly related to hardware

configuration, as required by AIO driver specification.

8)  Board numbering can be overridden to accomodate AIO.NLM bug

which requires that boards for a particular driver be loaded

in board number sequential order, starting with 0.

9)  Driver command line load information is available to the

user by entering 'load aiocomx ?' at server prompt.

(Note: the driver will not be loaded).