IntelDX4TM Processor

Performance Brief

Release 1.1

February 1994

Intel Corporation

# Table of Contents

µIntroduction	4

The IntelDX4TM Processor	5

IntelDX4 Processor Performance Summary	7

iCOMPTM Performance Index	8

UNIX CPU Benchmarks	9

DOS/Windows CPU Benchmarks	11

Overall System Performance Benchmarks	13

Graphics Performance Benchmarks	14

75 MHz IntelDX4 Processor Performance Summary For Mobile Applications	15

DOS/Windows CPU Benchmarks	15

Graphics Performance Benchmarks	17

Summary Tables of Benchmark Results	18

Summary	22

Appendix A - Test Configurations	23

100 MHz IntelDX4 Processor-based Platform	23

66 MHz Intel486 DX2 and 33 MHz Intel486 SX Processor-based

Platform	24

Pentium Processor-based Platform	25

Appendix B - Test Configurations	26

75 MHz IntelDX4 Processor-based Platform	26

50 MHz Intel486 DX2 and 25 MHz Intel486 DX Processor-based Platform	27

Appendix C - Measurement Details	28

SPEC Benchmark CINT92 Summary	28

SPEC Benchmark CFP92 Summary	29

# Figure List

Figure 1: iCOMPTM  Index Ratings for Intel Processor...............................	8

Figure 2: Performance Comparison of Intel Processors for the UNIX SPECint92 Benchmark	10

Figure 3: Performance Comparison of Intel Processors for the UNIX SPECfp92 Benchmark	10

Figure 4: 100 MHz IntelDX4TM  Processor vs. Intel486TM Processor for the 16-bit DOS Benchmarks	12

Figure 5: 100 MHz IntelDX4 Processor vs. Intel486 Processor for the 16 bit Windows Benchmarks	12

Figure 6: 100 MHz IntelDX4 Processor vs. Intel486 Processor for System Performance Benchmarks	13

Figure 7: 100 MHz IntelDX4 Processor vs. Intel486 Processor for Graphics Performance Benchmarks	14

Figure 8: 75 MHz IntelDX4 Processor vs. Intel486 Processor for 16-bit DOS Benchmarks	16

Figure 9: 75 MHz IntelDX4 Processor vs. Intel486 Processor for 16-bit Windows Benchmarks	16

Figure 10: 75 MHz IntelDX4 Processor vs. Intel486 Processor Floating Point Benchmarks	17

# Table List

Table 1:  100 MHz IntelDX4  Processor Benchmark Results	18

Table 2:  100 MHz IntelDX4 Processor Application Performance Benchmark Results	19

Table 3:  Comparison Of  PentiumTM Processor To 100 MHz IntelDX4 Processor Results	20

Table 4:  75 MHz IntelDX4 Processor Benchmark Results	21

Table 5:  75 MHz IntelDX4 Processor Graphics Performance Benchmarks Results	21

# Changes from Release 1.0

# The following additional information has been added in Release 1.1:

1. iCOMP Index and benchmark results for 100MHz and 90MHz Pentium Processor

2. BAPCO Sysmark93 benchmark results for a typical IntelDX4 Processor-based system

The following are trademarks of the Intel Corporation and may only be used to identify Intel Products:

i386TM, Intel386TM, i486TM, Intel486TM, IntelDX4TM,  PentiumTM, iCOMPTM.

# Introduction

The IntelDX4TM  Processor is the newest and highest performing member of the Intel486TM Processor Family.  At internal speeds of up to 100 MHz, the IntelDX4 Processor is the fastest 486, designed for users who want the best value in 486 desktop computing today.  This report provides test results of traditional benchmarks for IntelDX4 microprocessor-based systems, and compares them with other Intel486 processors such as the 66 MHz Intel486 DX2 and the 33 MHz Intel486 SX.  For reference, PentiumTM Processor performance data is also provided.  The Pentium Processor provides the best performance for all PC software.  Details of system configurations are described in the appendices.

The focus of this report is on performance results of the IntelDX4 Processor in  computing applications.  Widely accepted industry standard benchmarks have been used to position the performance of the IntelDX4 Processor with respect to other products in the Intel486 Processor Family.  Processor-intensive benchmarks such as SPECint92 and several popular DOS CPU benchmarks highlight the performance capabilities of the IntelDX4 Processor in demanding computing applications.  SPECint92 is an effective benchmark for evaluating CPU performance because of its large size and its application representative instruction mix.  Graphical performance improvements are also illustrated using WinBench.

Benchmark results are intended to give a standard measure of performance that can be used to predict how well application code will execute.  Good benchmark programs should be representative of the intended applications.  However, the performance measured will often reflect the combined characteristics of a given computer architecture and many other tightly coupled system software/hardware constituents rather than just the CPU.  Operating system, compilers, libraries, memory design and I/O subsystem characteristics may well dominate the results and make comparisons difficult.  This report is intended to show IntelDX4 Processor performance on standard benchmarks that can be used as a predictor of performance on real applications.

# The IntelDX4TM Processor

With an iCOMPTM Index Rating of 435, the 100 MHz IntelDX4 Processor outperforms the 66 MHz Intel486TM DX2 Processor (iCOMP Index Rating of 297) by as much as 50%.  With Intel's unique 5V tolerant input buffers, this performance improvement can be achieved with minimal modifications to existing i486TM processor-based system designs.  The combination of increased CPU performance and standard, low cost i486 processor design technology effectively positions the IntelDX4 Processor for mainstream business computing.

In addition, the low power consumption and intelligent power management of the IntelDX4 Processor provide OEMs with the opportunity to deliver true desktop performance in a mobile system.  The IntelDX4  Processor provides a new, unparalleled performance level for mobile applications.  Operating at 3.3V, the IntelDX4 Processor consumes less than 3.0 watts of power (75  MHz) on typical applications.  Intel's SL Enhanced technology not only provides robust power management for mobile systems, but also enables energy efficient desktop  systems that meet, and exceed, the EPA's Energy Star guidelines.

The IntelDX4 Processor is based on proven Intel486 technology and is compatible with the huge installed base of over 50,000 applications written for the Intel Architecture.  To ensure end-user investment protection,  IntelDX4 Processor-based systems can be verified for upgradability to a future PentiumTM OverDriveTM Processor.

Product Highlights

·	Fastest Intel486TM Processor with an iCOMPTM Index Rating of 435 (100  MHz)

Available versions:

- 100 MHz with a 50 MHz or a 33 MHz external bus  (OEM selectable)

- 75 MHz with a 25 MHz external bus

- An 83 MHz version, with a 33 MHz external bus, will be available later in 1994.

·	3.3V operation results in low power consumption making the IntelDX4 Processor ideal for mobile systems

·	5V tolerant input buffers with TTL compatible outputs allow for direct interface with either 3.3V or 5V system logic and memory components

·	SL Enhanced technology provides robust power management designs which are O/S and user transparent

·	On-chip 16 KByte unified code and data cache

·	Compatible with an entire library of software based on operating systems such as MS-DOS*, Windows*, OS/2*, UNIX SVR4*, iRMX* operating system, iRMX* Real-Time Kernal, Windows NT*, NeXTstep 486*, Solaris 2.0*, SCO* UNIX

·	Speed-multiplying technology enables the internal Processor core to operate two to three times faster than the external memory bus, delivering high performance with low-cost system design

·	168-lead PGA package  and thermally enhanced, 208-lead, SQFP package

·	Common core instructions execute in a single internal clock cycle

·	On-chip floating-point unit compatible with Intel486TM DX2 processor

·	Burst data bus capable of a 160 Mbyte/sec transfer rate at 50 MHz bus clock

·	Supported by Intel Verification Program for upgradability to a future PentiumTM OverDriveTM Processor

*  MS-DOS, Windows, OS/2, UNIX SVR4.2, NextStep486, and Solaris 2.0 are trademarks of their respective companies.

# IntelDX4TM Processor Performance Summary

This section is divided into three sub-sections: iCOMPTM Index, 100 MHz IntelDX4 Processor Performance and 75 MHz IntelDX4 Processor Performance For Mobile Applications.  In the iCOMP Index section, the IntelDX4 Processor is compared to other members of the Intel family of processors.  In the 100 MHz IntelDX4 Processor Performance section, the 100 MHz IntelDX4 Processor performance is demonstrated using many industry standard benchmarks in the UNIX, DOS, and Windows environments.  The 75 MHz IntelDX4 Processor Performance section illustrates the performance of the processor in mobile applications.

# iCOMPTM Index Rating

The iCOMPTM (Intel COmparative Microprocessor Performance) index provides a simple relative measure of microprocessor performance.  It is not a system benchmark, but an index of relative processor performance intended to help non-technical end users decide which Intel microprocessor best meets their computing needs.

The iCOMP rating is based on the technical categories that encompass four separate aspects of both 16 and 32-bit processor performance: integer, floating-point, graphics and video performance.  Each category is weighted based on the estimated percentage of time it enters into the processing picture.  The higher the iCOMP index, the higher the relative performance of the microprocessor.

Figure 1 illustrates the iCOMP ratings for 11 Intel microprocessors.  The 100 MHz IntelDX4TM Processor with a rating of 435 delivers a new performance point between the 66 MHz Intel486TM DX2 and the PentiumTM  Processors.

µ §

Figure 1.  iCOMPTM Index Ratings for Intel Processors

# 100 MHz IntelDX4TM Processor Performance Summary

## UNIX CPU Benchmarks

The Systems Performance Evaluation Cooperative (SPEC) is a non-profit organization formed to develop a standard suite of benchmark programs that effectively measure the performance of computing systems in actual application environments.  CINT92 and CFP92 are test suites provided by SPEC in January 1992 to represent the performance of a processor.  SPECint92 is the geometric mean of the SPECratios from the six integer-intensive benchmarks in CINT92.  SPECfp92 is the geometric mean of the SPECratios from the fourteen floating-point intensive benchmarks in CFP92.

Although the SPEC92 benchmark suite is the industry standard for measuring performance of technical computing systems in actual application environments, today's commercial applications are still mostly comprised of integer-intensive programs.  Thus, SPECint92 represents an appropriate instruction mix for commercial applications and is a much more effective benchmark to predict 32-bit business performance than SPECfp92.

The 100 MHz IntelDX4TM Processor performance improvements on SPEC and other UNIX CPU benchmarks range up to 39 percent when compared with the 66 MHz Intel486TM DX2 processor.  The relative performance is even more dramatic when compared with the 33 MHz Intel486 SX processor, with gains of up to 245 percent.  Figure 2 and 3 show the SPECint92 and SPECfp92 performance.

µ §

Figure 2. Performance Comparison of Intel Processors for the

UNIX SPECint92 Benchmark

µ §

Figure 3. Performance Comparison of Intel Processors for the

UNIX SPECfp92 Benchmark

DOS/Windows CPU Benchmarks

DOS benchmarks are presented to illustrate the 16-bit integer performance of the 100  MHz IntelDX4TM Processor.  Many of these benchmarks are industry standards for the PC-DOS and PC-Windows environments.  These benchmarks are presented to demonstrate the high integer performance achieved with the 100 MHz IntelDX4 Processor.

For 16-bit integer DOS operation, the 100 MHz IntelDX4TM Processor offers performance gains of up to 37 percent, when compared to the 66 MHz Intel486TM DX2 processor.  Figure 3 illustrates the 100 MHz IntelDX4 Processor performance when executing Ziff-Davis Labs (a.k.a. PC Labs) PC Bench, Version 8.0, Processor Harmonic,   a popular and robust DOS benchmark.

For Windows operations, the 100 MHz IntelDX4 Processor offers performance gains of up to 50 percent, when compared to the 66 MHz Intel486 DX2 processor.  Figure 4 illustrates the 100 MHz IntelDX4 Processor performance when executing Landmark CPU Mark, Version 3.0, a popular and robust Windows CPU benchmark.

For floating point operation, the 100 MHz IntelDX4 Processor offers performance gains of up to 50% percent, when compared to the 66 MHz Intel486 DX2 processor.  Figure 4 illustrates the 100 MHz IntelDX4 Processor performance when executing  Landmark FPU Mark, Version 3.0, a popular and robust Windows FPU benchmark.

µ §

Figure 4.  100 MHz IntelDX4TM Processor vs. Intel486TM Processor for

16-bit DOS Benchmarks1

µ §

Figure 5.  100 MHz IntelDX4TM Processor vs. Intel486TM Processor for

16-bit Windows Benchmarks

## Overall System Performance

To demonstrate realistic application performance, SYSmark93 for Windows was chosen to gauge the performance of the IntelDX4TM Processor.  SYSmark93 for Windows is a suite of application software and associated benchmark scripts that have been developed by Business Applications Performance Corporation in order to provide a tool for accurate and realistic measurement of system performance of personal computers running popular business-oriented applications in the Microsoft Windows operating environment.   The scripts are developed to accurately reflect usage patterns of PC users in a business-oriented environment.

SYSmark93 for Windows includes benchmark scripts for the following applications selected from six categories of application software:

Word-processing	WordPerfect 5.2, Word For Windows 2.0, AmiPro 3.0

Spreadsheet	1-2-3 4.0, Excel 4.0

Database	Paradox 1.0

Desktop Graphics	Corel Draw 3.0

Desktop Presentation	Freelance 2.0, PowerPoint 3.0

Desktop Publishing	PageMaker 5.0

For overall system operation, the 100 MHz IntelDX4 Processor offers performacne gains of up to 33 percent, when compared to the 66 MHz Intel486TM DX2 processor.  Figure 6 illustrates the SYSmark93 for Windows Performance on the IntelDX4 Processor.

µ §

Figure 6.  100 MHz IntelDX4TM Processor vs. Intel486TM Processor for

System Performance Benchmarks

## Graphics Performance Benchmarks

To illustrate the impact a higher performing processor can have on system graphics performance, WinBench 4.0 was executed using the 100 MHz IntelDX4TM Processor.  WinBench 4.0 is a subsystem-level benchmark that tests the graphics performance of PC's running Windows-based applications.  WinBench 4.0 does not run actual applications; instead it performs test based on the  operations that applications perform.  WinBench 4.0 executes on top of Windows and reports results in WinMark measurements.  The graphics subsystem is impacted by the processor during data access to and from RAM.  The graphics subsystem tested includes both the video adapter and the Windows device driver.

For graphics performance, the IntelDX4 Processor will offer performance gains of up to 37%, when compared to the 66 MHz Intel486 DX2.  Figure 6 illustrates the graphical performance gains (compared to the 33 MHz Intel486TM SX and 66 MHz Intel486 DX2 processors) achieved when utilizing the 100 MHz IntelDX4 Processor in desktop systems.

µ §

Figure 7.  100 MHz IntelDX4TM Processor vs. Intel486TM Processor for

Graphics Performance Benchmarks

# 75 MHz IntelDX4TM Processor Performance Summary for Mobile Applications

The IntelDX4TM Processor continues Intel's innovation in microprocessors designed specifically for mobile system applications.  In addition to low power consumption and intelligent power management, the IntelDX4 Processor provide OEMs with the opportunity to deliver true desktop performance in a mobile system.    With the IntelDX4 processor, mobile users will be able to maintain pace with the ever-increasing performance on their desktops.  This will allow notebook users to take full advantage of technologies they have been using in their desktop environment, such as presentation graphics, in mobile environments including training sessions or on-site customer presentations.  The 75 MHz IntelDX4 processor consumes less than 3 watts of power in typical applications, enabling mobile system designs to achieve new thresholds of performance with long battery life.  With these capabilities the IntelDX4 processor becomes the processor of choice for professional performance notebook users.

## DOS/Windows CPU Benchmarks

DOS benchmarks are presented to illustrate the 16-bit integer performance of the 75 MHz IntelDX4TM Processor.  Many of these benchmarks are industry standards for the PC-DOS and PC-Windows environments.  These benchmarks are presented to demonstrate the high integer performance achieved with the 75 MHz IntelDX4 Processor.

For 16-bit integer DOS operation, the 75 MHz IntelDX4 Processor offers performance gains of up to 67 percent, when compared to the 50 MHz Intel486TM DX2 Processor.  Figure 8 illustrates the 75 MHz IntelDX4 Processor performance when executing Ziff-Davis Labs (a.k.a. PC Labs)  PC Bench, Version 8.0, Processor Harmonic, a popular and robust DOS benchmark.

For Windows operations, the 75 MHz IntelDX4 Processor offers performance gains of up to 73 percent, when compared to the 50 MHz Intel486TM DX2 Processor.  Figure 9 illustrates the 75 MHz IntelDX4 Processor performance when executing Landmark CPU Mark, Version 3.0, a popular and robust Windows CPU benchmark.

µ §

Figure 8.  75 MHz IntelDX4TM Processor vs. 50 MHz Intel486 DX2 Processor

for the 16-bit DOS Benchmarks

µ §

Figure 9.  75 MHz IntelDX4TM Processor vs. 50 MHz Intel486TM DX2 Processor

for 16-Bit Windows Benchmarks

For floating point operation, the 75 MHz IntelDX4TM Processor offers performance gains of 50 percent or greater, depending on the benchmarks, when compared to the 50 MHz Intel486TM DX2 processor.  Figure 10 illustrates the 75 MHz IntelDX4 Processor performance when executing the Landmark Version 3.0 floating point unit benchmark.

µ §

Figure 10.  75 MHz IntelDX4TM Processor vs. 50 MHz Intel486TM DX2 Processor

for 16-bit Floating Point Operations

## Graphics Benchmarks

To illustrate the impact a higher performing processor can have on system graphics performance, WinBench 4.0 was executed using the 75 MHz IntelDX4TM Processor.  Table 4 indicates the graphical performance gains (when compared to the 50 MHz Intel486TM DX2 Processor) achieved when utilizing the 75 MHz IntelDX4 Processor in mobile systems.  For WinBench 4.0, the performance gain is 43 percent.

## Summary Tables For Benchmark Performance

Table 1.  100 MHz IntelDX4TM Microprocessor Benchmark Results

µ §

Table 2.  100 MHz IntelDX4TM Application Benchmark Results1

µ §

1Note that these results are measured from the first available desktop IntelDX4 Processor-based system utilizing PCI local bus.  Higher performance is expected for subsequent production desktop IntelDX4  Processor-based systems.

Table 3.  Comparison of Pentium ProcessorTM  To

100 MHz IntelDX4TM   Benchmark Results

µ §

*For Pentium Processor (510/60) and Pentium Processor (567/66) - based systems, the highest recorded SPECfp92 result is 63.6 and 55.1 SPECmarks.  See September 1993 SPEC newletter

for full system description and disclosure reports.

**At time of printing, results for these systems were not available.

Table 4.  75 MHz IntelDX4TM Microprocessor Benchmark Results3

µ §

Table 5.  75 MHz IntelDX4TM  Graphics Performance Benchmark Results3

# µ §

# 3Note:  These benchmark measurements were taken on major brand-name notebooks.  They are configured without Level 2 (external) cache.  Since the measurements were taken on a first available IntelDX4 Processor-based system, higher performance is expected for subsequent IntelDX4 Processor-based notebook systems.

# Summary

# The IntelDX4 Microprocessor brings a new level of computing power to the popular Intel486 Processor Family.  The 100 MHz IntelDX4 Processor delivers SPECint92 results of 51.38  and an iCOMP of 435, up to a 50% increase over the 66 MHz Intel486 DX2 processor.  The IntelDX4 Processor combines high performance and affordable technology to provide the best value in 486 desktop computing and the highest performance for mobile computing systems.

# Appendix A - Test Configurations

100 MHz IntelDX4TM Processor-based Platform:

System:		Micronics M4P Desktop PCI System

CPU:			100 MHz IntelDX4 Processor

FPU:			Integrated

Cache:			16 KByte Unified (Instruction + Data) Internal

256 KByte Write-Back External

Chipset:		Intel 82420 PCISet

Memory:		32 MByte DRAM (UNIX)

16 MByte DRAM (DOS and Windows)

Operating Systems:	DOS 5.0

Windows 3.1

UNIX SVR 4.2

DOS/Windows

Environments:		All  processor measurements were executed with

a 2MByte SmartDrv cache.  BAPCo SYSmark93 for Windows 				used a 4MByte SmartDrv cache.

UNIX Compilers:	Intel Reference C Compiler, V1.3

Intel Reference FORTRAN Compiler 1.3

Disk:			WD2340 540MB Hard Disk with integrated ISA/IDE Controller

Video:			Diamond Stealth Viper PCI Graphics Card, 2MB VRAM

Viper Drivers, V1.7, 9/27/93

Graphics:		640 x 480 x 256 Colors Resolution

Bus:			PCI Local Bus

Bus Frequency:	33 MHz

66 Mhz Intel486TM DX2 Processor and

33 Mhz Intel486 SX Processor-based Platform:

System:		Micronics M4P Desktop PCI System

CPU:			66 MHz Intel486 DX2 Processor

33 MHz Intel486 SX Processor

FPU:			Integrated

Cache:			8 KByte Internal

256 KByte Write-Back External (66 MHz Intel486 DX2 Processor)

128 KByte Write-Back External (33 MHz Intel486 SX Processor)

Chipset:		Intel 82420 PCISet

Memory:		32 MByte DRAM (UNIX)

16 MByte DRAM (DOS and Windows)

Operating Systems:	DOS 5.0

Windows 3.1

UNIX SVR 4.2

DOS/Windows

Environments:		All  processor measurements were executed with

a 2MByte SmartDrv cache.  BAPCo SYSmark93 for Windows 				used a 4MByte SmartDrv cache.

UNIX Compilers:	Intel Reference C Compiler, V1.3

Intel Reference FORTRAN Compiler 1.3

Disk:			WD2340 540MB Hard Disk with integrated ISA/IDE Controller

Video:			Diamond Stealth Viper PCI Graphics Card, 2MB VRAM

Viper Drivers, V1.7, 9/27/93

Graphics:		640 x 480 x 256 Colors Resolution

Bus:			PCI Local Bus

Bus Frequency:	33 MHz

Intel PentiumTM Processor-based Platform for UNIX:

System:		Intel Xpress/MX Deskside

CPU:			Intel Pentium Processor 815/100 MHz

Intel Pentium Processor 735/90 MHz

Intel Pentium Processor 567/66 MHz

Intel Pentium Processor 510/60 MHz

FPU:			Integrated

Cache:			8 KByte (Instruction) + 8 KByte (Data) Internal

512 KByte Write-Back External (815/100 and 735/90 MHz only)

256 KByte Write-Back External (567/66 and 510/60 MHz only)

Memory:		64 MByte DRAM (UNIX)

Operating Systems:	UNIX SVR 4.2

UNIX Compilers:	Intel Reference C Compiler, V1.4.3

Intel Reference FORTRAN Compiler 1.4.3

Disk:			2 1.2 GByte DEC DSP SCSI Hard Disk

Video:			ATI Mach32, 2MB VRAM

Bus:			EISA

Bus Frequency:	66, 60 MHz

Intel PentiumTM Processor-based Platform for DOS/Windows:

System:		Dell OmniPlex System (Pre-Production)

CPU:			Intel Pentium Processor (567/66)

FPU:			Integrated

Cache:			8 KByte (Instruction) + 8 KByte (Data) Internal

256 KByte Write-Back External

Memory:		16 MByte DRAM

Operating Systems:	DOS 5.0/Windows 3.1

Disk:			Maxtor 540S SCSI Hard Disk

Video:			Matrox PCI Graphics Card, 2MB VRAM

Graphics:		640 x 480 x 256 Colors Resolution

Bus:			PCI

Bus Frequency:	66 MHz

System:		Gateway Pre-Production System

CPU:			Intel Pentium Processor (510/60)

FPU:			Integrated

Cache:			8 KByte (Instruction) + 8 KByte (Data) Internal

256 KByte Write-Back External

Memory:		16 MByte DRAM

Operating Systems:	DOS 5.0/Windows 3.1

Disk:			Maxtor 540S SCSI Hard Disk

Video:			Matrox PCI Graphics Card, 2MB VRAM

Graphics:		640 x 480 x 256 Colors Resolution

Bus:			PCI

Bus Frequency:	60 MHz

# Appendix B - Test Configurations For Mobile Applications

75 MHz IntelDX4TM Processor-based Platform:

CPU:			75 MHz IntelDX4 Processor

FPU:			Integrated

Cache:			16 KByte Internal

No external cache

Memory:		4 MByte DRAM

Operating Systems:	DOS 6.0

Windows 3.1

Disk:			209 MB Hard Disk

Software

Disk Cache:		SmartDrv 4.16, 512 KByte

Video Resolution:	640 x 480 pixels x 256 Colors

Video Driver:		Cirrus 6440, Ver.1.0

Bus:			ISA Bus

Bus Frequency:	25 MHz

50 MHz Intel486TM DX2  Processor-based Platform:

CPU:			50 MHz Intel486 DX2 Processor

FPU:			Integrated

Cache:			8 KByte Internal

No external cache

Memory:		24 MByte DRAM

Operating Systems:	DOS 6.0

Windows 3.1

Disk:			340 MB Hard Disk

Software

Disk Cache:		SmartDrv 4.16; 2,048 KByte

Video Resolution:	640 x 480 pixels x 256 Colors

Video Driver: 		C24S480c.drv 3.10; Size 145792

Bus:			ISA Bus

Bus Frequency:	25 MHz

# Appendix C - Measurement Details

µ §

µ §