Last Change 27.11.92

������������������������������������������������������������ͻ

�                                                            �

�            V O I C E   M A I L     V 1 . 0 0               �

�            = = = = =   = = = =     = = = = =               �

�                                                            �

�         Programm for a Voice-Mail-Box with the             �

�   Zyxel 1496 for a IBM-Compatible( or real IBM ???)        �

�                                                            �

������������������������������������������������������������ͼ

����������������������������������������������������������������������������ͻ

�      This Programm is copyrighted Shareware.                               �

�      No fee may be charged for its reproduction or distribution.           �

�      No liability accepted for any damages that may caused by this program!�

�      German law prohibits the use of this programm on public               �

�      telephone installations                                               �

����������������������������������������������������������������������������ͼ

I. INTRODUCTION

================

With a DTMF-Tone you can initiate programm execution on

IBM (or compatable) computer.  Thus enabling emulation of functions

such as Anseringmachine, Fax, BBS etc

eg  by starting a Programm

This programm is my first attempt  to build a Voice-Mail-Box for the

ZyXEL on the IBM-AT. Ith was originally designed for my own use,

it is'nt  perfect, but given time....

When you  register you contribute to the futher developement of this programm.

I'm open for tips and  contructive  suggestions for new features etc.

To much abusive feedback is the best way to kill all futher development.

II. COPYRIGHT

see above !

and:

For a commercial use contact you have to contact the autor  !!!

The autor:          Andreas M�nzenmaier

Gustav-Werner-Str. 6

7300 Esslingen

Germany

Voice: 49-711-3161493  (be carefull with that)

BTX  : 07113161493-0001

Bank : BLZ: 61150020

NR.: 766337

Kreisssparkasse  Esslingen

Fido:  Fellbox [2:244/21.5]

/********************************************************/

/* Thanx to GMSdata                                     */

/*          APTERyX Software!                           */

/*          Silke Greschner for her Voice!              */

/*          Walter Vitols for the english dokumentation */

/* who helped with the Documentation                    */

/********************************************************/

The Features of the BOX

- Time Limited recorded calls. (How Greate)

- Remote Control:  - Memory Playback

- Record Messages

- Record Vip Messages

- Change OGM (von 1-9)

- External Programmable Shut Down

RING counter to  (15)

- select OGM (1-9)

- Leave and Record VIP Messages

- Password protection for all Options !

- Automatic Call-Back ("Call back" phone number may be changed externally)

- Automatic Call-Back after a Preset number of calls

- Faxrecive  (automatic)

- Go to BBS-System possible (by T.30 Tones automatic)

- Give call into a Internal Telefone System to the   ???

Answermachine/FAX/BBS!

Posible Future developements

- FAX Pooling (for up to 9 Pages, select via TOUCH-TONE )

- Real Voice Mail Box with several Users.

- Time spelling

- Make Coffee, Cooking, Dry clothing, having Babies, Ironing etc...

- and many more

Explanation MEMORY PLAY BACK FUNTION

When a message is recieved, a it is entered in the "VOICE.LOG" File.

If you aktivate the Memory Playback the Zvoice plays the MSG which are

entered in the VOICE.LOG file. After the playback the contens of the

Voice.log file will be appended to the Memory.log file for arcive use!

Then the file VOICE.LOG is cleared.

1. Configuration  File:

===============

Passwords:

All funktions can be protected by passwords.

All funktions ( except the VIP messages ) may have the same password,

VIP messages must have different passwords. You can deaktivate a password

by setting the password flag to 0

The VIP Passwords must be different from all passwords!

(PW_VIPR1 not equal with PW_VIPR2). ?????  Who Where What

You can also enable all funktions with the same password simultaneously.

The best way to set up the baudrate is to lock the baudrate with a Fossil

driver program such as BNU or X00 !

The programm's default Baudrate ist set to 38400 Baud.

I do not recomend  19200 Baud (expect for the CELP Mode? users I don't have it) ?????????

it could be to slow on some systems!

Record Lenght (Break_time):

The break-time value is only an aproximate value.

The programm uses a 2K buffer to receive incoming data.

To improve  the speed of data acquistion a break signal will

only be reacted upon when the buffer is full.

The difference between real time and aproximate

time schould be not more than 1 sec!

Configfile valid line format:

'*' represents a  comment line. (Ignored line)

Comments at the end of an instruction have to begin with  '/*'

and should end with a '*/'

System Konfigurationen

======================

My System configuration is:

IBM-AT386SX/16Mhz/210MB

Zyxel 1496E Rom 5.02

A Internal Telephone System Pulse-Dialing from Keil Electronic

The COM is buffered with a 16550!

The programm also work on my 286/16Mhz! I don't have any experiance

with slower machines.

I will be delighted over your Tips about that (The other also)!

Callback Funktion

==================

Procedudde from the call back funktion:

Everytime a Voice-Call is recorded a counter is incremented. Exeeds the

counter a preset value after the aktuell call is finished the programm

does a call back.

The program dials the number given by the configfile. Then a message is

played and the program waits for your password entering. After

the password has been entered it plays the not played voice files.

The the logfile is updated (cleared). If the callback fails (busy or

no wron password the call back is repeated after each call!

V Special

=========

The handshake in the Voice-Mode is with  Xon/Xoff Protokoll ! (refer ZyXEL)

ERROR Handling

==============

Because this is only a testversion which is not completetd yet I'm using

a strong errortreatment. This means a error causes a programm break with

ERRORLEVEL 1 (a restart is possible with the start.bat file!

All error are loged in the ERROR.LOG file!

The .ZVD ( Zyxel Voice Data ) format

====================================

The voice file contains a file header and the compressed voice data.

The header is a 16-byte data, they are

Bytes 0 - 4 : title  -> "ZyXEL"

Byte  5     : 2

Byte  6 - 7 : reserved

Byte  8 - 9 : reserved

Bytes 10-11 : voice compress scheme,

0 - CELP

1 - 2 bits ADPCM

2 - 3 bits ADPCM

Bytes 12-13 : reserved

Bytes 14-15 : reserved

The voice data (0 to ff) is just the data received from U1496 Modem/Fax.

But not the 0x10 (DLE) the DLE shielded codes. Should a DLE transmitted

it is send twice. Is one DLE send the second is the shileded code, and it is

not part of the Voice Data.

Is the .ZVD resend to the ZyXEL the 16 Bit ist not Transmitted and for

the DLE you have to send the DLE twice.

Limits:

The call can be up to 9999! (also Data-Calls, Silent, Answered Calls)

VI Voice File names and there meaning

=====================================

Filename    Meaningg or Suggestion

OGM1.ZVD    /* Out going messages from 0 */

OGMx.ZVD

OGM9.ZVD    /* to 9 */

GOODBYE.ZVD /* Goodbye */

WRONG.ZVD   /* input not supported */

0.ZVD       /* Null pressed */

x.ZVD       /* x (0-9) pressed */

9.ZVD       /* upto 9 pressed */

ALLOK.ZVD   /* Input OK */

OGM_SEL.ZVD /* Enter the desired OGM */

MENUE.ZVD   /* Press

1 Memory Playback

2 f�r VIP 1

3 f�r VIP 2

4 Special

5 Ende Ohne Nachricht

6 Anruf Weiterleiten

7 f�r Fax

8 f�r DF�

9 f�r Fax Pooling

0 f�r Adresse

* f�r Menue

# f�r Abbrechen

OGM1.ZVD    /* Guten Tag Sie sind in einer Voice-Mail_box

Bitte dr�cken sie die Stern Taste f�r das Menue.

Falls sie keine Taste dr�cken, k�nne Sie nach dem Pfeifton

eine Nachricht f�r mich hinterlassen */

FAXPOOL.ZVD /* Auswahl-Menue f�r Fax Pooling 0 bis 9 */

FAXx.FAX    /* Fax-File for pooling x(0 to 9)

TEXT1.ZVD   /* Ihre Nachricht lautete */

TEXT2.ZVD   /* Ihre anruf wird weitergeleitet */

TEXT3.ZVD   /* Bitte VIP1 Message hinterlassen */

TEXT4.ZVD   /* Bitte VIP2 Message hinterlassen */

TEXT5.ZVD   /* Keine Berechtigung bitte Passwort eingeben */

TEXT6.ZVD   /* Es wurde keine Message hinterlassen        */

TEXT7.ZVD   /* Sie haben nun 10 secunden Zeit die Nummer zu hinterlassen */

TEXT8.ZVD   /* Passwort ok */

TEXT9.ZVD   /* Passwort ok */

TEXT10.ZVD  /* Bitte die Anzahl der Anrufe eingeben */

TEXT11.ZVD  /* Calling back my sysop! please enter password! */

TEXT12.ZVD  /* Falls sie noch eine nachricht hinterlassen m�chten bitte 1 dr�cken */

TEXT13.ZVD  /* Memory Play Back */

VIP1.ZVD    /* Die VIP 1 Message */

VIP2.ZVD    /* Die VIP 2 Message */

VERBIND.ZVD /* ICH Verbinde */

SPECIAL.ZVD /* Special Menue

1 f�r OGMnummer �ndern

2 f�r Record OGM

3 f�r Record VIP1

4 f�r Record VIP1

5 f�r Record VIP2

6 f�r Record Callback number

7 f�r Enter Call back after x Numbers

8 f�r Turn the unit off (15 Rings)

9 f�r Terminate Programm after call

RANDOMx.ZVD  /* random messages die als erstes file gespielt werden -> .cfg */

with the # Pushbutton you can skip the aktuall message !