Release Notes for DFDP3/plus v. 1.00

Release Date: August 24, 1992

Read this entire document before using DFDP3/plus.

New features of DFDP3/plus v 1.00

DFDP3/plus 1.0 incorporates bug fixes with DFDP3/plus v. 0.9

and several new features, including:

Filter Redesign

Quantization After the Design

Pole/Zero Output Files

Calculated Filter Zoom

Contour Plotter

Integrated Code Generator

Buttons

Filter Redesign

You can now redesign any designed filter.  Filter Redesign

replaces the Modify Last Design feature in DFDP3/plus v.

0.9.  You specify the name of the filter coefficient file,

and DFDP3/plus presents the proper parameter collection

window and the specifications used to create the filter.

Quantization After the Design

You can now quantize (or unquantize) filter coefficients

after you design them.  This means that you don't have to

redesign the filter just to change the number of bits of

quantization.

Pole/Zero Output Files

During an IIR design, DFDP3/plus 1.0 now generates a .PZ

file (in addition to the filter coefficient file and the

signal files) that corresponds to the pole/zero locations of

the filter.  This lets you manually adjust the pole/zero

locations of the designed filter, then input the modified

filter with the Pole/Zero Input feature of DFDP3/plus.

Calculated Filter Zoom

After designing a filter, you can magnify (or "zoom in" on)

filter response files -- such as magnitude, log magnitude,

group delay, phase, and error signals -- to see more detail

of the signal.  This differs from the "Zoom In" menu option

in that the signal is now recomputed over the region of

interest rather than interpolating the signal linearly

(looking at the same points expanded with straight lines

connecting them).  This lets you precisely view the filter

response over any region of interest.

Contour Plotter

The new contour plotter lets you display signals in a

topographic format. This is useful in time-frequency

analysis of signals such as the magnitude output of the

short-time Fourier transform.

Integrated Code Generator

There is a new Code Generator menu option under Filter

Design.  If you purchase and install a Code Generator, you

can select an I/O loop according to your ASPI hardware

setup, then create, compile, and download code to the ASPI

board.

To use CGEN to create and compile code, you must first copy

the ADP.PAR associated with your hardware installation to

the DFDP3\BIN directory (you only need to do this once,

unless you change your ASPI hardware).  Then you can get

into ASHELL, run DFDP3, and select an I/O loop.  This will

set up the proper I/O loop and the proper TMS320DL (for

downloading), and TMS320CL (for compiling with the C

interface), and TMS320AL (for compiling with the ASM

interface) files.

If you use a Banshee Board and have problems running the

GO._MC file generated by the Code Generator, then check that

the MACRO environment variable (defined in the ASPI_ENV.PAR

file in the DFDP3/BIN directory) includes the Banshee macro

directory.  In the ASPI_ENV.PAR file, modify this line:

MACRO=C:\DFDP3\MAC

to read:

MACRO=C:\DFDP3\MAC;C:\ASPI\MACRO

where C:\ASPI is the directory where you installed your

Banshee software.

The next time you enter ASHELL (either by running DFDP3.BAT

or ASHELL), your MACRO environment variable should be set up

properly.

Buttons

To make the user interface more uniform, DFDP3/plus now

includes "buttons" that you can select ("click on") with a

mouse.  These buttons and (if you do not use a mouse) their

equivalent keystrokes are:

OK  (End or Enter, depending on where you are)

Cancel  (Esc)

Reset  (F5)

Help  (F1)

OK accepts any values you input and continues with the

program.  Use End in Parameter Collection Windows and Enter

for messages.

Cancel backs you out of what you are doing.

Reset returns parameter values in a Parameter Collection

Window to their default value.

Help provides context-sensitive help wherever you are in

DFDP3/plus.