-----------------------------------------------------------------------

cFos Modem Command Set

-----------------------------------------------------------------------

?       Show a small help screen of AT Commands and important

S-registers.

A       Force Answer

Bn      Select protocol ONLY for outgoing calls

0       X75 without B3 protocoll, 64000bps      ("ISDNC")

s22=2048, s26=2, s28=2048

1       V110 async, 38400bps, 8n1               ("ISDNB")

s22=2048, s26=2, s28=2048

2       V110 async, 19200bps, 8n1               ("ISDNA")

s22=2048, s26=2, s28=2048

3       TELES 128kbps Bundle Protocoll

s22=2048, s26=2, s28=2048

4       X75, Framelen=256, Win.Size=7 (ELINK mode)

s22=framelen, s26=7, s28=256

5       Datex-J (BTX), Framesize=130, Win.Size=7

s22=framelen, s26=7, s28=128

6       same as 5, but switches VT-100 on

7       analogue modem, if possible

'framelen' refers to the framelen set by -b switch on

commandline; all values are checked not to exceed the

commandline given settings

D..     Dial

F       start a leased line connection

L       dial last number

S       dial stored number (hardcoded, 0=Zaphods BBS)

V       flag connection as SPV

0..9    digits

A..D#*  special "digits"

<nb>s   dial number <nb> and flag connection as SPV

En      Command Echo

0       echo off

1       echo on

Hn      On/Off Hook control

0       on hook, terminate an active connection

1       off hook, reject an incoming call

In      Inquiry

0       product identification and CAPI version and

manufacturer

1       status line output

2       last call statistics

4       last 10 CAPI info dumps

5       some CAPI infos (number of charging infos

and time/date infos)

Qn      Quiet mode, result codes display

0       display result codes

1       suppress result code; be Quiet

Sr=n    Set register r to value n (registers are unsigned 16bit ints)

Sr.b=n  Set bit b to n

Sr,b=n  Set bit b to n

Sr?     Query register r

SrH=n   Set high byte of register r to value n

SrL=n   Set low byte of register r to value n

Vn      Verbal mode

0       numeric result codes

1       verbal result codes

Xn      Result code options

0       modem looks like a standard 9600 bps modem

1       enables CONNECT <rate>

2       enables RINGING

3       enables RING <incoming caller's id>

4       enables CONNECT[...]/<protocols..>

5       enables ?

6       enables /<cause> on ERROR, NO CARRIER, NO ANSWER

and NO DIALTONE

Zn      Restore profile from number n (out of the file PROFILE).

This will further reregister the application at the CAPI.

If no number is given, 0 will be default.

Fn      \

Kn       |

Ln       |

Mn       |  ignored for compatibility

Nn       |

O        |

P        |

T       /

-----------------------------------------------------------------------

cFos extended Modem Command Set

-----------------------------------------------------------------------

*       &An,n,. Answer to these MSNs (CAPI 2.0). Default is all.

For example by AT&A123456,789012,345678 'cFos' will listen

to the MSNs 123456, 789012 and 345678. Up to 10 are

allowed. AT&A will answer to no MSN, AT&A* will answer to

all MSNs.

{++}    &Bn     Number of Channels to use for 'cFos' Channel Bundling, for

next dial command

&Cn     Ignored for compatibility

&Dn     Status line

0       disable status line (default)

1       enable status line (only if port is initialized)

2       enable status line (only if carrier is high)

&En     EAZ: n = '0' ... '9': changes register 18. Default is

EAZ 0.

&F      Reset to Factory defaults and reset (like ATZ)

&H<str> This sets the hangup string to "+++<str><cr>". E.g. this

enables setting the hangup  sequence  to  "+++ATHE1<cr>"

for the pcANYWHERE "Hayes compatible modem". Default  is

<str>   =   "ATH0",   i.e.   the   hangup-sequence    is

"+++ATH0<cr>".

&I..    Listen to what services; changes register 14; takes

'A' (Analog or Audio) and/or 'D' (Data or Digital) as

parameter.

&Lnnn   Listen to what EAZ's; changes register 13;

takes multiple digits or '*' for all. Default is EAZ 1.

&Mn     Select bundling mode.

0       static bundling

1       dynamic bundling

&Onnn   Outdial Number. This number will be used as prefix for

all phone numbers dialed with the ATD command. Default is

none.

&Rn     Reregister the application if all ports are idle; use

#R1 to force reregister (and lockout of all active

connections!)

* {++}  &Snnn   Outgoing Subaddress (CAPI 2.0). Default is none.

&T      TRANX: set the clock to the actual time/date (gotten from

the last connection attempt).

* {++}  &Unnn   Calling party number (CAPI 2.0 and X.31)

&V      View profile

&Wn     Write profile to number n (into a file named PROFILE). If

no number is given, it defaults to 0.

-----------------------------------------------------------------------

cFos Modem Register Chart

-----------------------------------------------------------------------

Reg#      Description                                     Default

S0      Auto Answer on (>=1) / off (==0)                0

S1      Ring count                                      0

S2

S3

S4

S5

S6      Column of callerid display (only if s10.1 == 1, 61

at&d1 or at&d2; used together with s11 (line))

S7      Number of seconds modem waits for a carrier     60

S8

S9      Bit-mapped register:                            32737

.0      enable any further result code

.1      enable CONNECT <rate>, otherwise

CONNECT 9600 is always returned

.2      enable RINGING

.3      enable RING <number>

.4      enable /X75.. after CONNECT

.5      enable ERROR/<cause>

.6      enable NO CARRIER/<cause>

S10     Bit-mapped register:                            Default: 0

.0      enable CONNECT.../ID=<callerid>

.1      disable full status line, just display the callerid

on the left side

.2      display numerical values fuer service indicator /

add. si instead of names

.3      display the callerid, requested eaz, si/add si on

a RING like RING 1234/EAZ1/07/00, in spite of the

default method: RING<cr>ID=1234/EAZ1/07/00

{+}             .4      enable +++ATH0<CR> hangup sequence in BIOS emulator

mode (and disable hangup on baudrate change)

.5      (obsolete)

.6      disable multiple CallerID's

.7      display the baudrate specified in register S47

on CONNECT messages

S11     Status line line number (0 - screen length-1)   0

Note: only displayed if AT&D1 or AT&D2

{++}    S12     Nummer of 1/18 sec. to wait before CONNECT      50

message.

S13     Serviced EAZ Mask                               2

Every bit from bit0.. bit9 enables RING on

this EAZ ('0' - '9')

ex:     0000000010b     answer on '1'

0100000000b     answer on '8'

0001111111b     answer on '0' - '6'

1111111111b     answer all calls

This register is also modified by the AT&L... command.

S14     Serviced SI Mask                                0x80

Every bit enables a service 'cFos' listens to

Bit 0    videophone

Bit 1    telephone

Bit 2    a/b-services

Bit 3    X.21-services

Bit 4    telefax (Group 4)

Bit 5    BTX (64 kbits/s)

Bit 7    data transmission (64 kbits/s)

Bit 8    X.25-services

Bit 9    teletex 64

Bit 10   mixed mode

Bit 15   BTX (new standard)

S15     B-Channel Identification (ffs. 1.TR.6,          0x83

3.2.3.4.4.7)

S16     Source Service Indicator                        7

S17     Source Additional Service Indicator             0

S18     Source EAZ (character: '0'=48.. '9'=57)         '0'

Sent with connect_req message; maybe this has

to be changed on PBX systems.

This register is also modified by an AT&En

command.

S19     Inactivity timer: modem disconnects if          0

no traffic in N minutes; 0==disabled

S20     B-Channel, level 2 protocol                     1

1       X.75 SLP Basis Operation Mode,

mit Implementation Rules nach T.90

2       Transparent-HDLC mit Bit-Stuffing,

Frame-Erkennung und CRC-Check

3       Bittransparent

4       SNA-SDLC

5       X.75 Btx

8       V110 mit transparentem B2 protokoll

9       V110 SNA-SDLC

10      V110 X75 SLP nach T.90

S21     B-Channel, level 3 protocol                     4

1       T70 NL fr Leitungsvermittlung (CSPDN)

2       ISO 8208 (DTE/DTE)

3       Level 3 according to T.90, Appendix II

4       transparent

S22     SELECT_B2_PROTOCOL_REQ: Data-length             2048

S23     SELECT_B2_PROTOCOL_REQ: Link-Address-A          3

S24     SELECT_B2_PROTOCOL_REQ: Link-Address-B          1

S25     SELECT_B2_PROTOCOL_REQ: Modulo-Mode             8

S26     SELECT_B2_PROTOCOL_REQ: Window-Size             2

S27     SELECT_B2_PROTOCOL_REQ: V110 user rate          64

S28     Maximum SEND B2-Framesize                       2048

S29     B3-Protocol-Overhead in Bytes                   0

b3 == 1 (t70nl): 2

b3 == 2 (iso8208): 3

b3 == 4 (transparent): 0

S30     SELECT_B3_PROTOCOL_REQ: LIC                     0

S31     SELECT_B3_PROTOCOL_REQ: HIC                     0

S32     SELECT_B3_PROTOCOL_REQ: LTC                     1

S33     SELECT_B3_PROTOCOL_REQ: HTC                     1

S34     SELECT_B3_PROTOCOL_REQ: LOC                     0

S35     SELECT_B3_PROTOCOL_REQ: HOC                     0

S36     SELECT_B3_PROTOCOL_REQ: Modulo-Mode             8

*       S37     SELECT_B3_PROTOCOL_REQ: Window-Size             2

(CAPI 2.0 only)

S39     Numbering Type/Plan                             0x80

S40     Controller                                      0

S41     Info-Mask-low                                   63

S42     Info-Mask-high                                  0

S43     Default V110 user rate & Protocol               64

S44     Audio Mode for incoming calls                   0

0       Voice mode

1       Modem mode

{++}    S45     X.25 / X.31 delivery confirmation               0

{++}    S46     X.25 / X.31 group/channel                       0

{++}    S47     Baudrate to return on CONNECT, if S10.7=1       9600

{++}    S48     Suppress the first <s48> RINGs.                 0

{++}    S49     number of digits of caller id to ignore         0

(from right to left) when called with CCB

S50    \

S51     |

S52     |   Default Additional Service Indicator and B2-Protocol

S53     |   for every EAZ:

S54     |   If the incoming AddSI is 0 then we get the AddSI

S55     |   from the low byte of register 50+EAZ; the B2-Protocol

S56     |   is got from the high byte.

S57     |

S58     |

S59    /

S60   charge limit, main port     \

S61   charge limit, aux. port      |

S62   time limit, main port (secs) |

S63   time limit, aux. port (secs) |

S64   lower cps limit              | {++} see CFOS.DOC, chapter 12

S65   upper cps limit              |

S66   charge simulation timer      |

S67   pre-off disconnect time      |

S68   connect tries                |

S69   delay between dyn.CCB tries /

-----------------------------------------------------------------------

The following registers, marked with (*) are reserved for cFos

for CAPI 2.0 (currently under development) and cannot be used yet.

*      S100  CAPI 2.0  B1 Protocol                             0

0        64 kBit/s, HDLC framing (CAPI 2.0 default)

1        64 kBit/s, bit transparent

2        V.110 async, w/ start/stop byte framing

3        V.110 sync, w/ HDLC framing

4        T.30 for FAX group 3

5        64 kBit/s inverted w/ HDLC framing

6        56 kBit/s, bit transparent

*      S101  CAPI 2.0  B2 Protocol                             0

0        ISO 7776 (X.75 SLP) (CAPI 2.0 default)

1        Transparent

2        SDLC

3        LAPD, Q.921 for D channel X.25

4        T.30 for FAX group 3

5        Point to Point Protocol (PPP)

6        Transparent (ignoring framing errors of B1)

*      S102  CAPI 2.0  B3 Protocol                             0

0        Transparent (CAPI 2.0 default)

1        T.90NL w/ compatibility to T.70NL

2        ISO 8208 (X.25 DTE-DTE)

3        X.25 DCE

4        T.30 for FAX group 3

*      S103  CAPI 2.0  B1 Protocol Config, Rate                0

*      S104  CAPI 2.0  B1 Protocol Config, Bits per Char       8

*      S105  CAPI 2.0  B1 Protocol Config, Parity              0

0=no, 1=odd, 2=even

*      S106  CAPI 2.0  B1 Protocol Config, Stop Bits           0

0=1 stop bit, 1=2 stop bits

*      S107  CAPI 2.0  CIP mask, low                           4

Bit 0       don't provide CIP value

(all other bits must be 0)

Bit 1       speech

Bit 2       unrestricted digital information

Bit 3       restricted digital information

Bit 4       3.1 kHz audio

Bit 5       7 kHz audio

Bit 6       video

Bit 7       packet mode

Bit 8       56 kBit/s rate adaption

Bit 9       unrestr. digital info. w/ tones

Bit 10..15  reserved

*      S108  CAPI 2.0  CIP mask, high                          0

Bit 0       telephony

Bit 1       FAX group 2/3

Bit 2       FAX group 4, class 1

Bit 3       teletex, basic & mixed mode

Bit 4       teletex, basic & processable mode

Bit 5       teletex, basic mode

Bit 6       international interworking f. videotex

Bit 7       telex

Bit 8       X.400

Bit 9       X.200

Bit 10      7 kHz Telephony

Bit 11      video telephony, first connection

Bit 12      video telephony, second connection

Bit 13..15  reserved

*      S109  CAPI 2.0  Info mask, low                        127

Bit 0       Cause info

Bit 1       Date/Time

Bit 2       Display

Bit 3       User-user info

Bit 4       Call progression

Bit 5       Facility

Bit 6       Charging info

Bit 7..15   reserved

*      S110  CAPI 2.0  Info mask, high                         0

Bit 0..15   reserved

*      S111  CAPI 2.0  CIP calue                               2

-----------------------------------------------------------------------

cFos Modem String Register Chart

-----------------------------------------------------------------------

*      &S0   CAPI 2.0  Bearer Capability, BC

string register of 14 bytes

*      &S1   CAPI 2.0  Low Level Compatibility, LLC

string register of 17 bytes

*      &S2   CAPI 2.0  High Level Compatibility, HLC

string register of 5 bytes

-----------------------------------------------------------------------

cFos Numerical Result Codes

-----------------------------------------------------------------------

0     OK

1     CONNECT

2     RING

3     NO CARRIER

4     ERROR

5     ( not used )

6     NO DIALTONE

7     BUSY

8     NO ANSWER

9     ( not used )

10     ( not used )

11     RINGING