Filename: AIODISK.DOC

Product:  NetWare(R) Connect(tm)

Date:     9 June 1995

=========================================================================

1.0 CONTENTS

============

2.0 What are AIO Drivers?

3.0 Installing the AIO Drivers

4.0 Listing of AIO Drivers

5.0 Trademarks

==========================================================================

2.0 WHAT ARE AIO DRIVERS?

=========================

AIO drivers allow third party asynchronous adapters to be used with

Novell(R) products such as NetWare Connect and NetWare Global MHS(tm).

These adapters provide a wide range of performance and packaging variations

depending upon your needs.

These drivers are provided as a convenience to ensure that you have third

party AIO drivers to function with your Novell product.

These drivers have been tested and certified under Novell Labs(tm)

certification program.  However Novell Inc. does not guarantee or support

these drivers.  Please contact the appropriate vendor if you have problems.

This disk, which is in standard DOS format, contains a number of

subdirectories, each dedicated to a particular vendor. Within these

subdirectories you will find the drivers, any support files and a

documentation file (with a name like AIOCOMX.DOC).  These documentation

files can be read or printed with the normal DOS utilities.

These files describe the particular driver, how to install it, any

particular hardware configuration details that might be relevant and also

any information that would be useful when using the driver with a

particular Novell product such as NetWare Connect or NetWare Global MHS.

In case of problems please call Novell technical support at

800 NETWARE (800-638-9273).

========================================================================

3.0 INSTALLING THE AIO DRIVERS

==============================

The documentation shipped with NetWare Connect and NetWare Global MHS

products describe the AIO driver installation in relation to the needs of

the respective products.  In general, the driver files need to be placed

in the SYSTEM directory on the server and then loaded by means of manual

commands at the server console or automatic commands within the

AUTOEXEC.NCF file.

The files are transferred onto the server by logging onto the server from

a workstation and copying from the workstation floppy disk into the

SYSTEM directory.

The driver is loaded after the AIO NLM(tm) (provided with the Novell

product) using the command line parameters (such as I/O ports and

Interrupts) as described in the driver documentation (or in the text file

provided with the driver on this disk).

Typically the driver will prompt the user for needed information (or use

defaults) if they are not entered on the command line. The driver will

usually give a message indicating correct (or incorrect) loading.  Having

successfully loaded the driver, the product using it can then be loaded

(for example, NetWare Connect).

It is recommended that the driver be loaded manually at first to verify

the parameters needed, once the correct operation is achieved the commands

can be incorporated in the AUTOEXEC.NCF file, to allow automatic loading

when the server is started.

Where multiple boards of the same type are used in the server the driver is

typically loaded once for each board with different parameters.

For example, to load the AIOCOMX driver for COM1, use the following

commands:

load aio

load aiocomx

AIOCOMX uses the defaults for COM1 (I/O port 3f8, Interrupt 4). If you

wish to use other settings they should be entered on the command line as

shown below:

load aiocomx port=2f8 int=3 name=faxport

This will use the hardware at the specified address and interrupt and give

the board the desired name.  Some applications such as Global MHS require

that board be named and if the driver does not give the board a name as

default, then it must be specified on the command line. (AIOCOMX gives the

default name COM1 to the first port and COM2 to the second.)

Once you have verified that the driver works by using it with the Novell

product, you should place the load statement into the AUTOEXEC.NCF file

using the INSTALL program.  When the server is started the driver should

be automatically loaded, however, make sure that you place the load command

after the AIO NLM is loaded.

Individual drivers may have additional parameters depending upon the

particular hardware and software.  See the driver documentation for those

details.

Each board using a particular driver will have a unique board number.

AIO requires that boards be numbered starting from zero and that all

boards using an individual driver have contiguous numbers.  This

requirement may lead to the need to include a "node=" parameter on the

command line with some drivers.  If it is not included AIO will not find

the board or the ports on that board. For example:

load aiocomx port=2f8 int=4 node=0 name=COM2

This will set the board number for this invocation of the driver to zero.

Succeeding load commands for the driver require "node=1", "node=2" ...

Normally COM2 would be assigned the board number 1 (leaving 0 for COM1).

Without this parameter AIO would not be able to find COM2 if COM1 was not

used; if for example it was allocated for use by a non-AIO application

such as UPS power-fail notification.

Most drivers will only require the "node" parameter under unusual

conditions such as the one indicated in the above example.  Some drivers

use the keyword "board" as synonomous with "node".  Not all drivers support

this parameter; in such cases additional caution is required in configuring

the hardware, for more details see the documentation on the individual

board.

===========================================================================

4.0 LISTING OF AIO DRIVERS

==========================

Following is a directory tree of drivers contained with this release:

�

����AIODISK.DOC                 This file

�

����AIOHWTYP.TXT                List of AIO drivers hardware types

�

����COMTROL                     Comtrol Driver(s)

�   �

�   ����ROCKET                  RocketPort-RA Directory

�           AIORCKT.NLM         Driver for RocketPort/RocketPort-RA Adapters

�           AIORCKT.DOC         Doc File for RocketPort Driver

�           AIORCKT.ADI         AIORCKT.NLM Driver ADI File

�

����CUBIX                       Cubix Driver(s)

�   �

�   ����MP2008                  MP2008 Directory

�           AIOCUBIX.NLM        Driver for MP2008 Adapters

�           AIOCUBIX.DOC        Doc File for MP2008 Driver

�           AIOCUBIX.ADI        AIOCUBIX.NLM Driver ADI File

�

����DIGI                        Digi, International Driver(s)

�   �

�   ����XALL                    PC/Xe, MC/Xi Directory

�   �       AIODGXI.NLM         Driver for PC/Xe and MC/Xi Adapters

�   �       AIODGXI.DOC         Doc for PC/Xe and MC/Xi Driver

�   �       AIODGMEM.NLM        Auxiliary Memory Manager

�   �       AIODGXI.ADI         AIODGXI.NLM Driver ADI File

�   �

�   ����XEM                     PC/Xm, PC/Xr, PC/Xem Directory

�           AIODGXEM.NLM        Driver for PC/Xm, PC/Xr, PC/Xem Adapters

�           AIODGXEM.DOC        Doc for PC/Xm, PC/Xr, PC/Xem Driver

�           AIODGMEM.NLM        Auxiliary Memory Manager

�           AIODGXEM.ADI        AIODGXEM.NLM Driver ADI File

�

����EQUINOX                     Equinox Driver(s)

�   �

�   ����SST                     SST Directory

�           AIOEQNX.NLM         Driver for SST Adapter

�           AIOEQNX.DOC         Doc File for SST Driver

�           AIOEQNXI.ADI        AIOEQNX.NLM ISA Driver ADI File

�           AIOEQNXE.ADI        AIOEQNX.NLM EISA Driver ADI File

�           AIOEQNXM.ADI        AIOEQNX.NLM MCA Driver ADI File

�

����MICRODYN                    Microdyne Driver(s)

�   �

�   ����WNIM3000                WNIM3000 Directory

�           AIOWNIM3.NLM        Driver for WNIM3000 Adapter

�           AIOWNIM3.DOC        Doc File for WNIM3000 Driver

�           AIOWNIM3.ADI        AIOWNIM3.NLM Driver ADI File

�

����NEWPORT                     Newport Systems Solutions Driver(s)

�   �

�   ����ACI                     ACI Directory

�           AIOACI.NLM          Driver for ACI Adapter

�           AIOACI.DOC          Doc File for ACI Driver

�           AIOACI.ADI          AIOACI.NLM Driver ADI File

�

����NOVELL                      Novell Driver(s)

�   �

�   ����COMX                    Comx Directory

�           AIOCOMX.NLM         Driver for Comx Ports

�           AIOCOMX.DOC         Doc File for Comx Port Driver

�           AIOCOMX.ADI         AIOCOMX.NLM Driver ADI File

�

����TCL                         Technology Concepts, LTD Driver(s)

�

����SUPER186                Superport-186 Directory

AIOTCL.NLM          Driver for Superport-186 Adapter

AIOTCL.DOC          Doc File for Superport-186 Driver

TCALOAD.NLM         Support Files for Superport-186 Driver

TCMANUAL.DOC        Doc File for TCLOAD Loader

TCALMBX.NLM         Support Files for Superport-186 Driver

PCCHW.DEF           Support Files for Superport-186 Driver

PCCID.BIN           Support Files for Superport-186 Driver

PCCASYNC.SYS        Support Files for Superport-186 Driver

AIOTCL.ADI          AIOTCL.NLM Driver ADI File

===========================================================================

5.0 TRADEMARKS

==============

Third-party trademarks are identified in this file with an asterisk (*).

Novell, Inc. has attempted to supply trademark information about company

names, products, and services mentioned in this file. The following list

of trademarks was derived from various sources.

Novell and NetWare are registered trademarks of Novell, Inc. NetWare

Connect, NetWare Global MHS, Novell Labs, and NLM are trademarks of

Novell, Inc.

(C) Copyright 1993-95 Novell, Inc. All rights reserved.

===========================================================================

** End of AIODISK.DOC **