Software Manual

Digi Intelligent Asynchronous

Serial Communications Boards

NetWare Device Drivers

92000162B

DigiWARE, AccelePort Xe, AccelePort 4e, AccelePort 8e, PC/Xe, PC/2e, PC/4e,

PC/8e, MC/Xe, MC/2e, MC/4e, MC/8e, PC/Xi, PC/8i, PC/16i, MC/Xi, MC/4i, MC/8i,

and the Digi logo are trademarks of Digi International Inc.  All other

brand and product names are the trademarks of their respective holders.

Digi International Inc.  1993-1995

All Rights Reserved

Digi International

6400 Flying Cloud Drive

Eden Prairie, MN 55344

Phone	(800) 344-4273

(612) 943-9020

FAX     (612) 943-5398

BBS     (612) 943-0550

Digi GmbH

Domkloster 1

50667 Koln

Germany

Phone	+49 221 920520

FAX 	+49 221 9205210

BBS     +49 221 9205211

Digi Asia Pte Ltd

13-06 Tower `A'

391A Orchard Road

Ngee Ann City

Singapore 0923

Phone	+65 732 1318

FAX 	+65 732 1312

BBS     +65 735 2460

Information in this document is subject to change without notice and does

not represent a commitment on the part of Digi International.

Digi provides this document "as is", without warranty of any kind, either

expressed or implied, including, but not limited to, the implied warranties

of fitness or merchantability for a particular purpose.  Digi may make

improvements and/or changes in this manual or in the product(s) and/or the

program(s) described in this manual at any time.

T

his product could include technical inaccuracies or typographical errors.

Changes are periodically made to the information herein; these changes may

be incorporated in new editions of the publication.

Novell NetWare AIO Driver

Digi's support for Novell compatible asynchronous applications is provided

through a universal driver conforming to the "AIO" (Asynchronous Input/Out-

put) specification.  Use the AIO driver when Digi hardware is being installed

in the file server (or machine with run-time NetWare 3.X).  When Digi hardware

is installed in a machine other than the server (for instance NetWare Access

Server v1.3) use the specific driver for that application.

AIO drivers are loaded on the file server from the system console (or RCONSOLE)

command line or the AUTOEXEC.NCF file.  Installation is simply a matter of

copying of the driver files AIODGXI.NLM and AIODGMEM.NLM. from the \NETWARE\AIO

directory of the distribution diskette.

The driver will support up to seven PC/Xe, AccelePort Xe and/or PC/Xi boards

on ISA and EISA bus servers.  PC/Xe, AccelePort Xe and PC/Xi boards may be

configured to use the same memory address space ("stacked") or may be con-

figured for unique addresses.  On Micro Channel bus servers the driver will

support up to seven MC/Xe and/or MC/Xi boards.  MC/Xe and MC/Xi boards may not

share memory address space.

PC/Xi boards must be configured for 64K of dual ported memory.  Refer to the

PC/Xi Installation Guide for information on configuring the PC/Xi board's

memory.

Installation

Hardware Installation

Before proceeding with the software installation for the Novell AIO device

driver, you must first install the board(s) according to the instructions in

the Installation Guide for your board.

Software Installation

To install the Digi AIO driver, simply copy the relevant files to your file

server (although the driver can be loaded from the file server floppy, this

will not be a typical way to set-up a system).  The Digi AIO driver files

(AIODGXI.NLM, AIODGXI.ADI and AIODGMEM.NLM) should be copied from the

\NETWARE\AIO directory of the distribution diskette to the system directory of

the file server, typically called F:\SYSTEM.  Copy the files from a workstation

to the file server with the commands:

COPY A:\NETWARE\AIO\AIODGXI.NLM  F:\SYSTEM

COPY A:\NETWARE\AIO\AIODGMEM.NLM F:\SYSTEM

COPY A:\NETWARE\AIO\AIODGXI.ADI  F:\SYSTEM

PC/Xe, AccelePort Xe and PC/Xi Boards

PC/Xe, AccelePort and PC/Xi boards may be installed in either ISA or EISA

platforms.  In either case, the driver must be loaded for each board you wish

to support.  The syntax for loading the driver is as follows:

load aiodgxi port=xxx mem=xxxxx [name=bname] [node=0]

Note:   The above is a single command, and must be entered on one command

line.

port=xxx

This parameter is the board's 3 digit hex I/O port address.

For example:

port=320.

Valid port addresses are 100, 110, 120, 200, 220, 300 and 320.

Each board must have a unique I/O port address, which must not be used by

any other device in the system, including other Digi products. All PC/Xe,

AccelePort Xe and PC/Xi boards may share the same memory addresses, since

only one board's memory is turned on at a time.

name=bname:

This parameter is optional and allows you to specify up to a 10 character

name to identify the board.  bname may consist of any combination of

alphanumeric characters.  If you do not specify a name, one will be

assigned by the driver.  Following is a list of the I/O port addresses and

the associated default names and board numbers.

Port Address        Default Name        Board Number

100h            DigiXi00                0

110h            DigiXi01                1

120h            DigiXi02                2

200h            DigiXi03                3

220h            DigiXi04                4

300h            DigiXi05                5

320h            DigiXi06                6

node=0

This parameter must be used if you are using AIO.NLM version 1.0.2 or

earlier.  These versions of AIO need sequential board numbers based on the

load order of the driver.  (The first board loaded will be assigned #0, the

second board will be assigned #1 and so on.)  Using the node=0 parameter

informs the driver to assign board numbers sequentially.  If this parameter

is excluded, the driver will assign board numbers based on the I/O port ad-

dress as defined above.  The node=0 parameter is only effective when it is

in the first load aiodgxi command.

MC/Xe and MC/Xi Boards

The driver supports up to seven MC/Xe and/or MC/Xi boards on Micro Channel

bus servers.

There are two ways to load the driver on these servers:  You may load the

driver for one board at a time or you may autoload the driver for all boards

in the system (if you do not specify values for port, mem or name, the driver

will auto-load all Digi intelligent serial boards present in the system).

To load the driver for individual MC/Xe or MC/Xi boards, use the following

command syntax:

load aiodgxi [port=xxx mem= xxxxx [name=bname]][node=0]

The command line parameters are the same as for the ISA board installation.

See above for explanations of the parameters.

To auto load the driver for all Digi intelligent serial boards in the computer,

omit the port, mem and name parameters; the driver will get the I/O port and

memory address information from the computer's POS registers and assign the

following default board names based on the board's I/O port address:

Port Address        Default Name        Board Number

108h            DigiXi00                0

118h            DigiXi01                1

128h            DigiXi02                2

208h            DigiXi03                3

228h            DigiXi04                4

308h            DigiXi05                5

328h            DigiXi06                6

Error Messages

The following error messages may occur while the driver is parsing the

parameters.

*Usage Error: AIODGXI port=XXXX mem=XXXXX [name=bname].

This error will appear if you load the driver with no parameters on ISA and

EISA bus servers.

*Usage Error: First parameter not 'port='.

The driver expects the first parameter to be port=xxx, but this was not the

case.

*Error: Invalid Port Address 0x310.

This message indicates that an invalid port address was specified.

*Error: Port Address 0x320 is already supported.

This message indicates that the driver is already supporting a board at this

port address.

*Usage Error: Second parameter not 'mem='

The driver expects the second parameter to be mem=xxxxx, but this was not

the case.

*Error: Invalid Memory Address 0xF0000.

This message indicates that an invalid memory address was specified.

*Error: Memory Address 0xC0000 is already supported.

This message indicates that the driver is already supporting a board at the

specified memory address.  This message applies only to MC/Xe and MC/Xi boards

since these boards must use different memory addresses.

The following error messages may occur while the driver is performing some

initial tests on the board.

Board 4: Port 220 Mem D0000 Port 220 not found.

This message indicates that the driver could not find a board with port

220h.  Check to insure that you have specified the correct I/O port or that

you have properly configured the board's port address.

Board 4: Port 220 Mem D0000 Hardware Reset Fails.

This message indicates that the driver could not place the board into a reset

state.  This is most likely the result of an I/O port address conflict or a

faulty board.  Be sure that there are no other I/O ports in your system at the

indicated I/O port address.

Board 4: Port 220 Mem D0000 Memory not found error x.

This message indicates that the driver could not find the board's memory.  x

indicates the number of memory errors found in a 256 byte block.

Board 4: Port 220 Mem D0000 Memory error x.

This message indicates that the driver has found the board's memory, but it

appears to be faulty.  x indicates the number of errors found in a 256 byte

block.

Board 4: Port 220 Mem D0000 Memory fails disable.

This message indicates that the board's memory could not be disabled.

This message will not occur on MC/Xe or MC/Xi boards.

The following messages may occur during the driver's attempt to initialize the

board.

Board 4: Port 220 Mem D0000 BIOS fails initialization.

This message indicates that the board's on-board BIOS did not initialize

properly.

Board 4: Port 220 Mem D0000 BIOS passes init but reports 0 ports.

This message indicates that the BIOS initialized successfully, but could not

find any of the asynchronous channels.

Board 4: Port 220 Mem D0000 NMI Fails during FEPOS code move.

This message indicates a failure during transfer of the on-board operating

system code.

Board 4: Port 220 Mem D0000 NMI Fails during FEPOS execution.

This message indicates a failure during startup of the on-board operating

system.

Board 4: Port 220 Mem D0000 FEPOS Fails Initialization.

This message indicates that the on-board operating system did not initialize

successfully.

Board 4: Port 220 Mem D0000 Command failure during Initialization.

This message indicates that the on-board operating system encountered a

failure during initialization of the asynchronous channels.

Digi Support Services

The Digi Bulletin Board System

Digi provides an electronic bulletin board service (BBS) for our customers.

This bulletin board provides general and technical information about Digi's

products.

The Digi BBS allows users to download software drivers as soon as they

become available.  There is also a feature to allow users with problems or

questions about Digi products to leave messages to Digi Technical Support.

Using the Digi BBS is easy.  Simply dial (612) 943-0550.  In Europe, dial +49

221 9205211; in Asia, dial +65 735 2460. The bulletin board accepts calls at

1200, 2400, 9600 and 14.4K baud.  V.32, HST 14.4, V.42 and V.42bis

standards are supported, with full MNP class 1-5 error correction and data

compression.

The recommended modem communications parameters are 8 bits, no parity and

one stop bit (8 N 1).  Other settings may also work.  Download protocols

include Zmodem, Xmodem, Ymodem, Kermit and others.

Internet FTP Server

Digi has set up an Anonymous FTP server for those with access to the Internet

network.  The address is ftp.digibd.com.  Log in as anonymous, and enter

your E-mail address when asked for a password.  Drivers and installation tips

are located in the /drivers directory.  A text file, download.doc, gives

information on uncompressing the files after downloading.  Tip:  Be sure to

enter "bin" before downloading, to ensure binary transfer of files.

World Wide Web Server

Product information, manuals, new product announcements, programs, application

stories and more can be obtained through the World Wide Web.  Our address is

http://www.digibd.com.

DigiFACTs FaxBack Server

Manuals and technical information can also be obtained by FAX.  To use the

FaxBack server, simply call (612) 943-0573 on a touch tone phone.

Technical Support

At Digi, we are proud of our products, and support them. Our dealers and

distributors are fully trained on our product line, so that they can help you

on a technical level should assistance be needed.

Your first level of support is your Digi dealer, the place where you purchased

your  Digi product.  Your dealer has the training to help you with any in-

stallation questions or difficulties you might have.

If you still experience difficulties (after contacting your first level of

support), Digi has a staff of Technical Support Specialists that can assist

you.  They can be reached at (612) 943-0578.  In Europe, call +49 221 920520,

and in Asia, call +65 732 1318.  FAX numbers are: (612) 943-0579 (USA),

+49 221 9205210 (Europe) and +65 732 1312 (Asia).

When you call Digi Technical Support, please call from a position where you

can operate your system.  Also, please fill out the form on the facing page

before calling, so your Technical Support representative can have a clear

picture of your system and any potential conflicts between devices.

Digi Technical Support can also be reached via Internet E-mail.  Please send

correspondences to support@digibd.com, and include your voice and FAX phone

numbers.

Customer Service

Digi also has a staff of Customer Service representatives to help you with

software and documentation update requests, as well as Returned Merchandise

Authorizations (RMAs) in case you need to return your board to Digi for repair.

They can be reached at (612) 943-0577.

Digi Customer Service can also be reached via Internet E-mail.  Please send

correspondences to cust_serv@digibd.com,  and include your voice and FAX phone

numbers.