Software Manual

Digi Xem Systems

Novell AIO Driver

92000137C-1

DigiWARE, PC/Xem, EISA/Xem, MC/Xem, PORTS/16em, PORTS/8em, PORTS/8emp,

Xr, and the Digi logo are trademarks of Digi International Inc.  All other

brand and product names are the trademarks of their respective holders.

Digi International  Inc.  1993-1995

All Rights Reserved

Digi International

6400 Flying Cloud Drive

Eden Prairie, MN 55344

Phone	(800) 344-4273

(612) 943-9020

FAX     (612) 943-5398

BBS     (612) 943-0550

Digi GmbH

Domkloster 1

50667 Koln

Germany

Phone	+49 221 920520

FAX 	+49 221 9205210

BBS     +49 221 9205211

Digi Asia Pte Ltd

13-06 Tower 'A'

391A Orchard Road

Ngee Ann City

Singapore 0923

Phone	+65 732 1318

FAX 	+65 732 1312

BBS     +65 735 2460

Information in this document is subject to change without notice and

does not represent a commitment on the part of Digi International. Digi

provides this document "as is", without warranty of any kind, either ex-

pressed or implied, including, but not limited to, the implied warranties

of fitness or merchantability for a particular purpose.  Digi may make

improvements and/or changes in this manual or in the product(s) and/or

the program(s) described in this manual at any time.  This product could

include technical inaccuracies or typographical errors.  Changes are

periodically made to the information herein; these changes may be

incorporated in new editions of the publication.

Introduction

Overview

Digi's support for Novell compatible asynchronous applications is provided

through a universal driver conforming to the "AIO" (Asynchronous

Input/Output) specification.  Use the AIO driver when Digi hardware is

being installed in the file server (or machine with run-time NetWare 3.X).

AIO drivers are loaded on the file server from the system console (or

RCONSOLE) command line or the AUTOEXEC.NCF file.  Installation is simply

a matter of copying of the driver files AIODGXEM.NLM and AIODGMEM.NLM.

Supported Boards

This driver supports up to eight EISA/Xem host adapters, seven PC/Xem host

adapters, or four MC/Xem host adapters.  Each Xem host adapter supports up

to four Digi PORTS/16em, PORTS/8em and/or PORTS/8emp EBI modules, so the

total number of ports supported by the driver is 512 (fully loaded EISA

configuration).

On Micro Channel bus servers the driver will support up to four MC/Xem host

adapters, but your server may be limited to two boards due to conflicts with

the network adapter card and ESDI hard disk controller.

Unsupported Hardware Features

This driver uses no interrupts from the Xem host adapter.  When configuring

the MC/Xem host adapter with the POS configuration utility, the board's

interrupt vector should be disabled.

This driver does not support the ninth (parallel printer) port on PORTS/8emp

modules.  The driver will report nine serial ports, but the ninth port is

unusable since AIO does not provide support for parallel ports.

Installation

To install the Digi AIO driver, simply copy the relevant files to your file

server (although the driver can be loaded from the file server floppy, this

will not be a typical way to set-up a system).  The Digi AIO driver files

(AIODGXEM.NLM, AIODGXEM.ADI and AIODGMEM.NLM) should be copied to the

system directory of the file server, typically called F:\SYSTEM.

Copy the files from a workstation to the file server with the commands:

COPY A:\AIODGXEM.NLM F:\SYSTEM

COPY A:\AIODGMEM.NLM F:\SYSTEM

COPY A:\AIODGXEM.ADI F:\SYSTEM

Loading the Device Driver

If you are running NetWare Connect 2.0, the driver load command line

is configured from within the NetWare Connect Console.  See the NetWare

Connect 2.0 manual for information on installing AIO device drivers.

Note that the AIODGXEM.ADI, AIODGXEM.NLM AND AIODGMEM.NLM files must

be in the SYS:SYSTEM directory so that Connect 2.0 can select

AIODGXEM.NLM as a loadable driver.

If you are running NetWare Connect 1.x, you must load the driver by hand.

The user supplies information to the driver through a set of command line

parameters at load time.  Some or all of the parameters are optional,

depending upon the type of board you are using.  On ISA and EISA bus

servers, the driver must be loaded for each board.  On Micro Channel bus

servers, the driver may be loaded for each board individually, or it may

be loaded once to support all boards in the server.  The load command has

the following general syntax:

load aiodgxem {parameters}

The final form of the command is determined by which Xem host adapter is

installed, and the bus type of the computer (EISA, ISA or Micro Channel).

The variants of the command syntax are shown below.  Parameters enclosed in

brackets [] are optional parameters.

PC/Xem boards in ISA machines:

load aiodgxem port=xxx mem=xxxxx [name=bname][node=0][altp[=mask]]

PC/Xem boards in EISA machines:

load aiodgxem slot=x port=xxx mem=xxxxx [name=bname][node=0] [altp[=mask]]

EISA/Xem boards in EISA machines:

load aiodgxem slot=x mem=xxxxx [name=bname][node=0][altp[=mask]]

MC/Xem boards:

load aiodgxem [port=xxx mem=xxxxx [name=bname]][node=0]

[altp[=mask[,mask[,mask[,mask]]]]]

(When the port= and mem= parameters are omitted, the driver will

automatically load support for all MC/Xem host adapters found in the

system at supported memory addresses, and the default names will be

used.)

AccelePort Xr Boards:

load aiodgxem port=xxx mem=xxxxx [name=bname][node=0][altp[=mask]]

[fastbaud]

port=xxx

This parameter is the board's 3 digit hex I/O port address.

For example: port=324.  Valid port addresses are 104, 114,

124, 204, 224, 304 and 324 for PC/Xem host adapters, or

108, 118, 128, 208, 228, 308 and 328 for MC/Xem host

adapters (I/O port addresses are not specified for EISA/Xem

host adapters).

Each host adapter must have a unique I/O port address, which

must not be used by any other device in the system.

If this parameter is specified for MC/Xem host adapters, the

mem= parameter must also be specified.

slot=x

The host adapter's EISA slot number.  For example, slot=3.

mem=xxxxx

This parameter is the 5 digit hex starting address of the

board's dual-ported memory.  For example: mem=d0000.  The

device driver supports the following 32K starting addresses:

80000, 88000, 90000, 98000, A0000, A8000, B0000,

B8000, C0000, C8000, D0000, and D8000typically, the

addresses from C0000 to D8000 will be the only addresses

available in your server.  PC/Xem and EISA/Xem host

adapters may share the same memory address, since only one

board's memory is turned on at a time.  MC/Xem host

adapters must each have their own address.

If this parameter is specified for MC/Xem host adapters, the

port= parameter must also be specified.

name=bname:

This parameter is optional and allows you to specify up to a

10 character name to identify the host adapter.  bname may

consist of any combination of alphanumerical characters.  If

you do not specify a name, one will be assigned by the driver.

On MC/Xem host adapters, you can only include the name=

parameter if you also include the port= and mem=

parameters.  Following is a list of the board's I/O port

addresses or slot numbers and the associated default names

and board numbers:

For host adapters in ISA machines:

Port Address        Default Name        Board Number

104h            DigiXem00               0

114h            DigiXem01               1

124h            DigiXem02               2

204h            DigiXem03               3

224h            DigiXem04               4

304h            DigiXem05               5

324h            DigiXem06               6

For host adapters in EISA machines:

Slot Number         Default Name        Board Number

1               DigiXem00               0

2               DigiXem01               1

3               DigiXem02               2

4               DigiXem03               3

5               DigiXem04               4

6               DigiXem05               5

7               DigiXem06               6

8               DigiXem07               7

For host adapters in MCA machines:

Port Address        Default Name        Board Number

108h            DigiXem00               0

118h            DigiXem01               1

128h            DigiXem02               2

208h            DigiXem03               3

228h            DigiXem04               4

308h            DigiXem05               5

328h            DigiXem06               6

node=0

This parameter must be used if you are using AIO.NLM version

1.0.2 or earlier.  These versions of AIO need sequential board

numbers based on the load order of the driver.  (The first board

loaded will be assigned #0, the second board will be assigned #1

and so on.)  Using the node=0 parameter informs the driver to

assign board numbers sequentially.  If this parameter is omitted,

the driver will assign board numbers based on the I/O port address

as defined above.  The parameter is effective only when used in

the first load command, and determines the method by which

subsequently loaded boards will be numbered.

altp[=mask]

The standard wiring for Digi PORTS modules equipped with RJ-

45 connectors places the modem control lines RI (Ring Indicator)

and DCD (Data Carrier Detect) on pins 1 and 10, respectively,

which makes them inaccessible to an 8-pin RJ-45 plug.  While RI

is seldom used, DCD is necessary for most modem applications.

To facilitate the use of 8-pin RJ-45 plugs, a software feature

called "ALTPIN" has been included with this device driver.  When

ALTPIN is active, DCD is swapped with DSR (Data Set Ready),

which is on pin 1 of an 8-pin RJ-45 plug.

This parameter is optional and allows you specify that you want to

activate the driver's ALTPIN option.  When this parameter is

entered as just "altp", all ports on the board will operate with the

ALTPIN option active.  When entered as "altp=mask" the driver

uses the hex mask to determine which ports will use the ALTPIN

feature.

For example :

altp=f0

This will activate ALTPIN on ports 5-8. The mask can be up to 16

hex digits long to activate ALTPIN on up to 64 ports.

altp[=mask[,mask[,mask[,mask]]]]

When autoloading support for all boards in an MCA server, up to

four masks may be specified, one for each host adapter installed.

For example:

altp=1,8,10,80

In this example the first port on the first board, the fourth port

on the second board, the fifth port on the third board and the eighth

port on the fourth board will use ALTPIN.  The mask can be up to 16

hex digits long to activate ALTPIN on up to 64 ports.  When

autoloading, the driver uses the MCA bus's POS (Programmable Option

Select) capabilities to determine the MC/Xem host adapters that are

installed in your server, starting with slot 1.  The first ALTPIN

mask will activate ALTPIN on the board in the lowest numbered slot,

the second mask for the next lowest slot, and so on.

fastbaud

To operate an AccelePort Xr board and 230k baud, include the fastbaud

parameter and set the port's baud rate to 300.  fastbaud maps 300

baud to 230k baud.

Important:

IRQs on MC/Xem host adapters must be set to "Disabled", by using the

POS configuration utility on the Reference Diskette.

Error Messages

The following error messages may occur while the driver is parsing the

parameters.

*Usage Error: AIODGXEM port=XXXX mem=XXXXX [name=bname][node=0].

*Usage Error: AIODGXEM slot=X [port=XXXX] mem=XXXXX [name=bname][node=0].

These errors will appear if you load the driver with no parameters on

ISA and EISA bus servers, respectively.

*Usage Error: First parameter not 'port='.

The driver expects the first parameter to be port=xxx on ISA machines,

but this was not the case.

*Usage Error: First parameter not 'slot='.

The driver expects the first parameter to be slot=x on EISA machines,

but this was not the case.

*Error: Invalid Port Address 0x310.

This message indicates that an invalid port address was specified.

*Error: Port Address 0x304 is already supported.

This message indicates that the driver is already supporting an Xem

host adapter at this port address.

*Error: Invalid Slot 10.

This message indicates that an invalid slot number was specified while

loading the driver on an EISA computer (slots 1-8 are valid).

*Error: Slot 2 is already supported.

This message indicates that the driver is already supporting an Xem

host adapter in this slot on an EISA computer.

*Usage Error: Second parameter not 'mem='

The driver expects the second parameter to be mem=xxxxx in ISA

machines, but this was not the case.  The driver also expects this to be

the second parameter in EISA machines if the port= parameter was

not the second parameter (when using an EISA/Xem host adapter).

*Usage Error: Third parameter not 'mem='

The driver expects the third parameter to be mem=xxxxx in EISA machines

when the port= parameter is the second parameter (specified for ISA/Xem

host adapters).

*Error: Invalid Memory Address 0xF0000.

This message indicates that an invalid memory address was specified.

*Error: Memory Address 0xC0000 is already supported.

This message indicates that the driver is already supporting an Xem

host adapter at the specified memory address.  This message will only

be seen on MCA bus servers since MC/Xem host adapters must use

different memory addresses.

*Error: Failed POS Hardware Registration.

This message indicates that the MCA POS registration was not

accepted by NetWare.

*Error: AllocateResourceTag failed.

This message indicates that the driver could not allocate resources from

NetWare.  Unload unnecessary NLMs and reload the driver.

*Error: AIORegisterDriver failed.

This message indicates that NetWare failed to permit registration of the

driver.

*Error: AllocateSemiPermMemory returns NULL.

This message indicates that the driver's request to allocate memory

failed.  Unload unnecessary NLMs and reload the driver.

No usable boards found. / Installed boards already supported. This

message appears only on MCA servers, and indicates that the driver

is already supporting the board(s) that were found.

The following error messages may occur while the driver is performing some

initial tests on the Xem host adapter.

Board 4: Port 304 Mem D0000 Failed Hardware Registration.

This message indicates that NetWare failed to accept the driver's

attempt to register the board's hardware resources.

Board 4: Port 304 Mem D0000 Port 310 not found.

This message indicates that the driver could not find a board with port

304h.  Check to insure that you have specified the correct I/O port or

that you have properly configured the board's port address.

Board 4: Port 304 Mem D0000 Hardware Reset Fails.

This message indicates that the driver could not place the host adapter

into a reset state.  This is most likely the result of an I/O port address

conflict or a faulty board.  Be sure that there are no other I/O ports in

your system at the indicated I/O port address.

Board 4: Port 304 Mem D0000 Memory not found error x.

This message indicates that the driver could not find the host adapter's

memory.  x indicates the number of memory errors found in a 256 byte

block.

Board 4: Port 304 Mem D0000 Memory error x.

This message indicates that the driver has found the host adapter's

memory, but it appears to be faulty.  x indicates the number of errors

found in a 256 byte block.

Board 4: Port 304 Mem D0000 Memory fails disable.

This message indicates that the host adapter's memory could not be

disabled.  This could be the result of a memory overlap with some

other memory in the system, or a faulty board.  Ensure that no memory

conflict exists.  This message will not occur on MC/Xem host adapters.

The following messages may occur during the drivers attempt to initialize the

board.

Board 4: Port 304 Mem D0000 BIOS fails initialization.

This message indicates that the host adapter's on-board BIOS did not

initialize properly.

Board 4: Port 304 Mem D0000 FEPOS fails initialization.

This message indicates that the on-board operating system did not

initialize successfully.

Board 4: Port 304 Mem D0000 FEPOS finds 0 ports.

This message indicates that the on-board operating system initialized

successfully, but found no ports.  Check to see that the PORTS

modules are connected to the Xem host adapter.

Board 4: Port 304 Mem D0000 Command failure during Initialization.

This message indicates that the on-board operating system encountered

a failure during initialization of the asynchronous channels.

Digi Support Services

The Digi Bulletin Board System

Digi provides an electronic bulletin board service (BBS) for our customers.

This bulletin board provides general and technical information about Digi's

products.

The Digi BBS allows users to download software drivers as soon as they

become available.  There is also a feature to allow users with problems or

questions about Digi products to leave messages to Digi Technical Support.

Using the Digi BBS is easy.  Simply dial (612) 943-0550.  In Europe, dial

+49 221 9205211; in Asia, dial +65 735 2460. The bulletin board accepts calls

at 1200, 2400, 9600 and 14.4K baud.  V.32, HST 14.4, V.42 and V.42bis

standards are supported, with full MNP class 1-5 error correction and data

compression.

The recommended modem communications parameters are 8 bits, no parity and

one stop bit (8 N 1).  Other settings may also work.  Download protocols

include Zmodem, Xmodem, Ymodem, Kermit and others.

Internet FTP Server

Digi has set up an Anonymous FTP server for those with access to the

Internet network.  The address is ftp.digibd.com.  Log in as anonymous,

and enter your E-mail address when asked for a password.  Drivers and

installation tips are located in the /drivers directory.  A text file,

download.doc, gives information on uncompressing the files after

downloading.  Tip:  Be sure to enter "bin" before downloading, to ensure

binary transfer of files.

World Wide Web Server

Product information, manuals, new product announcements, programs,

application stories and more can be obtained through the World Wide Web.

Our address is http://www.digibd.com.

DigiFACTs FaxBack Server

Manuals and technical information can also be obtained by FAX.  To use the

FaxBack server, simply call (612) 943-0573 on a touch tone phone.

Technical Support

At Digi, we are proud of our products, and support them. Our dealers and

distributors are fully trained on our product line, so that they can help

you on a technical level should assistance be needed.

Your first level of support is your Digi dealer, the place where you purchased

your  Digi product.  Your dealer has the training to help you with any in-

stallation questions or difficulties you might have.

If you still experience difficulties (after contacting your first level of

support), Digi has a staff of Technical Support Specialists that can assist

you.  They can be reached at (612) 943-0578.  In Europe, call +49 221 920520,

and in Asia, call +65 732 1318.  FAX numbers are: (612) 943-0579 (USA),

+49 221 9205210 (Europe) and +65 732 1312 (Asia).

When you call Digi Technical Support, please call from a position where you

can operate your system.  Also, please fill out the form on the facing page

before calling, so your Technical Support representative can have a clear

picture of your system and any potential conflicts between devices.

Digi Technical Support can also be reached via Internet E-mail.  Please send

correspondences to support@digibd.com, and include your voice and FAX

phone numbers.

Customer Service

Digi also has a staff of Customer Service representatives to help you with

software and documentation update requests, as well as Returned Merchandise

Authorizations (RMAs) in case you need to return your board to Digi for repair.

They can be reached at (612) 943-0577.

Digi Customer Service can also be reached via Internet E-mail.  Please send

correspondences to cust_serv@digibd.com,  and include your voice and

FAX phone numbers.