# SuperPrint Table of Contents

Introduction

What is SuperPrint?

About this manual

What you absolutely need to know

Minimum System requirements

Conventions used in this manual

Register SuperPrint!

How to get Help

SuperQueue Reference

What is SuperQueue?

About this chapter

SuperQueue quick tour

The main SuperQueue window

The tool bar

The status bar

Printer windows

The queue

Messages

Sounds

Documents

Preview window

Printer activation

Filters

Filter specifications

Bitmap filters

PostScript filter

SuperDriver Reference

Chapter organization

What are SuperDrivers?

Common settings

Papers

Custom Papers

Options

SuperRIP Memory Management

Tricks

Image

Tips for users of color devices

Halftoning options

About...

Laser Printers

Duplex

HP LaserJets

Other PCL-type laser printers

Inkjet Printers

HP DeskJets

HP PaintJets

Canon Bubble Jets

Lexmark ExecJet IIc

Epson Stylus 800

Epson Stylus Color

Dot-Matrix Printers

Thermal Printers

Fargo Primera printer

Star Micronics SJ-144 printer

Star WinType 800C printer

Bitmap SuperDrivers

Installing a Bitmap SuperDriver

Bitmap SuperDriver properties

The Paper tab

Options

SuperDriver status

Masquerade reference

What is Masquerade?

Before installation

Installing a PostScript driver in Windows 95

Installing a PostScript driver in Windows NT

Running Masquerade

Un-doing everything Masquerade has done

Settings for Windows PostScript drivers

Spool settings

Graphics settings

Fonts

Other settings in Windows 95

Printer description files

SuperPrint and Networks

Adding a network printer in Windows 95

Adding a network printer in Windows NT

Sending raw data

Configuring SuperQueue to send raw data

Sending SuperMetafiles (SMFs)

Configuring SuperQueue to send SuperMetafiles

Printer server administration in SuperQueue

Working offline

Index

# Introduction

### What is SuperPrint?

SuperPrint 4.0 has been redesigned from the ground up for Windows 95 and Windows NT. SuperPrint delivers smooth multitasking during printing, with output quality and true 32-bit productivity tools you’ll find nowhere else.

Fast, true 32-bit printing. Get exceptionally fast output and the best multitasking and background printing with SuperPrint.

PostScript Level 2. Turn your non-PostScript printers into powerful, faster PostScript devices. With SuperPrint, there’s no need to buy expensive PostScript-equipped printers to print EPS and PostScript files. Includes screen previews, too.

SuperDrivers. SuperPrint provides 32-bit drives for all popular non-PostScript devices. Each driver is pre-tuned for your printer model by a Zenographics printing specialist to give you fast, high quality printer performance.

Improved File Drivers. We’ve added GIF and JPEG to our Bitmap SuperDriver so you can generate images quickly and easily from any Windows application.

SuperQueue. A completely new SuperQueue offers more control over print job processing, helpful job status information (including job logging), and enhanced, bi-directional network support. The new SuperQueue lets you preview and print bitmap and PostScript files using drag and drop — without the need for another application.

Efficient Network Printing. SuperPrint is the only print solution that lets you print by sending compact metafiles over any Windows compatible network. Our SuperPrint printing architecture reduces network traffic and optimized background printing performance on the client PC.

### About this manual

The SuperPrint manual is organized as follows:

Introduction – with information you absolutely need to know (below).

SuperQueue reference – details on background and batch imaging.

SuperDriver reference – complete explanation of SuperDriver for your printer and its options.

Network information – details on how SuperPrint can work with your network.

### What you absolutely need to know

Even if you don't like reading manuals, please read this section.

### Minimum System requirements

Microsoft Windows 95 or Windows  NT version 3.51 or later

Windows-compatible mouse

Hard disk with 4 Megabytes free disk space after SuperPrint is installed

The extra disk space is for temporary files. In general, you should have at least 4 megabytes free for SuperPrint's temporary files after installing SuperPrint. Much more additional free space may be needed by graphics-intensive programs, especially bitmap editors.

### Conventions used in this manual

#### “Do-this” text

Actions that we want you to perform are always set off by either a bullet or a number and are set in Helvetica Normal text. For example:

Choose the Printers icon in the Control Panel window.

#### What you type

Any text that you are to type verbatim is shown in italics:

Please enter A:\SETUP.

#### Menu items

Menu items and buttons are shown in bold text. For example:

Choose Add Documents… from the Printer menu.

### Register SuperPrint!

Please become a registered user by completing the enclosed, prepaid registration card and mailing it. You may also register electronically: see the box on the registration card that tells you how. Registered users can get SuperPrint help from the Zenographics Answerline and receive timely update information on new versions of SuperPrint.

### How to get Help

You must register SuperPrint in order to receive technical support.

Help for Zenographics SuperPrint is available from several sources.

1. This documentation – If it has to do with the normal operation of the product, or a problem that is not application-specific or hardware-related, chances are the answer can be found here. Be sure to check the index for the topic you need.

2. Help files – SuperPrint has its own help files. If you need a quick explanation of options or procedures while using SuperPrint, it can usually be found in the Help files.

3. SuperHelp –This is our “knowledge base” transferred to the standard Windows help file format. This helpful tips, tricks and troubleshooting file can be  accessed either by clicking on  a SuperHelp button in your SuperDriver's Help tab or by choosing the  SuperHelp icon from the SuperPrint group.

4. Electronic Help – Help is available in several areas:

Find us on our World Wide Web site at http//www.zeno.com.

For CompuServe: Enter GO ZENO or GO WINAPB to access  the Windows 3rd Party B Forum, then select the appropriate section.  Zenographics owns Section 12. Our User ID number is 76702,1351.

Visit us on the Microsoft Network use the Go Word Zeno.  There is on-line technical help available Monday thru Friday from 8:00 - 9:00 am Pacific Time and 2:00 - 3:00 pm Pacific Time in our Chat room.

5. AutoTech – If you have a FAX machine and a touch-tone telephone, you can get 24-hour help from our AutoTech system. When you call in, you will be guided through the system by a friendly voice. Information will be sent to you by FAX after you have finished the call. AutoTech can be reached at (714) 833-7472 (this is a voice line only).

6. FAX Help – You can send a complete description of your problem to our technical services department by FAX. This typically takes less phone time than calling in and describing your situation to our Answerline staff. Be sure to include your serial number, version numbers of all software involved, as well as your system's hardware configuration. Send technical FAX documents to (714) 833-7465.

7. The Answerline – The Answerline technicians accept calls from 7:00 am to 3:00 pm, Pacific Time on Monday, and 7:00 a.m. to 4:00 pm Tuesday through Friday except on U.S. holidays. Please check the About box in SuperPrint for your version and serial number and have it ready when you call.The Answerline phone number is (714) 851-2191.

# SuperQueue Reference

### What is SuperQueue?

SuperQueue provides substantial enhancements to the Windows 95 and Windows NT print spool viewers. In addition to a more complete view of the print queue, SuperQueue offers important new functionality such as:

- Direct support through external filters for file types other than Windows 95 Enhanced Metafiles (EMF), Windows NT “Journal” files, and “raw” printer data files. SuperQueue can print bitmap and PostScript files directly, without the need for the originating application. Additional filters can be installed at any time.

- Screen preview (with zoom) for filter-supported files.

- Clipboard copy of filter-supported files for pasting as Windows metafiles.

- Support for simultaneous printing on multiple devices.

- Added control over the print job queue.

- Job status and tracking information.

### About this chapter

This chapter will cover how to work with:

- Printer windows

- The queue (job list)

- Messages from filters

- Documents (individual jobs)

- Printer activation

- Filters and their options

- SuperQueue’s printing processes

#### Notice to users of Windows NT

This manual shows screen captures of SuperQueue running under Windows 95. You are running the same executable program file as a user of Windows 95, so there are only minor differences in appearance between when you will see and what’s shown here.

## SuperQueue quick tour

The SuperQueue window is similar to that of  the Windows 95 “Printers” folder or Windows NT Print Manager. Each installed printer is represented by a “child” window (this chapter will refer to these as “printer windows”). SuperQueue’s printer windows contain both the list of jobs (the queue) and messages from SuperQueue’s translation filters. Unlike Windows 95’s Printers utility, the individual printer windows in SuperQueue cannot stand alone as separate windows – they are always linked to and contained within SuperQueue’s main window.

Note that most choices you make using either the tool bar or the menus apply only to the currently selected printer. The title bar of the current printer window is highlighted and the name of the current printer normally appears in the status bar at the bottom of the SuperQueue window.

The invisible parts of SuperQueue are the print processing filters. Filters work in the background to translate native format files (bitmaps such as TIFF and BMP, plus PostScript language files) into Windows’ “native language” (GDI).

### The main SuperQueue window

## The tool bar

Each of the functions found on the tool bar is also found in a SuperQueue menu. The graphic on the following page shows the menu equivalent of each button. Note that for the rest of this chapter, most references will be to the menu functions; the tool bar buttons simply provide functionally equivalent shortcuts.

Tool Bar icons from left to right:

1. Add Document  2. Copy   3. Start Printer  4. Pause Printer  5. Set As Default Printer  6.Promote

1     	2        3       4        5         6      7        8       9     10    11    12     13     14

7.  Demote  8.  Properties   9. Release  10.  Hold   11. Remove  12.  Print Preview  13. Index  14. Context

#### How to hide and restore the tool bar

If you are more accustomed to using menus and you want to free up some on-screen space by hiding the tool bar, you can do so as follows:

- In the View menu, uncheck Tool Bar by choosing it.

If the tool bar is hidden and you want to restore it:

- In the View menu, check Tool Bar by choosing it.

## The status bar

The status bar runs along the bottom edge of the SuperQueue window. On the left side, one of two things is displayed:

- If the cursor is directly over a tool bar button or if a menu choice is highlighted, the function of that choice is described, or;

- If the cursor is elsewhere, the currently selected printer is indicated.

Printer-specific choices you make using either the tool bar or the menus apply only to the printer shown here.

On the right side of the status bar, the state of the currently selected printer is shown (Ready or  Paused) and the number of documents in the queue is displayed.

#### How to hide and restore the status bar

If you feel you do not need the information contained in the status bar and you want to free up some on-screen space by hiding it, you can do so as follows:

- In the View menu, uncheck Status Bar by choosing it.

If the status bar is hidden and you want to restore it:

- In the View menu, check Status Bar by choosing it.

## Printer windows

SuperQueue’s multiple window interface makes it possible to monitor the activity of each printer on your system. Each installed printer is represented by a window that displays a job list (or queue) for that printer. Multiple queues can operate simultaneously, one for each printer. Note that if job processing is underway, even if you minimize a printer window or close SuperQueue, the processing will continue in the background.

Each printer window is divided into two main sections: the queue area and the message area. The first time you open a new printer window, only the queue area is visible. If you change the arrangement of the windows or the queue/message areas of a window, SuperQueue remembers the changes for subsequent sessions.

At the bottom of each window is a status bar. This displays the progress of filter translations and also lets you know if any messages have appeared in the messages portion of the printer window.

#### How to view the queue and message areas

Each printer window displays a job list (or queue) for that printer plus a message section that delivers the information generated by the filters.

If only the queue is visible, you can make the message area visible as follows:

- Choose Messages from the View menu, or;

- Click on the dividing bar at the bottom of the queue area and drag it up.

If only the message area is visible, you can make the queue visible as follows:

- Choose Queue from the View menu, or;

- Click on the dividing bar at the top of the message area and drag it down.

In each case, the latter method lets you determine how much screen space you want to give to the queue area vs. the message area. The View menu selections force the entire printer window to be devoted to that choice.

If the printer window is currently divided, showing both the queue and message areas, both the Queue and Messages choices will be available on the View menu.

Explanations of the contents of the queue and message areas are found later in this manual under “The queue” and “Messages.” Also, the tabs found at the bottom of the message area are described in the “Messages” section.

#### How to change the screen font

The font that’s used to display queue and message information can be changed as follows:

- Choose Fonts… from the Options menu.

The Font dialog appears (as shown below).

- Make choices from the Font, Font Style, and Size list boxes.

A sample of your choice appears in the Sample box.

- When you’re finished, choose OK.

#### How to automatically arrange printer windows

SuperQueue’s printer windows conform to the Windows standard Multiple Document Interface (MDI).

- To automatically arrange all open windows so they overlap with their title bars showing, choose Cascade from the Window menu.

- To automatically arrange all open windows as non-overlapping tiles, choose Tile from the Window menu.

- To automatically arrange all minimized printer windows, choose Arrange Icons from the Window menu.

### The queue

#### Reading the queue information

There are six pieces of information displayed for each job in the queue: the document’s name, status, owner, and progress, the date and time the job was added to the queue, and the job’s data type. The bar across the top of the queue area controls the width of each field; by dragging the dividing bar, the width of a field can be changed.

The format of the information in the Document Name column changes depending on several factors. If the job was submitted by a Windows application, it typically begins with the name of the application followed by the document name. If the job is a bitmap file that was loaded using Printer/Add Documents… (or drag-and-drop), it typically shows the file’s directory and filename. If the job is a PostScript file which contains title information (in the form of a “%%Title” remark), that information is shown under Document Name. The implementation of the title remark in PostScript files varies widely by application.

Status displays Spooling while a job is being submitted by an application, Printing while the job is being sent to the printer, and Deleting if the job is being removed from the queue. In addition, if an individual job has been put on hold, the Status column shows Paused.

When printing on a network, the Owner column shows the network names of the client users who generated the files. On a standalone system, this column is blank. If you are on a standalone system, you may want to “collapse” the owner column to give more room to the other columns. To do this, drag the dividing bar on the right of the word Owner all the way to dividing bar on the left.

While a job is printing, Progress shows the sequence of job processing either in pages or in byte counts. For jobs waiting in the queue, the total number of pages or byte counts are displayed.

The date and time at which the job arrived in the queue is shown under Started At.

The last column shows the Data Type. Windows 95 (without SuperPrint) handles two basic data types: EMF (Enhanced MetaFile) and RAW (low-level printer data). Windows NT uses NT Journal files and RAW files. SuperQueue’s filters and print processors allow for many additional data types including:

  SMF (SuperMetaFile)
  PS (PostScript)
  BMP (Windows BitMaP)
  TIF (Tagged Image File Format)
  GIF (Graphics Interchange Format)
  JPG (Joint Photographic Experts Group)

As filters are added to SuperQueue, additional data types may be used.

Note that when printing over a network, this field may show an entry of “RAW” regardless of the actual data type of the job. This behavior is forced by the network messaging subsystem of Windows; it does not affect the actual job transmission or processing.

#### How to change the order of jobs in the queue

When you have several jobs waiting in the print queue, you can re-order (change the priority) of the jobs most easily with the mouse using drag and drop. Alternative methods using the tool bar, the menus, and the shortcut keys are also explained here.

To re-order a job using drag and drop:

- Click on the job’s listing in the Document Name column.

- Drag the job up or down to its new location.

If you are dragging a job up (promoting the job), the job will be inserted above the job onto which you drop it; if you are dragging a job downwards (demoting a job), the moved job will appear below the job onto which you drop it.

#### Other methods of changing a job’s priority

With the job’s listing in the Document Name column highlighted:

- Move the job one place at a time with the Document menu choices or the tool bar buttons called Promote Document (to move it up) or Demote Document (to move it down). You can also use the keyboard shortcuts CTRL+UP and CTRL+DOWN.

- Move the job to the top or bottom of the queue with the Document menu choices Move To Front or Move To Back. You can also use the keyboard shortcuts CTRL+HOME and CTRL+END.

Note that moving a job in front of the currently printing job in the queue will not cause the new  “top” job to interrupt the currently printing job. If you must print the new job right away, you need to drag it to the number two position, then delete the currently printing job using the DEL key or Remove Document as described below.

#### How to remove a single job from the queue

- Highlight the job’s listing in the Document Name column.

- Press the DEL key.

Alternatively, you can either choose Remove Document from the Document menu, or click on the Remove Document button on the tool bar.

#### Remove Document

Jobs which were created by printing from a Windows application are removed from the queue and deleted from disk; they must be reprinted from the application if you want to try printing them again. Jobs added using Add Document or drag-and-drop are removed from the queue, but the original files remain on the disk.

If there are jobs remaining in the queue, SuperQueue will immediately begin processing the new “top” job.

Removing a job that shows “Printing” in the Status column may result in your printer ejecting a partially printed page.

#### How to remove all jobs from the queue

- To clear the queue of all jobs, choose Purge Printer from the Printer menu.

If a job is in progress, it will be terminated. A dialog appears asking you to confirm this choice.

#### Purge Printer

As with Remove Document, jobs created by printing from a Windows application are removed from the queue and deleted from disk. Jobs added using Add Document or drag-and-drop are removed from the queue, but the original files remain on the disk.

#### How to add a job to the queue

When driver spooling is on, jobs which are created by printing directly from a Windows application appear automatically in the print queue. Data files, such as PostScript or bitmap files, must be added manually. There are two methods of adding a job to the queue: using Add Documents from the Printer menu (or tool bar), and “drag and drop.”

To add a job using Add Documents:

- Choose Add Documents from the Printer menu or click the corresponding button on the tool bar.

The Add Documents dialog appears.

- If you intend to preview the job or if you do not want to print the job immediately, check the box next to Hold selected document(s) in printer queue.

- Choose the desired format from the Files of Type list box.

- Navigate to the drive and folder that contains your file.

- Choose your file or files and click Add.

Multiple files can be added by using SHIFT+click for a contiguous range or CTRL+click for selecting multiple files from different parts of the list.

To add a job using drag and drop:

- Locate the data file listing using either Windows Explorer or My Computer in Windows 95, or File Manager in Windows NT.

- Drag the file listing onto the desired printer window within the SuperQueue window.

Note that you can drag a data file onto a minimized printer window (a “brick”) within SuperQueue. However, you cannot drag a file onto the task bar if the SuperQueue window is minimized.

If you drag and drop a file that SuperQueue does not recognize, it will present the following dialog. You must then manually choose the filter to be used for this job.

#### Add Documents

SuperQueue can load and print files in various formats. The acceptable file extensions are determined by the filter itself. Some filters designate more than one extension, so you may need to scroll through the Files of Type list box to find the correct one. If your file is of the correct data type but uses an extension other than the ones supported by the filter, you can either enter a revised wildcard in the File name box of the Add Documents dialog, or change your file's extension using Windows Explorer. If you change the wildcard, the Manual Filter Selection dialog (explained above) appears.

Each document can have its own set of filter preferences. If you haven’t turned off Ask for preferences each job, you can set the preferences in the pop-up dialog that appears when you add the job. If the pop-up did not appear, or if you want to change the settings after adding the job to the queue, you can still set filter preferences for any job that’s not currently being printed using Document Properties. See the section on “Document Properties” later in this chapter for more information.

At the top of the Print page (shown here), you are allowed to set multiple copies for the current document. For output devices set to print to a port, you can force the printer data to be sent to a file instead (for devices already set to FILE:, the Print to file checkbox is redundant). There is also a Properties button that accesses the standard printer properties dialog. This lets you set things like paper size and resolution on a per-job basis.

If the settings that appear in the pop-up dialog are correct, and if you expect that they will not need to be changed, you may want to check Don’t display these options again.

If you click Cancel from the Filter Preferences pop-up dialog, the job is not added to the queue.

The Image tab shows all the information that the filter can glean from the job file. There are no settings you can change; the Image  page is informational only.

The Enhancement tab will only appear if the document you are adding is either an 8-bit grayscale or 24-bit color bitmap.

For specific information on filter options, see the “Filter specifications” section later in this chapter for more information.

#### Adding multiple jobs

If you’ve added multiple jobs, the following dialog appears. You have these options:

•  Modify filter settings for this file – SuperQueue presents the Filter Preferences dialog for this job, then redisplays this Configure Filter dialog for the following job.

•  Accept default filter settings for this file – SuperQueue applies the filter defaults to the current job and redisplays the Configure Filter dialog for the following job.

•  Accept default settings for all remaining files – SuperQueue adds all the selected documents using filter defaults with no further prompts.

### Messages

The lower portion of a SuperQueue printer window provides messages only for jobs that are processed through a SuperQueue filter. This includes SuperMetafiles that are generated when printing from Windows 95 applications. It does not include Windows NT Journal files. Messages typically include the start and finish times for job processing and the total elapsed time with a page count. Other messages may include warnings generated by the filters (these typically do not stop printing) and filter errors (which may stop printing).

To keep a record of filter messages, enable Message Logging in the Options menu before printing. A log file will be created in the temporary folder on your hard disk. To view the message log for the active printer, select the Port tab (such as LPT1:) and choose Log File in the View menu. This option will be grayed and unavailable if there are no messages or if you have not selected a Port tab.

After having created a log file, if you subsequently disable Message Logging, a message box will be displayed asking if you wish to delete all existing message log files. If you say yes, all of the logs for all of your printers will be deleted.

If Message Logging is not enabled, filter messages will accumulate over the course of the current session, but when you close SuperQueue, all unsaved messages are lost. If you start SuperQueue while a filter job is printing, messages for that job will not appear.

Making the message area of the printer window visible is described earlier in this chapter under “How to view the queue and message areas.”

#### How to choose a message tab

SuperQueue “sorts” messages according to the print job’s destination. Typically, there are two tabs at the bottom of the message area: one marked “Preview Messages” and one marked with either the port designation of your local printer (typically LPT1) or the network address of a remote printer.

- To see the activity associated with screen preview (or local processing of a network job when Options/Remote Processing is off), click on the Preview Messages tab.

- To see the activity associated with local printing (or printing a network job with Options/Remote Processing on), click on the appropriate port tab.

#### How to copy messages onto the clipboard

The contents of the message window can be copied, in whole or in part, onto the Windows clipboard for pasting into other applications (typically text editors).

To copy only a portion of the text:

- Choose the tab that corresponds to the target destination (preview or port).

- Highlight the text you want to save

- Choose Copy from the Edit menu. (Alternatively, you can click on the Copy button on the tool bar or enter the keyboard shortcut CTRL+C.)

To copy all of the message text:

- Choose the tab that corresponds to the target destination (preview or port).

Even if the correct tab is on top, this action is necessary to put the “focus” on the message area.

- Choose Select All from the Edit menu.

- Choose Copy from the Edit menu.

#### How to find text in the message area

SuperQueue lets you locate particular words or phrases in the messages portion of the printer window. For example, this procedure makes it possible to locate all occurrences of  “BMP.”

- Choose the tab that corresponds to the target destination (preview or port).

Even if the correct tab is on top, this action is necessary to put the “focus” on the message area.

- Choose Find… from the Edit menu.

- Enter the desired word or phrase in the Find what: box and click Find Next.

Clicking on Find Next repeatedly will cause SuperQueue to find additional occurrences of the search string.

#### How to clear the contents of the message area

- Choose the tab that corresponds to the target destination (preview or port).

Even if the correct tab is on top, this action is necessary to put the “focus” on the message area.

- Choose Clear All from the Edit menu or use the keyboard shortcut CTRL+DEL.

#### Edit menu

All choices on the Edit menu are grayed (as are their tool bar equivalents) unless Windows’ “focus” is on the message area of the current printer window and the message area contains some text. The focus is most easily recognized by a flashing text cursor. Focus can be transferred to the message area by either clicking on the appropriate port tab or by clicking within the message text itself.

### Sounds

SuperQueue comes with a series of .WAV files that announce printer events such as, “Your printer is out of paper” and “Job complete.” If you have a sound card and speakers installed on your computer, you should hear these sounds automatically. If you do not want the sounds, uncheck Sound on the SuperQueue Options menu.

## Documents

For purposes of this chapter, “document” and “print job” are interchangeable. Each line in the print queue represents one document. Documents can be generated by printing from Windows applications or they can be in the form of data files (such as TIFF or PostScript). When spooling is on, documents from Windows applications are added to the queue automatically; data file documents are manually added as described earlier in this chapter under “How to add a job to the queue.”

#### How to preview a document

- Choose Add Document from the Printer menu.

- Check the box next to Hold selected document(s) in printer queue.

- Choose the desired format from the Files of Type list box.

- Navigate to the drive and folder that contains your file.

- Choose your file and click Add.

- Highlight the name of the job in the queue window.

- Choose Print Preview… from the Document menu.

During processing by the appropriate filter, a status dialog appears as shown. When processing is finished, the preview window appears. It defaults to a single-page, “fit to window” mode. You can resize the preview window and the preview page will expand accordingly.

### Preview window

At the top of the preview window is a row of buttons that give you access to several functions.

All previewed jobs can take advantage of  Zoom In and Zoom Out. Aside from the fit to window view, there are two zoom levels. If you are zoomed in all the way, the Zoom In button will gray. Similarly, Zoom Out is grayed when you are in fit to window mode. Note that the mouse cursor can be used to zoom in as well. The zoomed view is centered over where you click.

Multi-page jobs can take advantage of  Next Page, Previous Page, and Two Page. As you might expect, Next Page and Previous Page let you scroll through your job a page at a time. Two Page gives you a side-by-side view of two pages at once.

Copy places a Windows metafile copy of your artwork on the Windows Clipboard. You can then paste the artwork into other Windows applications. Programs that have an “ungroup” function (such as CorelDRAW) or graphics editing capabilities of their own (such as Microsoft Word) can be used to modify the artwork if you want. For multi-page jobs, the currently displaying page is the only one placed on the clipboard.

Save gives you a chance to save the metafile in either EMF (Windows 95 Enhanced Metafile) or WMF (older-style Windows Metafile) formats. You choice of formats may depend on the application into which you want to import the artwork.

Close returns you to the SuperQueue main window. You cannot view the main SuperQueue window and the preview window simultaneously.

#### How to hold and release a document

Putting a document on hold may be useful if, for example, you have an exceptionally long or complex document that you want to process at a later, presumably less busy time. Putting a document on hold is necessary to use the Preview function. To hold a document:

- Highlight the job’s listing in the Document Name column.

- Choose Hold Document from the Document menu.

A pause symbol appears next to the listing and “Paused” appears in the Status column.

To release a paused job:

- Highlight the paused job’s listing in the Document Name column.

- Choose Release Document from the Document menu.

#### Document properties

When you choose Properties… from the Document menu, or click the Document Properties button, the following dialog appears. Here you can both view information and change the filter options for the currently chosen job.

The first page, General, gives you a view of all the information that SuperQueue knows about the current job. Most of these attributes are self-evident. You cannot change any attributes on this page. Note that Owner and Queued From contain network related information; they will be blank for standalone systems.

The second page, Print, gives you the same options as the filter preferences pop-up. For details on this page, see “Add documents” earlier in this chapter. For information on specific filter options, see “Filter Specifications” section later in this chapter. If the chosen job is not already printing, you can change the filter preferences and even things such as paper size and resolution here.

The third page gives all the information that the filter can glean from the job file. There are no settings you can change; this  page is informational only.

If the currently selected document is an 8-bit grayscale or 24-bit color bitmap, a fourth page will give you options for Image Enhancement.  These options are explained in the “Image Enhancement” section of “Filter Specifications” later in this chapter.

## Printer activation

#### How to pause and restart a printer

To pause a printer:

- Make sure the printer window that represents the printer you want to pause is active.

- Choose Pause Printer from the Printer menu.

Pause Printer puts the selected printer queue “on hold” – incoming print jobs accumulate but are not processed until Start Printer is chosen.

To restart a printer:

- Make sure the printer window that represents the printer you want to start is active.

- Choose Start Printer from the Printer menu.

#### Pause printer

If Pause Printer is chosen while a job is in progress, SuperQueue temporarily stops processing that job. Printing resumes where it left off when you choose Start Printer. Note that on certain printers, a long pause in a data stream can be interpreted as a time-out which may cause the printer to eject a partially printed page. Subsequent resumption of the data stream can also cause printing problems if the printer is expecting “header” information to begin a new page; the mid-page data may be misinterpreted.

#### How to set a printer as the default

The Windows default printer is the one to which applications normally print (without specific action on your part to choose another printer). SuperQueue makes it easy to set the default printer.

- Make sure the printer window that represents the printer you want to make the default is active.

- Choose Set as Default Printer from the Printer menu, or click on the corresponding button on the tool bar.

Note that when the current printer is the default, there’s a check mark next to Set as Default Printer in the Printer menu, and the corresponding tool bar button appears to be depressed.

## Filters

Filters are the program modules that allow SuperQueue to accept native data file formats such as TIFF and PostScript.

#### How to add a filter

Filters that come with SuperPrint are generally installed automatically when you run SuperPrint’s Setup program. If you’ve removed a filter and want to re-install it, or if you have acquired a new filter and have already copied it to your hard disk, proceed as follows:

- Choose Filter Libraries… from the Options menu.

The Manage Filter Libraries dialog appears.

- Choose the Install… button.

The Install SuperQueue Filter Libraries dialog appears.

- Navigate to the drive and folder containing the SQF file.

- Highlight the desired filter and click OK.

Note that installing a filter requires that you restart Windows.

#### How to configure a filter for automatic operation

- Choose Filter Libraries… from the Options menu.

The Manage Filter Libraries dialog appears.

- Highlight the filter’s listing in the Installed Filter Libraries box.

- Choose the Setup… button.

Each filter has different options, so dialogs will vary. For details on a particular filter's options, please see the “Filter specifications” section later in this chapter.

One common feature is called Ask for preferences each job. If checked, a setup dialog appears each time you start processing a job with this filter. If the check box is cleared, the features you set here automatically apply to all jobs.

- When all options are set, click OK.

#### How to remove a filter

If you believe you will have no further use for a particular filter, you can remove it from the filter list.

- Highlight the doomed filter in the Installed Filter Libraries list box.

- Click the Remove button.

The chosen filter is removed from the list, but its files are not erased from disk. This allows you to re-install the filter later.

Note that removing a filter requires that you restart Windows to cause the change to take effect.

#### Filter Libraries

Options/Filter Libraries... brings up the Manage Filter Libraries dialog. At the top is a list of currently installed filters. When you highlight a filter, the function of that filter, along with its supported file extensions are shown below in the Library Contents (Filters) box.

The functions of the Install…and Remove buttons are explained earlier in this chapter under “How to add a filter” and “How to remove a filter.”

The Setup… button accesses the filter preferences dialog, explained under Filter Specifications below.

The About button brings up an informational dialog regarding the currently chosen filter. Included are the names of some very talented programmers and the version number, the latter being useful if you should need to contact Zenographics for technical support.

### Filter specifications

#### Ask for preferences each job

When you access the filter options using Options/Filter Libraries…/Setup in SuperQueue, each filter has a choice called Ask for preferences each job. If Ask for preferences each job is on, the other settings you choose here become the defaults, but you are given an opportunity to override these settings on a per-job basis. With Ask for preferences each job off, the settings are applied automatically to each filter job. They can still be overridden using Document Properties.

Use of this option depends on how you use the filter. If, for example, you are imaging slides from TIFF files, and you always want to fill the slide frame with the selected image, you'll probably want to choose Fit to page and clear the Ask for preferences each job check box. This will prevent SuperQueue from asking you for these options at the beginning of each job.

### Bitmap filters

#### Image sizing and positioning

Filters that translate bitmap file formats (such as TIFF, BMP, GIF, and JPEG) generally offer several common options (see screen capture for typical settings).

Turning Stretch to fit page on causes the filter to size the bitmap until either a maximum height or width printable area boundary is reached. For example, if you are printing a square bitmap on a portrait page, the filter will encounter the maximum horizontal printable area limit before it reaches the vertical limit. The filter will stop scaling when it reaches that horizontal limit. In that instance, the vertical dimension will not fill the page.

If Stretch to fit page is off, there are two possibilities for how the bitmap will print: If the bitmap header contains its own resolution information (thereby setting its size in real-world units of measurement), the filter will print the bitmap at that size. For example, if a bitmap is 450 pixels by 300 pixels, and the header specifies that its resolution is 100 dots per inch, it will print 4.5 inches by 3 inches, regardless of the resolution of the output device. (Many bitmap editing applications allow you to set this “internal” resolution.) If the bitmap header does not contain resolution information, the bitmap pixels are mapped directly to the output device pixels. So, using our 450 by 300 example, on a 300 dpi laser printer, the bitmap would print 1.5 inches by 1 inch.  For output devices with non-square pixels (where the dpi of the horizontal dimension doesn’t match the vertical), the bitmap pixels are directly mapped to the dimension of the smaller dpi number and stretched in the other dimension to accommodate the difference. The aspect ratio of the image is maintained on the page.

Note that Stretch to fit page works in both directions – expanding or shrinking the bitmap as needed. If you turn this option off and your bitmap is larger than the page, you will end up with a clipped image.

Center on page forces the image to print in the middle of the printable area of your page. If the device’s margins are not equal in the same dimension (for example, if the left margin is larger than the right), the larger margin is used on both sides. With Center on page off, the bitmap is placed in the upper left corner of the page.

Minimum Margin lets you set a margin around the image that’s larger than the printer’s hardware requires.  The units you enter into this box are in hundredths of an inch or hundredths of a centimeter (as indicated by the setting immediately below the box). So, for example, if you wanted to force a margin of 1 inch to accommodate a three-hole punch, you would set Minimum Margin to 100.  This setting affects the size of your output when Stretch to fit page is on, and increases the amount of clipping when you print a too-large bitmap with Stretch to fit page off. Setting a Minimum Margin that’s smaller than the printer’s own margins will accomplish nothing – software cannot override hardware limitations.

Under normal circumstances, you’ll probably want to leave Match page orientation to image on. With this setting, the orientation of the page is automatically set to the orientation of the image. If you prefer to set the orientation yourself, clear this checkbox and set the orientation you desire in the printer driver.

#### Image Enhancement

For 8-bit grayscale and 24-bit color images, the SuperQueue bitmap filters can perform automatic adjustments for both Lightness and Contrast. The filter examines each pixel of your image and creates an internal histogram – adding up the pixels that fall into each lightness value. Once this process is complete, the filter can modify the output settings dynamically to adjust for less-than-perfect images.

You can control whether or not the adjustments take place by checking or clearing the boxes next to Enhance Lightness and Enhance Contrast. The Preserve Shadows and Preserve Highlights controls work in conjunction with Enhance Contrast. The numbers represent the percentage of color values that are close to the extremeties (shadows and highlights) that the filter will preserve in your output. Decreasing either of these numbers increases the contrast of your output by allowing more output range to the mid-level input values and less to the extremities. Decreasing Preserve Shadows causes more dark pixels to become black; decreasing Preserve Highlights causes more light pixels to merge with white.

### PostScript filter

The PostScript filter is Zenographics' ZScript module. It is a full-featured, level 2 PostScript language interpreter that translates PostScript page descriptions and Encapsulated PostScript (EPS) files into Windows GDI. This filter offers settings for Language Level, Accurate Images, Accurate Strokes, Accurate text clipping, and Ignore screens from PostScript file.

#### Language Level

Since PostScript level 1 is a subset of level 2, you should generally leave this setting on Level 2. All level 1 files and well-behaved level 2 files will work properly.

In rare instances, some level 2 files may cause errors during translation or printing. If this happens, switching to Level 1 may help get the job done. Well-written level 2 files are supposed to inquire about the level of the interpreter and dynamically accommodate the lower level interpreter. This workaround may result in some differences in the output, but these are generally not significant.

#### Accurate Images/Accurate Strokes

These settings are primarily for use with non-SuperDrivers. If you are using SuperDrivers, you can usually ignore these settings, leaving them in the off state.

There are inherent differences between the PostScript imaging model and the Windows GDI imaging model. SuperDrivers can compensate for some of these differences in ways that other Windows printer drivers cannot. Among these differences are scaling of color or grayscale bitmaps and rendering of lines.

With Accurate Images off, ZScript lets the chosen printer driver scale the bitmap on its own. The results you get in both throughput and quality will vary depending on the driver. With Accurate Images on, ZScript does the work of scaling the image and passing it along to the printer driver at the printer's native resolution. Use this option if your output displays either poor quality or imaging artifacts such as fine lines or unwanted patterns.

If the line widths, caps (line-end shapes), miters (shapes of corners), or dashed lines on your output are not accurate, try turning Accurate Strokes on. With this setting, ZScript will translate lines into filled polygons of exactly the thickness and shape requested. Depending on the complexity of the drawing, this may add some processing time.

#### Accurate text clipping

In rare instances, you may have a piece of text that, as part of a design, is clipped (chopped off) in mid-character. Most printer drivers recognize this clipping function and reproduce the effect accurately; some, however, do not. Accurate text clipping forces the ZScript filter to render clipped text as a bitmap, thereby ensuring accurate results. Depending on the complexity of the clipping, this may increase processing time. Unless you experience problems with clipped text, you should leave this option off.

#### Ignore screens from PostScript file

Some applications allow you to specify halftone screens within your PostScript file (for example, 133 line screen with a 45 degree screen angle). Such screens are usually needed only for pre-press imagesetting, not for desktop printing.

On your desktop printer, you may find that you get better looking results by using the halftoning (or dithering) that your printer uses by default. Also, depending on your printer driver and the complexity of the file, you may find that generating a custom screen adds processing time. If having exact line screen matching is not important, you can cause the filter to ignore the defined screens by turning this option on.

With this option off, any screens in the PostScript file will be imaged to the best ability of your printer and printer driver. Be aware that setting a screen frequency that is too high for your hardware will result in dark and posterized output. Also note that some programs (such as Corel Ventura Publisher) let you specify screens for particular images without setting screen values for other page elements. In such a case, the graphics with unspecified screens will use your printer’s (or printer driver’s) default screen.

# SuperDriver Reference

### Chapter organization

The SuperDriver reference chapter is organized into five sections:

- What are SuperDrivers?  An overview

- Common settings for all SuperDrivers

- Special settings — Additional information by printer type (laser, inkjet, dot matrix and thermal) and, as applicable, by individual printer.

- Bitmap SuperDrivers.

- The SuperDriver Status “applet”

## What are SuperDrivers?

SuperDrivers are Windows-compatible printer drivers that use Zenographics' powerful SuperRIP technology to provide the following benefits:

- High-speed printing of complex graphics and bitmaps

- Superior halftoning

- Advanced color capability

- Seamless integration with both Adobe Type Manager and True Type

While you can use any of TrueType's scalable typefaces with SuperDriver, you will not be able to use your printer's resident fonts or font cartridges. However, with the possible exception of dot-matrix printers, text-only documents should print just as fast with SuperDriver as with the standard Windows driver.

## Common settings

The following section covers settings that are common to nearly all SuperDrivers. The SuperDriver settings are contained in the tab pages shown here. The dialog shown below is typical of what you will see when you choose Printer Setup from SuperQueue or another Windows application.

Many settings are shown in “drop-down” list boxes; only the active setting is displayed. If there are more choices than can be displayed in a  box, it will have scroll bars. Scroll up or down as needed.

### Papers

#### Paper Source and Paper Size

Your choice of printer model determines your available choices in these list boxes. All of the paper sizes, trays and bins supported by your printer model are shown. Use the scroll bars as needed to see additional choices. Where pertinent, additional information is available in printer-specific sections.

#### Orientation

The Portrait or Landscape setting should correspond to the layout of your document.

When printing bitmap files using SuperQueue’s filters, you have the option to override the Orientation setting on a per-job basis. See the SuperQueue chapter for more details.

Additional orientation information is available in the printer-specific sections later in this chapter.

#### Resolution/Print Quality

Resolution and Print Quality have to do with the appearance of the printed image. Resolution, expressed in dots per inch, is a common way to specify output quality for printers like laser and dot matrix printers. Higher resolution numbers deliver better quality output while the lower ones are more suited for draft output.

Some inkjet printers, notably the Hewlett-Packard DeskJets, have Print Quality features, expressed as High, Medium, Low and Draft that combine resolution with hardware options to control such things as print head movement, passes per band, and the amount of ink on the page.

More information is available on both Resolution and Print Quality in the printer-specific sections later in this chapter.

#### Restore Defaults button

The Restore Defaults button resets choices to the “factory” SuperPrint defaults for your printer model. This button is used primarily to give you a new start when troubleshooting.

Caution: the Restore Defaults button does not give you any confirmation message before resetting your driver. Its action can only be undone by clicking Cancel to exit this dialog.

### Custom Papers

Some SuperDrivers (e.g., dot matrix, plotters, etc.) have a Custom Papers... button. When you click on this button, the dialog shown here appears. This dialog lets you define new papers and remove papers that you have created. The Custom Papers feature can come in handy when:

- You have an unusual paper size installed in your printer and want to make sure the driver reports the correct dimensions to your printing application.

- Your application bases its document size on the chosen paper and you want to create a custom sized document.

#### How to create a new Paper Size

To create a new paper size:

- Click the New button and type a new name in the Name box.

- Change the units (inches or centimeters) as needed.

- Use the scroll arrows next to the Width and Height boxes to get to the desired numbers, or enter the numbers in the Width and Height boxes with the keyboard.

Note that the numbers you enter are expressed in hundredths of an inch or hundredths of a centimeter. So, for example, if you wanted to define a dimension of 5½ inches, you would enter 550. A dimension of 5.50 inches would be shown in the list under Paper Size:.

You must define the page using portrait dimensions; the dialog will not allow you to enter a width greater than the height. If you need to print landscape, simply set Landscape on the Paper page. Also, you will not be allowed to enter dimensions that exceed your printer’s hardware capabilities.

- To activate changes and continue, click on Apply. To activate changes and exit, choose OK.

Your new custom paper will be added to the Paper Name list and will be displayed in the Paper Size list box on the Paper page.

Note: the Cancel button will not undo any changes that have been saved with the Apply button. You can delete your custom papers with the Remove button as explained in the next section.

#### How to remove a Paper Size

If you've defined a custom paper that you no longer want,  you can remove its listing.

- Choose the correct name in the Paper Name list.

- Click on the Remove button.

The chosen Paper Name is removed and the next name in the list becomes the current one.

### Options

The only option that is shared by most SuperDrivers is Compression.

#### Compression

Compression works only for certain devices. Basically, it speeds the transmission of data and reduces printing time by sending repetitive data (such as scan lines that are all one color) to the printer faster. Usually, you should leave Compression in its default state. This option is not available for devices that do not support it.

You may wish to turn Compression off if your image is composed in large part of natural images (bitmaps) since this sometimes improves throughput. Also, if you are getting garbled-looking output, an “out-of-memory” message, or your printer “hangs,” turning Compression off is one thing to try during troubleshooting.

### SuperRIP Memory Management

When you click on the SuperRIP tab, the page shown here appears.

The top half of this page is informational. In Current image size, you are given the image size for the current resolution settings. Also shown here is the “pixel depth” —that is, how many bits are used for each pixel.

For Image memory required, the numbers in Current image size are multiplied. The result is reduced to Kilobytes.

The Available Windows memory (virtual) line shows a number that includes virtual swap file memory (if any). However, SuperRIP does not make use of the virtual swap file by default. It normally uses only physical RAM. The next line, Unused physical memory, shows how much actual RAM is available for SuperRIP's use. This number represents system RAM less everything currently in memory (system drivers, Windows, and currently running applications.

#### How SuperRIP allocates memory

The next section of the dialog lets you control how SuperRIP uses available memory. The amount of memory SuperRIP will use is shown in the Maximum SuperRIP allocation box. By default, Automatic memory management is turned on, so the number in the allocation box is pre-determined. This number is never larger than the one in Image memory required. In a system that does not have full-frame memory available, SuperRIP allows itself to take one-eighth of the free RAM at print time. The actual number can only be estimated; the memory available at print time will determine memory consumption and performance.

#### Changing memory allocation

There are a couple of reasons why you might want to change the automatic allocation. If the automatic allocation is just shy of full frame (the Image memory required figure), you could increase it. If you want to allow more memory for processing very complex graphics or for other applications, you could decrease it. To instruct SuperRIP to use a different amount of memory than the default:

- Clear the Automatic memory management box.

- Enter a new figure in Maximum SuperRIP allocation.

We urge you not to be too aggressive in changing this memory value. This new number cannot be lower than 64KB, and it rounds up to the nearest 64KB. Under normal circumstances, it should not be higher than Unused physical memory. In fact, it's usually best to leave some Unused physical memory free so Windows can access DLLs and other program modules without excessive swap file activity.

Note that available Windows memory can be increased by closing unused applications. To see the effect of closing an application, click on the Retry button.

#### Other controls

Once you have either changed Maximum SuperRIP allocation or closed another Windows application, you can force SuperRIP to re-evaluate your memory status by clicking on the Retry button. All numbers will be updated, as well as the information in the Expected Printing Performance box. This box displays either a green, “go” indicator (indicating GOOD performance with a full frame buffer) or a yellow “caution” indicator (indicating FAIR performance using buffering and banding). FAIR merely indicates that two or more bands will be used to send the file to the printer (up to a total of 64).

A red “stop” indicator is a signal that not enough memory has been made available for SuperRIP; you should close other applications or upgrade your computer.

### Tricks

When you click on the Tricks tab, the SuperDriver Tricks page appears as shown. The features on this page are necessary for certain applications to work correctly (or optimally) with SuperDrivers. The box has four sections: Windows/ SuperDriver Interface, Color Tricks, Application Fill Patterns and Font Engines.

#### Windows/Super-Driver Interface

Some applications do not recognize the inherent color-handling capabilities of SuperDrivers. It is for these applications that the option Report 1 bit/pixel, 1 plane as device bitmap exists. One application that requires this setting to be on is Microsoft PowerPoint. If you find that you are not getting correct color output from an application, you should try toggling this option.

Microsoft's specifications for communicating to Windows printer drivers are sufficiently “loose” that different software developers have invented (or borrowed) special driver “escapes” (unusual instruction codes). The set of escapes invented by Micrografx can provide substantial performance gains. Applications that either require or work better with Enable `Micrografx' escapes (Path, ExtCaps, ...) turned on include Lotus Freelance for Windows and all Micrografx software. In general, if performance when rendering complex graphics is either very slow, or yields unsatisfactory output, you should try toggling this setting.

#### Color Tricks

Most output devices rely on dithering to achieve shades of gray or light colors. With the Show accurate... options on (the default), SuperDriver gives you accurate grays or colors even if it has to dither them. You get exactly what you ask for.

However, small dithered text is difficult to read. Likewise, fine dithered lines appear dashed instead of solid. With the Show accurate... options off, SuperDriver maps text and lines to the nearest solid color. For black & white devices, grays darker than about 50% map to black; lighter grays map to white. For color devices,  colors map to the nearest solid ink. For example, an orange line (which would be solid yellow and dithered magenta) will come out solid yellow.

Note: This option has no effect on continuous-tone devices such as film recorders.

#### Application Fill Patterns

When an application sends a bitmap fill pattern (also called a “pattern brush”) to the printer, the expected, standard size is 8 pixels square. Some applications are able to send larger patterns. If Accept patterns larger than 8x8 pixels is on, and you encounter output where your bitmap fills have repeating pattern anomalies in them, try turning it off.

Bitmap fill patterns can be used for any repeated design, or even to simulate grayscale dithering. Some applications send box tints as bitmap patterns. One problem that arises with this is that some programs don't make allowances for the difference between screen resolution and printer resolution. (Another problem is that some do!) The typical result is that patterns printed to a LaserJet get very small, and tints get too dark.

SuperDriver has a process that automatically enlarges the bitmap fill patterns so that the output more closely matches the screen. For example, with the default, “automatic” setting of Pattern Scale: 0, patterns that come from a VGA system (96x96 logical pixels per inch) are enlarged three times to (almost) match the 300x300 dpi of a laser printer.

If you set Pattern Scale to 1, that tells SuperDriver to handle the brush patterns exactly as they come from the application—no scaling. You can also force SuperDriver to scale by 2, 3, 4, or more times by entering larger numbers.

#### Font Engines

The primary use of this feature is for troubleshooting (by a process of elimination) the current SuperDriver with a limited number of applications. If your printing works OK, or if TrueType is your only font manager, you should ignore these settings.

In some rare circumstances, certain applications can cause problems with SuperDrivers when the document uses typefaces installed in Adobe Type Manager software (ATM) or TrueType. (To check if your favorite applications are among these, please refer to SuperHelp.) If you have both font engines, you could also run into substitution problems.

The Adobe Type Manager checkbox accesses a “back door” to ATM. Turning this option on uses this back door to speed up handling of Type 1 PostScript fonts by handling them as text. With this option turned off, fonts are handled as graphics.

Clearing the TrueType check box allows you to completely turn off SuperDriver's access to TrueType fonts. Doing so makes all TrueType faces unusable with the current SuperDriver. If you do not have ATM, you should never clear this checkbox!

### Image

When you click on the Image tab, the page shown on the following page appears. Please note:

- Lightness, Contrast and Sharpness apply to all devices

- Saturation and Hue Matching apply only to color devices

NOTE: This feature has been implemented primarily to overcome hardware inadequacies or maladjustments. Default settings for the recommended “threshold array” halftone patterns have been extensively tested by Zenographics on the equipment available to us; your results may vary. If you decide to change the defaults, it may take some trial-and-error experimentation to achieve the desired results.

To some readers, some of the following may appear rather technical; to others, not technical enough. An attempt has been made to explain the function of these settings only to the extent that it makes informed experimentation possible.

#### Input / Output

When your application prints, it sends color values for every object, pixel, or piece of text. The SuperDriver accepts that information and passes it through these imaging correction settings.

All these settings affect only the output. Absolutely no changes are made to your original files or your on-screen display. Also, these settings are remembered by SuperDriver from one session to the next, so if you find some settings you like, you can set them and forget them.

Settings in the Image Correction box are “global” settings—that is, they affect the entire image and all colors. Before you adjust these settings, you should have an idea of just what is wrong with your output.

| Problem | Probable adjustment |
| --- | --- |
| Light and dark colors OK, mid-tones are too dark | Lightness |
| Output looks either too “flat” or too stark | Contrast |
| Colors either washed out or too vivid | Saturation |
| All color values (including what should be light tints) are too dark | Dot Gain (See section on “Halftoning Options”) |

Lightness

Lightness primarily affects the mid-range color or gray values of a graphic image. It does so in a non-linear fashion (that is, it does not affect every gray value equally—see the accompanying graphic).

Probably the best use for Lightness is to compensate for flaws in your original document or graphic. If, for example, your document contains a scanned image that's too light or too dark, you can compensate using Lightness.

If you have a 1-bit-per-pixel device and all your printing is coming out too light or too dark, consider manipulating the Dot Gain setting first. See “Halftoning options” later in this chapter for details about Dot Gain. For continuous-tone devices, changing Lightness may be useful if your mid-tones are consistently too light or too dark.

HINT: Try changing Lightness in 10% increments to start.

Contrast

Contrast is similar to the contrast knob on your television. If the distinctions between neighboring objects are not clear enough, try increasing Contrast. Conversely, if your image appears too stark, you can decrease Contrast to soften it.

If you increase contrast, lighter colors may tend to vanish into white, and darker colors may merge with black. The remaining middle tones will spread out along the entire range of grays so that the distinction between them should increase. Increasing Contrast all the way (100%) removes the middle grays and maps every gray into either black or white.

Decreasing Contrast, on the other hand, flattens distinction between gray values—eventually, nearly everything becomes a 50% gray (at -100% contrast).

HINT: Try changing Contrast in 10% increments to start.

#### Sharpness

Any time you print an image using halftoning (or dithering), a little of the sharpness or clarity is lost. This is primarily due to details being lost in the “cracks” between halftone dots. In traditional printing, when the highest quality results are needed, line screen frequencies are increased to 150 lines per inch or more. Unfortunately, on today's desktop printers, this is not an option—trying to print very fine line screens results in unacceptable dot gain and reduction of available gray levels.

SuperPrint includes a Sharpness filter that helps compensate for this. It is only useful with scanned photographs (natural images) that are either 24-bit color or 8-bit grayscale. A default Sharpness value has been supplied for your device; you can also change it if you want to. To change Sharpness:

- Enter a new value in the Sharpness box with the keyboard or by clicking on the scroll arrows.

Acceptable values are zero to 100, with zero representing no sharpening action. The effects of Sharpness will be noticeable primarily on the edges where areas of different lightness values converge. For example, on a photograph of a face, you would notice a greater definition of the irises against the “whites” of the eyes; details like eyelashes can also become more clear.

This Sharpness filter uses a process called “Unsharp Masking.” It masks out anything that's not sharp by examining an image pixel-by-pixel, looking for adjoining areas which differ in lightness. Where it finds such areas, it exaggerates the contrast at the point where the areas meet—the lighter pixel is made even lighter, the dark one is made darker.

If you've set Sharpness too high, you might notice that formerly minor variations within certain image areas become starkly defined. This may produce an interesting special effect, but for most work, you'll want to reduce Sharpness until such undesirable effects disappear.

### Tips for users of color devices

IMPORTANT! Adjusting these settings to other than the “factory” defaults can seriously affect output quality.  Unless you have the time and resources to spend on significant trial-and-error experimentation, we strongly urge that you leave these settings alone! Also, since color balancing is, by its very nature, subjective, it is unlikely that our telephone support staff will be able to remedy any difficulties arising from its use. Remember that if your Color Correction experiments go awry, you can always click the Restore Defaults button in the Hue Correction dialog to return your settings to the factory defaults.

#### Saturation

Saturation controls the intensity of color that's placed on the output page. It affects all hues equally. If output colors appear dull, increasing Saturation can make them more vibrant. Conversely, if colors are too intense, reducing Saturation will make them more subdued.

On ink-jet printers in particular, color intensity will be strongly affected by  choice of paper; a typical laser paper or cotton bond will absorb ink, making colors dull, while a coated paper will keep the inks on top of the paper itself, increasing their reflective properties. You can use Saturation to compensate for the properties of the paper you are using.

#### Hue Matching

Zenographics has made every effort to give you accurate color on your output device when you use the SuperDriver with its default settings. If, however, you would like to make changes to the individual color values of your output, you can use the Adjust Hues button to do this.

#### A little bit of theory – the color model

Internally, SuperDrivers use the Windows 24-bit RGB (Red, Green, Blue) color model to define all colors. These are the colors of the phosphors in your color monitor. When viewed on a monitor, they are considered additive colors because the colors are generated by adding light from one or more of these primary colors. Black is the absence of all light (R=G=B=0), and white is the presence of all light at maximum levels (R=G=B=255).

RGB values are used for output as well. Some output devices, such as film recorders, accept and use RGB values. Some devices, such as certain color thermal printers, accept RGB input and convert the colors to CMY or CMYK (Cyan, Magenta, Yellow, and Black) for output. Some printers require that the SuperDriver convert its RGB to CMY or CMYK before it's sent to the printer.

On printed output, C, M, Y, and K are referred to as subtractive colors. This name indicates that a colored area accepts white light, reflects only certain wavelengths, and subtracts all other wavelengths from the white light. In this case, white is the absence of all color (C=M=Y=K=0) and black can be either K=100% or C=Y=M=100%.

As you might expect, additive and subtractive colors work very differently when combined. For example, when you combine 100% values of two additive colors on your monitor, the resultant color is brighter. Combining 100% values of two printed colors results in a darker color.

Here are the relationships between 100% values of RGB and CMY:

| Additive color | plus additive color | yields subtractive color |
| --- | --- | --- |
| Red | Green | Yellow |
| Green | Blue | Cyan |
| Blue | Red | Magenta |
| Subtractive color | plus subtractive color | yields additive color |
| Cyan | Magenta | Blue |
| Magenta | Yellow | Red |
| Yellow | Cyan | Green |

But remember what we said about combining colors: the cyan you get on screen from combining green and blue becomes brighter, while the blue you get on paper from combining cyan and magenta is darker. If you are using a SuperDriver for a color hardcopy device, you should find that the color correction we have built in compensates quite well for this situation. If you want to fine tune your colors, however, we have given you the tools to do it.

#### Using Hue Correction

- First make sure that the Enable Hue Matching box is checked.

Hue Matching settings are ignored if this box is clear.

- Click on the Adjust Hues… button.

The dialog shown on the previous page appears.

- Choose the radio button next to the color you want to modify.

Note that changes you make to a particular color will not affect neighboring colors; if you want to make your blues more greenish, you can do so without affecting cyan or magenta.

There are four ways to modify your chosen color:

- Click inside the large, rainbow-hued box (called the “Color Matrix”).

- Drag the arrow next to the vertical gradient box (called the “Luminosity Bar”).

- Enter values in the Hue, Sat, and Lum boxes.

- Enter values in the Red, Green, and Blue boxes.

There is an interrelationship between Hue, Saturation, and Luminosity and Red, Green, and Blue. (To see this dynamically, just drag the cursor around in the Color Matrix or the Luminosity Bar. Click the Cancel button if you don't want your experiment to take effect.)

#### The Color Matrix

This box lets you graphically change the Hue and Saturation. The value in the Hue box is a numerical representation (from 0 to 239) of the color name (red, red-orange, orange, yellow-orange, yellow, etc.). The value in the Saturation box is a representation of the intensity of the color, from solid color (240) to absence of color (0), also known as gray.

#### The Luminosity Bar

This bar lets you graphically change the Luminosity of your color. The values range from 0 (black) to 240 (white). Using Luminosity could be helpful when you have, for example, a chart or other graphic element with clearly defined areas of color and you want to manipulate the lightness of just one color.

#### Red, Green, and Blue

While the Hue, Saturation, and Luminosity settings offer graphical representations to make your choices easier, it's the Red, Green, and Blue settings that really matter to Windows and SuperDriver. These settings are harder to manipulate manually, but if you choose to do so, you will be able to see the results on the Printer side of the Printer/Screen swatches.

#### The Restore Defaults button

The Restore Defaults button instantly returns all color values to your driver's “factory defaults”. There is no confirmation box. If you click Restore Defaults in error, simply click Cancel from the Hue Correction dialog; this will undo all actions you have taken while in that dialog box.

#### Color manipulation results

If you have manipulated a color value, the resulting displayed color should be used only as an indication of how you have skewed that individual color. For example, if your yellows were coming out a little too green using the default settings, you could use Hue Matching to move yellow toward red (away from green). The color box in the Printer column would display an orange-ish yellow as an indication that you have shifted this color. The goal is to make your output match as closely as possible the color shown in the Screen column; the revised colors shown in the Printer column are not intended for screen matching.

### Halftoning options

Choose the Halftoning tab for access to these options as shown on the following page.

The original halftoning process came from the traditional graphic arts. In that process, continuous-tone artwork is photographed through a glass screen that's etched with a cross-hatch pattern at a specific frequency (lines per inch). On the resulting film, the artwork is converted into a series of dots which vary in size according to the relative lightness of the original art. This conversion allows the shaded artwork to be reproduced on a printing press.

On a computer, halftoning (in some cases also called “dithering”) refers to the patterns in which printer spots are placed on a page to form shades of gray (for laser and most dot-matrix printers) or colors (color ink-jet and thermal printers). These patterns have traditionally consisted of small cells in an orderly arrangement that mimicked traditional graphic arts halftoning.

Recent advances in computer and graphic arts technology have made it possible to break free of the orderly rows of dots and achieve interesting and useful special effects with “halftoning.” SuperPrint incorporates the latest of these technologies to help you get the best output possible.

The Halftoning dialog has a single drop-down list box that contains the names of all the available halftone patterns. As you scroll through these names, three indicators give you information about the current choice:

- Representations of the output pattern appear next to the list box.

- A description of the choice appears in the area below the list box.

- A bitmap name and array size appear below the description. If no name or size appears, the current choice is algorithm based.

SuperPrint uses threshold arrays to achieve its halftoning. These are a close digital counterpart to the traditional graphic arts halftoning. A pattern, stored digitally in a BMP file (as opposed to a glass screen), is superimposed on the original digital image, producing different dot densities on the output.

Some threshold arrays are clustered dot patterns (for example, 3x3, 4x4, and 8x8 plus Dispersed-dot dither) while others produce a random screen appearance (SuperScreens 1 & 2, and the White noise threshold array).

Ordered patterns typically photocopy and fax well, but can exhibit artifacts like banding in gradient areas.

SuperScreens employ the latest FM (also called “scatter” or “stochastic”) screening for amazingly smooth looking output. They don't photocopy very well, but if you want to produce a short run of dazzling originals, you should definitely try these!

The White noise threshold array produces output that still has a random dot look, but its coarser grain creates an interesting special effect.

One advantage to the threshold array concept is that because the patterns are BMP file based, new screen patterns can be created and added easily in the future. See Appendix C for more information on adding your own threshold arrays.

#### Dot Gain

Dot Gain is the result of the difference between your printer's addressable pixels (the dots-per-inch resolution claimed by the manufacturer) and its optical pixels (the actual spot size made by the marking engine).

In a laser printer, for example, the laser beam can accurately target 300 spots per inch on the toner drum, but in the toner transfer process, the size of a single spot of toner becomes larger than 1/300 inch. An ink-jet or dot-matrix printer can accurately position its print head to within 1/240, 1/300, or even 1/360 inch. However, many variables, especially the properties of the paper being used, may cause the spot of ink to spread considerably (see diagram).

SuperDrivers have been given default settings for Dot Gain based on the printer model, the chosen Resolution and the chosen Media Type (for example, plain paper vs. coated paper or transparency). You should only adjust Dot Gain if it appears that your printer's marking engine is out of adjustment (that is, giving you consistently too dark or too light results across the entire color range). Setting Dot Gain higher will result in lighter output, setting it lower will give you darker output.

HINT: Try changing Dot Gain in 10% increments to start.

### About...

This tab brings up an informational page that includes the version number of your SuperDriver, the registered user and the Serial Number. You may need this information should you have to contact Zenographics for help.

## Laser Printers

This section covers features that are common to laser printer SuperDrivers. Specific sections follow for Hewlett-Packard LaserJet printers. SuperDrivers for other PCL lasers (Kyocera F-Series, Canon LaserBeam 430, Panasonic Laser, and Lexmark Laser) operate virtually identically to the LaserJet SuperDriver. Options in common with other SuperDrivers are covered earlier in the chapter.

#### Download Fonts & Rectangles

Download Fonts and Download Rectangles are found on the Options tab for laser printers using the Hewlett-Packard Printer Control Language (PCL).

When Download Fonts is checked, SuperDriver reads the text in your document and determines which characters need to be created. It then builds the character bitmaps and downloads them automatically to the printer as temporary soft fonts. When the print job is finished, the soft fonts downloaded by SuperPrint are automatically erased from the printer's memory.

When Download Rectangles is checked, SuperDriver looks for rectangles (and ruling lines from some applications) and downloads them as well. This is especially useful for drawing spreadsheet cells quickly. Not all applications send rectangles as rectangles, however— they may use other graphics primitives to accomplish a rectangle.

Only graphics areas are sent to the printer as raster lines when both Download features are active.

Downloaded objects always appear on top of graphics. If, for example, you place an Excel chart on top of a spreadsheet and print with Download Fonts and Download Rectangles on, any text or rectangles under the chart will show through on the printout. Another common situation is when a black rectangle has been used to create a drop shadow behind a white rectangle; with Download Rectangles on, the black rectangle obliterates the contents of the white rectangle. To avoid problems like these, turn the downloading options off when printing such documents.

## Duplex

Several laser printer models support automatic printing on both sides of the page. If the chosen printer does not have this option, the Duplex tab will be unavailable.

If you choose this tab, a page appears offering three options for Portrait pages:

- No Duplex prints on one side of the page only (default).

- Top Binding prints the second page upside-down in relation to the first.

- Left Binding prints both sides in the same orientation.

If you are printing Landscape pages, the definitions are switched, i.e., Top Binding prints both sides in the same orientation and Left Binding prints the second page upside-down in relation to the first.

### HP LaserJets

This section covers options for Hewlett Packard LaserJet printers and compatibles. If you are using a LaserJet compatible, make sure you consult your printer manual for correct printer emulation information.

#### Orientation

As stated earlier, Orientation should be set to correspond with the layout of your document. Unlike downloadable HP Soft Fonts, SuperDriver doesn't care whether your document needs portrait or landscape fonts. When using scalable typefaces, it simply builds the fonts in whatever orientation is required.

#### Resolution

Most LaserJets and compatible printers support multiple resolutions. Choosing a lower resolution will result in somewhat faster output at the expense of smooth-looking text and fine halftone screens.

If you are printing full-page graphics on a LaserJet that does not have sufficient memory, you may be forced to use a lower resolution. For example, printers that do not have enough free memory for a full page at 600 dpi (4 MB) will have to use the 300 dpi setting.

#### Hardware options

Certain LaserJet printers support hardware options that can be controlled from within the SuperDriver Options tab. For detailed information, see your Hewlett-Packard manual.

- RET (Resolution Enhancement Technology) refines the print quality of characters and graphics by “smoothing” the fine gradations along the angles and curves of the printed image. Interacts with Density.

- Density is a measure of darkness of print on the page. Lower numbers lighten; higher numbers darken. Interacts with RET.

- EconoMode reduces the amount of toner on the printed page so that the toner lasts longer.

- Page Protect reserves a full frame buffer in the printer's memory, protecting that memory from being used by other printer data or instructions (such as downloaded fonts).

#### LaserJet Printer memory

Different LaserJet models use varying compression schemes; for example, some allow you to print a full page of graphics at 300 dpi with only 512K RAM on board the printer.

However, a 300 dpi LaserJet printer with 1 MB of memory free (or a 600 dpi model with 4 MB free) will be able to print a normal SuperPrint generated page. The key word here is free. The printer itself uses some of the memory, as do other printing choices. For example, if you are printing a full-size, complex page with Download Fonts on and you’re using a halftoning choice such as SuperScreens that doesn't compress well,  you may need slightly more memory.

### Other PCL-type laser printers

The SuperDrivers for Kyocera, Panasonic,  PCL Canon LaserBeam and Lexmark Laser printers operate virtually identically to the SuperDriver for Hewlett Packard LaserJets.

## Inkjet Printers

Inkjet printers included in this section are Hewlett-Packard DeskJets, Hewlett-Packard PaintJets, Canon Bubble Jets,  Epson Stylus 800, Epson Stylus Color and Lexmark ExecJet IIc. Options common to all SuperDrivers are covered at the beginning of the chapter.

In general, for inkjet printers that allow use of plain bond and clay-coated paper, the coated paper provides better-quality output. Transparency stock that's specifically designed for use with inkjet printers can also be used; printers with a transparency setting usually give the best results.

### HP DeskJets

#### Orientation

Orientation should be set to correspond with the layout of your document. Unlike using downloadable HP Soft Fonts, SuperDriver doesn't care whether your document needs portrait or landscape fonts. When using scalable typefaces, it simply builds the fonts in whatever orientation is required.

#### Print Quality

Print Quality settings invoke a combination of resolution, print head movement, and ink depletion features that give you a choice of speed and quality options for your output. Some features are built into your printer, others are implemented in the SuperDriver. The choices for the DeskJet are High, Medium, Low and Draft. All settings use the highest dots per inch (dpi) of the printer except Draft, which prints at half normal resolution. Medium (the default) provides the best quality/speed combination. High provides the best quality but is twice as slow as Medium. You may wish to experiment to determine the best setting for your application.

#### Media

Several models have settings for Plain Paper, Transparency, and Glossy, and some include an option to pause between pages to give media time to dry. Color models also have a setting for Hewlett-Packard's special CX JetSeries and LX JetSeries papers. Make sure your selection matches the media loaded into the printer. Make sure transparency film is loaded into the printer with the print (rough) side down.

#### Color

Choose Color checkbox for color output; clear it for black and white. Some DeskJet printers have an option to add a color kit. If you have not added a color kit, make sure the Color checkbox in the SuperDriver Options dialog is clear.

#### Black Generation

Some models have a Black Generation option. The choices are:

- Composite Black makes black using cyan, yellow, and magenta inks only. The only choice for CX JetSeries paper.

- Pure Black uses black ink for black and pure gray, uses combinations of cyan, yellow, and magenta for all other dark hues.

- Blend intelligently displaces dark cyan, yellow, and magenta combinations with a black ink component.

Your use of this option depends on the document itself. The default, Blend, should produce overall good results, especially in graphics with gradient fills that make a transition from a solid color to black. In some cases, bitmaps may benefit from Composite Black. Some experimentation may be needed to determine the optimal setting for a particular document.

#### Performance considerations for DeskJets

While SuperDriver improves performance on DeskJets when printing complex graphics and bitmaps, you probably won't notice any significant speed increase with SuperDriver when printing text-only documents; SuperDriver feeds data to the printer as fast as it can go.

No extra printer memory is needed for use with SuperPrint. SuperPrint can generate all fonts within your system and send them to the printer as raster graphics; this is the default for most DeskJet models. For models that support downloaded fonts, any problems associated with not enough printer memory can be overcome by turning Download Fonts and Download Rectangles off. This will result in only a minor increase in total print time.

### HP PaintJets

#### Printer model

Choosing the correct model when adding a printer is especially important for this SuperDriver since PaintJets and PaintJet XL/XL300s speak different dialects of the Printer Control Language (PCL).

#### Print Quality

This setting is grayed; print quality is controlled by Presentation Mode.

#### Orientation

Orientation should be set to correspond with the layout of your document. Unlike using downloadable HP Soft Fonts, SuperDriver doesn't care whether your document needs portrait or landscape fonts. When using scalable typefaces, it simply builds the fonts in whatever orientation is required.

#### Presentation Mode

Presentation Mode uses a print interlacing technique to remove “banding” effects. While Presentation Mode improves output quality, it also doubles printing time.

#### Media Type

Choose from Paper and Transparency. The Transparency selection will instruct the printer to do a double pass per line, which causes more ink to be placed on the page. Follow instructions in the Hewlett-Packard manual for loading media.

#### Before printing with the PaintJet...

Be sure that you set the top of page correctly in your printer. SuperPrint automatically centers the image inside the margin areas, but it assumes that the Top of Form (TOF) is set at the very top of the page (at the perforation).

If, for some reason, you must abort printing on an already partially printed page, you should realign the paper in the printer so that the perforation is at the top, then press the SET TOF button on the PaintJet's front panel.

#### Memory

Since PaintJet printers use a moving print head and print “on the go,” no extra printer memory is needed for use with SuperPrint. SuperPrint generates all fonts within your system and sends them to the printer as raster graphics; extra memory to hold Soft Fonts is unnecessary.

### Canon Bubble Jets

#### Form Feed Each Page

Form Feed Each Page (available only on some models) inserts a "Form Feed" character (ASCII 12) at the end of each page.  This command ejects the current page. Turn it on if partially printed pages do not automatically advance to the end of the page. Turn it off if extra pages are ejected between printed pages.

#### Resolution

All resolutions are expressed in dots per inch. Where there are two numbers, the first is the horizontal measure, the second is vertical. Higher numbers yield better looking output; lower numbers give you faster output with some sacrifice in print quality.

On some models, the resolution may depend on Print Quality or Smooth Black settings.

#### Color

Enable the checkbox for color output. Clear it for black & white output.

#### Smooth Black

A checkbox option for several models, Smooth Black enables a hardware feature that refines output by smoothing jagged edges of characters and graphics. Available only for monochrome printing. (For color models,  enabling this checkbox will disable the color checkbox.)

#### Enhance Black

A checkbox option for some models. When enabled, it initiates commands in the printer to intensify black areas of the printed image.

#### Print Quality

Print Quality is an option for several models. Choices initiate hardware commands in the printer. Note: Windows keywords may differ from the terms used by your printer's documentation. Choices vary by model.

- High Quality – provides highest quality, slowest speed.

- Standard – provides fair quality, good speed.

- Draft – provides lowest quality, fastest speed.

- Color Non-Bleed – provides fair speed and very high quality.

- Smooth Black (720 x 360) – provides increased quality by smoothing jagged edges on output. It produces 720 x 360 dpi output.

When printing in monochrome the BJC-4000 and BJC-70, using the High Quality setting and enabling the Smooth Black checkbox produces 720 x 360 dpi output.

Resolution of 360 dpi is recommended with all settings except Draft, where 180 dpi is sufficient.

#### Media

Print Media is an option for several models. Choices vary by model:

- Plain Paper – for low ink-density image on plain paper.

- Coated Paper – for use with clay coated paper.

- Transparency – for use with inkjet transparency film.

- Glossy Paper – for use with high gloss paper.

- Textile Sheet – for use with paper with a high rag content.

Coated Paper is the best media choice with High Quality or Color Non-Bleed Print Quality settings. Glossy, rag-based, or extra-thick papers may have special paper-handling requirements. See your Canon printer manual for details.

#### BJC Print Modes

BJC Print Modes are only available for certain Bubble Jet models. Choices are:

- Coated paper – For use with clay-coated paper.

- Transparency – For use with inkjet transparency film.

- Plain – For low ink-density image on plain paper.

- Plain (enhance black) – Darker blacks than Plain.

- Plain (wait to dry) – For high ink-density image on plain paper. 20 minute print time.

- Draft – Half the normal ink coverage; twice the printing speed.

#### Bubble Jet 300/330 hardware

There are several DIP switch settings that must be correctly set for proper operation of the BJ-300/330 with SuperPrint. The DIP switches are located on the back of the printer. Refer to the Canon manual for exact information on setting these switches.

Your Bubble Jet printer must be configured for IBM Proprinter emulation (Emulation Mode 1).

If you are using the optional automatic sheet feeder, you must set switch 1-6 off. Switches 2-4 and 2-5 must be set for the length of paper you are using.

#### Bubble Jet BJ-10e hardware

There are three DIP switch settings that must be correctly set for proper operation of the BJ-10e with SuperPrint.

If you are using the optional automatic sheet feeder, you must set switch 1 on.

If you are using A4 (European) paper, you must set switch 4 on.

For use with SuperPrint, switch 10 must be off (IBM emulation).

### Lexmark ExecJet IIc

#### Color

Enable the Color checkbox for color printing; clear it for monochrome printing.

#### Media

There are five media type choices: Plain Paper, Transparency and Glossy, plus an option to pause after printing to allow paper to dry for Transparency and Glossy. Make sure your selection matches the media loaded into the printer. Transparency film should be loaded into the printer with the print (rough) side down.

### Epson Stylus 800

#### Paper Source

For Automatic Sheet Feeder, choices are Letter or A4 size paper. All paper sizes may be printed with Manual Feed.

#### Resolution (DPI)

Settings are shown in dots-per-inch. Choices are 180 dpi, a draft mode, and 360 dpi for final output.

### Epson Stylus Color

#### High Speed

Allows bi-directional printing. Clear the checkbox for uni-directional printing.  If the box is clear and you are printing in black and white, the printer will detect which mode is better and will use it.

#### Pause Between Pages

Pause Between Pages lets you initiate the printing of subsequent pages manually. You can allow time for ink to dry, preventing a freshly printed page from becoming smeared by a following page.

#### MicroWeave

When enabled, MicroWeave improves image quality by passing the print head across the page a number of times and laying down only a portion of the image with each pass. Since using MicroWeave does not significantly affect print speed for color output, you may want to use it for all color printing. It is slower for monochrome printing, so you may want to turn it on only for final output.

With MicroWeave on, SuperDriver loses some of its vertical printable area (approximately 1/8 inch less, top and bottom). Therefore, if you plan to use MicroWeave for your final output, you should design with this in mind or leave it on when printing preliminary output during the design process.

#### Media Type

Choices are Plain Paper, Transparency, Coated Paper (360 dpi) and Coated Paper (720 dpi), the last two of which are Epson Special Coated Papers. Media choice is limited to Coated Paper (720 dpi) when you choose 720 dpi resolution.

#### Resolution

Shown in dots per inch. Low resolution (180 dpi) is faster but of lower quality, useful for draft output. Use 360 dpi for final output with plain paper, coated paper (360 dpi) or transparencies.  720 dpi sets the Media Type to Coated Paper (720 dpi).

## Dot-Matrix Printers

Options common to all SuperDrivers are covered at the beginning of the chapter. This section covers only the options specific to IBM, Epson and Panasonic dot matrix printers (and compatibles).

#### Choose the correct model!

Most dot-matrix printers can perform either an IBM or Epson emulation. If your printer is not listed among models supported by SuperPrint, consult your printer's manual to determine if it emulates one of these types of printers. You may have to change some settings on your printer to accomplish this.

If you have a 24-pin dot-matrix printer that can only use Toshiba's emulation, you will not be able to use this version of SuperPrint with that printer.

If you have a 24-pin printer, do not choose a 9-pin emulation! These settings will not provide any time savings and will result in distorted output.

#### Resolution

The choices in the Resolution box will be different depending on whether you've chosen a 9-pin printer or a 24-pin printer. Resolutions are given in dots per inch (dpi). Where there are two numbers, the first is the horizontal measure, the second is vertical.

The best way to think of SuperPrint's Resolution settings is as edge resolution settings. The main difference you will see between the settings will be in the crispness of text and smoothness of curves and diagonal lines.

#### 9-pin resolutions

120 x 72 dpi is as close as SuperDriver gets to a “draft mode” setting. It makes one pass of the print head with the most fundamental use of the pins.

120 x 108 dpi and 120 x 144 dpi each produce very similar results. Due to the rather esoteric mathematics of producing 108 or 144 vertical pixels per inch from a 9-pin print head, it so happens that 120 x 108 dpi resolution requires 3 passes of the print head, while 120 x 144 dpi requires only 2 passes! However, the triple-interlacing of 120 x 108 dpi produces a more mathematically perfect arrangement of the dots, which may, in some cases, improve the look of the output. This is something best observed through experimentation. If you don't have time to experiment, use 120 x 144 dpi.

240 x 216 dpi will produce the best-looking text and line art your printer can produce. It does, however, require three slow passes of the print head for each line, so it is best saved for making final output.

Not all printers can use 240 x 216 dpi resolution. If you are experiencing problems, consult the “Specifications” section of your printer manual for resolution, print head movement and minimum vertical paper movement details.

#### 24-pin resolutions

120 x 180 dpi is as close as SuperDriver gets to a “draft mode” setting. It makes one pass of the print head with the most fundamental use of the pins.

180 dpi and 360 x 180 dpi resolutions each cause the printer to lay down more pixels during its movement across the page. This may result in slower print head movement on some printer models. The print head makes only one pass per band in either setting (unless Presentation Mode is activated).

360 dpi will produce the best-looking text and line art your printer can produce. It does, however, use a double pass to double the vertical resolution. This doubles the print time, so it is best saved for making final output.

Not all printers can use 360 dpi resolution. If you are experiencing problems, consult the “Specifications” section of your printer manual for resolution, print head movement and minimum vertical paper movement details.

#### Bi-directional Printing

Many dot matrix printers support bi-directional printing; that is, the print head makes one printing pass moving from left to right, advances the paper, and makes the next printing pass moving right to left.

In some dot-matrix printers (especially those that have seen a lot of use), there may be registration problems when printing graphics with bi-directional printing. This will be seen as a slight horizontal offset between the bands laid down during each pass of the print head. Turning Bi-directional Printing off may help alleviate this problem.

With Bi-directional Printing off, the print head will print from left to right, advance the paper and return to the left before printing the next band. In some printers, this action of returning to the left (“home”) position helps provide more accurate print head registration.

#### Form Feed Each Page

When Form Feed Each Page is on, a form feed character (ASCII 12) is sent to the printer at the end of each page printed. The usefulness of this option will depend on your printer and the application you are using.

- Turn Form Feed Each Page on if printing of second and subsequent pages starts too high up on the page or partially printed pages do not automatically advance to the end of the page.

- Turn Form Feed Each Page off if extra pages are being ejected between printed pages or you are printing forms such as continuous-feed mailing labels that are shorter than 11 inches.

#### Presentation Mode

Turning Presentation Mode on activates an interlaced, double-pass printing method designed to overcome the problem of horizontal “banding” produced by some printers, especially in areas of dense graphics. While it offers improved quality, it also doubles the amount of time it takes to print a page.

Presentation Mode is available only when you are using a black ribbon.

#### Ribbon Type

Make sure you have chosen the Ribbon Type that matches the ribbon you have installed in your printer.

## Thermal Printers

Options common to all SuperDrivers are covered at the beginning of the chapter.  This section covers only the options specific to Fargo Primera, Star SJ-144 and Star WinType 800C thermal printers.

### Fargo Primera printer

#### Paper Size

Select the correct paper size from the available choices. Note: if you are using standard letter size (8½" x 11") or A4 size (210mm x 297mm) paper, you must choose the Letter Small or A4 Small paper sizes. To use the regular Letter or A4 settings, you must have special paper that uses a tear-away header strip. This type of paper allows the printer's mechanism to grip the paper outside the printable area.

#### Buffer to disk

The Buffer to disk option can speed output of multiple copies when there is not enough memory to do full-frame rasterization. Buffering the output to disk means that the page does not have to be re-rasterized for each copy. In general, you should leave this option off. This setting is ignored if there is enough memory to perform full-frame rasterization.

#### Ribbon Color

Ribbon Color choices are Monochrome, 3-Color, and 4-Color. Make sure your choice here corresponds with the ribbon you have installed in your printer! If there is a mis-match, color layers may get “out of synch.”

### Star Micronics SJ-144 printer

#### Paper Size

Select the correct paper size from the available choices. Note that when the Strip Label Cassette is being used, the only paper choice by default is Strip Labels.

#### Custom Papers

The Custom Papers option is described in the “General Setup” section earlier in this chapter. Do not create a custom paper for the Strip Label Cassette.

#### Color Ribbon

Select the Color Ribbon check box when you have a color ribbon installed in the printer. Note: this option is ignored when you are using Strip Labels. Use a monochrome ribbon for labels.

#### Mirror Image

When Mirror Image is enabled, SuperDriver will mirror the image on output.

### Star WinType 800C printer

#### Buffer to disk

The Buffer to disk option can speed output of multiple copies when there is not enough memory to do full-frame rasterization. Buffering the output to disk means that the page does not have to be re-rasterized for each copy. In general, you should leave this option off. This setting is ignored if there is enough memory to perform full-frame rasterization.

#### Ribbon Color

Ribbon Color choices are Monochrome, 3-Color, and 4-Color. Make sure your choice here corresponds with the ribbon you have installed in your printer! If there is a mis-match, color layers may get “out of synch.”

#### Mirror Image

When Mirror Image is enabled, SuperDriver will mirror the image on output.

## Bitmap SuperDrivers

These drivers don't create printer output—they generate a variety of bitmap (paint) files from any Windows application. Now you can “print” bitmap files from your illustration and page layout programs for use in desktop publishing, or for enhancement in Paintbrush or other bitmap editors.

There are SuperDrivers for seven different file formats: Targa (.TGA), TIFF (.TIF), Windows BitMaP (.BMP), JPEG (.JPG), GIF, PCX and a multi-page variant of PCX called DCX.

The Targa file format was developed by TrueVision for their line of Targa and Vista graphics cards. It is commonly used by high-end paint and image retouching software.

The TIFF format was developed by Aldus to provide a machine-independent image format for desktop publishing. Programs such as Ventura Publisher and PageMaker have TIFF import capability.

The BMP format was developed for Windows 3.x and its Paintbrush application. When using Paintbrush or the Windows clipboard, it is perhaps the most well-behaved format in the Windows environment. It is the only format in Bitmap SuperDriver with a 4-bit color and an 8-bit grayscale setting.

The JPEG format was developed by the Joint Photographic Experts Group. It uses a “lossy” compression scheme that yields good results with considerably reduced file sizes. This is a common format for use over the Internet.

The GIF (Graphics Interchange Format) specification was developed by CompuServe. It is widely used there as well as on many other on-line services including the Internet. While the GIF SuperDriver represents itself to applications as a 24-bit color driver, the output it generates is 256-color palette based.

The PCX format was created by ZSoft for their PC Paintbrush and later, Publishers Paintbrush applications. The DCX format is a multi-page version of PCX, suitable for use in several digital FAX applications.

### Installing a Bitmap SuperDriver

- Follow the instructions in the “Getting Started” manual for installing a printer.

- When the list of SuperDrivers appears, choose Bitmaps from the Manufacturers list.

- Choose the file format you want from the Printers list and click Next.

- For nearly all installations, you should choose FILE from the Available Ports list. The only time you would choose a hardware port is if you have a special printer that has direct support for the chosen bitmap file format.

- Proceed with the remainder of the installation as you would for any other printer.

## Bitmap SuperDriver properties

### The Paper tab

The Paper tab page contains settings Paper Size,  Resolution, and Orientation.

#### Paper Size

Windows applications expect the printer driver to report the current paper size back to them. In many desktop publishing and graphics applications, this setting will determine the available on-screen workspace. It also relates to total file size; for any given resolution, a Legal page setting will produce a larger bitmap file than a Letter page.

#### Resolution

This list box shows the available preset resolutions. When choosing a resolution, keep in mind the size of the resulting bitmap file—higher resolutions yield more detail, but they also increase the file size.

There may be cases when you would want to use a custom resolution. For example, when creating 24-bit bitmaps, resolutions of 75 and above might create bitmaps too large for your needs. Another possibility would be choosing a resolution to match a particular device (for example, creating a 360 dpi bitmap for printing on a BubbleJet printer).

To enter a custom Resolution:

- Highlight the number in the Resolution box and enter a new value.

#### Orientation

This setting should correspond to the page orientation used by the application.

#### Custom Papers

The Custom Papers dialog is described in the “Common Settings” section at the beginning of the manual.

Using Custom Papers settings, it is possible to create bitmaps in custom sizes when using applications which support them. For example, if you've set your page size in CorelDRAW! to 8 x 4 in., you can create a match for those settings in Custom Papers. This way you will be assured of getting a bitmap that has no gaps around the edges or unwanted cropping. Do not use these settings to change the shape of the bitmap to something different than the page used by your application. You could end up with an unusable bitmap.

If you want to set the dimensions of your desired bitmap in pixels, you’ll have to do a little math. For example, for bitmaps intended to be displayed on a computer screen (e.g., 640 x 480 for VGA resolution), you can create a custom page size of 6.4 inches by 4.8 inches, then set Resolution to 100 dpi.

Options

When you choose the Options tab, the Options page appears. The page for the TIFF SuperDriver is shown; other formats may vary.

#### Compression

The Compression option enables image compression for those file formats that support it. This can dramatically reduce file sizes. Unless you are having problems using compressed files in your target application, leave Compression on.

The Compression setting can also affect the file type:

|  | BMP format | TIFF format | Targa format |
| --- | --- | --- | --- |
| Compression On | (not supported, setting ignored) | PackBits type 32773 | Top-down, file type 10 |
| Compression Off | Normal | Normal | Bottom-up, file type 2 |

#### Quality (JPEG only)

The Quality slider in the JPEG SuperDriver controls the amount of compression used in the output file. Higher numbers yield better quality and larger file sizes, while lower numbers reduce file sizes at the expense of  image detail. Since JPEG is a lossy compression format, low Quality numbers can cause unwanted image artifacts, and single-digit numbers can result in bad, even unrecognizable output. The default, which is 75, provides a good balance of compression and quality. You may want to experiment to find the balance you need.

#### Bits per pixel

For each File type, there is a different set of choices for Bits per pixel.

- One bit per pixel means that each pixel (short for picture element, also known as a dot) is either on or off, black or white. Grayscales are simulated using SuperPrint's current dither pattern.

- In the four bits per pixel image, each pixel can be one of 8 colors.  Additional colors are simulated.

- In an eight bits per pixel image, each pixel can be one of 256 shades of gray.

- Color images are saved in 24 bits per pixel format, using eight bits for each primary color (red, green, blue). Since each eight bits represents 256 possible color levels, this means that up to 16,777,216 colors can be represented (2563).

Remember that your choice here will also determine file sizes. 24-bit image files can become quite large, especially at higher resolutions. (For example, a letter-size BMP file in 24-bit color at 100 dpi is 2.8 MB!) Make sure there is ample disk space on the target drive before attempting to create a large bitmap.

The GIF SuperDriver shows 24 (RGB, 16.7 mil colors) in the Bits per pixel box. The actual output is a 256-color GIF. This 24-bit reporting is used to make sure applications send true color data. The colors are mapped to a 256-color palette by the GIF SuperDriver.

#### Anti-Aliasing

Anti-Aliasing is a process that gives the illusion of doubling the resolution of a 24-bit color bitmap or an 8-bit grayscale bitmap. Its best application is for making bitmaps containing text or object graphics.

The process works by first rasterizing the page at double the selected resolution (for example a 100 dpi graphic is rasterized at 200 dpi). Then, pixel-by-pixel, SuperDriver examines each target pixel and the surrounding eight pixels, then uses a weighted averaging algorithm to arrive at a new color value for the target pixel. Finally, the extra pixels are removed, leaving a bitmap in the originally requested resolution.

The effect is mainly noticeable on diagonal lines and curves—the “jaggies” are considerably reduced.

Anti-Aliasing provides almost no benefits for scanned (natural) images. Text and graphics can be improved considerably, but at a cost—it can nearly double the time it takes for output and memory requirements are increased.  Anti-Aliasing is unavailable for 1- and 4-bit bitmaps.

#### Output Path

The Output Path box allows automatic naming of bitmap files. There are three ways to print a bitmap file (all require the port to be set to FILE):

1. With no entry in the Output Path box, Windows will pop up a dialog asking for the filename for each job you print. This method is only suitable for single-page documents.

2.  With only a path in the Output Path box (for example, C:\TEMP\), the bitmap SuperDriver creates a file in the target directory using the filename (or document name) of the source with the correct file extension appended. For example, if you were to print “Myletter” from Microsoft Word to the TIFF SuperDriver, the resulting file would be “Microsoft Word - Myletter.TIF”. Again, this is only suitable for single-page documents. However, if you are imaging a long queue of one-page PostScript files, for example, this can be very handy.

3.  You can also place full path and filename entries in the Output Path box. If you use just a plain filename, you will end up with only the last job printed, since each job will overwrite the previous one. However, there is a mechanism to create multiple, numbered files – ideal for printing multi-page documents.

#### Multi-page files to a bitmap SuperDriver

Automatic numbering of bitmap filenames on output can be accomplished by specifying a filename that includes a formatting code. The code specifies how page numbers form the filename. Simply insert a percent sign (%) and a d (for decimal) in the filename.

For example, if your desired filename is bird.tif,  enter it as bird%d.tif. The resulting filenames will be bird1.tif, bird2.tif,...bird10.tif.

For a more complex numbering system,  insert a zero and a number (0n)  between % and d,  where 0n will create a constant width for the page number.  For example:

bird%02d.tif will result in bird01.tif, bird02.tif, ...
xx%04dyy.tif will result in xx0001yy.tif, xx0002yy.tif, ...

Numbering in hexadecimal format is also available using %X. For example:

hexy%04X.tif will result in hexy0001.tif, ..., hexy0009.tif, hexy000A.tif....

#### Halftoning options

Halftoning is only applicable to 1- and 4-bit formats. For 8-bit (grayscale) and 24-bit (color) formats, no dithering is needed.

## SuperDriver status

If you are interested in viewing the progress of a printing job, SuperPrint provides the SdStatus mini-application. It can be accessed by an icon in the SuperPrint group.

In Windows 95, it appears minimized whenever the SuperDriver is active. By default, it disappears after a few seconds of inactivity. You can force it to remain active by clicking on its icon in the tool bar.

The progress bars are allocated dynamically and can take on different labels during the course of a print job. Typically, you’ll see status of things like sub-object counts, scanlines sent to the printer, and throughput rates.

(A sub-object can be any of the attributes of an object, such as fill color, outline thickness, position, etc.)

The Setup button opens the SuperDriver’s printer properties dialog. This is a quick and easy alternative to going through the Windows 95 Printers folder or Windows NT Print Manager to change driver settings.

The Print button lets you load Windows spool jobs directly. These are jobs that have already been submitted by an application to Windows for printing. To intercept and save these .SPL files, you must print while the printer is paused, go into the Windows\Spool\Printers folder, and copy these files to another folder. Note: In Windows 95, the Spool folder is hidden by default. You may have to use the Windows Properties feature to reveal the folder.

If you choose, you can rename spool files that were created with a SuperDriver and give them a .SMF extension, since they are really SuperMetafiles. This provides one method of sharing print jobs with other computers. (However, most users find PostScript files more convenient.) The Print button can only be used for spool files and SuperMetafiles, not for any of the other file types supported by SuperQueue filters.

The Cancel button stops a print job that was started by using the Print button. It does not stop jobs submitted in any other way.

# Masquerade reference

### What is Masquerade?

The Masquerade utility is included with SuperPrint and works in conjunction with the ZScript PostScript language filter with SuperQueue. Masquerade allows any non-PostScript Windows printer driver (including SuperDrivers) to “masquerade” as a PostScript driver. For example, if you run the HP LaserJet 4 SuperDriver through Masquerade, a new driver will appear in your printers folder, “HP LaserJet 4 (Z) (PSCRIPT).” When you look at the new driver's Setup and Options dialogs, it appears to be a PostScript printer. Internally, however, it has all the attributes of the LaserJet 4 SuperDriver – the paper trays, printable area, fonts, resolution, and color capability. The new driver lets you print documents that contain PostScript information (such as EPS files) directly to your non-PostScript printer.

Note: The non-PostScript driver is referred to in this manual as the “target” driver; it's where the actual output will eventually be generated.

In addition, Masquerade provides the following benefits:

- Automatically handles devices with non-standard imaging areas, such as film recorders and some color thermal printers.

- Automatically creates a PostScript Printer Description (PPD) file for each printer driver you process through it.

## Before installation

While not absolutely necessary, we recommend you follow these steps before running Masquerade.

#### Typeface setup

Make sure that all your installed typeface libraries (TrueType or ATM) are up to date. That is, remove any typefaces you don't plan to use, or add any recently acquired typefaces. If you ever want to change your font selection, Masquerade makes it easy to do. The preparatory steps outlined here are offered to help make your initial installation go smoothly.

If you plan to use typefaces from TrueType:

- If your target driver is a SuperDriver, make sure TrueType is checked on in the Font Engines section of the SuperDriver Tricks dialog.

If you plan to use typefaces from Adobe Type Manager:

- Make sure ATM is turned on in the ATM Control Panel. If your target driver is a SuperDriver, make sure ATM is checked on in the Font Engines section of the SuperDriver Tricks dialog.

If you are using a Windows or third-party target driver that lets you install hardware fonts or cartridges:

- Make sure your typeface list in the target driver is current.

### Installing a PostScript driver in Windows 95

If Masquerade cannot find a PostScript driver already installed in Windows, it will prompt you to install one. The only way to get a PostScript driver installed is to use the Add Printer wizard and install a “dummy” PostScript printer. After the dummy printer is installed, it can be removed from the Printers folder, leaving the necessary driver files on your disk for Masquerade and ZScript to use.

For your convenience, here are detailed instructions on this process.

1.	Click the Start button.

2.	Choose Settings.

3.	Choose Printers.

The Printers folder appears.

4.	Double click on Add Printer.

The Add Printer Wizard appears.

5.	To begin installing the printer, click Next.

6.	Choose Local Printer and click Next.

The Wizard then asks you to choose the manufacturer and model of your printer. For purposes of installing a PostScript driver, any PostScript printer will do. Since Apple is the first manufacturer listed, and since they make PostScript printers, we suggest choosing one of them.

7.	Choose Apple LaserWriter NTX, then click the Next button.

You are asked for the port information. Since you will be removing this printer, the port doesn’t matter.

8.	Accept the default by clicking on Next.

After choosing your port, you are asked for a name for this printer and whether you want to make this printer the Windows default.

9.	Accept the default name and make sure the default printer status is set to No, then click Next.

Finally, you are presented with an option to generate a test page.

10.	Choose No for the test page and click Finish.

You are asked for your Windows CD-ROM or diskettes. Follow the instructions on the screen. After the printer has finished installing:

11.	With the new icon highlighted, choose Delete from the File menu.

Windows will ask, “Some files were used only for this printer and are no longer needed. Would you like to delete these files now?”

12.	Click on No.

Masquerade now has the files it needs to create custom PostScript drivers.

### Installing a PostScript driver in Windows NT

1.	Choose Print Manager from the Main window in Program Manager

2.	Choose Create Printer from the Printer menu.

The Create Printer dialog appears.

3.	Click the arrow next to the Driver drop-down list box, scroll down to any one of the Apple LaserWriter NTX models.

4.	Enter any name for your printer in the Printer Name box.

You can give it any name you want since it will be deleted shortly.

5.	Click OK.

The Windows NT Setup dialog appears requesting a path.

6.	Enter the path of the drive containing your Windows NT disk.

7.	Click OK.

After the printer has finished installing:

8.	With the new printer window highlighted, choose Remove Printer from the File menu.

Masquerade now has the files it needs to create custom PostScript drivers.

## Running Masquerade

The Masquerade icon can be found in the SuperPrint group in your Start/Programs menu.

In the Installed ZScript Printers box at the top is the list of PostScript drivers created by Masquerade with the ports to which they are connected. In the middle section is a listing of the Non-PostScript Printers already installed in Windows.

At the bottom is a list of the PostScript Drivers installed on your system which are compatible with Masquerade. Masquerade automatically detects and lists all compatible PostScript drivers.

If you're sure the settings for the desired non-PostScript driver are correct, you can proceed. If not, use the Setup... button to call up the Printers folder (or Print Manager in Windows NT) and make sure all properties in the target driver (especially resolution) are correct.

#### How to create a new driver

- Choose the target driver from the Non-PostScript Printers list box.

- Choose from among the available PostScript Drivers.

- Click on the Install button.

An informational status bar appears at the bottom of the Masquerade dialog showing the progress of the installation process. Note: If you want to abort the driver creation process, you can press the ESC key.

Note that because Masquerade is creating .PFM files for every typeface installed on your system, the more typefaces you have, the longer this process will take.

When it is finished, a new listing will appear in the Installed ZScript Printers list box. This new ZScript-specific printer driver will now appear in the Printers folder (or Print Manager in Windows NT) and every application that allows you to choose from among Windows printer drivers. The name will be the same as the target printer listing with a legend added to the end that corresponds to the driver chosen under PostScript Drivers (if the original target driver's name is exceptionally long, some letters at the end may have been clipped to make room for the new legend).

#### Making the new driver the default

If you plan to do most of your printing through ZScript using this new printer driver, you can make it the Windows default by clicking on the Set as Default Printer button. This has the same effect as choosing the similar settings in the Windows 95 Printers folder or Windows NT Print Manager.

#### Updating an existing driver

If you have added or removed typefaces from your target Windows printer driver or from TrueType or ATM, you'll want to update your Masquerade-generated PostScript driver. To do so:

- Choose the driver you want to update from the Installed ZScript Printers list box.

- Click on the Update Font List button.

Masquerade will update the appropriate PPD file.

#### Removing an existing driver

- Choose the driver you want to update from the Installed ZScript Printers list box.

- Click on the Remove button.

This has the same effect as removing the driver from the Printers folder or Print Manager.

### Un-doing everything Masquerade has done

There may be a situation where you want to remove all the Masquerade- generated drivers, PPD files, and WIN.INI font entries (de-installing SuperPrint would be one such instance). For that purpose, we have provided a Reset button.

A confirmation box pops up when you click on Reset, giving you an opportunity to cancel the request. If you click OK, Masquerade removes almost all traces that it was ever run.

## Settings for Windows PostScript drivers

The settings discussed here are all contained in the Windows PSCRIPT driver. These settings apply specifically to Masquerade-generated drivers.

### Spool settings

#### Spool settings in Windows 95

- In the Printers folder, right-click on the icon of the printer and choose Properties from the resulting menu.

- Click the Spool Settings… button on the Details page.

- Make sure Spool print job so program finishes printing faster is chosen.

Automatic printing will not work if Print directly to the printer is chosen! Note that Start printing after last page is spooled and Start printing after first page is spooled are ignored; printing always begins after the last page is spooled.

- Check that Spool data format is set to RAW.

- Click OK to exit the Spool Settings dialog

#### Spool settings in Windows NT

- In the Print Manager, highlight the printer window and choose Properties from the Printer menu.

- Click the Details button.

- Make sure Print directly to port is off.

Automatic printing will not work if Print directly to port is on! Note that Job prints while spooling is ignored; printing always begins after the last page is spooled.

- Check that Default datatype is set to RAW.

- Click OK to exit the Printer Details dialog

### Graphics settings

#### Graphics settings in Windows 95

On the Graphics page, you are given Halftoning options. Output will almost always be faster with Use printer’s settings chosen. You should only choose Use settings below if you have a special need for a particular screen angle and frequency.

In a Masquerade-generated driver, the Resolution number should automatically reflect the resolution of SuperPrint's target output device.

#### Graphics settings in Windows NT

The only settings for graphics are found by choosing Properties from the Printer menu in Print Manager, then clicking the Setup button. Output will almost always be faster with Use Printer Halftoning chosen. You should only turn this setting off if you have a special need for a particular Windows NT halftone pattern.

### Fonts

#### Fonts in Windows 95

The ZScript PostScript filter recognizes TrueType fonts as “resident” for use with Masquerade-generated PostScript drivers. You will get better performance with TrueType fonts by setting the driver not to download TrueType fonts as follows:

- In the Printers folder, right-click on the Masquerade-generated printer icon.

- Choose Properties from the resulting menu.

- Choose the Fonts tab.

- Click on the Send Fonts As… button.

- Using the drop-down list box, set TrueType fonts as to Don’t Send.

- Click OK until you return to the Printers folder.

If you are using PostScript fonts with Adobe Type Manager 3.02 or earlier (the latest version available at the time this manual was written), those fonts will be downloaded in the PostScript file regardless of the setting in the driver. This is controlled by Adobe’s “Softfont” entries in the WIN.INI file.

#### Fonts in Windows NT

In Windows NT, both TrueType and Adobe Type 1 fonts can be added directly into the Fonts utility in Control Panel. Using this utility, Type 1 fonts can be translated into TrueType format for use on the screen and when printing to non-PostScript printers.  When you install the font, you can instruct Windows NT to download the Type 1 outlines when printing to a PostScript device.  We recommend this approach for use with Masquerade-generated drivers.

Masquerade detects all installed Type 1 fonts and includes them in the PPD file. However, the Windows NT PostScript driver is “hard-wired” with the font metrics for the 35 “Plus fonts” (the fonts that are found as resident on most PostScript printers – dating back to the Apple LaserWriter Plus) plus a few additional fonts. If you have used Control Panel to install the Plus fonts, the PostScript driver can treat them as “resident” for use with the Masquerade-generated driver.  It has no provision for treating most other Type 1 fonts as resident. Therefore, most other typefaces must be downloaded in the PostScript data stream.

### Other settings in Windows 95

#### PostScript

Under PostScript output format: you can choose either PostScript (optimize for speed) or PostScript (optimize for portability - ADSC). The ADSC stands for Adobe Document Structuring Convention; this setting can help when printing multi-page jobs with large virtual memory requirements.

Print PostScript error information is not really needed since SuperQueue has a user-friendly message window; we recommend turning this option off.

We recommend using the default setting for the Device Options page. All other settings not specifically mentioned here can be changed to suit your needs.

### Printer description files

Masquerade automatically creates a PPD (PostScript Printer Description) file with printer-specific information such as paper sizes and trays, resolution, and color capabilities. PPD files are used not only with the Windows PostScript driver, but also with Adobe Systems' drivers and on Macintosh and NeXT computers and Sun workstations. If your system or application on these other machines can use a PPD file, you can create cross-platform PostScript files that will work cleanly with ZScript.

PPD files created by Masquerade can be freely distributed.

Newly-created PPD files in Windows 95 are placed in the Windows\System folder. In NT, the PPD files are placed in the Windows\System32\Spool\Drivers\w32x86 folder. Note that the Spool folder is hidden by default.

# SuperPrint and Networks

There are two ways to use SuperPrint over a network:

- Send already-processed “raw” printer data through a virtual port

- Send SuperMetafiles as page description files

The first option puts the burden of creating the printer data on the originating (client) workstation, and usually ends up sending more data across the network. The second option, using SuperMetafiles, puts the printing burden on the remote (server) computer, and typically results in less data traveling over the network.

### Adding a network printer in Windows 95

1.	Click the Start button and choose Settings/Printers.

2.	Double click the Add Printer icon.

The Add Printer Wizard appears.

3.	Click Next to get started.

4.	Choose Network Printer and click Next.

5.	Enter the Network path or queue name.

It may be easier to use the Browse button to locate your remote printer.

6.	Click Next.

If you are connecting to a printer server that’s also running Windows 95, the Add Printer Wizard will proceed to the next screen automatically since it can get the driver files from the printer server.

If you are connecting to a Windows NT printer server or some other form of network print server architecture (such as Novell), you will be asked for the manufacturer and model of the printer you are using. If you are using a SuperDriver, you should choose Have Disk  and follow the instructions on the SuperPrint Getting Started card for installing a SuperDriver.

7.	Give the printer a name and choose whether or not you want to make it the default printer, then click Next.

8.	Choose whether or not you want to print a test page, then click Finish.

If the network printer server to which you are connecting is running Windows 95, the driver it is using is automatically copied to the client workstation.

IMPORTANT! Unless a site license has been issued by Zenographics, SuperPrint comes with a single-user license. It is not legal to copy the SuperDriver in this manner to a computer that does not already have a licensed copy of SuperPrint.

When connecting to a printer server other than Windows 95, you will be asked to insert the disks needed to install the driver locally.

### Adding a network printer in Windows NT

If you are using Windows NT on the client workstation and intend to use a SuperDriver to print to a network printer server that is not running Windows NT, you must first install the appropriate driver files using Print Manager’s Create Printer (as explained in the SuperPrint “Getting Started” manual). This is because the Windows NT Connect to Printer setup routine has no capability to get the necessary files from a third-party installation disk. Using Create Printer causes the necessary files to be copied to your hard disk, but it leaves an unusable “phantom” local printer on your system – one that could cause confusion. Therefore, when you have finished installing the driver, you will use Printer/Remove Printer in Print Manager to get rid of the phantom printer; the driver files will remain on the hard disk for use in network printing.

If you are printing from a Windows NT workstation to a Windows NT printer server, you do not need to install the printer driver in advance; the files will be copied automatically. (Please see the legal caveat below.)

1.	Open Print Manager from the Main window.

2.	Choose Connect to Printer… from the Printer menu.

3.	Locate the desired printer from the Shared Printers list.

If you are connecting to a non-Windows NT server, you’ll get the following message:

Because you previously installed the appropriate driver, it will be in the list that appears when you click OK from this dialog. Choose the correct name from the Driver drop-down list box and the installation will be complete.

If the printer server to which you are connecting is also running Windows NT, the correct driver is copied automatically from the server, completing the installation.

WARNING! Unless a site license has been issued by Zenographics, SuperPrint comes with a single-user license. It is not legal to copy the SuperDriver in this manner to a computer that does not already have a licensed copy of SuperPrint.

## Sending raw data

If the computer that serves your network printer also needs to print from DOS or other non-Windows applications, or if SuperQueue has not been installed on it, you must configure SuperQueue to create the printer data on the client workstation and send it as raw printer data. If the printer server does have SuperQueue, but you would rather distribute the rasterization processing to the client workstations, you can still use SuperQueue on both the client and printer server computers.

While this method of printing is the most straightforward and will work with virtually all networks, it does have one drawback – if your network printer does not support a “high-level” language (like PostScript or PCL), each page will be a full-page graphic. If you are sending multi-page documents, your printer server should have substantial hard disk space available for buffering the raster data. Since the print server has no idea where individual pages end, it must buffer the entire print job before it begins to send data to the printer.

### Configuring SuperQueue to send raw data

With the appropriate printer window in SuperQueue highlighted:

- Make sure Remote Processing in the Options menu is off.

#### Sending raw data to a non-Windows printer server

If the printer server computer is not running either Windows 95 or Windows NT (for example, a Novell server), there is one more step you must take to enable printing of raw data.

Under Windows 95:

From the Printers folder (Start/Settings/Printers), right-click on the icon of the network printer.

- Choose Properties from the resulting menu.

- Choose the Details tab.

- Click the Spool Settings button.

- Change the Spool data format to RAW.

- Click OK until you return to the Printers folder.

Under Windows NT:

- In Print Manager, choose Properties from the Printer menu.

- Click the Details button.

- Change the Default Datatype to RAW and click OK.

#### Messaging when sending raw data

When the client computer is creating the raw printer data, SuperQueue filter messages appear in the Preview tab of the appropriate printer window.

## Sending SuperMetafiles (SMFs)

If the computer that serves the network printer is running SuperQueue, you will be able to take advantage of using SuperMetafiles over the network.

SMFs are almost always much more compact than raw printer data, requiring less transmission time. Another advantage is that processing of the page takes place in the printer server computer, not in the client workstation. You can even have more than one type of printer hooked up to the printer server.

### Configuring SuperQueue to send SuperMetafiles

With the appropriate printer window in SuperQueue highlighted:

- Make sure Remote Processing in the Options menu is on.

When the workstation prints, it sends a SuperMetafile to the spool directory on the printer server. When the new file is received, the Windows print processor automatically begins processing it.

#### Driver settings

The printer and document options you’ve chosen on the client workstation (things like font downloading, copies, resolution, etc.) are transmitted within the SuperMetafile. They will override the settings on the printer server.

#### Messaging when sending SuperMetafiles

When the printer server computer is processing the print job, SuperQueue filter messages appear in the tab that corresponds to the remote printer port.

#### Multiple Printers on the SMF Server

Since a SuperMetafile also contains references to the output device for which it is intended, it is possible to send SMFs for multiple devices to a SMF Server. If, for example, the SMF Server is hooked up to a Canon Bubble Jet on LPT1 and a Hewlett-Packard LaserJet on LPT2, the SMF Server could accept files intended for both devices. SuperQueue would then direct the output to the appropriate device automatically.

### Printer server administration in SuperQueue

#### Re-ordering jobs

Print jobs sent to the printer server may not have equal importance or priority. SuperQueue makes it easy to re-order jobs in the queue. To change the order in which the jobs will print, just move them where you want them. Changing the order of jobs must be done from within SuperQueue on the printer server; re-ordering jobs from the client workstations has no effect.

To re-order a print job:

- Click on the job you wish to move and drag it to its new location in the list.

The job will be inserted at the point where you release the mouse button.

Note that the job that is currently being processed cannot be interrupted by placing a job ahead of it on the queue. If you drag another job to the top, it becomes the next job to be processed.

You can also use Promote Document, Demote Document, Bring to Front, and Send to Back as explained in the SuperQueue chapter.

#### Holding and deleting jobs

Jobs can be held, released, and deleted from both the printer server and the client workstations. The exact process is described in the SuperQueue chapter under “How to hold and release a document” and “How to remove a single job from the queue.”

### Working offline

If SuperQueue on the client workstation cannot establish communications with the printer server, it will automatically set itself for working offline. This is easy to spot; the title bar of the printer window reads, “User Intervention Required – Work Offline.” You can continue to generate print jobs; they’ll simply accumulate on your hard disk, waiting for the printer to be set back to an on-line condition.

You can also force this condition by choosing Work Offline in the Printer menu. This can be a handy way to do batch printing, since you can continue to generate print jobs without tying up any network resources.

Once the printer server is ready to accept jobs, simply uncheck Work Offline in SuperQueue’s Printer menu. All waiting jobs will be sent to the printer server.

# Index

A

About

in SuperDriver	42

Accurate Images/Strokes	26

Accurate text clipping	26

Add Document

multiple jobs	14

Add Documents	12

Adding typefaces	63

Adobe Type Manager	34, 60

Anti-Aliasing

in Bitmap SuperDrivers	56

Application Fill Patterns	34

Arrange Icons	9

Ask for preferences each job	22, 23

Automatic memory management	32

B

Bi-directional Printing

on dot matrix printers	50

Bitmap filters	24

Bitmap names, automatic	57

Bitmap SuperDriver

installing	53

Bitmap SuperDrivers	53

Bits per pixel	56

BJC Print Modes	47

Black Generation	45

BMP files

defined	53

BMP filter	24

Buffer to Disk

on Fargo Primera	52

on Star WinType 800C	52

C

Canon Bubble Jets	46

Canon laser printers	44

Cascade	8

Center on page	24

Clear All	17

Close, in the preview window	19

Color checkbox

on Canon BubbleJets	47

on HP DeskJets	45

on Lexmark ExecJet	48

color correction	38

Color Matrix	39

color model	37

color problems	33, 35, 37

Compression	31

in Bitmap SuperDrivers	See

Configure Filter	14

Connect to Printer	68

Contrast	36

in bitmap filters	25

Copy

in the Preview window	19

on the Edit menu	16

Custom Papers	29

in Bitmap SuperDrivers	54

removing	30

tutorial	30

D

Data Type	9

DCX files

defined	53

Defaults button

in Hue Correction	39

Demote Document	10

Density on HP LaserJets	44

DeskJets	44

dithered text and lines	33

Document Name	9

Document Properties	19

Dot Gain	41

Dot-Matrix Printers	49

Download Fonts/Rectangles	42

drag and drop	12

Duplex tab	43

E

EconoMode on HP LaserJets	44

Edit menu	17

EMF	19

Enable Micrografx escapes	33

Enhance Black on Canon Bubble Jets	47

Enhance Lightness/Contrast	25

Epson Stylus	48

Expected Printing Perfomance	32

F

Fargo Primera	51

file extensions	13

Files of Type list box	13

Filter Libraries	21

filter preferences

pop-up	14

Find	16

Font Engines	34

Fonts	8

Form Feed Each Page

on Canon Bubble Jets	46

on dot matrix printers	51

G

GIF files

defined	53

GIF filter	24

H

Halftoning	40

in Bitmap SuperDrivers	58

too fine or too coarse	34

High Speed on Epson Stylus	49

Hold Document	19

Hold selected document(s) in printer queue	17

How

to add a filter	21

How to

add a job to the queue	12

automatically arrange printer windows	8

change the order of jobs in the queue	10

change the screen font	8

choose a message tab	15

clear the contents of the message area	17

configure a filter for automatic operation	22

copy messages onto the clipboard	16

find text in the message area	16

hide and restore the status bar	6

hide and restore the tool bar	6

hold and release a document	19

pause and restart a printer	20

preview a document	17

remove a filter	23

remove a single job from the queue	10

remove all jobs from the queue	11

set a printer as the default	21

view the queue and message areas	7

HP DeskJets	44

HP LaserJets	43

printer memory	44

HP PaintJets	45

top of form	46

Hue Matching	37

I

Ignore screens from PostScript file	26

Image Enhancement	25

Image tab	34

inkjet printers	44

Install button	62

install filter	21

Installed ZScript printers	62

J

JPEG files

defined	53

JPEG filter	24

K

Kyocera laser printers	44

L

Language Level	25

laser printers	42

LaserJets	43

printer memory	44

Lexmark ExecJet	48

Lexmark laser printers	44

Library Contents (Filters) box	23

Lightness	36

Luminosity Bar	39

M

Manage Filter Libraries	21

Manual Filter Selection	13

Match page orientation to image	25

Media

on Canon Bubble Jets	47

on Epson Stylus	49

on HP DeskJets	45

on HP PaintJets	46

on Lexmark ExecJet	48

memory management	31

Messages	15

Micro Weave on Epson Stylus	49

Minimum Margin	25

Move To Front/Back	10

N

Network

in Windows 95	67

in Windows NT	68

overview	67

sending raw data	69

sending SuperMetafiles	70

Next Page	19

Non-PostScript printers	62

O

Options tab	31

in Bitmap SuperDrivers	55

Orientation	28

on HP DeskJets	44

on HP LaserJets	43

on HP PaintJets	46

Output Path in Bitmap SuperDrivers	57

Owner	9

P

Page Protect on HP LaserJets	44

PaintJets	45

top of form	46

Panasonic laser printers	44

Paper Size	28

custom	29, 30

in Bitmap SuperDrivers	54

on Fargo Primera	51

on Star SJ-144	52

Paper Source	28

Paper tab

in Bitmap SuperDrivers	53

Papers tab	28

Pattern Scale	34

patterns in bitmap fills	34

Pause Between Pages on Epson Stylus	49

Pause Printer	20

PCX files

defined	53

PostScript drivers	62

PostScript filter	25

PPD files	66

removing	63

Presentation Mode

on dot matrix printers	51

on HP PaintJets	46

Preserve Shadows/Highlights	25

Preview	17

Preview Messages	15

Previous Page	19

Print Preview	17

Print Quality	29

on Canon Bubble Jets	47

on HP DeskJets	44

Print to file	14

Printer drivers

PostScript	64

printer fonts

with SuperDriver	27

printer windows	7

Promote Document	10

Purge Printer	11

Q

Quality setting in JPEG SuperDriver	56

queue window	9

R

RAM	31

RAW data type	10

rectangles on top of text	42

Release Document	19

Remove a filter	23

Remove Document	11

Report 1 bit/pixel...	33

Reset	63

Resolution	29

in Bitmap SuperDrivers	54

on Canon Bubble Jets	46

on dot matrix printers	50

on Epson Stylus	49

on HP LaserJets	43

Restore Defaults button	29

RET on HP LaserJets	44

Retry button	32

Ribbon Color

on Fargo Primera	52

on Star WinType 800C	52

S

Saturation	37

Save preview	19

screen fonts	8

Select All	16

Set as Default Printer	21

in Masquerade	63

Setup filter	22

Sharpness	36

Show accurate text/line color	33

slow printing	33

on dot matrix printers	50

Smooth Black on Canon Bubble Jets	47

Sound	17

SQF files	22

Star SJ-144	52

Star WinType 800C	52

Start Printer	21

Stretch to fit page	24

SuperDriver

definition	27

status bar	6

SuperRIP tab	31

T

Targa files

defined	53

text clipping	26

text on top of objects	42

thermal printers	51

threshold arrays	41

TIFF files

defined	53

TIFF filter	24

Tile	8

tool bar	5

Toshiba dot matrix printers	49

Tricks tab	33

True Type	34

TrueType	59

Two Page	19

Typefaces	59

adding	63

U

Update font list	63

V

View menu	7

W

WAV files	17

Window menu	8

Windows NT	4

Windows/SuperDriver Interface	33

WMF	19

Z

Zoom In/Out	19