AIORCKT.NLM V2.01

Comtrol Corporation

(March 1995)

The AIORCKT.NLM driver is a NetWare Loadable Module (NLM) that

provides asynchronous input/output (AIO) driver support for

Comtrol(TM) RocketPort(R) and RocketPort RA high-performance serial boards.

The AIORCKT.NLM driver is loaded onto the NetWare server along

with AIO.NLM.

AIORCKT.NLM provides the low-level software interface between AIO and

the Comtrol hardware, which includes:

o	Handling all enqueuing and dequeuing of serial data

o	Modem control signal functioning

o	Hardware and software flow control

o	Port configuration

o	Status information

Most of these operations are performed by the custom ASIC on the

controller, which considerably reduces system overhead on the NetWare

server, while at the same time, increasing serial I/O performance.

AIORCKT.NLM supports up to four RocketPort or RocketPort RA boards in one

server. The boards can have any configuration of from 4 to 32 ports

each, for a maximum of 128 ports. Different controller configurations

may be mixed in one server. For example, one 8-port and two 16-port

RocketPort or RocketPort RA boards is a valid configuration.

Baud rates from 50 to 230.4Kbps are supported on all ports and all ports

fully support hardware/software flow control and full modem signaling

(excluding the RocketPort 8J).

Installation

------------

Use the following information to configure and install your Comtrol

RocketPort or RocketPort RA board.

o	Select an I/O address range that you want the controller

to occupy. The procedure for setting DIP switches

depends on whether this is the first or the second, third,

and fourth board you are installing. Part of the controller

address is selected by hardware and part by software. The DIP

switches take care of the hardware part and the driver takes

care of the software part.

If this is the first RocketPort controller you are installing,

then you must choose an I/O address range containing

68-byte contiguous, free I/O addresses from the following list:

1) 100-143	4) 200-243	7) 300-343

2) 140-183	5) 240-283	8) 340-383

3) 180-1C3	6) 280-2C3	9) 380-3C3

Refer to the "SW1 Settings Chart" and set the I/O DIP switch

according to the I/O address choice you have made.

If you are installing the second, third, or fourth controller,

then the DIP switch setting depends on the setting of the first

board installed and not the address range that the second, third,

or fourth board is going to occupy. Refer to the RocketPort

"SW1 Settings Chart" and set the DIP switches for the second,

third, or fourth board according to the chart and the I/O

address range you chose for the first installed board.

For the second, third, and fourth board you must choose

a 64-byte, contiguous I/O address range and supply this

information to the AIORCKT.NLM driver. Choose this range from

the following list excluding whatever ranges have already been

used for the first or subsequent RocketPort or RocketPort RA

cards.

1) 100-13F	4) 200-23F	7) 300-33F

2) 140-17F	5) 240-27F	8) 340-37F

3) 180-1BF	6) 280-2BF	9) 380-3BF

Note:	Be sure that addresses you choose do not conflict

with other hardware in your NetWare server.

You can load the AIORCKT.NLM driver either manually from the NetWare

console or automatically from the AUTOEXEC.NCF file when the server boots.

The NetWare Administration Guide provides instructions on how

to modify AUTOEXEC.NCF to load drivers. Follow those instructions

and load AIORCKT in a similar manner, making appropriate changes in

the command line syntax as detailed in the following table.

AIORCKT.NLM command line syntax is as follows:

load aiorckt port=IOADDR {node=NNN} {name=SSS} {nports=PP} {fastbaud}

AIORCKT.NLM Parameters

---------------------

port=IOADDR

IOADDR specifies the hexadecimal I/O address of the

RocketPort or ROcketPort RA board. This must match the setting

you chose earlier when installing the board.

node=NNN

This option overrides the default assignment of board

numbers that are passed to AIO for this particular

board. Normally, the I/O address for the board is used

as its identification to AIO. Some applications may

require assignment of a different board number.

name=SSS

This option sets the textual name for the controller. If

omitted, the textual name is blank. Some applications require a

board name and access boards by this name rather than by number.

nports=PP

This optional parameter allows the count of the actual number of ports

on a board to be overridden with a different value. You can not override

with a greater number than are actually present on the board

but you can specify a smaller number. This is required in the case

of a 4-port RocketPort board because the board is really an 8-port

board with a 4-port interface box attached. If an nports=4 override

is not specified in this case, AIORCKT.NLM will think that 8 ports

exist when in reality only 4 do.

fastbaud

This optional parameter remaps the 50 baud selection to 230.4K baud

since AIO does not have a direct 230.4K selection. When 'fastbaud'

is enabled, applications choosing 50 baud will set the ports

to 230.4 baud.

If AIORCKT.NLM is loaded without any command line parameters,

it prompts for required information.

Here is an example of how to load AIORCKT.NLM for two RocketPort

boards.  The first board will be set for 0x180 (the default)

and the second for 0x100.  Set the switches for both boards

according to the diagram below referencing the 'I/O ADDRESS 180 HEX'

section. The switches on the second board are set according to

the 0x180 range and NOT according to the 0x100 range.  DO NOT try to

interpret the switch settings as a representation of the I/O address

of the board.  This is not the case.

The two load lines for Netware will read as follows:

load aiorckt port=180 fastbaud

load aiorckt port=100 fastbaud

Novell and NetWare are registered trademarks of Novell, Inc.

RocketPort is a registered trademark of Comtrol Corporation.

Comtrol is a trademark of Comtrol Corporation.

RocketPort SW1 Settings Chart

===============================================================

I/O ADDRESS 100 HEX

8 7 6 5 4 3 2 1			8 7 6 5 4 3 2 1

---------------  off			---------------  off

X   X			      X   X   X

X X X X X   X  			X X X   X   X

---------------  on			---------------  on

First board				  Second board

8 7 6 5 4 3 2 1			8 7 6 5 4 3 2 1

---------------  off			---------------  off

X     X   X			    X X   X   X

X X   X X   X  			X X     X   X

---------------  on			---------------  on

Third board				  Fourth board

===============================================================

I/O ADDRESS 140 HEX

8 7 6 5 4 3 2 1			8 7 6 5 4 3 2 1

---------------  off			---------------  off

X X  			      X   X X

X X X X X     X			X X X   X     X

---------------  on			---------------  on

First board				  Second board

8 7 6 5 4 3 2 1			8 7 6 5 4 3 2 1

---------------  off			---------------  off

X     X X  			    X X   X X

X X   X X     X			X X     X     X

---------------  on			---------------  on

Third board				  Fourth board

===============================================================

I/O ADDRESS 180 HEX

8 7 6 5 4 3 2 1			8 7 6 5 4 3 2 1

---------------  off			---------------  off

X X X			      X   X X X

X X X X X      			X X X   X

---------------  on			---------------  on

First board				  Second board

8 7 6 5 4 3 2 1			8 7 6 5 4 3 2 1

---------------  off			---------------  off

X     X X X			    X X   X X X

X X   X X      			X X     X

---------------  on			---------------  on

Third board				  Fourth board

===============================================================

I/O ADDRESS 200 HEX

8 7 6 5 4 3 2 1			8 7 6 5 4 3 2 1

---------------  off			---------------  off

X     X			      X X     X

X X X X   X X  			X X X     X X

---------------  on			---------------  on

First board				  Second board

8 7 6 5 4 3 2 1			8 7 6 5 4 3 2 1

---------------  off			---------------  off

X   X     X			    X X X     X

X X   X   X X  			X X       X X

---------------  on			---------------  on

Third board				  Fourth board

===============================================================

I/O ADDRESS 240 HEX

8 7 6 5 4 3 2 1			8 7 6 5 4 3 2 1

---------------  off			---------------  off

X   X  			      X X   X

X X X X   X   X			X X X     X   X

---------------  on			---------------  on

First board				  Second board

8 7 6 5 4 3 2 1			8 7 6 5 4 3 2 1

---------------  off			---------------  off

X   X   X  			    X X X   X

X X   X   X   X			X X       X   X

---------------  on			---------------  on

Third board				  Fourth board

===============================================================

I/O ADDRESS 280 HEX

8 7 6 5 4 3 2 1			8 7 6 5 4 3 2 1

---------------  off			---------------  off

X   X X			      X X   X X

X X X X   X    			X X X     X

---------------  on			---------------  on

First board				  Second board

8 7 6 5 4 3 2 1			8 7 6 5 4 3 2 1

---------------  off			---------------  off

X   X   X X			    X X X   X X

X X   X   X    			X X       X

---------------  on			---------------  on

Third board				  Fourth board

===============================================================

I/O ADDRESS 300 HEX

8 7 6 5 4 3 2 1			8 7 6 5 4 3 2 1

---------------  off			---------------  off

X X   X			      X X X   X

X X X X     X  			X X X       X

---------------  on			---------------  on

First board				  Second board

8 7 6 5 4 3 2 1			8 7 6 5 4 3 2 1

---------------  off			---------------  off

X   X X   X			    X X X X   X

X X   X     X  			X X         X

---------------  on			---------------  on

Third board				  Fourth board

===============================================================

I/O ADDRESS 340 HEX

8 7 6 5 4 3 2 1			8 7 6 5 4 3 2 1

---------------  off			---------------  off

X X X  			      X X X X

X X X X       X			X X X         X

---------------  on			---------------  on

First board				  Second board

8 7 6 5 4 3 2 1			8 7 6 5 4 3 2 1

---------------  off			---------------  off

X   X X X  			    X X X X X

X X   X       X			X X           X

---------------  on			---------------  on

Third board				  Fourth board

===============================================================

I/O ADDRESS 380 HEX

8 7 6 5 4 3 2 1			8 7 6 5 4 3 2 1

---------------  off			---------------  off

X X X X			      X X X X X

X X X X        			X X X

---------------  on			---------------  on

First board				  Second board

8 7 6 5 4 3 2 1			8 7 6 5 4 3 2 1

---------------  off			---------------  off

X   X X X X			    X X X X X X

X X   X        			X X

---------------  on			---------------  on

Third board				  Fourth board