WNIM3000 AIO Driver Information

July 1994

Microdyne Corporation

3601 Eisenhower Avenue

Alexandria, VA, 22304, USA

TEL: (800) 578-3270       FAX: (408) 376-3531

TEL: (408) 432-1191       BBS  (408) 376-3662

Copyright 1994 Microdyne Corporation

All rights reserved.

INTRODUCTION

The AIOWNIM3.NLM is the AIO Driver for Microdyne's Wide Area

Network Interface Module 3000 (WNIM3000) adapter. This document

contains the necessary information to load the WNIM3000 AIO

Driver and lists status/error messages that may display while

loading and unloading the AIO Driver.

I/O ADDRESS SWITCH SETTINGS

I/O Address:   WNIM3000s shall not share I/O addresses. Choose

1 of 16 address blocks (SWITCH selectable).

In the following table, 0 is Switch on, and 1 is

Switch off.

Device Address range             SWITCH Positions

hexadecimal notation             4    2    3    1

280     -    28F                 0    0    0    0

290     -    29F                 0    0    0    1

2A0     -    2AF                 0    0    1    0

2B0     -    2BF                 0    0    1    1

2C0     -    2CF                 0    1    0    0

2D0     -    2DF                 0    1    0    1

2E0     -    2EF                 0    1    1    0

2F0     -    2FF                 0    1    1    1

300     -    30F                 1    0    0    0

310     -    31F                 1    0    0    1

320     -    32F                 1    0    1    0

330     -    33F                 1    0    1    1

340     -    34F                 1    1    0    0

350     -    35F                 1    1    0    1

360     -    36F                 1    1    1    0

370     -    37F                 1    1    1    1

LOADING THE WNIM3000 AIO DRIVER

To load the WNIM3000 AIO Driver, use the following syntax:

load AIOWNIM3 port=pppp mem=mmmm name=nnnn

where

pppp  is the device I/O address of the WNIM3000

adapter that you are loading. Available device

I/O addresses are 0280, 0290, 02A0, 02B0, 02C0,

02D0, 02E0, 02F0, 0300, 0310, 0320, 0330, 0340,

0350, 0360, and 0370.

NOTE:  When installing WNIM3000 adapters in a

Novell server, make sure that you reserve

device I/O address ranges that you will

be using or problems may occur.

mmmm  is the base memory address of the WNIM3000

adapter that you are loading. Available base

memory addresses are

A0000, A2000, A4000, A6000, A8000, AA000, AC000, AE000,

B0000, B2000, B4000, B6000, B8000, BA000, BC000, BE000,

C0000, C2000, C4000, C6000, C8000, CA000, CC000, CE000,

D0000, D2000, D4000, D6000, D8000, DA000, DC000, DE000,

E0000, E2000, E4000, E6000, E8000, EA000, EC000, EE000.

NOTE:  If you are loading more than one WNIM3000

adapter in the server PC, then the base memory

address specified on the command line for the

first board must be used for the second, third,

and fourth boards.

Furthermore, if you load the first board without

specifying any parameters, then the default base

memory address (D0000) will be used; therefore,

you must use D0000 for any other WNIM3000

adapters you install.

nnnn  is the user-defined name to be assigned to the

WNIM3000 adapter you are loading. The name may

not contain space characters.

If you omit all of the parameters, then the AIO Driver will load

each WNIM3000 adapter (in order) using the defaults as follows:

Device I/O Address        Base Memory Address   Name

Board 0          0280                      D0000          None

Board 1          0290                      D0000          None

Board 2          02A0                      D0000          None

Board 3          02B0                      D0000          None

WNIM3000 AIO DRIVER STATUS AND ERROR MESSAGES

Following is a list of status/error messages that may display

while loading and unloading the AIO Driver. Also included with

each message is an explanation of the message and what to do when

it occurs.

NOTE:  There are no messages displayed during the normal

operation of the AIO Driver. That is, after the driver

load operation is complete and before the driver

unload operation is initiated, no messages will be

displayed.

IMPORTANT

If the action instructions provided for a given error do not

correct the error or there is no action provided for the error,

then the error may have been caused by either incorrectly keyed

in commands (at the server PC's keyboard), or the driver was

loaded incorrectly.

Message 1

AIO WNIM3000 Driver unloaded.

Reason: The driver unload operation initiated by the

operator is complete.

Message 2

Error: AIODeregisterDriver failed (ccode=xxxx).

Reason: A call to the system function  "AIODeregisterDriver"

failed during the unload operation.

Action: Use the system monitor to verify whether or not the

AIO Driver is resident in the server PC's memory.  If it is, try

the unload operation again.

Message 3

*Error: AIORegisterBoard failed (ccode=xxxx).

Reason: A call to the system function "AIORegisterBoard" failed,

causing the driver load operation to abort.

Action: Use the driver unload command to unload the AIO Driver

from the server PC's memory. Then use the driver load command to load

each WNIM3000 adapter in the following order:

Board Number             Port I/O Address

0                        0280h

1                        0290h

2                        02A0h

3                        02B0h

Message 4

Error: AIORegisterDriver failed (ccode=xxxx).

Reason:   A call to the system function "AIORegisterDriver"

failed, causing the driver load operation to abort.

Action:   Check to see if an AIO Driver has already been

loaded and registered.

Message 5

*Error: AllocateResourceTag failed.

Reason:   A call to the system function "AllocateResourceTag"

failed, causing the load operation to abort.

Action:   No action is possible.  The system is out of resource

tags.

Message 6

*Error: Unable to register Hardware Options for WNIM3000.

Reason:   The AIO driver is unable to register the specified

hardware configuration with the NetWare operating system, causing

the driver load operation to abort.

Action 1: Verify that the I/O address (device I/O address)

is not being used by another device that is installed in the

server PC.

Action 2: Also verify that the memory address (base memory

address) is not being used by a device which cannot share the

memory with other devices installed in the server PC.

Message 7

Error: Unable to release Port yy (ccode=xxxx).

Reason:   The driver was unable to release the port during

the driver unload operation.  The error code returned from the

release port function may indicate that the AIO Driver's tables

are corrupted.

Action:   None.

Message 8

*Error: WNIM3000 did not initialize.

Reason:   The WNIM3000 adapter failed to complete its

initialization sequence within the specified time, causing the

driver load operation to abort.

Action 1: Verify that the WNIM3000 adapter's hardware

configuration for the port I/O address (device I/O address) match

the port I/O address specified with the driver load command.

Action 2: Verify that each WNIM3000 adapter installed in the

server PC is set to a unique port I/O address.

Action 3: If a valid port I/O address and memory address

were specified with the driver load command, then the WNIM3000

adapter may be defective. Try loading a different WNIM3000

adapter using the same port I/O address that was used for the

WNIM3000 adapter that failed.

Message 9

*Error: WNIM3000 Driver called too many times.

Reason:   The driver AIOWNIM has been called more than four

times.  A maximum of four WNIM3000 adapters (boards) can be

initialized by the AIO Driver.

Action:   Verify that the AUTOEXEC.NCF file contains no more

than four load commands for the driver (AIOWNIM) or make sure

that no more than four load commands for the driver are entered

via the server PC's keyboard.

Message 10

WNIM3000 Board x using port yyyy at address zzzzz is

installed.

Reason: The AIO Driver has successfully initialized the

specified WNIM3000 adapter (board).

where:

x is the number of the WNIM3000 adapter (board)

that has been initialized

yyyy is the port I/O address (device I/O address)

used by the specified WNIM3000 adapter

zzzzz is the memory address (base memory address)

used by the specified WNIM3000 adapter

Message 11

*Error: Unable to Parse Driver Parameters.

Reason:   The load command contained parameters that the

driver didn't recognize.

Action:   Verify that the WNIM3000 driver load command

contains only those options specified above.

Message 12

Error: Board presence test failed (ccode=xxxx).

Reason:   The WNIM3000 AIO driver was unable to detect the

presence of a WNIM3000 at the specified port I/O and memory

addresses.

Action 1: Verify that the WNIM3000 adapter's hardware

configuration for the port I/O address (device I/O address) match

the port I/O address specified with the driver load command.

Action 2: Verify that each WNIM3000 adapter installed in the

server PC is set to a unique port I/O address.

Action 3: If a valid port I/O address and memory address

were specified with the driver load command, then the WNIM3000

adapter may be defective. Try loading a different WNIM3000

adapter using the same port I/O address that was used for the

WNIM3000 adapter that failed.

Message 13

*Error: WNIM3000 board returned bad number of ports=xxxx.

Reason:   The driver read an invalid number for the ports

supported by the WNIM3000 board being loaded.

Action 1: Verify that the WNIM3000 adapter's hardware

configuration for the port I/O address (device I/O address) and

memory address do not conflict with other hardware installed in

the computer.

Action 2: The WNIM3000 adapter may be defective. Try loading

a different WNIM3000 adapter using the same port I/O address that

was used for the WNIM3000 adapter that failed.

Message 14

Error: AIODeregisterBoard failed (ccode=xxxx).

Reason:   A call to the system function "AIODeregisterBoard"

failed during the unload operation.

Action:   Use the system monitor to verify whether or not

the AIO Driver is resident in the server PC's memory.  If it is,

try the unload operation again.

TRADEMARKS

WNIM3000 is a registered trademark of Microdyne Corporation.

Novell and NetWare are registered trademarks of Novell, Inc.