CD-BlitzTM CD-ROM drives caching software test report

ãCopyright 1993 Blitz 'n' Software, Inc. All rights reserved.

SYSTEM CONFIGURATION

Platform used to test the CD-ROM drive performance

All tests are done using Acer Power 486 DX/33 with 64KB external cache and 8 MB RAM.

CD-ROM drives tested

1	Sony CDU-31A (AT bus)

DEVICE = SLCD.SYS  /D:MSCD001  /B:340  /M:P  /V  /C   (V1.22A)

2	Sony CDU-561 (SCSI)

DEVICE=\PROAUDIO\TSLCDR.SYS /D:MSCD001 /W3

3	Panasonic 523-B  (AT bus)

DEVICE = \SBPRO\DRV\SBPCD.SYS /D:MSCD001 /P:220  (V2.00)

4	Panasonic 532  (SCSI)

DEVICE = FDCD.SYS  /D:MSCD001  (V2.22)

5	Panasonic 562  (AT Bus, double speed)

DEVICE = CDMKE.SYS  /D:MSCD001  (V0.73)

6	Mitsumi LU005S (AT bus)

DEVICE = MTMCDE.SYS  /D:MSCD001  /P:300   /A:0  /M:20 /T:5  /I:10 	(V2.10b)

7	NEC-84j (SCSI)

DEVICE=\PROAUDIO\TSLCDR.SYS /D:MSCD001 /W3

8	Philip CM205 (AT bus)

DEVICE=\FUSION16\DD250.SYS /D:MSCD001 /P:340 /I:3 (V1.30.00)

CD-BlitzTM CD-ROM cache software configuration

Allocate 1024 KB RAM as cache buffer to CD-BlitzTM

Items tested

1	CD-ROM drive data transfer rate (CD-ROM drive low level test)

Test the data transfer rate of CD-ROM drives under MS-DOS and MS-Windows.

2	CD-ROM file directory walk test (CD-ROM file system level test)

Test the access speed of CD-ROM drives file system (MSCDEX) under MS-DOS

3	CD-Title program execution speed test (CD-ROM application level test 1)

Test the system performance of CD-Titles under MS-Windows

4	CD-ROM text retrieval speed test (CD-ROM application level test 2)

Test the text retrieval speed under MS-Windows

TEST METHODOLOGY

CD-ROM drive data transfer rate (CD-ROM drives low level test)

Two tools were utilitized:

Microsoft DOSSPEED.EXE tests CD-ROM drives speed under MS-DOS

Microsoft WINSPEED.EXE tests CD-ROM drives speed under MS-Windows

CD-ROM file directory walk test (CD-ROM drives file system level test)

In this test we walk through all the files in Microsoft BookShelf CD-ROM (in 	drive E:) with batch file:

> TM  START   (Norton Time Mark)

> DIR  E:  /S  > NUL

> TM  STOP

CD-Title program execution speed test (CD-ROM drives application level test 1)

Microsoft BookShelf is used to test execution speed of CD-ROM drives. The 	recorder of Windows 3.1 records a procedure that runs BookShelf

applications. Same procedure is applied repeatively to all CD-ROM drives.

Procedures

1. Click on MS BookShelf icon  (run BookShelf)

2. Click on HAMMOND ATLAS (run this book title)

3. Click back (return to main screen)

4. Click on American Heritage Dictionary  (run this book title)

5. Click back (return to main screen)

6. Click on The Concise Columbia Encyclopedia  (run this book title)

7. Click back (return to main screen)

8. Click on Familiar Quotations of John Bartlett  (run this book title)

9. Click back (return to main screen)

10. Click on The World Almanac 1992 (run this book title)

11. Click back (return to main screen)

12. Click on The Concise Columbia Dictionary of Quotations (run this title)

13. Click back (return to main screen)

14. Click on Roget's II (run this book title)

15. Click back (return to main screen)

16. Close BookShelf

CD-ROM text retrieve speed test (CD-ROM drives application level test 2)

A procedure is recorded and applied to search MS BookShelf database, computing 	time elapsed to complete this procedure. The Recorder of Windows 3.1 records 	the procedure, which is applied repetively to all CD-ROM drives.

Procedure

1. Open MS BookShelf

2. Click Search

3. Select phrase to retrieve phrase "COMPUTER"

4. Wait till all matched records read complete

TEST RESULT (RAW DATA)

CD-ROM drive data transfer rate (CD-ROM drives low level test)

DOSSPEED.EXE in MS-DOS: The file selected for testing is "demo386.mmm"

under MS-BookShelf.

Larger is better in this test

| CD-ROM Drive | 1st KB/s | 2nd KB/s | 3rd KB/s | 4th KB/s | Avg KB/s | % Improved |
| --- | --- | --- | --- | --- | --- | --- |
| Sony CDU-31A Sony CDU-31A + CD-BlitzTM | 103 110 | 135 6857 | 134 6857 | 136 6858 | 127 5170 | 4,942% |
| Sony CDU-561 Sony CDU-561 + CD-BlitzTM | 207 182 | 274 6857 | 271 6857 | 280 6857 | 258 5188 | 2,349% |
| Panasonic 523-B Panasonic 523-B + CD-BlitzTM | 111 111 | 134 7025 | 140 7025 | 141 6857 | 131 5254 | 4,763% |
| Panasonic 532 Panasonic 532 + CD-BlitzTM | 122 143 | 138 6857 | 137 6857 | 134 6857 | 133 5179 | 4,869% |
| Panasonic 562 Panasonic 562 + CD-BlitzTM | 218 277 | 296 6857 | 293 7025 | 285 7025 | 273 5296 | 2,273% |
| Mitsumi LU-005S Mitsumi LU-005S + CD-BlitzTM | 118 119 | 132 7025 | 137 7025 | 130 7025 | 129 5299 | 5,028% |
| NEC-84J NEC-84j + CD-BlitzTM | 224 135 | 271 6857 | 255 6857 | 269 6857 | 255 5177 | 2,430% |
| Philip CM205 Philip CM205 + CD-BlitzTM | 111 111 | 136 7025 | 133 7025 | 133 6857 | 128 5254 | 5,065% |

WINSPEED.EXE under MS-Windows. The file selected for testing is 	"demo386.mmm" under MS-BookShelf.

Larger is better in this test

| CD-ROM Drive | 1st KB/s | 2nd KB/s | 3rd KB/s | 4th KB/s | Avg KB/s | % Improved |
| --- | --- | --- | --- | --- | --- | --- |
| Sony CDU-31A Sony CDU-31A + CD-BlitzTM | 110 124 | 127 5113 | 130 5113 | 137 5113 | 126 3865 | 3,632% |
| Sony CDU-561 Sony CDU-561 + CD-BlitzTM | 204 139 | 255 5113 | 255 5113 | 268 5113 | 246 3877 | 1,808% |
| Panasonic 523-B Panasonic 523-B + CD-BlitzTM | 113 116 | 127 5113 | 137 5208 | 141 5113 | 129 3887 | 3,594% |
| Panasonic 532 Panasonic 532 + CD-BlitzTM | 127 137 | 127 5113 | 130 5113 | 130 5113 | 129 3869 | 3,833% |
| Panasonic 562 Panasonic 562 + CD-BlitzTM | 204 269 | 255 5113 | 284 5208 | 269 5113 | 253 3926 | 1,734% |
| Mitsumi LU-005S Mitsumi LU-005S + CD-BlitzTM | 106 110 | 137 5113 | 137 5113 | 127 5113 | 127 3862 | 3,632% |
| NEC-84J NEC-84j + CD-BlitzTM | 224 135 | 271 6857 | 255 6857 | 269 6857 | 255 5177 | 2,430% |
| Philip CM205 Philip CM205 + CD-BlitzTM | 113 112 | 134 5113 | 130 5113 | 130 5113 | 127 3863 | 3,716% |

CD-ROM file directory walk test (CD-ROM drives file system level test)

Smaller is better in this test (except Improved field)

| CD-ROM Drive | 1st Seconds | 2nd Seconds | 3rd Seconds | 4th Seconds | Avg Seconds | % Improved |
| --- | --- | --- | --- | --- | --- | --- |
| Sony CDU-31A Sony CDU-31A + CD-BlitzTM | 14 11 | 13 3 | 12 3 | 12 3 | 12.75 5 | 333% |
| Sony CDU-561 Sony CDU-561 + CD-BlitzTM | 9 6 | 8 5 | 9 5 | 9 6 | 8.75 5.5 | 80% |
| Panasonic 523-B Panasonic 523-B + CD-BlitzTM | 9 6 | 9 3 | 9 3 | 9 3 | 9 3.75 | 200% |
| Panasonic 532 Panasonic 532 + CD-BlitzTM | 11 6 | 11 3 | 10 3 | 11 3 | 10.75 3.75 | 267% |
| Panasonic 562 Panasonic 562 + CD-BlitzTM | 7 5 | 6 4 | 6 4 | 6 3 | 6.25 4 | 100% |
| Mitsumi LU-005S Mitsumi LU-005S + CD-BlitzTM | 16 14 | 14 10 | 14 10 | 13 10 | 14.25 11 | 40% |
| NEC-84J NEC-84j + CD-BlitzTM | 14 12 | 14 6 | 15 7 | 14 7 | 14.25 8 | 150% |
| Philip CM205 Philip CM205 + CD-BlitzTM | 14 12 | 13 4 | 13 3 | 13 4 | 13.25 5.75 | 333% |

CD-Title program execution speed test (CD-ROM drives application level test 1)

Smaller is better in this test (except Improved field)

| CD-ROM Drive | 1st Seconds | 2nd Seconds | 3rd Seconds | 4th Seconds | Avg Seconds | % Improved |
| --- | --- | --- | --- | --- | --- | --- |
| Sony CDU-31A Sony CDU-31A + CD-BlitzTM | 60 46 | 59 20 | 58 20 | 59 20 | 59 26.5 | 195% |
| Sony CDU-561 Sony CDU-561 + CD-BlitzTM | 43 34 | 41 20 | 40 20 | 40 20 | 41 23.5 | 100% |
| Panasonic 523-B Panasonic 523-B + CD-BlitzTM | 53 41 | 52 20 | 52 20 | 52 20 | 52.25 25.25 | 160% |
| Panasonic 532 Panasonic 532 + CD-BlitzTM | 57 40 | 53 20 | 53 20 | 54 20 | 54.25 25 | 165% |
| Panasonic 562 Panasonic 562 + CD-BlitzTM | 38 30 | 37 20 | 37 20 | 36 20 | 37 22.5 | 85% |
| Mitsumi LU-005S Mitsumi LU-005S + CD-BlitzTM | 68 50 | 62 21 | 63 21 | 62 20 | 63.75 28 | 215% |
| NEC-84J NEC-84j + CD-BlitzTM | 47 39 | 47 20 | 46 20 | 45 20 | 46.25 24.75 | 130% |
| Philip CM205 Philip CM205 + CD-BlitzTM | 60 42 | 59 20 | 58 20 | 59 20 | 59 25.5 | 195% |

CD-ROM text retrieval speed test (CD-ROM drives application level test 2)

Smaller is better in this test (except Improved field)

| CD-ROM Drive | 1st Seconds | 2nd Seconds | 3rd Seconds | 4th Seconds | Avg Seconds | % Improved |
| --- | --- | --- | --- | --- | --- | --- |
| Sony CDU-31A Sony CDU-31A + CD-BlitzTM | 177 42 | 176 5 | 178 5 | 175 5 | 176.5 14.25 | 3,420% |
| Sony CDU-561 Sony CDU-561 + CD-BlitzTM | 107 22 | 106 5 | 106 5 | 106 5 | 106.2 9.25 | 2,020% |
| Panasonic 523-B Panasonic 523-B + CD-BlitzTM | 142 39 | 144 5 | 143 5 | 143 5 | 143 13.5 | 2,760% |
| Panasonic 532 Panasonic 532 + CD-BlitzTM | 155 44 | 154 5 | 155 5 | 153 5 | 154.2 14.75 | 2,980% |
| Panasonic 562 Panasonic 562 + CD-BlitzTM | 82 22 | 81 5 | 81 5 | 81 5 | 81 9.25 | 1,520% |
| Mitsumi LU-005S Mitsumi LU-005S + CD-BlitzTM | 202 46 | 200 5 | 199 5 | 201 5 | 200.5 15.25 | 3,900% |
| NEC-84J NEC-84j + CD-BlitzTM | 137 26 | 131 5 | 132 5 | 129 5 | 132.2 10.25 | 2,520% |
| Philip CM205 Philip CM205 + CD-BlitzTM | 180 52 | 180 5 | 179 5 | 179 5 | 179.5 16.75 | 3,500% |

APPENDIX

Conventional memory (RAM) used for CD-ROM device drivers

CD-BlitzTM				13 KB

MSCDEX:				35 KB

Sony CDU-31A (AT bus):		22 KB

Sony CDU-561 (SCSI):		13 KB

Panasonic 523-B (AT bus):		10 KB

Panasonic 532  (SCSI):		23 KB

Panasonic 562  (AT, double speed):	12 KB

Mitsumi LU005S (AT bus):		57 KB

NEC-84J (SCSI):			13 KB

Philip CM205 (AT bus):		20 KB