Trak2WAV can import audio tracks (redbook) as raw digital audio

into a .WAV file.   This is a 16 Bit / Stereo / 44.1 KHz sample

.WAV file that can be recorded back out to an Audio CDR as Redbook.

This utility only works with the TOSHIBA 3401B or Toshiba XM4101 as a

source drive.  Only these two Toshiba 2x SCSI readers are supported.

( note , use TRAK4WAV for the XM4101 and TRAK2WAV for the XM3401 )

SET TRAKWAV = ID# of the SCSI address of the Toshiba CD Player.

ASPI4DOS or ASPI2DOS is required to be installed for the controller.

A logical device driver like ASPICD.sys is not needed.  !   DLV

TOC.exe - the CD "Table of Contents" (toc)

Also, use the TOC (table of contents) utility to determine the

starting and ending times of all of the tracks on the CD.

This does require MSCDEX or SUPERCDX to be installed for TOC.

syntax:

TRAK2WAV MM:SS:BB MM:SS:BB infile.wav

where MM:SS:BB is the start record and end record times in

Minutes:Seconds:75th Second Blocks

MM range is 00 to 74

SS range is 00 to 59

BB range is 00 to 74

The minumum start time is 2 seconds or just past the pregap at 00:02:00.

Any start time, ( or end time ) less than 00:02:00 is invalid.

TRAK2WAV alone with no paramenters echos the syntax.

TRAK2WAV was designed first for the Toshiba XM3401,

TRAK4WAV is for the new Toshiba XM4101 and otherwise identical.

thank you,

American InfoScience

1948-B South Interstate Hwy. 35

Austin,  Texas  78704

(512)440-1132

(fax)440-0531