INSTALLATION INSTRUCTIONS:

To install this Disk Driver:

1)    Copy IDE.DSK to the boot-up directory of the server. (Same location as

your SERVER.EXE)

2)    After the system is brought up,  type: LOAD IDE.DSK

NOTES:

The IDE disk driver was written to work with ATA/IDE disk drives.  The

ISADISK driver should not be used with these disk drives.  This driver

is a self configuring driver and does not rely on the BIOS for disk

drive support.

The driver supports Read-after-write verify at the software level.

Using the VERIFY ON mode will cause writing to the disk to be a

little slower than normal.

The driver will also support up to eight ATA/IDE disk drives in a

file server.  This is possible by using addressable paddleboards.

These paddleboards allow the user to configure the base I/O address

and IRQ.  (Primary I/O address at 1F0h and IRQ 14, second address at

170h, third address at 1E8h, and fourth  address at 168h.)  IRQs 9,

10, 11, and 15 may be used with the second, third, and fourth base

I/O addresses.  It is not recommanded to use IRQ 12 because of

possible conflicts with the mouse port.

Not all paddleboards will work correctly in all file servers.  Refer

to your hardware manufacturer or system integrator for recommended

configurations.

LIMITATIONS:

* The IDE disk driver will not work with ESDI and the ST506/412 bus

controllers.  The ATA/IDE, ST506, and ESDI interfaces look similar

to the system software; however, they are handled differently by

the disk driver.

* If a disk drive is to have a DOS partion installed, it will need

to be installed BEFORE the NetWare partion is installed.