Here is the necessary information to perform communications with

British Telecomm Tymnet.

This is a sample of a Logon.

Filename: bttlogon

/$NETCMD PROFILE TYPE2

/$NETCMD SIGNON EDIBSC01;BSC*EDI

USERID;PASSWORD;B140;

Note: USERID is your BT TYMNET Mailbox Id.

PASSWORD is your Network Password

This is the Cleo Job File used to transmit a file and receive a file

from BT TYMNET.

Filename: bttsr.job

CO btt.cfg               <== MODEM configuration file for CLEO

AUTODIAL 18002282403 R5  <== BT TYMNET BYSYNC 2400 BAUD Phone number

TE bttlogon              <== BT TYMNET Logon

RE BTT_MSG               <== Logon answer back

TE BTTDATA               <== EDI File sent to BT TYMNET

RE EDI_BTT               <== EDI File received from BT TYMNET

RE BTT_MSG               <== BT TYMNET Statistical message of session.

VO

QU

This is the Cleo Job File used to transmit a file to BT TYMNET.

Filename: btts.job

CO btt.cfg

AUTODIAL 18002282403 R5

TE bttlogon              <== BT TYMNET Logon

RE BTT_MSG               <== Logon Answer back

TE BTTDATA               <== EDI File sent to BT TYMNET

RE BTT_MSG               <== Statistical message of session.

VO

QU

This is the Cleo Job File used to receive a file from BT TYMNET.

Filename: bttr.job

CO btt.cfg

AUTODIAL 18002282403 R5  <=== 2400 Baud Bysync phone number

TE bttlogon              <=== Your BT Logon Info

TE bttnodat.msg          <=== No data to send message

RE EDI_BTT               <=== Your Received EDI Data

RE BTT_MSG               <=== Statistical message of session

VO

QU

This is a sample No Data Message.

Filename:bttnodat.msg

THERE IS NO EDI DATA CURRENTLY AVAILABLE TO SEND

Note: This record must be less the 80 bytes.

This is the modem configuration file for BT TYMNET.

Filename: btt.cfg

80

4

4

60

125

0

2

15

0

40

0

5

0

0

1

0

1

This is the batch file to send and receive data to BT TYMNET.

Filename: runbttsr.bat

3780Plus -J bttsr.job -D COM1

This is a batch file to receive a file from BT TYMNET.

Filename: runbttr.bat

3780Plus -J bttr.job -D COM1

This is a batch file to send a file from BT TYMNET.

Filename: runbtts.bat

3780Plus -J btts.job -D COM1