NetOpPM version 4.21

--------------------

If you are a NetOpPM version 4.20 user you can upgrade to version 4.21 by

faxing copy of license certificates and shipping address to Danware at:

int+45 44531551

We will then mail the upgrade to you free of charge.

Please read the NetOpPM manual carefully, especially if you encounter

problems in controlling a PC or a specific application.

This file will provide you with additional information on NetOpPM.

==========================================================================

Please save your original NetOpPM disks and programs. You will need

those for future updates.

==========================================================================

Shutdown in OS/2 2.x

--------------------

You cannot shutdown an OS/2 PC running RemPCOS2 while it's being

remotely controlled.

===========================================================================

DOS RemPCs

----------

The program SETUP.EXE will allow you to configure all available REMPC

parameters.

===========================================================================

Remote control of SuperVGA

--------------------------

NetOpPM support remote control of DOS and WINDOWS based PCs running

SuperVGA mode 800*600, 16 color. Support is available for a number of

graphics adapters.

You must load RemPC with the parameter /G. Both the Master PC and the

Remote PC must have compatible SuperVGA adapters.

NetOpPM support adapters that use one of the following chips:

ATI, EVEREX, NCR 77C22E, TRIDENT, VIDEO7, GENOA, PARADISE/WESTERN DIGITAL,

CHIPS & TECHNOLOGY, TSENG ET3000, TSENG ET4000, Ahead A, Ahead B, Oak,

Cirrus.

If you have an unrecognized SVGA adapter try the following:

On the RemPC: Start RemPC with the /G parameter. In the [DISPLAY]

section of your SYSTEM.INI file enter the line SVGAMODE=106.

If this is succesful you will be able to control the SVGA adapter, under

Windows.

Make sure you use the Windows SVGA display driver. Check this by starting

the Windows SetUp program from the Program Manager's Main Group.

If you experience problems, you are welcome to fax those to Danware at:

int. + 45 44 53 15 51

Remote control of CGA and MCGA

------------------------------

NetOpPM only supports CGA and MCGA adapters in text mode.

Remote control of 256 colors from 16 colors

-------------------------------------------

If the PC running NetOpPM is in a 16 color mode (eg. VGA) and

the remote PC is running in a 256 color mode (eg. an XGA mode) the colors

might not display correctly.

==========================================================================

Remote control of XT PCs

------------------------

Please make sure the adv. keyboard feature is turned OFF if you

encounter any problems.

==========================================================================

Microsoft Windows

-----------------

If you run Netware and WINDOWS make sure the statement "network=VIPX.386" is

removed from the SYSTEM.INI file on the remote PC.

If you remote control Windows in 386 Enhanced mode please note that you can

remote control DOS applications that are defined to run FULL SCREEN and

EXCLUSIVE. (they can be defined to run as a background task).

Loading the RemPCwin program in real mode under WINDOWS 3.0 can cause the

PC to hang.

If you run WINDOWS 3.0 (NOT WINDOWS 3.1) and your network adapter is using

hardware int 2, int 9, or an interupt higher than 9, then you must install

the file VPICDA.386. Find the line [386 enh] device=*vpicd and substitute

with device=vpicda.386. VPICDA.386 is a Microsoft patch for Windows 3.0 and

must be placed in the WINDOWS subdirectory SYSTEM.

Microsoft Windows for Workgroups

--------------------------------

If you have problems running on a Windows for workgroups network

try running the command NET START NETBEUI before you start RemPC.

Please note that if you have enabled sharing the screen might flash when

controlling Windows.

Mouse Support for Windows

-------------------------

If your mouse does not function properly under Windows, try opening

the control panel and set the mouse tracking speed to the middle position.

If the remote mouse still does not work correctly, for example if you

can only move the mouse on a limited area of the the screen, try to add the

following section to your WINDOWS MOUSE.INI file located on your remote PC:

[NETOPPM]

X-SCALE=1

Y-SCALE=1

If this does not correct the problem, try with different values for X and Y

SCALE.

===========================================================================

IBM LAN Support

---------------

If you experience slow performance (several seconds per keystroke), you

should make sure that you have disabled 'Piggy Back Acknowledgement'.

Under DOS append the parameter PBA=0 to the line in your CONFIG.SYS where

the NetBIOS is loaded (DEVICE=<path>\DXMT0MOD.SYS).

Under OS/2 make sure the statement PIGGYBACKACKS=0 is in your PROTOCOL.INI

in the IBMCOM directory.

OS/2 and IBM NetBios

--------------------

NetOpPM, RemPCOS2 and NetCopy2 will detect the type of NetBios support

available when they load up. They will detect ES mode or LM mode. It is

possible to force the program to use a specific NetBios mode. In case you

encounter problems you can try to force the program to use ES mode or LM

mode. Use the following parameter when loading the program:

/XNE = Force the program to use ES mode

/XNL = Force the program to use LM mode

/XNA = Force the program to use automatic detection (default)

IBM PC-LAN

----------

The PC-LAN support programs (NetBios) must be loaded. It is not necessary

to load PC-LAN. If PC-LAN is loaded it should be loaded before REMPC is

loaded. Using RemPC names identical with PC Lan names might create

problems so we advise that you do not do that.

DEC NET

-------

Remember to run the network with the netbios option. You should use latest

version (4.0 or above).

3Com 3Plus

----------

You must have 3COM version 1.2 or later and you must use the full NETBIOS

with /M option (do not use NB). Load MINSES12 and not MINSES

Novell Netware/IPX

-------------------

You should have Advanced Netware version 2.1 or above if you want to use

NetOp across bridges. If you experience problems, please configure REMPC to

use minimum buffer (use the parameter /B:1 when loading REMPC).

You can also try to load NetOpPM with the following command :

NETOPPM   /L

To obtain the best result, please make sure you are using latest version

of IPX/SPX, Shells and Netware.

10Net

-----

Use Netbios version 4.1 or later. Remote reboot with Ctrl Alt Del is not

possible on netbios version 4.1.

Torus

-----

Load RemPC using the Torus.Ini file.

==========================================================================

Danware Data A/S

93.10.13