Using Kali with ODIPKT

This documentation is Copyright 1996, Kali, Inc.

Written by Jay Cotton (jay@kali.net)

Latest revisions 3/4/96

******************

* Ethernet users *

******************

Please see below for use with the Klos PPP driver.  Also, please note that

you can't just go out and buy an ethernet adapter in order to get a faster

connection.  You must be located in the same building as an internet router

or gateway in order to use an ethernet connection.  If you are at home, and

your connection to the internet is via modem, then you must use the modem

and the PPP driver.

1. Find an ODI driver for your ethernet card.  This will be found on the

setup disk that came with the ethernet card.  If you can't find it, then

there's no need to read anything else... :)

2. Put your ODI driver in a directory along with startnet.bat, lsl.com,

odipkt.com, and net.cfg (a sample net.cfg should have been included).

3. Edit net.cfg for the correct settings.  These settings may be different

depending on your particular ethernet card.  Read the ODI instructions

and sample net.cfg file that came with your ethernet card for more info.

Sample net.cfg for a 3c509 driver:

Link Support

Buffers         4 1500

MemPool         4096

link driver 3C509

Int             5

Port            300

Mem             c8000

Frame           Ethernet_II

Note: replace "3C509" with the name of your own ODI driver (do not add

the .exe suffix).

4. Edit startnet.bat to run your own ODI driver.  For example:

@echo off

lsl

3c509

odipkt 0 96

5. Run startnet.bat by typing "startnet" at the DOS prompt.  You should

get messages like this:

NetWare Link Support Layer v2.05 (930910)

(C) Copyright 1990-1993 Novell, Inc.  All Rights Reserved.

BUFFERS 4 1500

MEMPOOL 4096

Configuration File "C:\KALI\ODI\NET.CFG" used.

Max Boards 4, Max Stacks 4

Buffers 4, Buffer size 1500 bytes, Memory pool 4096 bytes.

3Com 3C509 EtherLink III MLID w/ DME  v1.00 (920706)

(C) Copyright 1992 3Com Corp.  All Rights Reserved

Int 10, Port 300, Mem EC000, Node Address 608C65B294

Max Frame 1514 bytes, Line Speed 10 Mbps

Board 1, Frame ETHERNET_II

ODIPKT 2.1

(c) Copyright Daniel Lanciani 1991-1993.  All rights reserved.

This software is provided with NO WARRANTY.

Using Ethernet framing, class 1

ODIPKT is installed and ready.

6. Run Kali like normal.

7. Unload odipkt by typing "termin 0x60".  Make sure Kali is unloaded

before doing this since termin doesn't check.

***********************

* Klos PPPSHARE users *

***********************

1. Edit net.cfg to set the frame type to Ethernet_II like this:

Frame	Ethernet_II

You can also do this from inside pppmenu, but you'll need to exit

and unload and reload before this will take effect.

2. Edit your load.bat file to replace the IPSTUB driver with ODIPKT.

Your new load.bat might look like this:

@echo off

lsl

ppp

pppmenu

pppstate wait ip

pppwat

odipkt 0 96

3. Run the load.bat file in the usual way.  You should now be able to

run ping while Kali is loaded and have a little more memory available.

4. To unload odipkt you must enter "termin 0x60".  You can modify your

unload.bat file like this:

@echo off

kali /u

termin 0x60

ppp u

lsl u