Windows 95 FAQ

September 8,1995

The following document contains 35 questions.  These are many of the same

questions that Media Vision technicians answer on a daily basis.  This document is approximately 18 printed pages long.

This document is prepared with the assumption that the user is running the final released version of Windows 95.  Beta release versions of Windows 95 are not supported.  All protected mode drivers for Media Vision products are included with Windows 95 final released version. These protected mode drivers are not distributed by Media Vision, therefore will not be posted on Media Vision forums such as found on CompuServe, America OnLine, or the Media Vision BBS.

Media Vision is committed to providing the finest technical support in the

industry.  Please feel free to contact us via any of the following methods.

Internet:	TECHSUPP@MEDIAVIS.COM

World Wide Web:	HTTP://WWW.MEDIAVIS.COM/

FTP		FTP.MEDIAVIS.COM

CompuServe:	GO MEDIAVISION

America Online:  Keyword: MEDIA VISION

BBS:		(510)  770 - 0527

24 lines

24 hr. / day

Up to 21.9k baud and downward compatible

Z - Modem recommended

Fax:		(510)  623 - 5742

Mail:		Media Vision Technical Support Department

47900 Bayside Parkway

Fremont,  CA  94538

Tech. Support		800 - 638 - 2807

Live Agent Tech Support	503 - 882 - 1187

If you find it necessary to contact Technical Support, please have

the following information ready:

Customer ID #__________

The ID # is provided to you during your first call.

Multimedia Product purchased, and Date of purchase

Name and version of Multimedia Software installation disk

Type of computer (386, 486, etc.)

Memory (RAM) installed (2 MB, 4 MB, etc.)

Other devices in your system and their IRQ, DMA, and I/O addresses

if relevant to the problem or call.

Bus mouse

Scanner

Sound Card

CD-ROM drive

Fax/modem

Game card

MIDI device

Video Capture card

Other

1. GENERAL INFORMATION

1.1. Is my Media Vision Product supported under Windows 95?

1.2. How do I install my sound card and or CD ROM drive under Windows 95?

1.3. Should any Media Vision drivers be present in my CONFIG.SYS and AUTOEXEC.BAT?

1.4. Can I use the Media Vision Pocket Tools?

1.5. My sound card uses MidiSoft’s Sound Impressions to control my sound card.  Do I still use this under Windows 95?

1.6. In general, how do I view or change the settings for my sound card?

1.7. I see that I can choose from several different Basic Configurations.  Which one should I choose?

1.8. Should I run my Media Vision installation utilities, like INSTALL.EXE?

1.9. Autodetection is not setting up my hardware.  How can I correct this?

1.10. How do I view settings for my Midi Mapper?

1.11. How do I configure my DOS applications with sound to run under Windows 95?

1.12. My PC uses the Symphony Chipset.  How do I load SYMPFIX.COM under Windows 95?

2. Pro Audio Spectrum 16 and Pro Audio Studio Sound Cards

2.1. Sound Blaster emulation is not being setup.  How do I correct this?

2.2. My card now makes banging and crashing sounds or, sometimes, no sounds.  How do I correct this?

2.3. How do I add the /W:0 switch, to my configuration under Windows 95?

2.4. How do I add the T:1 switch, to my configuration under Windows 95?

2.5. I can’t get the microphone to record.  How do I change the mixer to use the microphone to record?

2.6. Where do I change my DMA and IRQ values?

2.7. What are the values that I will find under the Basic Configurations?

3. Premium 3D, Deluxe, and Prosonic 16 Sound Cards;  MV1100, MV1200, MV2100, MV2200, MV3100, MV4000, MV4100 and MV5000 MultiMedia Kits

3.1. I have lost the use of my Midi Port.  How can I enable it again?

3.2. I don’t think my Wavetable card is working. What can I do?

3.3. What about SRS sound? How does this work?

3.4. What about the BASS, Treble, and Enhanced sound modes for my card?  How does this work?

4. PCMCIA Sound Card

4.1. Is my PCMCIA Sound Card supported under Windows 95?

5. CD-ROM Host Adapter Issues

5.1. I have a PAS 16, my sound works, but my CD-ROM drive is not being recognized.  I’m not getting CD-Audio.  What do I do?

5.2. My CD drive still is not getting a drive letter, or not able to read data from CDs.  How do I correct this?

5.3. My drive constantly spins when I put a CD into the drive.  How can I stop this?

5.4. How do I adjust my settings to match my CD-ROM performance?

5.5. I’m using a Panasonic, Reveal, Mitsumi or Sony CD ROM, and Windows 95 did not install the driver for my drive.  What can I do?

5.6. My Sony CD ROM drive is not recognized anymore.  What can I do?

5.7. After installing the MV600 CD ROM drive, I’m getting intermittent Protection Fault errors in Module ESDI at (some address).  How can I correct this?

5.8. My Phillips CM205 or CM206 drive is not recognized by Autodetection.  How can I install it?

5.9. The CD drive connected to my Premium 3D SCSI sound card is not recognized by Autodetection.  How can I install it?

5.10. How do I Install the PCMCIA SCSI Adapter?

5.11. I have the Reno drive with the 8 bit SCSI adapter card.  Do I still need to load ASPI3X80.SYS into CONFIG.SYS?

# General INFORMATION

## Is my Media Vision Product supported under Windows 95?

Here is a table showing the status of all Media Vision products under Windows 95.

* 	Uses the same driver as was used under Windows 3.1..  For the 	ProMovie Video 	capture cards use 	PROMOVIE.DRV.  Remove the 	Microsoft Video for Windows program from the PC along with the Pro 	Movie spectrum program from DOS.  Then reinstall Video for 	Windows in the Win 95 Environment. Run MSDOS from the Win 95 	environment. put the Pro Movie Spectrum Installation disk in the 	appropriate drive and run the install where it came up to the Configure 	Hardware.  Run the shut down proceedure and restart the computer.

±	Windows 95 installs a Cirrus Logic video driver.  Optionally the 	PG1024 version 1.5 drivers or the PG1280 Windows 3.1 drivers 	may be installed.

## How do I install my sound card and or CD ROM drive under Windows 95?

Windows 95 runs an Autodetection to set up your hardware during the installation of the Operating System.  Normally it will detect your hardware and make all the correct settings automatically. Letting Windows 95 Autodetection detect your hardware should avoid these problems (and is recommended).

## Should any Media Vision drivers be present in my CONFIG.SYS and AUTOEXEC.BAT?

In general, the answer is no.  Add the command word REM, followed by a space, to the beginning of any Media Vision driver lines from your system files.  There are some notable exceptions to this rule.  For instance, the MINI406A.SYS and CDROM.SYS drivers used on the Premium 3D SCSI and Pro 3D SCSI sound cards, and the SLCD.SYS driver used with our sound cards which feature a Sony CD host adapter.

Here are some examples of lines which should have REM followed by a space inserted at the beginning of the line.

Sound...

DEVICE=C:\some directory name\MVSOUND.SYS D:3 Q:7 S:1,220,1,5 M:0 J:1

DEVICE=C:\some directory name\PROS.SYS D:1 I:7 H:5 P:330 Q:2

DEVICE=C:\some directory name\PA3D.SYS D:1 I:7 H:5 P:330 Q:2

CD ROM...

DEVICE=C:\some directory name\TSLCDR.SYS /D:MVCD001 /R

DEVICE=C:\some directory name\MCAM950.SYS

DEVICE=C:\some directory name\FDCD.SYS /D:MVCD005

Some lines may start with DEVICEHIGH, rather than DEVICE.  You would still insert the REM, followed by a space, at the beginning of the line.  You must shut down and restart Windows 95 after making changes to your CONFIG.SYS and AUTOEXEC.BAT system files.

## Can I use the Media Vision Pocket Tools?

You may still use the Pocket CD and Pocket Recorder utilities. The Pocket Mixer will not work properly and it is not the recommended mixer to use.  Use the Windows 95 MultiMedia Volume Control, (mixer) instead.

## My sound card uses MidiSoft’s Sound Impressions to control my sound card.  Do I still use this under Windows 95?

No.  Use the Windows 95 MultiMedia applications, CD Player, Media Player and Volume Control.  If you still wish to use Sound Impressions, do not use its mixer function at all.  It is recommended that you disable the display of the Sound Impressions Mixer by selecting Options, and taking the check off the Mixer selection.  Sound Impressions Wave Editor will function OK under Windows 95.

## In general, how do I view or change the settings for my sound card?

You should be able to make these changes through Control Panel.

Click Start, go up and click Settings.

Select Control Panel.  After Control Panel loads, select System.

Click on the Device Manager Tab.

Scroll down to the Sound Card and select it, (you'll need to expand the branch out by clicking on the + to the left of the Sound Card device name).

Now click on Properties,  then click on the Resources Tab.

Highlight the setting you wish to change.  If Automatic Configuration is checked, take off the check, then select Change Settings.

You will see a list of resources currently assigned to the sound card.  Below that may be a list of Basic Configurations.  First, scroll through the Basic Configurations to see if one of these choices includes the change you desire to make.  Within many of the Basic Configurations you can select an individual setting, like an IRQ, and change its value.  Not all settings can be changed; if a setting cannot be changed it will be grayed out, and that setting will not respond when you attempt to select it with your mouse pointer.

## I see that I can choose from several different Basic Configurations.  Which one should I choose?

For the Pro Audio Spectrum 16 and Pro Audio Studio family of cards there are three Basic Configurations.  The first is Basic Configuration 0, which sets up only the PAS 16 native mode driver, but has no Sound Blaster emulation nor does it have SCSI CD host adapter support.  Basic Configuration 1 adds the SCSI CD host adapter.  Basic Configuration 2 adds  the Sound Blaster emulation, as well as the SCSI CD host adapter.

For the Premium 3D, Deluxe, and Prosonic 16 family of cards there are two Basic Configurations.  Basic Configuration 0 sets up the native mode driver for the sound card.  Basic Configuration 1 adds the settings for MPU401 emulation.

## Should I run my Media Vision installation utilities, like INSTALL.EXE?

In general, you should never run these utilities under Windows 95.  These utilities will attempt to install drivers that are not compatible with Windows 95.

## Autodetection is not setting up my hardware.  How can I correct this?

Windows 95 has a CONFIG.SYS and AUTOEXEC.BAT file.  Remove any Media Vision hardware drivers from those lines; reboot the PC, and rerun the Add New Hardware Wizard in Control Panel.

## How do I view settings for my Midi Mapper?

Go into the Control Panel and select MultiMedia.  Click on the Midi Tab.

## How do I configure my DOS applications with sound to run under Windows 95?

Most applications will run fine.  If you need to make special settings, select the short cut by clicking on it with the right mouse button.  Select Properties, then click on the Program Tab.  Select Advanced.  You can add any necessary real mode drivers (just as if you would add them to your CONFIG.SYS or AUTOEXEC.BAT files).  Drivers added in this manner  will not load during a normal Windows 95 session; the drivers only load if this particular shortcut is run.

## My PC uses the Symphony Chipset.  How do I load SYMPFIX.COM under Windows 95?

Load SYMPFIX.COM from your AUTOEXEC.BAT file just as you did previously.  It is recommended to place the file SYMPFIX.COM in your root directory, and place the Sympfix command line as the first line in the AUTOEXEC.BAT file.  Here’s an example of the line:

C:\SYMPFIX.COM

The SYMPFIX.COM file can be downloaded from the Media Vision BBS, or from America Online as well as CompuServe.  The filename to download is SYMPFIX.EXE.

# Pro Audio Spectrum 16 and Pro Audio Studio Sound Cards

## Sound Blaster emulation is not being setup.  How do I correct this?

Under the Device Manager - Sound Card - Properties - change the configuration to "Basic Configuration 1".

If this is not successful, remove all sound drivers from the device manager.  Rerun the Add New Hardware Wizard.

You may also have to Select Settings, then Control Panel.  Select the MultiMedia icon, and click the Advanced Tab.  Expand the Audio Devices branch, and select Properties.  Disable the audio functions.  This allows the 32 bit sound driver in Windows 95 to relinquish control of the sound card to the application.

## My card now makes banging and crashing sounds or, sometimes, no sounds.  How do I correct this?

An application is contending for the control of the sound card’s mixer.  This condition was initially reported with After Dark version 3.0.  Disable any screen savers.  Even though the 3.0 version seems to work fine in all other

respects, it resets the MIDI volume level to maximum if you enable sounds.  Two ways have been found to get around the problem.  You can either turn After Dark sounds off, or you can leave the Volume Control dialog open.

## How do I add the /W:0 switch, to my configuration under Windows 95?

Go into the Device Manager and click on the parent device for the PAS 16.

Click on properties.  Click on the "Settings" tab.  There will be a setting for

"Enable Warm Boot"; click on it.  Place a check in the box.  Restart.

## How do I add the T:1 switch, to my configuration under Windows 95?

Go into the Device Manager and click on the parent device for the PAS 16.

Click on properties.  Click on the "Settings" tab.  There will be setting for

"Enable 28Mhz Clock”.  Place a check in the box.  Restart.

## I can’t get the microphone to record.  How do I change the mixer to use the microphone to record?

Run two instances of the volume control application.  On the first one, select Options, Properties. Click on Recording.  Make sure the Microphone is checked in the list of items below.  Click OK.

On the second, instance, select Options Properties.  Playback is probably already

selected; make sure you have the Microphone selected in the list below.

Click OK.

## Where do I change my DMA and IRQ values?

You will change your DMA, IRQ, and I/O settings via the Device Manager.  Where do you find Device Manager?  In the Control Panel, System Icon.  To get to the Control Panel -

Click Start

Select Settings

Select Control Panel

Select the System Icon

There will be a Tab labelled "Device Manager"

This will bring up a list of your devices.  Go down to Sound Cards, and click on the + to expand the branch.  Now you'll see your sound card listed as a Pro Audio Spectrum 16.  Select it, then click Properties.  Now click the Resources Tab.

You can change the settings via the Basic Configurations.  If you have the Automatic Settings box checked, you will not be able to modify the settings.   To modify the settings, just take the check off of the box marked Automatic Settings.

## What are the values that I will find under the Basic Configurations?

Make sure you have chosen the correct configuration for your sound card.  There are three separate configurations for the PAS 16.  Some of the configurations support the SCSI interface, and others do not.  Here are the configurations.

Basic Configuration 0	(default)	you only get PAS 16 support

Basic Configuration 1		you get PAS 16 support and SCSI support

Basic Configuration 2		you get PAS 16 support, SCSI support, and Sound Blaster support

For the PAS 16, you will find:

(note: these are in the order you will find them)

Basic Configuration 0   (default)	you only get PAS 16 support

address value

address value

address value

IRQ	__	this is the PAS 16 IRQ

DMA	__	this is the PAS 16 DMA

address value

address value

address value

Basic Configuration 1		you get PAS 16 support and SCSI support

address value

address value

address value

IRQ	__	this is the PAS 16 IRQ

DMA	__	this is the PAS 16 DMA

address value

address value

address value

IRQ	__	this is the phantom SCSI IRQ, you must set this, try 9

Basic Configuration 2		you get PAS 16 support, SCSI support, and Sound Blaster support

address value

address value

address value

IRQ	__	this is the PAS 16 IRQ

DMA	__	this is the PAS 16 DMA

address value

address value

address value

IRQ	__	this is the phantom SCSI IRQ, you must set this, try 9

I/O	__	this is the Sound Blaster emulaton port address, try 220

IRQ	__	this is the Sound Blaster emulation IRQ, try 7

DMA	__	this is the Sound Blaster emulation DMA, try 1

# Premium 3D, Deluxe, and Prosonic 16 Sound Cards;  MV1100, MV1200, MV2100, MV2200, MV3100, MV4000, MV4100 and MV5000 MultiMedia Kits

## I have lost the use of my Midi Port.  How can I enable it again?

If you cannot use your Midi Port, go into Control Panel, System, and click on the Device Manager Tab.  Go down to sound cards, and select the Prosonic 16, Deluxe, Jazz16 driver and select Properties, then select the Resources Tab. Make sure you select Basic Configuration 1 in Device manager. Configuration 0 does not provide the resources for MIDI support. Configuration 0 is the default the install Wizard uses for specific devices.

## I don’t think my Wavetable card is working. What can I do?

You'll have to set your MIDI device.  Select Settings, then Control Panel.  Select the MultiMedia icon, and click the Advanced Tab.  Expand the Midi Devices and Instruments branch, and add the external port.  Some sound cards will allow you to also select the daughter board option.

## What about SRS sound? How does this work?

The mixer provided with Windows 95 will allow you to access this feature.  To access this control, double click on the loudspeaker icon on the Task Bar to bring up the full mixer applet.  Click on the Advanced button.

## What about the BASS, Treble, and Enhanced sound modes for my card?  How does this work?

The mixer provided with Windows 95 will allow you to access this feature. To access this control, double click on the loudspeaker icon on the Task Bar to bring up the full mixer applet.  Click on the Advanced button.

# PCMCIA Sound Card

## Is my PCMCIA Sound Card supported under Windows 95?

As of the date of  this writing, there is no support for the Media Vision PCMCIA Sound Card under Windows 95.

# CD-ROM Host Adapter Issues

## I have a PAS 16, my sound works, but my CD-ROM drive is not being recognized.  I’m not getting CD-Audio.  What do I do?

Make sure you have chosen the correct configuration for your sound card.  There are three separate configurations for the PAS 16.  Some of the configurations support the SCSI interface, and others do not.  Here are the configurations.

Basic Configuration 0	(default)	you only get PAS 16 support

Basic Configuration 1		you get PAS 16 support and SCSI support

Basic Configuration 2		you get PAS 16 support, SCSI support, and Sound Blaster support

## My CD drive still is not getting a drive letter, or not able to read data from CDs.  How do I correct this?

You need to reload the Trantor Device Driver.  You will need to access the System icon in your Control Panel to find the Device Manager Tab.  Here is how:

Click Start

Click Settings.

Select Control Panel.

After Control Panel loads, select System.

Click on the Device Manager tab.

Select the Trantor Driver

Click on Properties

Select Change Driver

Re-Select Current Driver

Click OK

## My drive constantly spins when I put a CD into the drive.  How can I stop this?

You have your drive set to Auto Detect when a CD is inserted into the drive.  This is very easy to fix.  You will need to access your Device Manager to do this.  You will need to access the System icon in your Control Panel to find the Device Manager Tab.  Here is how:

Click Start

Click Settings.

Select Control Panel.

After Control Panel loads, select System.

Click on the Device Manager tab.

Click the CD-ROM type of device.

Your CD-ROM will now be listed

Select the CD-ROM drive.

Click on Properties

Select the Settings tab.

Remove the check from the Auto Insert Notification box

Click OK

## How do I adjust my settings to match my CD-ROM performance?

Click Start

Click Settings.

Select Control Panel.

After Control Panel loads, select Performance.

Click on the File System button.

Click on the CD-ROM drive tab.

Increase or decrease the Supplemental Cache Setting.  Also try changing the setting of Optimize Access to "No Read Ahead", or experiment with each of the settings.

## I’m using a Panasonic, Reveal, Mitsumi or Sony CD ROM, and Windows 95 did not install the driver for my drive.  What can I do?

It may be necessary to load the Real Mode CD host adapter driver into CONFIG.SYS.  Here are some  examples of how the line should appear:

Sony

DEVICE=C:\some directory name\SLCD.SYS /D:MVCD001 /B:1F88 /M:P

Mitsumi

DEVICE=C:\some directory name\MTMCDAE.SYS /D:MVCD001 /P:390 /A:0 /M:30 /I:10 /T:6

Panasonic

DEVICE=C:\some directory name\SGCDU.SYS /D:MVCD001 /P:390 /M:30 /V /N:1

In the AUTOEXEC.BAT file add the MSCDEX.EXE line:

C:\WINDOWS\COMMAND\MSCDEX.EXE /D:MVCD001 /M:10 /V

As of the date of this writing, version 2.25 MSCDEX.EXE is the most current version.  It will report as version 2.25 when loaded as a real mode driver; it will report as version 2.95 when loaded as a protected mode driver.

## My Sony CD ROM drive is not recognized anymore.  What can I do?

Users who have a Sony CDU-31 or CDU-33A drive being driven from a PAS 16 card’s Sony interface must change the address specified for the Sony driver, in Device Manager.  Here's how to see the address:

In Device Manager open up CD-ROM controllers, select Sony Proprietary CD-ROM Controller, then click on Properties. Select the 'Resources' tab. For

'Input/Output Range' it will probably have 340-343 with 'Setting based on: Basic

configuration 1'. This is not correct for the PAS16 CD-ROM interface.

Un-check the box: 'Use automatic settings'.

Then, change the Setting based on the 'Basic configuration 0'; change the

Input/Output range to 1F88-1F8B and the Interrupt Request to an unused IRQ.  The PAS 16 host adapter’s Sony interface does not require an IRQ, but the ‘Basic Configuration 0’ requires an IRQ assignment.

After rebooting, the CDU-31A or CDU-33A drive will be available for use.

It may also be necessary to load the Real Mode SLCD.SYS driver into CONFIG.SYS.  Here is an example of how the line should appear:

DEVICE=C:\some directory name\SLCD.SYS /D:MVCD001 /B:1F88 /M:P

In the  AUTOEXEC.BAT file add the MSCDEX.EXE line:

C:\WINDOWS\COMMAND\MSCDEX.EXE /D:MVCD001 /M:10 /V

As of the date of this writing, version 2.25 MSCDEX.EXE is the most current version.  It will report as version 2.25 when loaded as a real mode driver; it will report as version 2.95 when loaded as a protected mode driver.

## After installing the MV600 CD ROM drive, I’m getting intermittent Protection Fault errors in Module ESDI at (some address).  How can I correct this?

1)  Go into System and select the Device Manager Tab.  Select the CD drive and click on Properties.  Click on the Settings tab, and remove the check in the box Auto Insert Notification.  If you wish to leave Auto Insert Notification checked, then place any readable CD (audio or data) into the drive.  By simply leaving a readable CD in the drive whenever Windows 95 launches this error is circumvented.

If you are still experiencing problems, then you may try the following:

2)  Go into Control Panel, System and select Performance, then File System.  Select the CD ROM Tab and select Quad-speed or higher for the Optimize Access Pattern setting.

3)  Go into System and select the Performance Tab.  Select File System and check the box to Disable 32 bit protected mode drivers.

4)  Go into Device Manager, select the MV600 CD drive (OTI.C2 Stingray 8322IDE) and select Properties.  Now select the Settings Tab and see if there is a check on Disconnect, under Options.  Take the check off.  Disconnect should only be enabled if the CD drive is a SCSI device.  Click OK, and click OK again at the next window to exit.  Restart WINDOWS 95.

## My Phillips CM205 or CM206 drive is not recognized by Autodetection.  How can I install it?

You must load the DD250MS.SYS or DD260.SYS driver in your CONFIG.SYS, just as you previously did in DOS.  The Phillips CD drives and host adapter do not have Windows 95 protected mode drivers at this time. Here are the lines for your CONFIG.SYS file:

rem use the following line for the CM205 CD-ROM drive

DEVICE=C:\FUSION16\DD250MS.SYS /D:MVCD001 /P:340 /I:5

rem use the following line for the CM206 CD-ROM drive

DEVICE=C:\FUSIONLX\DD260.SYS /D:MVCD001

In the AUTOEXEC.BAT file add the MSCDEX.EXE line:

C:\WINDOWS\COMMAND\MSCDEX.EXE /D:MVCD001 /M:10 /V

## The CD drive connected to my Premium 3D SCSI sound card is not recognized by Autodetection.  How can I install it?

You must load the MINI406A.SYS and CDROM.SYS drivers into your CONFIG.SYS, just as you previously did in DOS.  The NCR 406A SCSI host adapter chip (on the sound card) does not have Windows 95 protected mode drivers at this time.  Here are the lines for your CONFIG.SYS file:

DEVICE=C:\MEDVSN\MINI406A.SYS /IO=230

DEVICE=C:\MEDVSN\CDROM.SYS /D:MVCD001

In AUTOEXEC.BAT add the MSCDEX.EXE line: C:\WINDOWS\COMMAND\MSCDEX.EXE /D:MVCD001 /M:10 /V

## How do I Install the PCMCIA SCSI Adapter?

Windows 95's Install New Hardware Wizard doesn't properly load the SCSI adapter drivers automatically. It needs the file SPARROWX.MPD from the \WIN95 directory on the Windows95 CD.  You must expand this file.  In the command example below, it will be expanded into the Windows\Command subdirectory:

EXTRACT.EXE /A /L C:\WINDOWS\COMMAND WIN95_02.CAB SPARROW.MXD <enter>

Then unplug the SCSI card, restart Window95 without the card, shut down again, and put the card back in.

Now restart Windows95, start the Install New Hardware, then if prompted, tell it where to find SPARROWX.MXD, (find it in C:\WINDOWS\COMMAND).

## I have the Reno drive with the 8 bit SCSI adapter card.  Do I still need to load ASPI3X80.SYS into CONFIG.SYS?

You must load the ASPI3X80.SYS, ADTC-CD.SYS, and ASCSI.SYS drivers into your CONFIG.SYS, just as you previously did in DOS.  This particular SCSI host adapter does not have Windows 95 protected mode drivers. Here are the lines for your CONFIG.SYS file:

DEVICE=C:\RENO\ASPI3X80.SYS /MD800

DEVICE=C:\RENO\ADTC-CD.SYS /D:MVCD002

DEVICE=C:\RENO\ASCSI.SYS

In AUTOEXEC.BAT add the MSCDEX.EXE line:

C:\WINDOWS\COMMAND\MSCDEX.EXE /D:MVCD002 /M:10 /V