SEARS Communications Script for Crosstalk MK.4 release 2.0

These are the instructions necessary to properly Set Up and use

Crosstalk MK4 with the SEARS.XTS script to send and/or receive EDI

data to/from the SEARS Communications Company Network.

This script is designed to be run in an unattended mode and

requires no human interaction during the communication session.

The communications Script is designed to automatically perform a

send and receive session with Sears, if an EDI file for Sears is

present.  It will perform a receive only session if the outbound

Sears file is not present.  The script also provides

status information about the communication session in a message

file (SEARS.MSG) which can be named and routed by assigning the

appropriate variable names.

If an error is encountered during a communications session a

second attempt will be made if an alternate dialing directory

option is specified.  If you attempt to connect to Sears using

both a 9600 (V.32) baud connection and a 2400 baud connection

two MODEM's will be required.  The script will select two dialing

directory options from the current crosstalk dialing directory.

You will be required to set up the dialing directories and update

the script variables for dialing directory names, user id, password,

EDI file names and directory locations as well as the name and location

of your SEARS message file.

The script will create a new message file for each session.

The script does not check for the existence of a current Inbound

EDI file and will overwrite the existing file. The script will not

delete the outbound EDI file after it is sent.

The maintenance of the users message and EDI files should be maintained

by the user.  The following dialing directory and script file variables

must be configured.

The dialing directory options must be set up as follows;

Enter Crosstalk by typing in XTALK <enter>

For V.32:

<enter> Alt S

Number: 18003472347

Capture: off

DownLoadDir: <user defined>

Local: off

Printer: off

Script: SEARS

Filter: None

Keys:

Device: MODEM

Port:  <user defined>

Speed: 9600

WordFormat: 8-N-1

Terminal: ANSI

BBS: off

BackSpace: off

Protocol: XMODEM

PMODE: xmodem-1K

<enter> PgDn

UserId: N/A

Mode: call

Keyboard: N/A

Log: off

Password: N/A

Echo: off

Keyclick: off

Accept: all

NetID: N/A

Graphics: off

Review: 8000

LPT: <user defined>

Answerback: N/A

Enterstring: N/A

RedialCount: 5

RedialWait: 15

Patience: 60

DialModifier: <user defined>

<enter> Alt U

LWait: none

CWait: none

LineDelim: ^M

blankEx: <blanks>

TabEx: off

LineTime: 0

For 2400:

<enter> Alt S

Number: 18003669640

Capture: off

Local:off

Printer: off

Script: SEARS

Filter: None

keys:

Device: MODEM

Port:  <USER DEFINED>

Speed: 2400

WordFormat: 8-N-1

Terminal: ANSI

BBS: off

BackSpace: off

Protocol: XMODEM

PMODE: xmodem-1K

<enter> PgDn

UserId: N/A

Mode: call

Keyboard: N/A

Log: off

Password: N/A

Echo: off

Keyclick: off

Accept: all

NetID: N/A

Graphics: off

Review: 8000

LPT: <user defined>

Answerback: N/A

Enterstring: N/A

RedialCount: 5

RedialWait: 15

Patience: 60

DialModifier: <user defined>

<enter> Alt U

LWait: none

CWait: none

LineDelim: ^M

blankEx: <blanks>

TabEx: off

LineTime: 0

Once the dialing directories are completed, update the communications

script variable as follows:

<enter> Alt E

Enter the script name:  SEARS.XTS

Page down to the Initialization Section.

Make the appropriate Changes to the following variables.

All variable except rec_switch are enclosed in double quotes,

and comments are preceded by /* and succeeded */:

/****** Initialization Section **************/

edi_in_file = "ISEARS.EDI"  /* Sears EDI inbound file 	name */

edi_out_file = "OSEARS.EDI" /* Sears EDI outbound file name */

edi_in_dir = "\MENTOR\IN\"      /* Sears inbound EDI directory */

edi_out_dir = "\MENTOR\OUT\"    /* Sears outbound EDI directory */

msg_path = "\xtalk4\"          /* Sears directory path for message                                  files */

msg_file = "sears.msg"         /* Sears Message file name */

prime_dial_dir = "SEARS"       /* Sears primary dialing directoryname */

alt_dial_dir = "NONE"          /* Sears alternate dialing directory name.

If you are using a 9600 baud v.32  modem and

do not have a 2400 baud modem hayes

compatible modem installed do not attempt to

use the alternate dialing option with the

2400 baud setting.  The alt _dial_dir

variable should be set to NONE or re-enter

the primary dialing directory */

user_id = "USER,"  	     /* Sears User id calma delimited and

upper case.

example:"ABC," */

word = "PASSWORD"    	     /* Sears user password, upper case*/

rec_switch = 1		    /* Set receive switch to 0 if you do not

want to receive any edi data : set Receive

switch to 1 if you wish to receive EDI data */

Note: All directory names should include both starting and ending slashes.

Both the user_id and pword must be upper case.

To run the Sears communications session the following command must be

entered at the command line:

xtalk do sears <enter>

Crosstalk MK.4 is a registered tradsemark of Digital Communications

Associates, Inc.

Sears and Sears Communications Company are registered trademarks of Sears,

Roebuck and Co.

Files provided by American Business Computer.

SEARS_X.DOC

SEARS.XTS

SEARS.XTC