Still Image Capture

STI Architecture and Interfaces

Draft Specification

2/25/97 8:55 PM

The information contained in this document represents the current view of Microsoft Corporation of the issues discussed as of the date of publication. Because Microsoft must respond to changing market conditions, it should not be interpreted to be a commitment on the part of Microsoft, and Microsoft cannot guarantee the accuracy of any information presented after the date of publication.

This document is for informational purposes only. MICROSOFT MAKES NO WARRANTIES, EXPRESS OR IMPLIED, IN THIS DOCUMENT.

Microsoft Corporation may have patents or pending patent applications, trademarks, copyrights, or other intellectual property rights covering subject matter in this document. The furnishing of this document does not give you any license to the patents, trademarks, copyrights, or other intellectual property rights except as expressly provided in any written license agreement from Microsoft Corporation.

Microsoft does not make any representation or warranty regarding specifications in this document or any product or item developed based on these specifications. Microsoft disclaims all express and implied warranties, including but not limited to the implied warranties or merchantability, fitness for a particular purpose and freedom from infringement. Without limiting the generality of the foregoing, Microsoft does not make any warranty of any kind that any item developed based on these specifications, or any portion of a specification, will not infringe any copyright, patent, trade secret or other intellectual property right of any person or entity in any country. It is your responsibility to seek licenses for such intellectual property rights where appropriate. Microsoft shall not be liable for any damages arising out of or in connection with the use of these specifications, including liability for lost profit, business interruption, or any other damages whatsoever. Some states do not allow the exclusion or limitation of liability or consequential or incidental damages; the above limitation may not apply to you.

Microsoft, MS-DOS, Visual Basic, Win32, Windows, and Windows NT are registered trademarks and ActiveMovie, ActiveX, DirectDraw, Direct3D, DirectSound, and DirectX are trademarks of Microsoft Corporation in the United States and other countries.

© 1996 Microsoft Corporation. All rights reserved.

This document is provided under a Non-Disclosure Agreement (NDA). Do not redistribute.

Introduction

This document specifies the architecture and components that provide for acquisition of images from devices that are primarily used for capturing still pictures. Examples of devices that use the still image architecture (STI) are:

- flatbed scanners including those with an automatic document feeder, transparency adapter, or other options

- flatbed scanners with "start" button

- sheet fed scanners with paper inserted detection mechanism

- hand held scanners

- still image digital cameras including those with stored images and associated data (thumbnails, audio, etc.)

This document is limited in scope to those items that are intended to be released in both Memphis (win97) and NT 5.0. Further work on upper level components and other devices (such as multi-function scanner/printer/fax machines) is anticipated but not documented within.

Architecture

A major design goal of this architecture is to reduce the software investment an IHV makes. It reduces the amount of work that a vendor has to do and focuses the work they do specifically for the device features itself, not system related tasks. In addition, this architecture puts as much vendor supplied code in user mode (ring 3) as is possible so as to avoid having a vendor deal with kernel code development.

A picture of the architecture is seen below.

Still Image Architecture

The components are:

- Application. Two are two primary types of applications that use still images. One is for editing of image data with typical applications of this type being Adobe's Photoshop or Microsoft's PictureIt!. The other type of application is for authoring of documents that include image data but do not focus on the editing of the image data itself. Typical applications of this type would be a word processing or page layout application such as Microsoft Word or a presentation application such as Microsoft PowerPoint. These later applications may be more interested in the text data resulting from an OCR interpretation of the image data rather than image data itself.

- Still Image API. The APIs of TWAIN, ISIS and Adobe Systems' Acquire interfaces are the common APIs in use today. An IHV has to supply a device specific component which implements a "driver" for each of these APIs they indent to support. These IHV components will require a slight revision to conform to the Still Image architecture's components.

- Color Management API. This interface and implementation maintains device color profiles and provides for color space conversion. For more information on the specific functionality and interfaces see the ICM 2.0 color matching specification.

- Still Image Control Panel. Devices are not only tracked via the Device Manger but also accessed by control panels. Printers, modems, displays, etc. appear in the control panel. We are adding still image devices in a similar manner. This control panel includes the following functionality for installed still image devices:

- Listing the installed still image devices

- Adding and removing still image devices

- Test validation of a selected device

- Vendor specific configuration/control (by attaching property sheets)

- Control of the association between specific device and applications that are to be notified of these events. This is used the for 'push model' support (see below)

- Still Image Event Monitor. This is a separate application to support push model behavior with the following functionality:

- Detect events coming from installed still image devices

- Dispatch a set of those events to a control center.

- Still Image Control Center. This is supports push model by implementing a specific policy on how still image device events are distributed to applications.

- Still Image DDI. This device driver interface is used to communicate with a particular device. The still image APIs, the Still Image control panel and the Still Image event monitor utilize the DDI. The DDI uses the color management system as a repository for the color profile supplied for a specific device. The DDI provides interfaces for

- enumeration

- device information (primitive capabilities and status)

- test activation

- data and command I/O

- notification for device events including polling for device activity

- retrieval of an ICM color profile

- User mode minidriver. This vendor written module is a small device specific component that is used to implement the device specific DDI functionality (primarily test, status, and data I/O). It informs the DDI of it capabilities (requires polling for event, for example) and uses the standard interfaces of CreateFile, ReadfFile, WriteFile, and DeviceIoControl to communicate with a specific kernel device driver.

- Still Image kernel driver. The kernel driver functions as a way of packaging a command or data for delivery on a specific bus type. There are not expected to be many of these, and perhaps none produced outside of Microsoft. Although the kernel drivers need not be WDM based, any new kernel driver that Microsoft adds will be WDM based. If the device does not conform to the standard Microsoft kernel drivers for a specific bus, the vendor will need to supply a kernel mode driver.

- Bus driver. This is the specific bus driver used for actual communication with the still image device. Examples are the USB or SCSI driver stacks as well as the serial and parallel port drivers.

Here is an expanded view showing more of the possible specific components within this architecture.

Kernel Drivers

STI kernel drivers will be provided for still image devices of the following types. Although the drivers will be written for specific devices it is expected that they are reusable for other devices of the same type.

- SCSI devices of either processor or scanner type. SCSI devices that work via read/write SCSI commands are of type processor. All HP SCSI SCL scanners fall into this category. These devices can use readFile and writeFile to send information to the device and a SCSI command block will be created by the kernel driver. SCSI devices that work via SCSI commands specific to scanners are of type scanner. Many existing scanners are of this type (the Canoscan 300 for example). These devices use deviceIOControl to send information to the kernel driver. This use is much like WinASPI in that the vendor code (typically the TWAIN data source) forms SCSI command information and sends it through the user mode minidriver to the SCSI bus. The SCSI ID and logical unit number however are controlled by the Still Image SCSI kernel driver.

- USB devices. The USB kernel driver is rather constrained from the device communication point of view but very flexible from a protocol point of view. We are implementing a driver for a USB device which has:

- a single interface

- a control end point

- a bulk input end point

- an optional bulk output end point

Data is exchanged in a "pass through mode", that is, there is no packet or message protocol used. This driver just sends/get sets of bytes. It is up to the user mode components that are vendor supplied to implement a command set or message protocol.

- Parallel devices including those using the ECP and EPP modes. Sample code for the Parallel scan module will be placed in the DDK so it can be customized for vendor specific needs.

- Serial devices. These devices will use the current serial port drivers included in the Microsoft operating systems.

In addition to these buses, this architecture should be able to support the following devices:

- 1394 still image devices

- Network still image devices

- Infrared capable still image devices

- Multi-function devices, that is ,those devices which are both scanners and printers.

Setup and Installation

To provide for easy and consistent installation the still image project includes

- Installation wizard for legacy devices on parallel and serial ports

- WDM PnP detection when installing or using hot attach/detach operations.

- Installation is extensible by vendor (calibration or other installation specifics)

IHV provided components

An IHV has the responsibility to supply the following components:

- User mode minidriver which provides an implementation of the test(), escape(), getStatus() COM interfaces and some private interfaces to the DDI.

- Installation information via an "inf' file.

- Upper level software (TWAIN data source or ISIS driver) which uses the DDI.

In addition, the vendor can supply the following optional components

- Property sheets for the control panel

- A dll used to extend the installation process for vendor specific installation needs.

Push Model Behavior

The Still Image architecture provides a consistent model in which a still image device can initiate data transfer to an application (push model) instead of an application having to request data from the device (pull model). There are two functional components that make this behavior possible:

- An event monitor to detect events coming from a device and,

- A control center to decide how to react to those events

Microsoft will implement the event monitor and provide a default control center that implements a specific policy on how device events are interpreted. A picture of this push model support is seen below.

The Still Image architecture components provide the following services used by the Still Image control panel, control centers and applications:

- Maintain the event types that are associated with devices. Some of these events are identified by the IHV as "Push" events to be sent to a control center.

- Dispatch push events to subscribed control centers

- Maintain a list of all push model aware applications

- Maintain a list of the running push model aware applications

- Launch or notify an application on behalf of a control center and provide information to the application as to which device and event initiated the notification

Event Monitor

The event monitor maintains a list of active push model devices. Devices have a capability that indicates they support push model events. The event monitor responds to the installation of new devices by responding to PnP messages for attaching and detaching devices as well as those devices that are added or removed via the Still Image control panel.

The event monitor opens all devices that support push model. The open is done in 'control' mode to allow another application to open the device in 'data' mode (for example, a pull model use of a device). The event monitor waits for events on these devices. Currently few devices are capable of actually generating an event directly from the device when data transfer has been initiated. In most cases a separate thread within the Still Image DDI implementation will simulate this behavior by polling the device and causing an event when the device becomes active.

In the event monitor there is a stack maintained of control center applications that have subscribed to push events from a device. When an event comes in from a device the event monitor looks at the subscribe stack for that device, and it passes the event along to the control center on the top of the stack. The first control center to subscribe is the default Microsoft control center.

## Control Center

A control center implements a specific policy for how device "Push" events are interpreted. The control center uses the system list of applications registered as push model aware, and the list of those applications that are running. The Still Image control panel is used for user control of some of the control center behavior. The control center adds a property page to the control panel per device and this allows the user to alter the behavior of its control center.

When a control center decides that it needs to notify an application is must do so via the I_STI subsystem. This provides a consistent mechanism ensuring compatibility between any control center and all push model aware applications. An application will be launched with arguments of Event:event_name and Device:STI_device_friendly_name on the command line. The application doesn't parse these arguments off the command line, however. Instead there is an method called getLaunchInformation() which returns the device event and the device friendly name (in a structure).

## The Default Microsoft Control Center

The Microsoft control center implementation allows the user to control the association between device events and applications to be notified when these events are detected. Using the control panel the user selects a device and one of the push model events for this device. Given this device/event pair, the user selects the subset of push model aware applications that can be used. Here is an example of how the control panel is used to associate device events with applications:

Using this method, for example, the user can easily associate a specific push event on a specific device for use with an OCR application and another event on the same device for use with a photo editing application. The user can just as well associate a specific push event on a specific device for use with an OCR application and another event on a different device for use with a photo editing application.

The Microsoft control center uses the following simple policy:

- Upon receiving a device event from the event monitor the control center obtains the subset of all registered applications selected by the user to respond to this device event (PREFERREDAPPS). The control center maintains this information.

- If there are PREFERREDAPPS selected for this device/event and PREFERREDAPPS has only one item, it is launched, otherwise a dialog containing PREFERREDAPPS is displayed and the user selects which application is to be launched,

- otherwise the user is informed that there is nothing associated with the event. If this is the case, the device has essentially been disable for push model behavior.

This control center does not consider an application that it notices has registered for the first time to be in any selected subset for push events from any device. The user must explicitly select the events using the control panel property page for each device before the application will be able to receive push events.

## Push Model Aware Applfications

Applications will have to be revised to make use of the STI push model behavior. Applications which are push model aware will call the I_STI->Register(Appname, pNotifyStructure) interface. This will

- put the application on the permanent list of push model aware applications

- add the application to the list of running push model aware applications

An application can detect if it were launched via the control center. If I_STI->GetLaunchInformation() API fails then the application was not launched via the control center. An application, TWAIN data source or other software can decide to reject using the device that caused the push model event, however, it must notify the user in some manner as to why the device is not acceptable. This allows the user to utilize the Still Image control panel so deselect this particular application for use with this device.

If an application were notified via the control center the application may need additional information to locate the device via it's image API. TWAIN and ISIS are the two APIs that we define information for, but any API can use the STI mechanism for obtaining additional information. On installation, the .inf file will place information into the registry that provides this supplementary information. A key/value string such as the following is used to specify this information such as

TWAIN=logiscan.ds

ISIS=xxx.xxx [need information here on how ISIS locates devices]

The application then has to use

retCode = GetDeviceValue(deivce-name, ValueName, ValueData-type, &value, ValueSize)

to obtain this data . In the case where Key == TWAIN, value would be logiscan.ds.

Once a TWAIN data source has been loaded it can obtain the device and event via the same getLaunchInformation() API since it is in the original address space. With this information it can open the device and start communication providing a seamless transition from push event, application launch, and image acquisition.

Once an application (or a TWAIN data source) has a device open all events will be sent to this application. The control center will not be aware of device activity until the device is closed.

When an application shuts down it will call I_STI->DeRegister(Appname, FALSE). This will remove the application from the running list of applications. When an application is uninstalled it will call I_STI->DeRegister(Appname, TRUE) to remove the application from the permanent list of push model aware applications. Only applications will register and deregister themselves, the user may not add or remove an application from the control panel, they may only select or deselect those to be associated with particular devices.

## Issues

- Should a non push model aware application be allowed to be added via the user in the control panel? This is easy to do, but the application itself will never 'register' and be known on the running application list. This will cause problems in the rules for notification. We say no.

- Should there be a user selectable behavior in which the "default" application for a device should always be notified instead of a running, selected application. We can see both actions being valid but have prefered the running applications over the default.

Color Management

The Still Image architecture is designed for use with the color management facility provided by the ICM 2.0 set of APIs. A vendor supplies one or more color profiles with their device. During installation of the device these profiles are installed in the system and associated with the still image device. A client of the DDI (for example a TWAIN data source) will use the ICM API EnumColorProfiles() to locate the appropriate color profile to use.

Dual Mode Devices

The Still Image architecture is designed for devices that are used to capture still pictures, such as scanners.  It can also be used to accommodate devices that are primarily used for motion video but capable of generating a single, high resolution picture. The Activemovie 2.0 API for video capture has methods that provide for obtaining a single frame of video. If a vendor wishes to have such a device appear in the Still Image Control Panel or use the Still Image Event Monitor the device must have a user mode minidriver implemented. To allow a still image application to access this device a TWAIN data source also needs to be written. To synchronize device access between video and still image use of this device the video capture kernel minidriver is accessed via an Activemovie filter graph instantiated inside the still image user mode minidriver.

This solution allows for a display of motion video by the TWAIN data source during image preview and a high resolution still image to be returned when requested. At the moment of  request the resolution of the device is set and single frame of video is captured (we presume out of real time). The following diagram illustrates the use of the still and video architectures with a dual mode device.

It seems that the TWAIN data source and the user mode minidriver would work with all video capture devices unless a vendor had added custom interfaces on the VideoCapture filter. Microsoft may produce these components in the future.  Although this example has focused on using the Activemovie 2.0 video capture interfaces, it is also possible to use an existing Video for Windows driver in a similar solution.