HumanCAD Systems Inc.

ManneQuinPRO Initial Tutorial

By Don Helt

This tutorial was has been provided to you as a quick and easy source to introduce you to the various functions of ManneQuinPRO. Through simplified step-by-step instructions, explanations and illustrations, you will develop your own three dimensional human, exposing yourself to the variety of ergonomic functions and CAD tools. ManneQuinPRO can offer any designer, engineer or human factors professional.

The first step in this tutorial is to become familiarized with ManneQuinPRO’s primary functions, buttons and screen commands. The second step is a crash-course introduction to the application and its features. The third step in this tutorial is to actually create your own customized three dimensional human in all the available body forms including stick, human, skeleton, humanoid and robot figures. The fourth and final step in this tutorial is to pose the mannequin in various positions, draw and import different 3D objects, apply and edit external loads to the mannequin’s body and perform a series of analysis including biomechanical prediction and NIOSH (National Institute of Occupational Safety and Health) calculations and simulations.

Our primary sample for this tutorial is a very simple, but common activity called “Lifting a Box” (Figure T1). The use of this example is the quickest and easiest way to comprehend the applications of ManneQuinPRO and its various functions.

Figure T1

### Table of Contents

## The Primary Principles of ManneQuinPRO		p. 3

All About the Main Screen				p. 4

All About the Buttons					p. 5

ManneQuinPRO Crash-Course				p. 6

Sample Ergonomics Issue					p. 10

Let’s Draw a Mannequin					p. 12

Let’s Pose and Manipulate a Mannequin		p. 13

It’s Time to Draw a 3-Dimensional Object		p. 15

Putting the Pressure on a Mannequin			p. 16

Analyze the Forces on a Mannequin			p. 17

How Much Weight Should Be Carried?		p. 18

Looking Through The Mannequin’s Eyes		p. 20

## The Primary Principles of ManneQuinPRO

The key to successfully and productively using any computer application is understanding its primary principles and methods of operation. For ManneQuinPRO there are only three principles you need to remember to take you where you wan to go. They are:

- Remember to ZOOM EXTENTS your drawing. Right after you create a mannequin, change views or perform a series of tasks, select ZOOM/EXTENTS from your VIEW toolbar option. This automatically scales your drawing to fit the screen. It helps you to work more efficiently as it brings your ideas into a more usable perspective. Try it and you will see.

- Put every thing into the correct view. Before you begin importing objects or moving your mannequin, be sure to have selected the screen view that will benefit you the most. Examples: 1) if you are going to bend your mannequin forward at the waist, you would view your drawing from the right or left, 2) if you want to spread a mannequin’s legs away from each other or raise/lower the arms to the sides, view your drawing from the front or back, and 3) if you want to manipulate your mannequin in a variety of ways, perhaps you should view your drawing either from two or four views at once (split-screening).

- Remember to choose your mannequin or object before you try to move it. Place yourself into your drawing as though it were a real life situation. If you wanted to bend someone’s arm or move a cup, the first thing you would do is choose who’s arm you are going to bend and which cup you are going to move. The same principle applies to ManneQuinPRO. Select your 2D or 3D object first, then move, scale, rotate or edit that object. Your virtual realistic world becomes realistic and your realistic world becomes virtually realistic using HumanCAD applications!

Though these three principles are not the only ones needed operate ManneQuinPRO efficiently and productively, they are three of the most important ones. Most errors encountered while using ManneQuinPRO occur because of these three reasons.

All About the Main Screen

Illustrated below is the main operating screen for ManneQuinPRO. It displays the entire button cluster and tool bar menu. A brief synopsis of most of its primary features, excluding the buttons, is written below.

Figure T2

#### File Menu: contains all those functions required for file opening, saving, importing/exporting and program authorization.

- Edit Menu: contains all those functions required to alter or manipulate any 2D or 3D object

- View Menu: contains all those functions required to alter view formats and display characteristics for both the screen and its objects

- People Menu: contains all those functions required for mannequin specific editing or image form display (e.g. skeleton, human, etc.)

- Drawing Menu: contains all those functions required to draw 2D and 3D objects

- Tools Menu: contains the miscellaneous functions required to specifically setup ManneQuinPRO, including user defined preferences

- Animation Menu: contains all those functions required to create and edit animations

- Biomechanics Menu: contains those functions required for biomechanical analysis

- Help Menu: contains general program information and user help files

##### All About the Buttons

### Select Object

Rotate Object

Move/Reposition Object

Scale Object

Edit External Force

Biomechanical Summary Report

Maximum Vision

Right Arm Reach Envelope

Left Leg Kick Envelope

Right Hand Reach

Create a 3-Dimmensional Sphere

Move similar body parts in opposite directions

### Draw dimensions

View drawing from the Front

### View drawing from the Right

### View drawing from the Top

### View drawing from the User Defined View

View drawing from four perspectives

View drawing through the mannequin’s eyes

Show the floor level in drawing

### Select the mid-point of a line

## Create a Mannequin

## Zoom in by Window

Capture a Frame for Animation

Erase/Delete Object

Add an External Force

Add Foot Prediction Force

Optimal Vision

Left Arm Reach Envelope

Left Leg Kick Envelope

Left Hand to Reach

Insert Text on a drawing

Move similar body parts simultaneously

Hide Dimensions

## View drawing from the Back

View drawing from the Left

## View drawing form the Bottom

View drawing from the Camera View

View drawing from two perspectives

Displays stage set in Tools/Set Stage Selects Select the closest point to mouse cursor

Display a grid on the screen

Move similar body parts the same button

###### Figure T3

ManneQuinPRO Crash-Course

ManneQuinPRO also has a file import/export function. Through this the user has the capability to import any MQPro file (*.MQW), HumanCAD file (*.HCD) or CAD file (*.DXF – either 2D and 3D DXF’s supported). These objects can be imported in accordance to their axes of design and then be grouped according to various properties including object color. As a quick “jump start” to many of the features of MQPro, follow this example of importing another file:

- Select the “front” view  from the bottom of the side tool bar.

- From the top pull-down menus, select FILE/IMPORT/Labwrkst.mqw. NOTE: The Import function adds an object to your current drawing while the Open function starts a new drawing in an empty drawing space.

- Notice that a DXF Import Options menu appears after you have selected a DXF file. From here you can select the appropriate units for the file being imported (inches, feet, millimeters, centimeters or meters), align MQPro’s axes with AutoCAD’s axes for this file, or if the file consists of multiple objects, group each object with another according to their similar properties (e.g. color).

- A cross hair appears on the screen. This indicates the center of your file being imported. Place it in the center of the screen and click your left mouse button. An image similar to Figure T4 should appear on the screen.

Figure T4

- From the top pull-down menus, select VIEW/ZOOM EXTENTS. This enlarges your drawing to fit the screen for easier working.

- Form the lower left-hand corner of the screen click on the “right”  view button, then the “front” , “top”  , and “user”  buttons. Use these to view your file easily form different perspectives.

- You can also view your drawing form 2 or 4 views simultaneously. To do so, click on either the “2-view”  or “4-view”  buttons from the lower left-hand corner of the screen. Your screens should look similar to Figure T5 or Figure T6:

Figure T5                                                       Figure T6

- You can also define these views by selecting VIEW/Set 2-VIEW or 4-VIEW from the top pull-down menus. One of these two windows will appear:

Figure T7                                                       Figure T8

- To define the appropriate view settings, first click on a circle from the VIEWPORT box and then click your cursor in the desired view box. The letter in the center of the chosen box should change accordingly. Then re-click either the “2-view”  or “4-view”  buttons.

Reach/Range Cones

To demonstrate the reach and kick cones available within MQPro, follow these simple instructions:

- Click on the “front”  view button from the bottom left-hand corner of the screen and then from the top pull-down menus select VIEW/ZOOM EXTENTS.

- Click on the “select object” button  and double click on the sitting mannequin for editing. He should highlight completely.

- From the center of the side tool bar, select the “left arm reach”  button, then the “right arm reach” , “left leg range”  and “right leg range”  buttons. Also click on the “optimal vision”  and “maximum vision”  buttons to view the vision range of the mannequin. Your final images should look similar to Figures T9 below.

Optimal Vison                                              Maximum Vision

Reach/Range

- Perform step 3 an additional three more times while in the “right”, “top” and “user” views. Remember to  ZOOM EXTENTS your drawing before selecting the reach/range icons.

Look Through The Mannequin’s Eyes

Another unique feature of MQPro is its ability to illustrate the perspective of the mannequin. With this the user can actually see what the mannequin sees. To demonstrate this, follow these steps:

- Select the “front”  view icon from the side tool bar and then ZOOM EXTENTS.

- Click on the “select object” button  and double click on the sitting mannequin for editing. He should highlight completely.

- From the side tool bar click on the “eyes”  button. Your screen should be similar to Figure T10. Now you and the mannequin are looking through the same pair of eyes.

Figure T10

- From the top pull-down menus select PEOPLE/MOVE/HEAD. Axes appear on the head of the mannequin illustrating an X for flexion/extension, Y for rotation and Z for lateral flexion/extension.

- Click once on the X-Axis with the left mouse button. Swipe the mouse left-right and the mannequin’s head should flex/extend accordingly.

- When the desired posture is reached, click on your left mouse button again to freeze the position.

- Once again select the “eyes”  button from the side tool bar to view what the mannequin sees. To switch to an alternative view, simply click one of the other view buttons:

Sample Ergonomics Issue

The file Labwrkst.mqw displays a mannequin sitting at a lab workstation in a comfortable and healthy posture. The workstation’s dimensions are setup in accordance to his anthropometrics (body measurements). But what would happen if someone with significantly different anthropometric values were to sit at this workstation? Well you are going to find out right now and it is up to you to determine how to improve the situation. Follow these steps to get started:

- Click on the “select object” button  and double click on the sitting mannequin for editing. He should highlight completely.

From the tool bar menu select PEOPLE/CHANGE BODY and select the following options as described in Figure T11. Notice a significant change? The new mannequin is much smaller than the original. Her feet do not rest on the foot rail and she is not even on the seat. Figure T12 and Figure T13 illustrate the before and after affect of this alteration.

Figure T11

Figure T12                                                     Figure T13

Now that the origin of possible ergonomic risks is identified, a number of things must be done to improve the situation: 1) lower the table height, 2) raise the chair height, 3) raise the foot rest, 4) shift the chair closer to the table and 5) rearrange the equipment on the table. But before anything can be done, place the new mannequin on the seat properly and use that as your starting point. To do so follow these steps:

- Click on the “select object” button  and double click on the sitting mannequin for editing. He should highlight completely.

- Select the “reposition/move” button .

- Place the mouse cursor over the mannequin and click-and-drag to move the mannequin properly onto the seat. Be sure to toggle through all the other views (front, back, right, left, etc.) to ensure the mannequin was moved properly.

- Once properly seated, begin to make the adjustments suggested above by using the various mannequin tools. Do not limit yourself to those suggestions only. Be creative and move anything you want to. If you feel as though you made an error, select EDIT/UNDO from the top pull-down menus to undo your last action.

- Check to see if the computer monitor is at the right height by clicking on the “eyes”  button form the side tool bar.

- Also check to see if the phone, keyboard and other office accessories are within reach by clicking on the “left reach”  and “right reach”   buttons. After clicking on one of these two buttons, place and click the cursor where you would like the mannequin to reach to. If that point is within reach, the mannequin will change his/her posture so that the selected hand is placed at that point (Figure T14 and Figure T15). If you do not want the mannequin to move but would like to see the reach envelope of that individual, select the “left arm reach”  button or “right arm reach”  button.

Figure T14                                                         Figure T15

##### Let’s Draw a Mannequin

Remember the illustration of the mannequin holding the box at the beginning of this tutorial? Well the time has come to create it! Just follow these steps and your true ManneQuinPRO experience will begin:

- Select FILE/NEW from the toolbar and click on the “create a mannequin” icon

- From the first window, select which type of mannequin you wan to create. For now click OK and you will use the default settings explained here:

Sex: Male

Size: Average – can be referred to as a general measure of height

Customize: No – permits user specific body measurements or automatic calculations based on height and weight

Body Type: Average – refers to girth or mass

Range: Normal – refers to the range of motion limits of the mannequin

Database: U.S.A (1988 US Army) – refers to the population selected for study

Figure: Human – the graphical form of the figure

###### Figure T16

Age: Adult – if child is chosen, an age between 3 and 12 may be selected

- Click OK on the body information window

Choose a posture for the mannequin. For now we will just click OK for the default posture (anatomical posture). But before doing so, click MORE… to view other pre-installed postures. After viewing the list, click PREVIOUS and then OK. Make sure that in the FACING box the same view is chosen as in the lower left corner of the main screen (default is FRONT)

Figure T17

- A cross hair now appears and the message box at the bottom left corner of the screen displays “Mannequin: Select Center”. Place the cross hair where you would like the center of your mannequin to be placed. For now, place them in the center of the screen.

- Now from the tool bar select VIEW/ZOOM EXTENTS to have your mannequin fit the screen.

##### Let’s Pose and Manipulate a Mannequin

On your main screen you should have a mannequin posed very similar to the one illustrated to the left in Figure T18. What we wan to do is view the mannequin from the right side, just like in Figure T1, so to do so simply click on the “right”  button at the bottom left hand corner of the screen. You image should appear similar to the one illustrated to the right in Figure T18.

Figure T18

Now that the mannequin is seen from the most beneficial view we can begin rotating his lower arms. Once again just follow these simple steps:

- Click on the “both same” icon

- Click on the  “select object” icon

- A small box appears as the mouse cursor. Place that on the mannequin, over the lower arm. Click the left mouse button until only that part is highlighted.

- Click your right mouse button and an image similar to Figure T19  appears at the mannequin’s elbow. These are your axes of motion for that body part.

Figure T19

- Choose the X-Axis to bend the lower arm up. If you swipe your mouse left-to-right, you will see that the arm raises and lowers. Simultaneously, in the lower left corner of the screen, the degree of rotation is displayed numerically. Swipe the mouse until both arms are at approximately 90 degrees. Then click your left mouse button. Your mannequin should look similar to Figure T20.

Figure T20

- Now click on the “front” view button  and select the Y-Axis to rotate the hands, palms facing into each other (clapping hands position). Once again swipe the mouse left-to-right to accomplish this and click the left button when completed. See  Figure T21

Figure T21

It’s Time to Draw a 3-Dimensional Object

The next step in our tutorial is to draw a three dimensional object, specifically a box, and place it in the hands of our mannequin. Follow these simple steps and your drawing portion of this ManneQuinPRO experience will be complete:

- From the tool bar menu select DRAWING/BOX and a cross hair appears on the screen.

- Using the left mouse button, click-and-drag the cross hair near the mannequin’s hands to establish the width and height of the box. When you let go you will notice a small cross hair at the first corner you selected. MQPro is now waiting for you to define the depth of your box. To do so, place the mouse at any distance from this point (horizontal, vertical or diagonal). This distance is the depth of your box. Click the left mouse button when you have chosen your measurement.

- If you now toggle through all the views, (front, back, right, left, top and bottom) you will see that you have successfully created a 3-dimendsional box.

- An alternative to creating a box with this method is doing so using the F3 button. When you first select to draw the box and it asks you to define its points, press the F3 button to manually enter the box dimensions and locations.

Now we must place the box in the hands of the mannequin. You may have noticed when you toggled through the other views that the box was not in-between the hands. Let’s move the box following these steps:

- Click o the “right” view button  to see the mannequin and box from the side

- Click on the “select object” icon  and place the small mouse cursor box over any one of the box’s lines.

- Press the left mouse button to select the box (notice that it is now highlighted)

- From the buttons on the left select the “reposition/move”  button

- Place the mouse cursor in the center on the box and click-and-drag it to its new position in between the mannequin’s hands.

- Click on the “front” view button  and perform the same procedure.

- Toggle through the other views to make sure the box is properly located.

Now we are going to group the box with the right hand. While in user view , use the select button to click on the right hand (Figure T22). Then while holding down on the SHIFT key, click on one of the box’s lines. Both of these objects should be highlighted. If so, from the tool bar select EDIT/GROUP. Click your right mouse button when done (as instructed by the message bar in the bottom left corner of the screen). Now whenever the right hand is moved, the box moves with it.

Figure T22

Putting the Pressure on a Mannequin

The time has come for us to put-the-pressure on the mannequin by applying external loads to its body. We will place these in both the right and left hands. Follow these easy steps to begin understanding biomechanics and its many applications:

- With the display in user view , select the mannequin using the “select object” button  and by double clicking the left mouse button. The whole body should highlight. Don’t forget about VIEW/ZOOM EXTENTS!

- Once selected, click the “foot prediction force” button  to see the mannequin’s center of mass and foot reaction force. Two arrows pointing upwards should appear at the feet and a small red dot near the waist.

- Click on the “add external force” button , place the cross hair on the right hand of the mannequin and click the left mouse button. A window appears with columns and rows. Place your cursor in the Y-column and FORCE-row. Type in –10.00 for the force value. This places an external force of 10 pounds acting downward on the right hand. Click OK and an arrow will appear on the hand pointing down.

- Repeat step 3 for the left hand. You need not select the “add external force” button again for it is already highlighted. Simply place the cross hair on the left hand.

- Click the “foot prediction force” button  to see the mannequin’s new center of mass and foot reaction force. Toggle through the different views to see the location of the forces. You should notice that as the forces were applied to the hands, the foot prediction forces moved closer to the toes. This is because a greater reactive force is required by the mannequin’s feet in order to remain standing. See Figure T23

Figure T23

Analyze the Forces on a Mannequin

What we are going to do now is to see the forces acting on the mannequin’s body quantified (referred to as biomechanics). Specifically we are going to view the torque (force x distance) at each joint of the body and the force applied to each body part. The units for these values are specified in the TOOLS/SET UNITS option of the tool bar. Follow these few steps to view the biomechanical summarization chart:

- Make sure that the mannequin is in the proper position for evaluation and all the external forces have been applied.

- Make certain to click the “foot prediction force” button  after every posture or force change.

- Click on the “biomechanical summarization” chart button  to view all the forces on the body and the torque reaction forces. If you followed all the preceding instructions, your summary table should be similar to that shown in Figure T24.

Figure T24

##### How Much Weight Should Be Carried?

ManneQuinPRO is conveniently equipped with the Revised NIOSH Lifting Equation for lifting task analysis. If properly used, this formulation can provide a recommended lifting weight for a specified activity. To access this function, select BIOMECHANICS/NIOSH from the tool bar menu. We will perform two different tasks unitizing this tool: 1) determine a recommended lifting weight and 2) simulate a lifting task. Follow these steps to make it all happen:

- Make sure you are viewing the drawing from the right view and use the bottom, horizontal scroll bar to shift the image to the right. This will leave room on the left for the NIOSH table.

- Select BIOMECHANICS/NIOSH from the tool bar menu and click the INITIAL button to save this image of the mannequin as the first frame of the simulation.

- Click the “select” object button  and click on the chest and while holding the SHIFT key, click on the box also (both objects should highlight). Then click the right mouse button, select the X-Axis and bend the mannequin’s chest downward toward the floor by swiping the mouse left-to-right. (see Figure T25).

- Click on the “both the same” button  from the left tool bar and then the “select object” button  again. Place the selection box over the mannequin, select his/her upper arm and while holding the SHIFT key click on the box. Then click the right mouse button again and rotate the arms up and away from the body using the X-Axis (see Figure T26). Click the FINAL button to save this image as the final frame of the simulation.

Figure T25                                     Figure T26

- From the NIOSH window click on the SIMULATE button and MQPro will perform the lift for you. If you like, you can also change the number of frames by clicking the FRAMES button. This analysis can then be saved for later viewing.

- To perform an actual NIOSH calculation, click on the CALCULATE button and enter the necessary information (similar to that illustrated in Figure T27). Then click the CALCULATE button again. The Recommended Weight Limit as prescribed by NIOSH is displayed at the bottom of this window.

Figure T27

##### Looking Through The Mannequin’s Eyes

An interesting feature included with this HumanCAD application is the Mannequin View button. By clicking this, you can see exactly what the mannequin sees. This includes both the optimal and maximal range of vision. Here is how you do it:

- Click on the “select object” button, place it over any part of the mannequin and double click. The entire mannequin should now be highlighted.

- From the side tool bar select the eye icon . An image similar to Figure T28 should be displayed on the screen. This illustrates the mannequin’s central vision, peripheral vision (in gray) and blind spots (in black). To return to normal viewing, click on any of the other views (front, right, bottom, etc.)

Figure T28

This concludes this tutorial for the ManneQuinPRO ergonomic software. Be sure to check our website for future tutorials focussing on industry specific issues and occupationally specific examples, e.g.: design engineering, surgical, rehabilitation/medical, industrial engineering.