chapter 7

Job Options

TapeWare uses parameters on a jobÕs Options tab to control various features necessary for running a job. TapeWareÕs default values are designed to be easy and secure to use, but you can modify the job option parameters to meet your particular needs.

### In This ChapterÊ

n	Overview

n	Backup Job Options

n	Advanced Options

n	Restore and Verify Job Options

n	Troubleshooting

# Overview

This chapter covers an important tab on the property sheet of a TapeWare job: the Options tab. This tab controls various features of the job, such as what media is used, what backup device is used, whether files can be overwritten, and how alerts are handled.

Although this tab is relevant to all job types, the Options tab is especially important for backup jobs. Some types of backup schedules, such as automatic rotation jobs, treat certain job option parameters differently than do others, such as manual rotation jobs.

The first part of this chapter focuses on backup job options, while the second part explains the advanced options parameters used by backup and verify jobs. Finally, these concepts are reviewed for both restore and verify jobs.

# The Backup Job Options Tab

The Options tab on the property sheet of backup job controls various parameters important to how TapeWare runs a backup job. Although there are numerous choices, the default values have been chosen to provide the maximum degree of security and ease of use. Use the default values unless your particular backup needs require different settings.

The following section describes each option parameter and some of the possible ways each might be used.

### Backup Mode

The Backup mode is either Full, Incremental, or Differential. For scheduled automatic rotation jobs, TapeWare uses the backup mode type for each backup set as indicated on the Schedule tab; for unscheduled or manual jobs, TapeWare uses the parameters set by the user. For more information, see ÒBackup Options Automatically UpdatedÓ below.

FullÊThis parameter instructs TapeWare to back up all selected files.

DifferentialÊThis parameter instructs TapeWare to back up all selected files that have changed since the last full backup.

Incremental This parameter instructs TapeWare to back up all selected files that have changed since the last backup.

For more information on the differences between incremental and differential backup jobs and the importance between them for data recovery, see the previous chapter.

### Write Mode

The Write mode is either Append to all, Append to first overwrite others, or Overwrite all. For scheduled automatic rotation jobs, TapeWare defaults to Overwrite all mode; for unscheduled and manual jobs, TapeWare uses the parameters set by the user. For more information, see ÒBackup Options Automatically UpdatedÓ below.

This mode determines whether the old data on the media is overwritten with new data, or whether the new data is appended to the end of the old data. When media is overwritten, all of the data previously stored on it is lost. Appending data will preserve the old data.

Unless the media is meant to be stored permanently, select Overwrite all. This is because when tapes or media are rotated (reused), TapeWare overwrites it. If you have appended data to the media, overwriting will result in the loss of not just the oldest material, but all of the data on the media, including the most recent. For this reason, use Overwrite all for media you wish to reuse through rotation, such as tapes that are part of a set of daily incremental backups, and Append to all or Append to first, overwrite others for media meant for permanent storage.

Appending is useful if the number of tapes is limited or if the tapes are several times larger than the size of the job. For example, a one gigabyte tape could hold the contents of four jobs that are less than 250 Mbytes if these jobs were appended. However, if overwrite mode is selected, only one job will be stored on a tape at once. Similar comments apply to other types of media.

Append to allÊThis parameter instructs TapeWare to append all data to the end of the media. No data is overwritten. Select this parameter for permanent storage.

Append to first, overwrite othersÊThis parameter instructs TapeWare to append data to the end of the first media, but to overwrite all media that follows. For example, TapeWare will not overwrite the first tape inserted, but will overwrite the second, third, and later tapes. This parameter is useful if you have a set of media with old data you no longer need. By selecting this option, TapeWare preserves your most recent data on the first media, but overwrites older, unneeded media.

Overwrite allÊThis parameter instructs TapeWare to overwrite all media. All data on media that is overwritten is lost. Use this option for tapes that are going to be recycled.

### New Media Name

The New media name is the name TapeWare gives to any new media it creates while running the job. For scheduled automatic rotation jobs, TapeWare automatically updates this parameter to match the mediaÕs place in the rotation schedule. For example, if the media is the first media in the yearly rotation set, TapeWare names it ÒYear Set 1:1Ó. (This means the media was used with a Yearly backup job and that it was the first tape in the first set.)

For manual rotation and unscheduled jobs, TapeWare assigns any new media it creates the name listed in this field. This is also true for automatic rotation jobs that are ÒforcedÓ to run. For more information, see ÒBackup Options Automatically UpdatedÓ below.

### Auto Format Mode

The Auto format mode is either No auto format, Auto format all, or Auto format blank.

This mode determines whether or not TapeWare will format the tape automatically. Before data can be written to a tape, the tape must be formatted. When tapes are formatted, any data on them is lost. Tapes and other media are formatted when TapeWare does not recognize the media, that is, when it has no information in its database about that particular media. This will occur when the tape is blank, it has been erased, it is first used, or it has been deleted from the database.

Auto formatting speeds up jobs and allows jobs to be run with no one attending them. On the other hand, disabling auto format can help insure that no data is lost by accidentally formatting a tape.

No auto formatÊWhen this parameter is selected, if TapeWare encounters media that needs to be formatted (either blank or unrecognized media), it sends an alert to the alert window. While waiting for a user reply, TapeWare scans the network for devices with the media it was expecting.

Auto format allÊThis parameter instructs TapeWare to automatically format all of the media inserted into the tape drive which require formatting. With this parameter selected, TapeWare will automatically format all new (or blank) tapes and all unrecognized media. Select this option when you are creating a job that will run unattended.

Auto format blankÊThis parameter instructs TapeWare to automatically format all new or blank media. However, if TapeWare encounters unrecognized media, it sends an alert to the alert window and then scans the network for the media it was expecting. This parameter can help prevent data from being accidentally destroyed by formatting, while not needlessly querying the user before formatting a blank tape. Under almost all circumstances, setting the Auto format mode to this parameter is sufficient protection against data loss and is preferable to No auto format.

### New Media Location

The New media location is the folder in which TapeWare will store any new tapes created while the job is running. By default, TapeWare stores media in the current User/Group folder; the media will appear there on the Media and Database tabs. You can also store the media in a Media folder inside the User/Group folder.

Select the folder in which you wish to store any new media or tapes by clicking on the BrowseÉ button and then selecting the folder from the Browse window. If you do not already have folders set up in which to store the media, use the Media tab to first create additional folders.

When TapeWare runs a scheduled automatic rotation job, it automatically creates new media folders for the job. The folders are organized by the name of the job and the various rotation sets in that job. There is no reason to create these folders manually. TapeWare will automatically create these folders for you.

### Number of Passes

This parameter determines how many times TapeWare will attempt to open a file in order to back up that file.

Sometimes when TapeWare is attempting to back up a file, the file may already be open, that is, currently being used by another user. Because that user may be modifying the file, TapeWare will attempt to wait until that user is through using the file. Each time TapeWare attempts to open a file is called a pass. When TapeWare is unable to back up a file on the first pass, it attempts to back up that file on subsequent passes. On the last pass, TapeWare opens the file in shared mode.

For example, suppose the number of passes is set to 5. On the first pass, TapeWare attempts to back up all of the files in the file selection list. If, on the first pass, it encounters any files being used by other users, it skips them and attempts to back them up on the second pass. If, on the second pass, these files are still being used by other users, TapeWare schedules them for the third pass, and so forth until the final pass. On the final pass, in this case the fifth pass, any file not previously backed up is opened in shared mode, regardless if it is currently being used by other users.

The default value is 5. If your historical usage shows that many files are open during backup or if the backup job will run when other users are likely to be working with the files to be backed up, set the value to a higher number. This will increase the number of times TapeWare attempts to open a file and may result in fewer files being opened in shared mode during the final pass.

### Delay Between Passes

This determines how many seconds TapeWare waits before attempting the next pass. If your historical usage suggests that many files are being opened on the last pass, consider increasing this parameter to a higher number.

### Log option

The Log option is either None, Log only failed, Log only completed, or Log all.

TapeWare keeps a log of which files are backed up while running a job. After a job is run, you can view or print the log to see which files were successfully or unsuccessfully backed up. The default value is Log only failed, which writes in the log any files which were not successfully backed up. This is particularly useful for locating any problems with running the backup job.

NoneÊThis parameter instructs TapeWare to not keep a log of the backup job as it runs.

Log only failedÊThis parameter instructs TapeWare to log the name of any file selected for backup but, for some reason, not backed up. Use this option to check if a backup job is running correctly.

Log only completedÊThis parameter instructs TapeWare to log the name of any file selected for backup and successfully backed up. You might use this option to produce a list of files backed up for archive purposes, for example.

Log allÊThis parameter instructs TapeWare to log the name of every file selected for backup and whether or not that file was successfully backed up. You can use this option to be certain that a backup job is running correctly as you planned.

### Auto Verify Mode

The Auto verify mode is either Full verify, No verify, or Quick verify.

After TapeWare backs up a file onto a tape, it can check to see that this file was backed up correctly. To verify if the file was correctly backed up, TapeWare reads the file from the tape and compares it to the original file. If any discrepancies between the two files are found, the file is considered to have failed the backup.

It is strongly recommended that the Auto verify mode be set to Full verify. Verification that data has been correctly written to the tape is an essential part of a comprehensive backup program. Further, verifying the files insures that the tape and the tape drive are working correctly. Restoring data after a disaster is no time to discover that the data was incorrectly stored to begin with.

Full verifyÊThis parameter instructs TapeWare to compare every file on tape with the original file from the workstation or file server. This default value is strongly recommended.

Quick verifyÊThis parameter instructs TapeWare to be certain that every file backed up onto the tape is in readable condition. It does not check to see if the data is correct, only that the data stored on the tape (incorrect or not) can be read. While selecting this option can save time, it is nonetheless not recommended.

No verifyÊThis parameter instructs TapeWare to skip the verification step. It is not recommended.

### Device

The Device option specifies which tape drive or other removable media device TapeWare will use to run the backup job.

By default, TapeWare sets this parameter to the network container. When running the job, TapeWare will use whatever devices it finds on the network. If there is only one device in your database zone or if you only have permissions to one device, there is no reason to change this parameter.

However, if there are several devices on your network and you need to select a specific device, specify which device the job should use by selecting it from the Device list. (If a machine has only one device, you need not select the device, only the machine.)

Note that TapeWare will automatically attempt to use multiple devices whenever there are multiple backup streams. Using multiple devices can speed up a backup job tremendously. However, you may wish a job to use only one specific device or exclude a particular backup device. Use the Device list to specify which devices a job should use.

For further information on using multiple streams, see “Maintaining the Flow of Data,” Chapter 10, and “Storage Tab,” Chapter 12..

Note If you are using an autoloader, the physical machine will have two (or more) drivers associated with it: the device driver (or drivers) and the autoloader driver. You may specify either driver in the Device list. If you specify the autoloader driver, for example, TapeWare will automatically select backup devices in the autoloader. On the other hand, if you select the device driver, the autoloader will also be automatically selected.

If you wish to use a device that is not shown in the Device list, click the AddÉ button and select the new device from the Browse window.

### Media

The Media option specifies the database folder in which the tape or other removable media are stored in the database. TapeWare will look here for media to use with this job.

The default folder is the current User/Group folder. If you wish to use media from another folder, specify which folder by selecting it from the Media list.

If you wish to use a folder that is not shown in the Media list, click the AddÉ button and select the new folder from the Browse window.

### Media PasswordÉ

When a job creates new media, you can assign that media a password. A password prevents the media from being imported into another TapeWare database and can be an important part of your overall security plan.

To assign new media your job will create a password, click the Media PasswordÉ button and type and confirm your password.

Note that passwords can only be assigned when media is formatted. Additionally, media passwords are only required when importing media.

### Change Mode

This option determines what action TapeWare will perform when it fails to find the media it was expecting to use for a job. When TapeWare runs a job, if the job uses specific media, TapeWare scans the network for devices with that media. If it does not find the media it expects, its response is determined by the Change mode parameter.

Skip deviceÊThis parameter instructs TapeWare to skip the designated device and look for other devices on the network with the proper media. TapeWare continues to scan for the correct media until it is found. As a result, the job may never run if the correct media is not found.

Force to appendÊThis parameters instructs TapeWare to append data to whatever media it finds in the designated backup device. If it cannot find the correct media, TapeWare appends data to whatever media is available. This option will insure that the job runs, if the media contains enough room to complete the job.

PromptÊThis parameter instructions TapeWare to continue to scan for the expected media and to send an alert warning that the proper media has not been found. This option will not allow a job to run with any other media except with the expected media. Additionally, this option will not search for another device that might contain the proper media.

## Backup Options Automatically Updated

The parameters shown on the Options tab fall into two categories: first, parameters that are updated automatically when TapeWare runs a scheduled automatic rotation job (built-in or custom), but specified manually in unscheduled and manual rotation jobs; and second, parameters that are always specified manually by the user.

For further information, see “Forcing Scheduled Jobs to Run,”
 Chapter 8.

Recall from the previous chapter that TapeWare jobs can be scheduled in four distinct ways, either as Not Scheduled, Manual Rotation, Automatic Rotation with Built-in Schedule, or Automatic Rotation with Custom Schedule.

For further information, see “Backup Job Scheduling Concepts,” Chapter 6.

When an automatic rotation job is scheduled, the job is placed on the Queue tab with an indication of the date and time the job is scheduled to run. When TapeWare runs these scheduled jobs on the Queue tab, it automatically updates three of the parameters on the Options tab: Backup mode, Write mode, and New media name.

Note however that TapeWare does NOT automatically update these fields when you manually ÒforceÓ a scheduled job to run. For example, when TapeWare automatically runs a scheduled backup job on a Monday, it changes (updates) the Backup Mode from Full to Incremental. But when this job is ÒforcedÓ to run before its scheduled time, TapeWare does not automatically update these fields.

For further information on how forcing a job to run effects job parameters, see “Forcing Scheduled Jobs to Run,” 
Chapter 8.

Note additionally that when you run an unscheduled or manual rotation job, TapeWare always uses the parameters selected by the user.

# Advanced Options

The Advanced OptionsÉ button on the Options tab allows the user to specify certain parameters controlling how files are stored on media. In particular, these options specify whether the data on the media is stored in the same form as it was transmitted across the LAN. TapeWare can either store the data in a format specific to a particular network platform or in a generic format. Similarly, TapeWare can store all of the data it receives, or filter out some of the data used by particular network platforms or operating systems.

The advanced options control how data transmitted across the LAN is stored on media. It can be used to filter out certain data and to change data stream format.

In general, the default values should be used. These options are provided only for advanced users who need to customize their backup jobs for unique circumstances. These options might be used in one of two circumstances: when transferring data from one network platform or OS to another; or when demands on network traffic require that a backup job be run as quickly as possible. Unless you have specific needs that require changes to the advanced options, leave the default values unchanged.

These options are applicable to both backup and restore jobs. Note that both job types can filter out certain data, such as security information. However, restore jobs cannot add data that was not originally stored on the media.

### Native data streams formatÊ

Different network software transmits data across the network to TapeWare in different formats. In particular, Windows NT and NetWare use different data stream formats. If you are going to share data from one LAN platform to another, the data should be stored on media in a common data format, not in the native data streams format.

Check this option when you do not plan to share data between different LAN platforms. When this option is checked, TapeWare generally runs backup jobs more quickly.

Clear this option when you plan to share data between different LAN platforms, such as from a Windows NT server to a NetWare server.

File compression and security are also issues to consider when checking this option. If files are compressed, such as on a NetWare platform, when this option is checked, the files are backed up in compressed mode; when the option is unchecked, the files are decompressed before backing up. In this case, clearing this option increases the time needed to run the backup job. Similarly, when this option is checked, TapeWare backs up all security information that network software (such as Windows NT) includes in the data stream. If the option is unchecked, TapeWare uses a generic format that removes the security information. Again, this increases the time needed to run the backup job.

### Parent SecurityÊ

When checked, TapeWare includes NetWare and Windows NT parent security information, that is, the access control list and trustee information that controls who can see and modify the directories. If this option is unchecked, TapeWare filters out the parent security information that it receives from the network during a backup job and that it would transmit across the network during a restore job.

### Child SecurityÊ

When checked, TapeWare includes NetWare and Windows NT child security information, that is, the access control list and trustee information that controls who can see and modify the files. If this option is unchecked, TapeWare filters out the child security information that it receives from the network during a backup job and that it would transmit across the network during a restore job.

### Volume restrictionsÊ

NetWare controls the maximum amount of space a user can use on a volume. When this option is checked, TapeWare includes this information about the volume in the backup media. If this option is unchecked, TapeWare filters out the volume restrictions that it receives from the network during a backup job and that it would transmit across the network during a restore job.

### Space restrictionsÊ

NetWare controls the maximum amount of space a directory can use on a volume. When this option is checked, TapeWare includes this information about the directories in the backup media. If this option is unchecked, TapeWare filters out the space restrictions that it receives from the network during a backup job and that it would transmit across the network during a restore job.

### OS/2 extended attributesÊ

When this option is checked, TapeWare includes the extended attributes for files and directories on workstations or file servers running operating systems that employ extended attributes, such as IBMÕs OS/2 operating system. Mac OS, Windows 95, and Windows NT also use extended attributes and this option also affects file servers and workstations running these operating systems. If this option is unchecked, TapeWare filters out the extended attributes that it receives from the network during a backup job and that it would transmit across the network during a restore job.

### Macintosh FinderÊ

When this option is checked, TapeWare includes the Finder information for files and directories on workstations or files servers running the Macintosh operating system. If this option is unchecked, TapeWare filters out the Finder information that it receives from the network during a backup job and that it would transmit across the network during a restore job.

### NFS informationÊ

When this option is checked TapeWare includes NFS information for files and directories on workstations or file servers running a UNIX version of NFS. If this option is unchecked, TapeWare filters out the NFS information that it receives from the network during a backup job and that it would transmit across the network during a restore job.

### Object ownerÊ

When this option is checked, TapeWare includes the object owner information for files and directories on workstations or file servers running NetWare. If this option is unchecked, TapeWare filters out the object owner information that it receives from the network during a backup job and that it would transmit across the network during a restore job.

### Maintain last access dateÊ

The access date is the last time a file or directory was opened or modified. Because TapeWare opens the file, the access date changes to the date and time the backup job was run. You may not wish this date to be changed, however. For example, you may want to maintain the last access date in order to know if a file should be archived. Checking this option preserves the last access date.

When this option is checked, TapeWare reads the access date before opening the file. After the file is backed up and closed, TapeWare restores the original access date to the file. In general, if you use access date information to manage files on your LAN, check this option.

When this option is unchecked, TapeWare does not restore the original access date after backing up and closing the file. Clearing this option results in backup jobs running faster.

# Restore Job Options

There are fewer and simpler options for restore jobs. In general, these options are similar to options for backup jobs.

### Number of Passes

This parameter determines how many times TapeWare will attempt to access a file on the network.

Sometimes when TapeWare is attempting to access a file, the file may already be open, that is, currently being used by another user. Each time TapeWare attempts to open a file is called a pass. When TapeWare is unable to access a file on the first pass, it attempts to back up that file on subsequent passes. For restore jobs, TapeWare attempts to open the file as many times as there are passes. When the file cannot be opened on the last pass, that file fails the restore job.)

The default value is 5. If your historical usage shows that many files are open while a restore or verify job is being run, set the value to a higher number. This will increase the number of times TapeWare attempts to open a file.

### Delay Between Passes

This determines how many seconds TapeWare waits before attempting the next pass. If your historical usage suggests that many files are being opened on the last pass, consider increasing this parameter to a higher number.

### Log options

The Log options are either None, Log only failed, Log only completed, or Log all.

TapeWare keeps a log of which files are restored while running a restore job. After a job is run, you can view or print the log to see if the job was completed successfully. The default value is Log only failed, which writes in the log any files which were not successfully restored. This information can be vital for checking to see if your job ran successfully.

NoneÊThis parameter instructs TapeWare to not keep a log of the job as it runs.

Log only failedÊThis parameter instructs TapeWare to log the name of any file selected but, for some reason, not restored. Use this option to check if a job is running correctly.

Log only completedÊThis parameter instructs TapeWare to log the name of any file selected that was successfully restored.

Log allÊThis parameter instructs TapeWare to log the name of every file selected and whether or not that file was successfully restored. You can use this option to be certain that a job is running correctly as you planned.

### Device

The Device option specifies which tape drive or other removable media device TapeWare will use to run the job.

By default, TapeWare sets this parameter to the network container. When running the job, TapeWare will use whatever device it finds on the network. If there is only one device in your database zone or if you only have permissions to one device, there is no reason to change this parameter.

However, if there are several devices on your network and you need to select a specific device, specify which device the job should use by selecting it from the Device list. (If a machine has only one device, you need not select the device, only the machine.)

If you wish to use a device that is not shown in the Device list, click the AddÉ button and select the new device from the Browse window.

### Advanced Options

For restore jobs, you may also specify advanced options. Generally, these options work just like they do for backup jobs, that is, as filters that exclude certain types of data.

However there are some changes to the advanced options of a restore job from those of backup jobs.

n	The Native data streams format does not function with restore jobs. TapeWare does not change the data stream format from its stored format. This option is important for backup jobs when you wish to share data from one network platform to another.

n	The Maintain last access date does not function with restore jobs. TapeWare does not change the access date of a file when restoring it, but lets the network or operating system specify that date.

n	Note that the data filters, such as security information and directory attributes, cannot add data that was not originally stored on the media. For example, if when the original backup job was run, the Volume restrictions box was unchecked, checking the box when running the restore job will have no effect. Because the volume restrictions are not stored on the media, they cannot be restored.

# Verify Job Options

There are fewer and simpler options for verify jobs. In general, these options are similar to options for backup jobs.

### Number of Passes

This parameter determines how many times TapeWare will attempt to access a file on the network.

Sometimes when TapeWare is attempting to access a file, the file may already be open, that is, currently being used by another user. Each time TapeWare attempts to open a file is called a pass. When TapeWare is unable to access a file on the first pass, it attempts to back up that file on subsequent passes.

The default value is 5. If your historical usage shows that many files are open while a verify job is being run, set the value to a higher number. This will increase the number of times TapeWare attempts to open a file and may result in fewer files failing to be verified.

### Delay Between Passes

This determines how many seconds TapeWare waits before attempting the next pass. If your historical usage suggests that many files are being opened on the last pass, consider increasing this parameter to a higher number.

### Log options

The Log options are either None, Log only failed, Log only completed, or Log all.

TapeWare keeps a log of which files are verified while running a verify job. After a job is run, you can view or print the log to see if the job was completed successfully. The default value is Log only failed, which writes in the log any files which were not successfully verified. This information can be vital for checking to see if your job ran successfully.

NoneÊThis parameter instructs TapeWare to not keep a log of the job as it runs.

Log only failedÊThis parameter instructs TapeWare to log the name of any file selected but, for some reason, not verified. Use this option to check if a job is running correctly.

Log only completedÊThis parameter instructs TapeWare to log the name of any file selected that was successfully verified.

Log allÊThis parameter instructs TapeWare to log the name of every file selected and whether or not that file was successfully verified. You can use this option to be certain that a job is running correctly as you planned.

### Verify Mode

The Verify mode is either Full verify, No verify, or Quick verify.

When TapeWare runs a verify job, it checks to see if the data on the media is readable and whether or not it matches data from the original source (that is, from workstations or file servers).

Full verifyÊThis parameter instructs TapeWare to compare every file on the media with the original file from the workstation or file server. This default value is strongly recommended.

Quick verifyÊThis parameter instructs TapeWare to be certain that every file backed up onto the media is in readable condition. It does not check to see if the data is correct, only that the data stored on the media (incorrect or not) can be read. While selecting this option can save time, it is nonetheless not recommended.

### Device

The Device option specifies which tape drive or other removable media device TapeWare will use to run the job.

By default, TapeWare sets this parameter to the network container. When running the job, TapeWare will use whatever device it finds on the network. If there is only one device in your database zone or if you only have permissions to one device, there is no reason to change this parameter.

However, if there are several devices on your network and you need to select a specific device, specify which device the job should use by selecting it from the Device list. (If a machine has only one device, you need not select the device, only the machine.)

If you wish to use a device that is not shown in the Device list, click the AddÉ button and select the new device from the Browse window.