NAT32

Connecting Private LANs to the Internet

Reference Manual

Windows NT Version

Ó Dipl.-Ing. Christopher Chlap (chrisc@fir.canberra.edu.au)

Legal Notices

This document describes the NAT32ntÒ software package Version 4.0.The current version of the package can be downloaded from:

http://willow.canberra.edu.au/~chrisc/nat32.html

Information in this document is subject to change without notice and the author assumes no liability for errors and omissions.

Redistribution

The above software may be freely redistributed, provided that:

1. You do not alter the contents of the distribution file in any way.

2. You do not charge for the package except to cover handling costs.

Software License

1. The author grants you the right to install and test the software on one computer on

one network.

2. The software is provided "as is" and all liability for any damages resulting from the

use of the software is excluded.

3. As supplied, the software will run for 60 minutes to allow rigorous evaluation. If you

use the software for longer than 60 minutes you must register the software by     paying the registration fee to the author and agreeing to the Registration License Agreement as outlined elsewhere in this document.

Canberra, Australia.								          18. April, 1998.

Table of Contents

1. Introduction

1.1 Windows NT IP Routing Limitations

1.2 The Private LAN Address Problem

1.3 Proxy Servers

2. The NAT32nt Package

2.1 A Sample Configuration

2.2 Network Address Translation

2.3 Port Mapping

3. Installing NAT32nt

4. Configuring NAT32nt

4.1 Network Interface Configuration

5. Fixing Configuration Problems

6. Registration

# 1. Introduction

With the release of Windows for Workgroups 3.11 in 1993, many PC users began installing Ethernet Adapters in their home and office computers in order to use Microsoft Networking for sharing  resources such as files, disks, CD ROMS, fax machines and printers. Users quickly became accustomed to the benefits of networking and the resulting increase in demand for Ethernet Adapters soon caused significant price drops, culminating in a price of less than $50.00 per connection today.

Two years later, Microsoft released Windows 95 and many users began using TCP/IP on their private networks, mainly in the form of Dial-up Networking, which, in the Windows 95 implementation, allowed only a single PC to access the Internet. Naturally, those users soon began looking for ways of sharing the Dial-up Networking resource so that all of their networked PCs could have Internet access. Full details of how this can be achieved under Windows 95 can be found in the Windows 95 version of this document.

With the introduction of cheap, powerful Pentium systems, many users switched to the Windows NT Operating System and also upgraded to fast Cable Modem services to achieve Internet connectivity. Now it became even more important to be able to share the high-speed Internet connection with other machines on a private local area network.

The most obvious way of achieving this is to somehow make the machine with the Internet connection act as a gateway. That machine would then forward packets belonging to the other machines over its Ethernet or Cable Modem connection to the Internet.

# 1.1 Windows NT IP Routing Limitations

Many users had discovered that the Windows NT IP software supports IP Routing. One only had to check the appropriate box  in the TCP/IP Properties Dialog using the Control Panel and, theoretically at least, Windows NT would route between the private LAN and the Internet. However, users of private LANs quickly discovered that routing only works correctly if the machines on their private LAN use registered IP addresses.

# 1.2 The Private LAN Address Problem

Most home and small-business networks use private IP addresses as defined in RFC 1597, and packets using those addresses cannot be routed within the Internet because they are not globally unique. The only registered IP address which most users have at their disposable is the IP address supplied by their Internet Service Provider (ISP) for use on the Cable Modem interface.

# 1.3 Proxy Servers

One possible solution to the problem is to run a Proxy Server on the machine with the Internet connection. Applications on the other machines then no longer communicate directly with Internet hosts. Instead, the applications are reconfigured so that they communicate only with the local Proxy, which then performs the required transfers on their behalf. While this solution does work, it has two major drawbacks:

1. All networking applications on all hosts must be reconfigured to use the Proxy Server.

2. The Proxy Server must be reconfigured whenever new applications become available.

In spite of these limitations, several Proxy Server Packages (such as WinGate) are available and are in widespread use. However, many users would prefer a better solution, and are probably reading this document in the hope of finding one.

# 2. The NAT32nt Package

NAT32nt is a WIN32 application which allows multiple hosts on a private LAN to transparently share a single connection to the Internet. That connection is typically a Cable Modem connection but NAT32nt can also be configured to act as a gateway between a private Ethernet and a registered Ethernet. In the latter case, only a single registered IP address is required on the registered Ethernet.

NAT32nt is an enhanced IP Router and runs on just a single machine on the private LAN.  It uses one IP Address on the private LAN and all machines on that private LAN must be configured to use that IP Address as the gateway.

# 2.1 A Sample Configuration

The following diagram shows a private network consisting of three machines, X, Y and Z. Machine X is a Windows NT PC with an Internet connection via an NDIS3 Network Interface Controller. NAT32nt is to be installed on machine X so that X, Y and Z can communicate over the Internet.

Host X

Apps

2

NAT32nt	         Winsock

3                   	1	  	   Host Y			 Host Z

Network Adapter			192.168.1.2		           192.168.1.3

1 = 192.168.1.1	              Private LAN

2 = 192.168.1.10

3 = 137.92.100.61

Internet    GW 137.92.100.1

Fig. 1:  NAT32nt Sample Configuration

NAT32nt runs as an application on Host X and is actually a self-contained TCP/IP sub-system which accesses the Network Adapter using a custom device driver. The connection to the Internet is accessed by the same custom device driver, which must be NDIS3 compatible. Network applications (e.g. Winsock Apps) running on X, Y and Z interact with NAT32nt via TCP/IP, using its IP address 192.168.1.10. NAT32nt is implemented as a completely independent protocol stack, acting as an Internet gateway for all machines on the private LAN.

Network applications running on Host X continue to use the standard Microsoft Winsock API. The only difference is that Windows NT is configured to use NAT32nt as a gateway. This is done either by using the Network Applet from the Control Panel or by simply adding a permanent default route which points to NAT32nt's IP address 192.168.1.10.

TCP/IP protocol stacks running on the other hosts (Y and Z) are also configured to use NAT32nt as the gateway. No alteration or reconfiguration of any application is required.

Once you have understood that your private network now has a gateway, and that the address of that gateway is NAT32nt's IP address, you will find NAT32nt configuration to be quite straightforward. A typical configuration run will be described in Section 4, using the above configuration as an example.

# 2.2 Network Address Translation

Network Address Translation is described in RFC 1631. Basically, NAT is a technique which replaces the IP Source Address of outgoing packets from a private LAN with a registered IP address before the packets are actually sent to the Internet. For packets arriving from the Internet, the substitution is reversed before they are forwarded to the destination host on the private LAN.

NAT is best implemented in gateways. That way, network applications running on the computers on the private LAN are completely unaware of the translation and no reconfiguration of those applications is required. The TCP/IP protocol stacks on all machines must be configured to use the NAT machine as the gateway, but this can be done quickly and easily using the Windows Control Panel.

RFC 1631 NAT does not allow more than one private IP address to be mapped to a single registered IP address, so the NAT32nt package also supports a technique called Port Mapping.

# 2.3 Port Mapping

Port Mapping is a technique which allows multiple hosts on a private LAN to share a single registered IP address. Port Mapping works by keeping track of UDP and TCP source port numbers and by changing those port numbers in all outgoing packets. Of course, the outgoing packets are also address mapped prior to transmission. The port mappings are stored in a table so that the original host address and port number of incoming packets can be restored.

Port Mapping is also best implemented in gateways. That way, networking applications remain totally unaware of the translation. The technique works fine with all "well-behaved" TCP and UDP applications such as WWW, TELNET and many others. However, it also works with protocols such as FTP, which transfer low-level information such as IP addresses and Port Numbers within the data field of TCP segments. The requirement is that the FTP PASV mode be turned on in your FTP Client software.

# 3. Installing NAT32nt

Download the latest NAT32nt.ZIP file from:

http://willow.canberra.edu.au/~chrisc/nat32.html

Store NAT32nt.ZIP in a directory of your choice (e.g. c:\nat32nt) and then type:

pkunzip -d nat32nt.zip

in that directory to unpack the distribution. You should then have the following files in your NAT32nt directory:

c:\nat32nt

readme.txt

p32.dll

c32.dll

p32.exe

oemsetup.inf

packet.sys

save.bat

startup

various pif files

one shortcut file

c:\nat32nt\man

various man pages

c:\nat32nt\doc

nat32nt.doc

You can erase the NAT32nt.ZIP distribution file because it is no longer needed. If you ever decide you no longer need NAT32nt, just drag the entire NAT32nt directory to the Recycle Bin and remove the PACKET.SYS driver using the Control Panel.

NAT32nt requires no installation, nor does it modify any Registry variables. However, you do need to install a custom device driver called PACKET.SYS in order to give NAT32nt access to Network Adapters.

Installing PACKET.SYS

- Log on to your NT 4.0 system as Administrator.

- Install your Network Adapter(s). Be SURE all Network Adapters are working correctly

before continuing.

- Select Control Panel/Network/Protocol and Add the driver. Click on Have Disk and specify  \nat32nt as the path. Then click OK to complete the installation.

- Reboot your NT system.

If all is well, the Packet Driver should now be bound to each Network Adapter and you can proceed to the next step: Windows NT TCP/IP Configuration.

Windows NT TCP/IP Configuration

Open the Windows Control Panel, select Network, then select the TCP/IP protocol on your Ethernet Adapter. Then click Properties, IP Address and specify the IP address 192.168.1.1 and the Subnet Mask 255.255.255.0. Now click Advanced and add the Gateway 192.168.1.10.

Now click OK and select the DNS tab. Add the Name Server address which you obtained from your Internet Service Provider.

Finally, click the Routing tab and turn OFF IP Forwarding.

Click OK and Windows will request a reboot. You must reboot before continuing.

While your machine is rebooting, go to the other machines on your private network and also configure their TCP/IP stack to use 192.168.1.10 as the Gateway and to use the same Name Server you specified above.

You can now begin configuring NAT32nt.

Note that Windows lets you add any address you like to the Gateway List. For NAT32nt to work properly, you must specify an address which is on the same network as the Windows TCP/IP protocol stack. For the sample network in Fig. 1, 192.168.1.10 is a valid address, but 192.168.2.10 would be an invalid address.

At this point, let us summarise what we have done and what we hope to ultimately achieve:

- We have installed an Ethernet Adapter on our system

- We have installed a special driver PACKET.SYS which allows standard WIN32 applications (NAT32nt) to access that Adapter.

- We have configured the TCP/IP Protocol to use a private IP address 192.168.1.1 and a Gateway 192.168.1.10. We have also specified a DNS address.

- At this point, the Gateway 192.168.1.10 does not actually exist. Instead, our NAT32nt application will later act as that Gateway. In order to do this, we will need to configure a NAT32nt logical interface which responds to this address. When a Windows Winsock application like Netscape tries to access an Internet site, it will send its packets to the 192.168.1.10 gateway and NAT32 will then forward the packets to the Internet via a second logical interface, which we have yet to configure.

- Therefore, this second NAT32nt logical interface must be configured to use the IP address and Gateway address of our ISP. Note that in the case of a Cable Modem service, the gateway is physically accessed via the same Ethernet Segment which we are using to access our other PCs. That Ethernet Segment is actually carrying packets to/from our ISP Gateway AS WELL AS packets to/from our private network.

- Note that we can’t use Windows NT to do all this directly because:

- Windows NT does NOT allow more than one TCP/IP protocol stack to be bound to an Adapter.

- Windows NT does NOT support IP Address Translation or Port Mapping

To configure the NAT32nt Logical Interfaces, we simply run the P32 program, specifying that two logical interfaces are to be used. The first time P32 runs, it will present you with two Network Configuration Dialog Boxes. The entire configuration process is described in detail in Section 4.

# 4. Configuring NAT32nt

When you run NAT32nt for the first time several configuration dialog boxes appear. These dialog boxes contain default values, many of which will be suitable for your computer. However, some values may need to be altered, so you should gather the following information before you begin:

1. The IP address of your machine on your ISP's network.

2. The Network Mask used by your ISP.

3. The IP address of the Gateway on your ISP's network.

4. The IP address of a Name Server.

Now open a Windows Console and change to the directory in which you have installed NAT32nt.

Start NAT32nt by typing the following command: p32 2

This command will start NAT32nt and request it to use two Logical Interfaces.

A configuration dialog box will appear for each NAT32nt Interface.

# 4.1 Network Interface Configuration

The following dialog box is used to configure a NAT32 Network Interface:

The first field, Name, is the Windows NT Network Adapter Name.

On my system, \Device\Packet_EE161 is an Ethernet Adapter, and NAT32nt is to use that Adapter to communicate with the local LAN.

The IPaddr field is used to specify the IP address which this Interface is to have on the private LAN. If your Microsoft TCP/IP configuration is correct, this address should be the Gateway Address you specified during that configuration.

The Mask field is used to specify the Network Mask. In the case of a Class C IP address (as in this example), the default value of 255.255.255.0 is correct and you can leave this field unaltered. The mask to use for each class is as follows:

Class A		255.0.0.0

Class B		255.255.0.0

Class C		255.255.255.0

Again, if you installed your Microsoft TCP/IP stack correctly, the mask value shown should be correct.

The Gateway field is used to specify the IP address of a gateway which is directly connected to this Adapter. Because there is no other gateway on the 192.168.1.x subnet, this value should be 0.0.0.0.

Note that even though the ISP Gateway is on the same physical LAN, it is on a different logical network (137.92.x.y in the above example). This second logical network is handled by a second NAT32nt Logical Interface, which will be configured later.

The Name Server field contains the IP address of your ISP's Name Server. Enter that address here. If you don't know the IP address of a Name Server, you may leave this field unaltered. NAT32nt will then use a Name Server located in Canberra, Australia. To speed  things up, I strongly recommend that you enter the address of a Name Server closer to home.

The Microsoft Ipaddr field is determined by reading the Windows NT Registry, so the suggested value should be correct.

The other fields in the Dialog Box should be left unaltered.

When you are sure that all fields are correct, click OK and a similar dialog box for the second Logical Interface will appear:

In this dialog box, select the same Adapter Name as you used for the first Interface. Then click the Use DHCP checkbox. NAT32nt will then attempt to use the DHCP Protocol to configure this Interface.

Note that if your ISP does not support the DHCP protocol, you must enter all the values manually. For the example configuration, the values should be set as follows:

Note that the Primary Interface checkbox must be set. When you are sure that all fields are correct, click OK.

NAT32nt is now configured and the NAT32nt gateway is running. All of your computers can now communicate with any host on the Internet. The NAT32nt Console can now be minimised or hidden.

# 5. Fixing Configuration Problems

Unfortunately, configuration can sometimes go wrong and you may be faced with the task of finding  the problem. The following set of tests should help you track down problems quickly.

But first:

Be sure you have configured the TCP/IP stacks on all machines to use the NAT32nt IP address as the gateway. In Sample Configuration 1, that gateway address must be 192.168.1.10. Also be sure you have specified a Name Server correctly.

If you made an error during the configuration process, terminate NAT32nt by typing exit at the NAT32nt command prompt. Then erase the files netcfg* in your NAT32nt directory and restart NAT32nt. You will then be presented with the Network Configuration Dialog Boxes.

If you are sure that you have configured everything correctly and you still can't get things to work, perform the following tests from the NAT32nt Console:

Test 1:	Type ping 192.168.1.1 (the address of the Microsoft TCP/IP stack)

If this ping fails, you can be sure that the address which you told NAT32nt

to use is not on the same subnet as your private LAN. Reconfigure NAT32nt

and then repeat this test.

Test 2:	If Test 1 was successful, try to ping other computers on your private LAN.

If any machines can't be reached, check that their IP addresses are valid.

Correct the problem and repeat this test.

Test 3:	If Test 2 was successful, try to ping 137.92.11.125.

If the ping fails, it means that NAT32nt can’t communicate with the Internet. Check the configuration of the NAT32nt Primary Interface. Correct any bad settings.

Test 4:	Now ping fir.canberra.edu.au. This is the host you contacted in Test 3. If the

name can't be resolved, your Name Server IP address is incorrect. Reconfigure

NAT32nt.

Test 5: Now type ipmapa. You should receive the following response:

Mapping 192.168.1.0 to 137.92.100.61 on interface 3

Port mapping: ON

This response indicates that NAT32nt is mapping your Microsoft TCP/IP subnet addresses to

your ISP-provided Internet address (137.92.100.61) on the second interface (IFN 3).

If you didn't receive this response, you should erase netcfg* from the Windows command

line and then reconfigure NAT32nt.

If you still cannot get NAT32nt to work, please do not hesitate to email me. Send as many details as you possibly can, including the contents of the netcfg files, ALL Microsoft TCP/IP settings, ALL adapter types, a list of which tests passed and which failed, and anything else you consider relevant. The more information you send me, the more likely I can solve your configuration problems.

# 6. Registration

The unregistered version of NAT32nt runs for 60 minutes to allow you to rigorously test its features. When 55 minutes have elapsed, a pop-up message box will remind you that NAT32nt will terminate in 5 minutes.

If you decide to continue using NAT32nt, please register by sending the author your email address and US $40 (preferably cash but I will accept Australian currency cheques for AUD $60.00):

Christopher Chlap

10 Edman Close

FLOREY, A.C.T. 2615

AUSTRALIA

Registration can also be performed by sending Credit Card details to the above address. Note that Credit Card orders can not be accepted via email.

Registration buys you the following:

1. A Registration Number which retains validity for all future upgrades to this product.

2. Full technical support per electronic mail, TELNET and SNMP.

3. The right to install NAT32nt on one computer on one network.

4. The right to use NAT32nt services from any number of computers on that network.

By registering, you explicitly agree to the following terms and conditions:

1. You will not disclose your Registration Number to any other party.

2. You will not use the Registration Number on any other computer.

3. You will not hold the author responsible for any loss or damage resulting from the use of the

software.

4. You have tested the software thoroughly and will not request a refund under any circumstances.

The software is supplied "as is" with absolutely no warranty of any kind. Because you had the opportunity to install and test the software before registering, the author cannot be held responsible for any loss or damage arising from the use of this software. In particular, the registration fee is not refundable under any circumstances.