1. FTP Clients

FTP clients must be set to use the PASV mode. Nearly all clients support this mode.

Notable exceptions:	Microsoft Command Line FTP

2. FTP Servers

FTP servers normally listen at Port 21.

FTP servers must support the PASV mode. This is because ALL WEB Browsers use the passive mode for FTP transfers.

NAT32 mapping: 	ppmap add tcp 21 <private IP address> <port>

Example: To map incoming FTP connection requests to Port 21 on machine 192.168.1.2:

ppmap add tcp 21 192.168.1.1 21

3. WEB Servers

WEB servers normally listen at Port 80. If other port numbers are used, use the following format in your

URLs:

http://willow.canberra.edu.au:8080/~chrisc/nat32.html

NAT32 mapping:	ppmap add tcp 80 <private IP address> <port>

Example: To map incoming HTTP connections to port 1080 on machine 192.168.1.3:

ppmap add tcp 80 192.168.1.3 1080

4. X-Windows Servers

X-servers normally listen at Port 6000. To contact an X-server from an X-client, the following command can be used on a machine running SUN OS:

xterm -display host:0.0

This command would contact the specified host via Port 6000.

NAT32 mapping:	ppmap add tcp 6000 <private IP address> port

The following command could be used to contact a second X-server on your private LAN:

xterm -display host:1.0

NAT32 mapping:	ppmap add tcp 6001 <private IP address> port

Example: To map one incoming connection to 192.168.1.2 and another to 192.168.1.8:

ppmap add tcp 6000 192.168.1.2 6000

ppmap add tcp 6001 192.168.1.8 6000

5. VDO Live Video

VDO live video streams can be obtained via the HTTP protocol, in which case no particular mappings are necessary. However, for better performance, the player can request delivery via a UDP Port (typically 7000), in which case the following mapping should be used

NAT32 mapping:	ppmap add udp 7000 <private IP address> port

Example: To map incoming VDO datagrams to port 7000 on machine 192.168.1.5:

ppmap add udp 7000 192.168.1.5 7000

6. Mapping ALL unsolicited incoming UDP Traffic

Incoming, unsolicited UDP datagrams on all ports can be forwarded as broadcasts on a specified interface. Note that explicit port mappings as illustrated above have precedence over the following mechanism:

NAT32 mapping:	ipmapb ifn

Interface numbers are assigned as follows:

ifn	Interface

1	PPP connection

2	Adapter 1

3	Adapter 2

4	Adapter 3

5	Adapter 4

This technique can also be used for RealAudio and various other applications which send UDP packets to dynamically assigned port numbers.

7. Miscellaneous Server Issues

A Windows machine may have more than one TCP/IP protocol stack installed, even in cases where only a single Network Adapter is present. In these cases, servers must be explicitly told on which IP address they are to listen. Check the configuration options of your server software.

Example: WAR FTPD lets you specify both the IP address and the PASV mode. It has excellent security features too. It is freeware and can be downloaded from your nearest TUCOWS mirror.

8. NetMeeting and other H.323 Applications

All H.323-based applications are notoriously difficult to support on private LANs and LANs connected to the Internet through firewalls. However, an Australian software company, EQUIVALENCE, supplies a low-cost package which allows H.323 apps to be used on private networks and networks behind firewalls. This package has been tested with NAT32 and works perfectly. It allows any machine on your private LAN running NetMeeting (or similar) to contact (an be contacted by) any machine on the Internet.

You can achieve limited NetMeeting functionality by following the guidelines in the following article:

http://support.microsoft.com/support/kb/articles/Q158/6/23.asp

9. Security Issues

When a private LAN is connected to the Internet, all computers on that LAN become reachable from the outside world. However, the following basic rule applies:

If a machine on your private LAN has NO servers running, outside machines have

ping access only.

A problem can arise for users who have Windows File and Printer Sharing enabled. This automatically enables a server which listens for incoming NETBIOS requests, so our basic rule is violated.

NETBIOS uses UDP and TCP to support File and Printer sharing. Participating machines broadcast certain packets on the local network. Normally, these broadcasts are NOT propagated by routers, so outside access should not be possible.

To determine whether or not these NETBIOS broadcasts can reach your machines, you need to know a few details of your ISP's configuration.

A typical ISP Setup

Gateway					  Remote Access

Box

....		Modems, PPP

(not shown)

Internet			    PC x		        NAT32

Private LAN

In the above setup, your PC running NAT32 is connected to your ISP via a Dial-Up PPP link. Windows will NOT propagate NETBIOS broadcasts to the Remote Access Box, so no problems arise. If, for some malicious reason, an outside user were to get NETBIOS broadcasts coming in to the NAT32 box, NAT32 will block them, they will not reach your private LAN.

This only applies if NAT32 has exclusive access to the Dial-Up Adapter. If you share that Adapter with Microsoft TCP/IP, then Windows will reply to incoming NETBIOS broadcasts and the machine running NAT32 will appear in the Network Neighbourhood of the machine which sent the NETBIOS packet. You must use strong passwords on all resources to guard against this.

NAT32 and Windows could be configured so that ALL traffic goes through NAT32. But I do not recommend this configuration because the machine running NAT32 then no longer has unmodified access to the Internet.

A typical Cable Modem Setup

Cable Modem LAN

Gateway

Internet			    PC x		        NAT32			User X

Private LAN

In this configuration, all subscribers to the service are effectively joined into one big LAN. All machines connected to the service use the same subnet address. Therefore, a NETBIOS broadcast from User X in the diagram will also reach your NAT32 box. But again, NAT32 will not pass these packets to your private LAN, so you are safe.

This only applies if NAT32 has exclusive access to the Cable Modem Ethernet Adapter. If you share that Adapter with Microsoft TCP/IP, then Windows will reply to incoming NETBIOS broadcasts and the machine running NAT32 will appear in the Network Neighbourhood of all the other machines on that Cable Modem segment. You must use strong passwords on all resources in this case.

Again, NAT32 and Windows could be configured so that ALL traffic goes through NAT32. But I do not recommend this configuration because the machine running NAT32 then no longer has unmodified access to the Internet.

Running TCP or UDP servers on your Private LAN:

Many users need to make machines on their private LAN accessible from the Internet. In order to do this, you must explicitly allow connection requests on certain ports to be forwarded by NAT32. Several examples of this were given in preceding sections of this document.

The basic rules to remember are:

Assign strong passwords to all services offered by your machine.

Change these passwords regularly.

10. NAT32 Filters

NAT32 Version 5.5 will support filtering functions. Use these functions to block any incoming traffic which you consider harmful on your private LAN.

(to be continued)