NAT32

Connecting Private LANs to the Internet

Installation Guide

Dial-Up Networking

Windows 98 Version

Legal Notices

This document describes the NAT32Ò software package Version 5.5.The current version of the package can be downloaded from http://willow.canberra.edu.au/~chrisc/nat32.html

Author

Dipl.-Ing. Christopher Chlap (chrisc@fir.canberra.edu.au)

Copyright Notice

(C) 1998 Christopher Chlap. All rights reserved.

Redistribution

This software may be freely redistributed, provided that:

· You do not alter the contents of the distribution file in any way.

· You do not charge for the package except to cover handling costs.

Evaluation License

·	The software is provided "as is" and all liability for any damages resulting from the use of the software is excluded.

·	The author grants you the right to install and test the software on one computer on one    network only.

·	The software will run for 60 minutes to allow rigorous evaluation. If you continue to use the software after the evaluation period, you must register by paying the registration fee to the author and agreeing to the Registration License.

Registration License:

By registering, both parties explicitly agree to the following TERMS and CONDITIONS:

·	On receipt of the License Fee, the LICENSOR shall:

· supply a Registration Number per e-mail within two working days.

· provide technical support per e-mail for a period not exceeding 30 days from the date of

registration.

· guarantee the validity of the Registration Number for all future upgrades to the product for

the same platform.

· On receipt of the License Fee, the LICENSEE shall:

· use the Registration Number on one computer on one network only.

· not reveal the Registration Number to any other party.

· not hold the LICENSOR responsible for any loss or damage resulting from the use of the

software.

· not request a refund under any circumstances.

Canberra, Australia.								  25. July, 1998.

Information in this document is subject to change without notice and the author assumes no liability for errors and omissions.

Contents

Introduction

Before Installing NAT32

Installing NAT32

Configuring NAT32

NAT32 Network Interface Configuration

Using NAT32

Technical Hints

Troubleshooting Tips

Technical Help

Registration

Appendix

NAT32 and the Windows 98 DUN Server

Installing the DUN Server

Removing TCP/IP from the DUN Adapter

Configuring the DUN Server

Configuring NAT32

Configuring your Home PC

Connecting an entire LAN to your Office Server

# Introduction

NAT32 is a software package which lets all computers on your private LAN  share a single connection to the Internet. You can then run most standard networking applications on any computer on your LAN, without any reconfiguration of those applications.

NAT32 works with Windows 98 Dial-Up Networking (using either standard modems or ISDN adapters), Cable Modems and Ethernet Adapters connected to a LAN with an Internet Gateway.

This Installation Guide tells you how to install and configure NAT32 to share a Dial-Up Networking Connection. If you follow these instructions carefully, you'll be up and running in a few minutes.

Installation of the DUN Server option is described in the Appendix.

Installation instructions for Cable Modem configurations are contained in the document:

NAT32 Cable Modem Installation Guide (\nat32\doc\nat32c.doc)

# Before Installing NAT32

Please make sure that both your LAN and your Dial-Up Networking Connection are working correctly.

If you are upgrading from a previous version of NAT32, please restore your original system.ini file and then run clean.bat. Now reboot your machine.

Please make sure that you are using private IP addresses on your private LAN. If you aren't, then NAT32 Autoconfiguration will not work. Private IP addresses are:

10.x.x.x	172.16.x.x - 172.31.x.x		192.168.x.x

To test your LAN:

From a Windows Console (sometimes affectionately called a DOS Box), run the ping program and ping all machines connected to your LAN. If any machines can't be reached, check the configuration of the TCP/IP protocol stack on the unreachable machines.

To test your Dial-Up Networking Connection:

Start the connection and then ping any desired host on the Internet. If you get no response, 	your Dial-Up Networking configuration is incorrect and you should fix it before continuing.

# Installing NAT32

· Download the latest NAT32.ZIP file from: http://willow.canberra.edu.au/~chrisc/nat32.html

· Create a directory such as c:\nat32 and then unzip NAT32.ZIP into that directory.

· Erase the NAT32.ZIP distribution file.

If you don't have an unzip program, you might like to download WINZIP from http://www.winzip.com

# Configuring NAT32

Start NAT32 by typing the following commands into a Windows Console:

cd \nat32

nat32

The above commands mean: change to the \nat32 directory and then run the program nat32.exe.

NAT32 will start and display the Evaluation Dialog Box:

Enter your name and then click Continue... If an UNREGISTERED message box appears at any time, simply click OK.

The following Options Dialog will appear:

· To use Dial-Up Networking, select Client and then click Continue...

· If you would like NAT32 to dial a particular connection each time it starts, select a connection from the drop-down list.

· To use the Dial-Up Networking Server, please see the Appendix.

· To save these settings, click the Create Shortcut checkbox.

A NAT32 Console Window similar to the following will now appear:

The screen shot shows that the file system.ini has been modified. The original file was saved as system.old. The system.ini file is modified because the Device Driver for the Windows Networking Adapters needs to be installed when Windows boots up. Each time NAT32 is run, it checks the validity of system.ini and modifies it only if necessary.

Note the message WINDOWS REBOOT REQUIRED in the above screen shot. This indicates that a reboot is required to install the NAT32 Network Device Driver. Type exit to terminate NAT32 and then reboot your system.

Start NAT32 by double-clicking the NAT32 shortcut on your desktop.

Note that NAT32 detected that the required driver was installed correctly, that the Dial-Up Adapter was found and that a Windows Network Interface Controller (NIC) was found.

In the unlikely event that any error messages appear, please write them down and e-mail them to me.

If all is well, a Network Configuration dialog box will appear.

# NAT32 Network Interface Configuration

The NAT32 package is actually a software router and works as follows:

· PCs on your private LAN are configured to use NAT32 as their gateway to the Internet.

· NAT32 forwards this traffic to the Internet via the Windows Dial-Up Networking Adapter, after

performing appropriate Address Translations and Port Mappings.

· Traffic coming in from the Internet is forwarded to the correct PC on your private LAN.

In order to perform its tasks, NAT32 uses a Logical Network Interface which must now be configured. A dialog box similar to the following will appear:

All of the values you see in the dialog box have been carefully extracted from the Windows Registry and will be correct for most configurations. If any fields (except the two checkboxes) are blank, please consult the explanation of the fields by clicking Help. Most users will be able to use the supplied values and can continue by clicking OK.

One value deserves special mention: the NAT32 IP address. This value was determined by replacing the last part of the IP address which you are using on your private LAN with the value 100. If you already have a machine on your private LAN which uses this address, then please specify an appropriate value in the NAT32 IPaddr field.

The settings in the above dialog box will be saved in a file called \nat32\netcfg2 when you click OK. That file will be used on subsequent runs of NAT32.

That's it, NAT32 is now configured and almost ready for use. But first, exit NAT32 again and  proceed as described in the next section.

# Using NAT32

First start your Dial-Up networking connection to the Internet. Then click on the NAT32 shortcut. The NAT32 Console will now be similar to the following:

You will now want to test your other machines to see if they really can access the Internet. But before you do, you must configure the TCP/IP protocols on those machines to use the NAT32 IP address as their gateway. The needed address is shown in the lower-right corner of the NAT32 Console Window.

On each of your other machines, open the Control Panel, select the Network Applet and then set the Gateway to (in this example) 172.16.2.100.

You will also need to Enable DNS. You can use the NAT32 IP address as the DNS Server if you like, or you can use the same DNS Server which your ISP specified for your Dial-Up Networking Connection.

You can now shrink or hide the NAT32 Console by clicking the X button in the top-right corner of the NAT32 Console Window.

NAT32 is now ready for use.

# Technical Hints

· If the Windows Dial-Up Networking Connection disconnects for any reason, simply click Reconnect in the pop-up Dialog Box. Now type:

ppp close

ppp open "Connection name"

ipmapa

· NAT32 contains an extensive command set which allows you to interrogate the system and perform useful operations. All of the commands you will need are documented in UNIX-style Manual Pages, which you can view by typing man <command> at the NAT32 command prompt.

· Windows Dial-up Networking on your machine is not modified by NAT32. It will work exactly as before, even if NAT32 isn't running. In fact, you need only run NAT32 whenever you wish to share the connection with other machines on your private LAN.

· The file \nat32\doc\app.doc contains useful information for advanced users.

· Should you ever decide to remove NAT32 from your system, simply drag the NAT32 folder to the Recycle Bin and copy system.old to system.ini.

# Troubleshooting Tips

TCP/IP problems:

· Be sure you have configured the TCP/IP stacks on all machines (other than the machine running NAT32) to use the NAT32 IP address as the gateway (172.16.2.100 in the sample configuration).

· Be sure you have enabled DNS and specified a valid Domain Name Server.

· Perform all of the ping tests described in the section: Before Installing NAT32

· If any tests fail, please contact a local expert for help on installing Microsoft TCP/IP and Microsoft Dial-Up Networking.

NAT32 configuration problems:

If you made an error during the configuration run, terminate NAT32 by typing exit at the NAT32 command prompt. Then run the file \nat32\clean.bat from a Windows Command Box. This will erase all NAT32 network configuration files and you can then start a new configuration run.

If Dial-Up Networking and the TCP/IP stacks are configured correctly, then NAT32 configuration problems are extremely unlikely. However, if you do have problems, restart NAT32, establish a DUN Connection and then perform the following tests from the NAT32 Console:

Test 1

Type ping 172.16.2.1 (the address of the Microsoft TCP/IP stack on the NAT32 machine). If this ping fails, it may be because you don't have 32-bit NDIS3 drivers installed on your Ethernet Adapter. NAT32 will NOT work if only 16-bit NDIS3 drivers are installed. Check this using the Control Panel Network Applet and clicking on the Properties of your Ethernet Adapter.

Test 2

If Test 1 was successful, try to ping other computers on your private LAN. If any machines can't be reached, check that their Network Adapters and TCP/IP protocols are installed correctly.

Test 3

If Test 2 was successful, try pinging the Internet address 137.92.11.125. While this ping is executing, observe the Modem SD and RD lights. If SD blinks but RD doesn't, it means that host 137.92.11.125 can't be reached. Re-test with an IP address on the Internet which you know is reachable. If both SD and RD blink, your ping is being answered but NAT32 isn't getting the reply. I suggest you reboot your computer in this case.

Test 4

Ping fir.canberra.edu.au. This is the host you contacted in Test 3. If the name can't be resolved, your Name Server address is incorrect. Reconfigure NAT32.

Test 5

Now type ipmapa. You should receive a response similar to the following:

Mapping 172.16.0.0 to 137.92.100.70 on interface 1

Port Mapping: ON

This response indicates that NAT32 is mapping IP addresses on your private LAN to your ISP-provided Internet address (in this case 137.92.100.70) on the PPP interface. If you didn't receive this response, you should run clean.bat from the Windows command line and then reconfigure NAT32.

Test 6

Start a WEB Browser or a Telnet Client on any machine. If pages don't open or the Telnet connection closes shortly after it was established, then you most likely forgot to reboot Windows after NAT32 modified system.ini. Reboot Windows to correct the problem.

# Technical Help

If you can't get NAT32 to work, please read the NAT32 WEB Pages and the NAT32 MAN Pages.

Registered users are entitled to e-mail help. Your e-mail must contain your registration code and the contents of the following text files:

· \NAT32\winipcfg.out

· \NAT32\netcfg2

· \NAT32\netcfg3 (if relevant).

The more details you send me about your problem, the more likely I can find a solution.

My e-mail address is: chrisc@fir.canberra.edu.au (text e-mail only).

Because of limited resources, I cannot answer technical queries from unregistered users.

# Registration

Registration details are displayed when you click the How to register... button in the first dialog box which NAT32 displays when it starts. This will display a WEB Page with the current registration information, including price, methods of payment and other details. The page also contains a link to a Web Site through which you can register NAT32 on-line.

Appendix

# NAT32 and the Windows 98 DUN Server

The Windows 98 DUN Server, in conjunction with NAT32, can be used to give one or more remote computers full TCP/IP access to the Internet via a connected Windows 98 computer in your office or at your place of work.

NAT32 will automatically start and stop the DUN Server at specified times on specified days, allowing your office phone to be used for normal calls during business hours.

# Installing the DUN Server

Before configuring NAT32 for Server operation, please confirm that you have the DUN Server installed on your office computer. Clicking on the Connections menu item in the Dial-Up Networking folder should show the following menu items:

If the Dial-Up Server... menu item is not shown, you should first install one of the later versions of Microsoft DUN (preferably DUN1.2b, available free of charge from Microsoft).

If the Dial-Up Server... menu item is present but inactive, it could be because:

· You don't have File Sharing installed. Use the Control Panel Network applet to install it.

· Your System Administrator has set NoDialin and/or NoFileSharing in the registry. Use

regedit to modify the settings..

# Removing TCP/IP from the DUN Adapter

Open the Windows Control Panel and start the Network Applet. If a Microsoft TCP/IP protocol stack is bound to the Dial-Up Adapter, select it and click Delete.

# Configuring the DUN Server

Start the Dial-Up Server by clicking the  Dial-Up Server  menu item in the Dial-Up Networking folder. You should see a Dialog Box similar to the following:

Select the desired modem and click the Allow caller access button. Click Change Password and  assign a strong password. This will ensure that no unauthorised access to your DUN Server can take place. Click No caller access and then OK. This will close the DUN Server.

If more than one modem was shown, as in the above example, use the Control Panel Modem Applet to remove all modem types except the one which is actually to be used.

# Configuring NAT32

NAT32 Configuration consists of several steps:

- Start NAT32 by typing: nat32 server 1 from a Command Box or by clicking an appropriate shortcut on your Desktop.

- When the Network Configuration Dialog Box appears, confirm that the settings reflect the values report by the Windows winipcfg program and that the Primary Interface checkbox is set. The settings are determined automatically and no modifications should be necessary.

- Click OK and NAT32 is then ready for operation. Dial-in from home to test the connection.

# Configuring your Home PC

To configure your home machine, proceed as follows:

·	Using the Control Panel Network Applet, configure the TCP/IP protocol which is bound to your Dial-Up Adapter as follows:

IP address:		172.16.1.253

Gateway Address:	172.16.1.254

Subnet Mask: 		255.255.255.0

DNS Address:		as used on your office LAN

·	The above values must be used. No other values will work.

·	Now open your Dial-Up Networking folder, right-click on the connection which you will use to connect to your office and select the Properties option. Under the Server Types tab you will find a dialog like the one shown below. Be sure your settings match those shown.

· Now click the TCP/IP Settings... button and you will see the following dialog box:

The crucial setting here is the Use IP Header compression checkbox.

BE SURE IT IS UNCHECKED.

The IP Address and the Name Server Address can be set in this dialog box, but I recommend you set these values from the Control Panel Network Applet as described above.

You should now dial your office and test the connection. If all went well you now have your own Dial-Up connection to the Internet.

# Connecting an entire LAN to your Office Server

If your home PC is also fitted with an Ethernet Adapter, you can configure it to give ALL computers on your home LAN access to the Internet through your office machine.

After configuring the machine with the modem as described above, you now need to perform the following additional configuration steps on that machine:

·	Use the Control Panel Network Applet to configure the TCP/IP protocol bound to your Ethernet Adapter as follows:

IP address:		172.16.2.1

Gateway Address:	none

Subnet Mask: 		255.255.255.0

DNS Address:		as used on your office LAN

· Click OK, but do not reboot just yet.

· Start regedit and add the value EnableRouting to the following Windows 98 Registry Key:

HKLM\System\CurrentControlSet\Services\VxD\MSTCP

You do this as follows:

· Select the above key.

· Click Edit, New, String Value and name it EnableRouting

· That value will then appear in the list of values for the key.

· Now double-click the EnableRouting value and change it to 1.

· Exit regedit.

It is crucial that you have an updated vip.386 driver installed on your machine. The driver is available from Microsoft as part of the Windows DUN 1.2 upgrade. If you are running the original vip.386 driver installed from the Windows 95 CD ROM, your machine will crash on reboot, in which case, do the following:

· Boot up in Safe Mode by pressing F8 just as Windows 95 starts booting.

· Set EnableRouting to 0 using regedit.

· Download the DUN 1.2 upgrade and install it.

The final step is to open the Properties of the DUN connection you will be using to connect to your office machine, select Server types and then click the TCP/IP Settings... button. Clear the Use default gateway on remote network check-box.

Now reboot your machine and, while it is rebooting, go to your other machines and configure their TCP/IP protocols as follows:

IP address:		172.16.2.x			(x represents any unused address)

Subnet Mask:		255.255.255.0

GW address:		172.16.2.1

DNS Address:		as used on your office LAN

Test your setup and then register NAT32.