PACTOR

Copyright 1998 by Brian Beezley, K6STI

All Rights Reserved

PACTOR.........................................1

Setting Up.....................................1

Operating......................................2

Monitor Mode...................................3

FEC Mode.......................................4

Linked Mode....................................5

Displays.......................................7

Receive Options Menu...........................7

Transmit Options Menu..........................8

Protocol Details..............................10

---- PACTOR ----------------------------------------------------

PACTOR is a data-communications protocol developed by

Spezielle Communications Systeme GmbH in Germany and licensed

for use in RITTY.  This document describes PACTOR mode.  See

RITTY.DOC for general information about RITTY and details about

RTTY mode.

PACTOR uses two-tone FSK like RTTY but offers higher

data rate, data compression, error correction, and the extended-

ASCII character set.  Instead of transmitting individual

asynchronous characters, PACTOR sends packets of information

using synchronous transmission.  Each packet contains a special

tag that permits the receiver to determine whether data has been

correctly received.  The receiver can request retransmission of

bad packets.  PACTOR transmits at 100 or 200 baud, automatically

changing speed according to channel conditions.  It can use

Huffman coding to compress text for faster throughput.

---- SETTING UP ------------------------------------------------

The PACTOR protocol requires your transceiver and

amplifier to switch quickly between transmit and receive.  VOX

can't be used because it switches too slowly.  Instead, use a

serial-port output bit to key your push-to-talk (PTT) line.  See

RITTY.DOC for details.

You can use AFSK or FSK with PACTOR.  Unlike RTTY,

PACTOR FSK requires an auxiliary serial port to provide timing.

This port cannot be the one used for data/PTT output.  However,

since no connections are made to the auxiliary port, you can use

the serial port of an existing device (for example, a modem or

mouse).  Select serial ports in the Setup menu.  RITTY assumes

that auxiliary serial ports 1-4 use standard IRQs.  Select port

Y to specify a nonstandard port address or IRQ.

PACTOR works best with a frequency shift of 200 Hz.

RITTY always emits narrow-shift AFSK at this shift.  If you use

FSK and your transceiver generates 170-Hz shift, signal-to-noise

ratio (S/N) will degrade about 1.5 dB for a matched-filter

demodulator.  Use FSK only when you can't use your CW filter in

LSB mode.  RITTY provides a sharp input bandpass filter for use

instead of a CW filter.

Set Output Delay in the Transmit Options menu to the

time it takes your transceiver and amplifier to switch from

receive to transmit.  The best way to determine this number is

to examine RF output with an oscilloscope while applying a

steady audio tone and repeatedly switching from receive to

transmit.  If you can't make the measurement, the 25-ms default

should work with most equipment.  An unnecessarily high value

can inhibit long-distance PACTOR links.

Set Rated TX Duty Cycle in the Setup menu to your

transmitter's rated power-output percentage for long-term,

1

constant-power transmissions.  This parameter allows RITTY to

transmit brief control signals at full power while reducing

output for packets and FEC to avoid overheating your transmitter

(AFSK only).  For example, if your 100-watt transceiver is rated

at 50 watts continuous output for extended transmissions, enter

50%.  RITTY then will output AFSK at the following relative

power levels:

Control Signals   100 watts

LP Packets         73

SP Packets         65

FEC & RTTY         50

---- OPERATING -------------------------------------------------

Use Alt-/ to switch between RTTY and PACTOR modes.

Press S for the Setup menu, R for the Receive Options menu, and

T for the Transmit Options menu.

PACTOR has three operating modes:  In monitor mode you

can tune around and monitor QSOs and CQs.  FEC mode lets you

call CQ or broadcast a message in much the same way as with

RTTY.  Finally, linked mode is for QSOs.  When linked you

communicate with one station and the PACTOR protocol provides

error correction, data compression, etc.  When you end a link or

finish a FEC transmission, RITTY returns to monitor mode.

PACTOR links require both callsigns.  Your callsign is

provided by your RITTY.LIC license file.  To establish a link to

another station, press L and enter his callsign.  RITTY begins

to transmit a connect request when you press Enter.  RITTY

listens between the request packets for a response.  When

"Linking" on the bottom screen line changes to "Transmit" you're

linked and you can begin to type a message.  RITTY transmits

your message in packets about one second long.  Each packet is

acknowledged by a brief control signal (CS) from the other

station.  Your transceiver switches to receive to listen for the

CS and then switches back to transmit.

When you're ready for the other station to respond to

what you've typed, press the Pause key to turn the link around.

"Transmit" will change to "Receive" and you'll begin to display

his response.  Normally the other station will turn the link

back to you when he's done typing, but you can interrupt and

grab the link by pressing Pause.

To terminate a link, press ^Q (Ctrl-Q) while

transmitting.  This sends a special QRT packet, which is then

acknowledged.  If signals are weak and you can't receive the

acknowledgment, press ^Q again to return to monitor mode.  If

you press ^Q while receiving, RITTY returns to monitor mode

without notifying the other station.

To call CQ, press Pause in monitor mode to enter FEC

mode.  RITTY then transmits whatever you type.  It repeats each

2

packet to facilitate reception.  (Duplicate text isn't displayed

by someone monitoring your transmission.  You can change the

number of repeats in the Transmit Options menu.)  Press Esc or

Pause to return to monitor mode and await a connect request.

When someone connects to you, his callsign appears in gray in

the lower-left screen corner.  The station initiating the link

transmits first.

Press F1 to F12 in monitor mode to prepare a canned

message.  (PACTOR and RTTY use separate message buffers.)  Press

the function key while transmitting to send the message.  Press

Alt-F1 to Alt-F12 while transmitting to edit a message.  You can

access the Transmit Options and Receive Options menus while

transmitting by pressing Alt-T and Alt-R.

---- MONITOR MODE ----------------------------------------------

RITTY uses concurrent 100- and 200-baud demodulators,

each with optimal SIN(x)/x channel filters matched to the baud

rate.  When a monitored signal changes speed, RITTY

automatically displays the output of the proper demodulator.

The callsign of the station someone is trying to link

with will appear in the lower-left screen corner in dark blue.

The callsign is transmitted only during connect attempts, not

during QSOs in progress.

Three red dots in text indicate that at least one packet

could not be decoded and text is missing.  This is common during

signal fades in monitor mode.

It's normal to see a few garbage packets of nonsense

text each hour when monitoring noise.  This occurs when the

special CRC (cyclic redundancy check) packet tag erroneously

indicates that decoded noise constitutes a valid packet.

Once RITTY receives two consecutive CRCs, it locks

(synchronizes) to the bitstream and preferentially decodes

synchronized packets.  It remains locked until it receives

several consecutive bad packets (Unlock Count in the Receive

Options menu).  A small count lets RITTY quickly relock to a new

bitstream but prevents it from hanging in as long for marginal

signals.  RITTY immediately unlocks when it detects link

turnover or QRT.  RITTY erases the baud rate annotation when it

unlocks.

As described later for linked mode, when locked to a

bitstream RITTY can repair bad packets.  It displays repaired

text in dark blue.  Because many PACTOR systems transmit FEC

with irregular timing, RITTY does not attempt to lock to FEC

bitstreams and therefore cannot repair FEC packets.

To extract maximum information when monitoring a weak

signal, enable Monitor Bad Pkts in the Receive Options menu.

When locked to a bitstream RITTY then will decode all packets

3

regardless of CRC.  It displays text from bad packets in red.

You'll often find useful information within the junk.

"Good" means a packet was received without error or was

repaired.  "Bad" means at least one error remained.  These

annotations use the same color scheme as the text.

When QRM is heavy use the Receive Options menu to engage

the input bandpass filter.  This 200-tap, linear-phase, FIR

filter is optimized for the 200-baud PACTOR signal spectrum.

The filter is 500 Hz wide at -6 dB, 800 Hz wide at -60 dB, and

has an ultimate rejection of more than 100 dB.  This filter

should work better than a CW IF filter (bandwidth is about the

same, shape factor and ultimate rejection likely are better, and

group delay is much better).  The BPF slightly degrades receive

sensitivity.  However, the effect is so minor you probably can

leave the filter engaged all the time without noticing anything

amiss.  The filter requires considerable computation and may not

run on slower computers.

Tidbits:  Tuning is less critical for PACTOR than for

RTTY because the channel filters are wider.  PACTOR interchanges

the mark and space channels each packet to allow use of LSB or

USB without concern for tone reversal.  Some stations use AMTOR

FEC to call CQ rather than PACTOR FEC even though they expect a

PACTOR QSO.  You won't be able to copy these CQs.

---- FEC MODE --------------------------------------------------

FEC stands for forward error correction.  PACTOR

actually performs only error detection in this mode.  The

receive gap between transmit packets is eliminated and each

packet is repeated before the next is sent.  (FEC Repeats in the

Transmit Options menu determines the number of repetitions.

Most stations send each packet twice.)  Repetition gives the

receiver a chance to recover text when an error occurs.

While FEC mode is most commonly used to call CQ, it's

also handy when you have problems linking.  Use FEC to obtain a

callsign or to QSY to another frequency.  It's also sometimes

used to make a final remark after link termination.  Finally,

FEC provides the only way to have a roundtable PACTOR QSO

involving more than two stations.

As with RTTY, you can embed ^R in a message to end

transmission and return to monitor mode.  If you enter ^R while

typing ahead, RITTY will switch to monitor mode when it

encounters the character.

In both FEC and linked modes RITTY avoids Huffman

encoding whenever it can do so without incurring a throughput

penalty.  This strategy allows a receiver to more easily recover

text from corrupted packets.

4

---- LINKED MODE -----------------------------------------------

When linked, PACTOR provides a brief interval during

which the receive station replies with a control signal.

However, this window is too short to permit the CS to be

received by long-path propagation.  PACTOR provides an alternate

timing scheme with a wider window for use in long-path QSOs.  To

initiate a link with LP timing, press ! when entering the

callsign.  LP timing also is useful for short-path QSOs with

stations using long transmit delays.

When sending a connect request, RITTY searches the

entire receive window for a control signal from the other

station.  This provides a great opportunity for noise to

masquerade as a valid response.  When this happens RITTY

switches from linking to transmit.  If it doesn't recognize

another CS within three frames, it reverts to sending a connect

request.

When it receives a second CS, RITTY stays linked.  It

then can tolerate errors in the CS1 and CS2 control signals that

acknowledge packets.  If it can't recognize a control signal,

RITTY must repeat the last packet.  Allowing one or more CS bits

to be incorrect speeds data for poor signals.  You can specify

the number of hits allowed with the Transmit Options menu.

RITTY allows no hits in CS3 (turnover) or CS4 (speed change)

because false recognition of these control signals may cause

link failure.  (These signals aren't frequent enough to warrant

error tolerance anyway.)

Tips:  To enter a callsign without immediately linking,

press L, type the callsign, then press Esc.  Later, to initiate

the link press L, then Enter.  Press Esc to terminate a link

attempt before it times out.  Press L then ! to toggle LP timing

without reentering the callsign.  Embed ^R in a message to turn

the link around or ^Q to QRT the link.  Use the backspace key to

correct typos even after they've been received and displayed by

the other station.

If you alter the BPF setting while linked, the signal-

delay change will kill the link.  This also will occur if you

engage your CW filter, although you may be able to get away with

it at 100 baud.  Set your filters before you link.

The number displayed in the lower-right screen corner is

the delay between the end of your transmit packet and the

beginning of the received control signal.  This figure includes

equipment delay at both ends and propagation delay.  The short-

path limit is about 135 ms when Output Delay is 25 ms.

The ppm number on the bottom line is the difference

between the far-end protocol clock and your clock in parts per

million.  RITTY displays this number only when someone connects

to you and only while you're transmitting.  The measurement

takes a while to stabilize.  If you notice a consistent bias for

5

many stations, modify the Clock Offset value in the Transmit

Options menu in the same direction.

RITTY implements Memory-ARQ.  This error-correction

technique works as follows:  When a packet and its

retransmission both have errors, RITTY combines the packets by

summing the demodulated waveforms.  Combining packets increases

S/N because the signals add coherently but the noise adds

incoherently.  (S/N increases 3 dB when the packets have the

same signal and noise powers.)  If the combined packet still

isn't error free, RITTY saves it and requests another

retransmission.  RITTY keeps adding repeated packets to the

combined signal until a received packet is good or the

combination is good (or either can be repaired, as described

next).  RITTY segments packets and uses separate summing

coefficients to independently combine the parts to maximize

combined-signal S/N.

RITTY can correct single-bit errors without requesting

retransmission.  It does this by flipping each message bit in

turn and retesting the CRC.  If the CRC becomes valid, it

accepts the packet.  Packet repair speeds links during difficult

conditions.  RITTY also applies packet repair to the Memory-ARQ

signal.  This provides six chances to recover a packet repeated

once.  Packet repair does increase the risk of decoding a packet

with undetected errors, but this seldom occurs while linked.

You can disable packet repair from the Receive Options menu.

Packet repair uses another trick to correct multiple-bit

errors under certain circumstances.  Packets for hand-typed text

usually contain fill characters to pad the packet to the proper

length.  Bit errors in the fill characters cause CRC failure

even though the characters aren't displayed.  When a packet

needing repair contains a fill character, RITTY first overwrites

the remainder of the text with fills.  It then attempts to

correct single-bit errors.  This strategy permits text recovery

even when the padded part of a packet contains many errors.

Although RITTY does not perform fill-replacement for Huffman-

encoded packets, most PACTOR systems do not Huffman-encode hand-

typed text.

"Good" means a packet was received without error or was

corrected.  "Bad" means the packet could not be recovered and a

retransmission was requested.  RITTY can display bad packets

(red text) in monitor mode but does not do so while linked.

RITTY colors error-corrected text like this:  Dark blue

is packet repair, tan is Memory-ARQ, and light blue is Memory-

ARQ with packet repair.  RITTY colors "Good" the same way.  It

colors "Bad" purple when it repairs a packet but certain control

bits contradict other information.  (Although it obtained a

valid CRC, RITTY asks for a repeat since there's evidence it

incorrectly reconstructed important control data.)  During

transmit RITTY colors the CS annotation dark blue when it

recognizes CS1 or CS2 with errors.

6

RITTY can send and receive ANSI graphics but does not

display color or blinking.

Unlike ordinary demodulators, RITTY's matched filters

provide a 3-dB S/N advantage at 100 baud compared with 200 baud.

This is equivalent to doubling transmit power.  RITTY switches

to 200 baud after receiving several consecutive completely

filled packets without error (Upspeed Count).  It switches to

100 baud after receiving several consecutive bad packets

(Downspeed Count).  Links begin at 200 baud when conditions

warrant.

---- DISPLAYS --------------------------------------------------

The left-hand display is a spectral tuning indicator

just like that for RTTY.  PACTOR receive-filter frequencies do

not autotune.  Use the arrow keys to alter them.

During receive the waveform display on the right shows

either the packet header (alternating bits) or the data samples

and decision threshold (ATC trace) for the entire packet.  The

header display provides a high-resolution snapshot of the

demodulated waveform.  It can reveal selective-fading

distortion, transmit start-up anomalies, leading-bit suppression

by your noise blanker, and random PK-232 header errors.  The

packet display shows the signal envelope and ATC action.  It

reveals flat and selective fades, static crashes, QRM, receiver

AGC artifacts, and last-bit truncation from early PTT release.

During transmit the waveform display shows the received control

signal.

---- RECEIVE OPTIONS MENU --------------------------------------

Upspeed Count

This parameter is the number of consecutive, good,

completely filled packets RITTY must receive before switching to

200 baud.  Enter a large number to force RITTY to stay at 100

baud.

Downspeed Count

This parameter is the number of consecutive bad packets

required to switch to 100 baud.  Enter a large number to keep

RITTY at 200 baud.

Unlock Count

This parameter applies only to monitor mode.  It is the

number of consecutive bad packets RITTY must receive before

unlocking from one bitstream and becoming more receptive to

7

another.  When monitoring bad packets, RITTY can't recognize

another bitstream at all until it unlocks.

Callsign Hits

RITTY can tolerate errors in the received callsign (your

callsign) during a connect attempt.  This permits much faster

connects.  This parameter is the number of errors allowed in the

72-bit callsign comparison.  The default is 1.  Use 0 to connect

only when your callsign has been perfectly received.

Repair Packets

Select Yes to enable RITTY to reconstruct received

packets to correct transmission errors.

Monitor Bad Pkts

Select Yes to display text in monitor mode for packets

with invalid CRC.  This also inhibits relocking to a new

bitstream until several consecutive bad packets (Unlock Count)

have been received, or the link turns over or QRTs.

BPF

The bandpass filter is optimized for 200-baud PACTOR

signals.  You can select it independently of the RTTY BPF.  The

filter lowers receive sensitivity by a small amount.

ATC

Disable automatic threshold correction only for severe

polar flutter.  (Strong static crashes that pump your receiver's

AGC can fool ATC, but it's better in this case to disable the

AGC.)  ATC greatly improves copy during ordinary selective

fading, so normally it should be enabled.

Waveform

Select Header to view the packet header (alternating

bits).  Select Packet to show the data samples and decision

threshold for the entire packet.

---- TRANSMIT OPTIONS MENU -------------------------------------

Timeout Count

This parameter is the maximum number of times RITTY will

send a packet without receiving acknowledgment.  It also is the

8

maximum number of consecutive bad packets during receive.

Finally, it is the maximum number of connect attempts.  When a

timeout count is exceeded, RITTY returns to monitor mode.

CS1/CS2 Hits

This is the number of incorrect bits allowed in CS1 and

CS2 while linked.  The default is 1.  To recognize CS1 and CS2

only when perfectly received, set the number to 0.  Control

signals are 12 bits long and differ in 8 bit positions.

FEC Repeats

This is the number of times RITTY transmits a packet in

FEC mode.  (If you want to repeat it once, enter 2.)  Nearly all

stations transmit FEC packets twice.

FEC Baud Rate

Nearly all stations transmit FEC at 100 baud.

Clock Offset

This parameter compensates for sample-rate variation

among sound cards.  Change its value when you notice a

consistent bias in the displayed ppm figure (mild offset) or

when links always hang (severe offset).  To measure your clock

offset, enter a value of 0, output a 1000-Hz RTTY AFSK tone

(Center Freq 1000 Hz, Freq Shift 0 Hz, AFSK Autotune off), and

measure its frequency with a high-resolution frequency counter.

A reading of 1000.145 Hz, for example, means your clock is 145

ppm fast.  Enter an offset of -145 ppm to compensate.

Output Delay

After asserting PTT RITTY waits for your transceiver and

amplifier to switch before sending data.  You can enter a delay

of up to 99 milliseconds.

Callsign Buffer

RITTY can automatically place the other station's

callsign into one of the message buffers.  You can then refer to

it in messages by using %n, where n is the buffer number.  Enter

the buffer number (for example, enter 10 for F10).  Enter 0 to

inhibit this feature.

9

---- PROTOCOL DETAILS ------------------------------------------

Timing            SP      LP

------            --      --

Cycle           1.25    1.40    seconds

Packet          0.96    0.96

CS window       0.29    0.44

CS              0.12    0.12

Max CS delay    0.17    0.32

Baud             200     100

----             ---     ---

Packet           192      96    bits

Text              20       8    bytes

SP rate         16.0     6.4    char/sec (ASCII)

~28     ~11             (Huffman)

LP rate         14.3     5.7             (ASCII)

~25     ~10             (Huffman)

CS                 -      12    bits

Control Signals

---------------

CS1     CS1 then CS2 or CS2 then CS1:  Acknowledge packet

CS2     CS1 then CS1 or CS2 then CS2:  Request retransmission

CS3     Change transmission direction

CS4     Change speed

10