- The Pool Professor

- Software

- Version 2.0

By:  Roy R. Fielding

- Copyright 1993-98 Roy R. Fielding

Sold by

Fielding Scuba Tours, Inc.

6701 Hollow Oak Drive

Charlotte, NC 28227

Fax Number (704) 573-0031

Email address: thepoolprofessor@worldnet.att.net

- Table of Contents

Program Overview							3.

System Requirements							3.

Installation								4.

Sample Screen								4.

File Menu and Pool Setup Menu					5.

Help Menu								6.

Treatment Menu							7.

Specials Menu								8.

Report Menu								9.

Menu Map								10.

License Agreement							10.

Hot Keys								11.

- DISCLAIMER

All calculations performed in the Pool Professor are on standard formulas used in determining the correct water chemistry for your swimming pool or spa.  Failure of accuracy on your part will result in incorrect answers in the program.  There is no error checking done by the program that is your responsibility.  The accuracy need is mainly involved, but not limited to your pool volume and your water test results.

I N NO EVENT SHALL FIELDING SCUBA TOURS, INC. OR ROY R. FIELDING BE LIABLE OR RESPONSIBLE FOR ANY PROBLEMS THAT ARISE BECAUSE OF YOUR USE OR DEFECTS WITH THE POOL PROFESSOR SOFTWARE.

- All Rights Reserved

- The Pool Professor

- The software application for determining the proper

water chemistry for your swimming pool or spa.

- Program Overview

The Pool Professor is a state of the art software application used in the daily operation of an aquatic facility.  It will help the pool operator maintain water chemistry in an acceptable range.  The Pool Professor will alert users of possible water problems that may cause premature corrosion or scale formation of the pool walls, plumbing, and equipment.  It will also recommend corrective measures to be taken.  The Pool Professor offers the following features:

- A graphically oriented user interface that allows easy retrieval of pool or spa treatment information.

- Easy selection of chemicals that the operator may use in their swimming pool or spa.

- The use of Hot keys for experienced users who do not want to use a mouse.

- A print utility that prints recommended chemicals dosages, water tests, recommendation for superchlorination or neutralization, saturation index, pool volume, date and time, and general pool reports.

- An archive file that will maintain a history of all water tests saved by date and time, ad general pool reports.

- Warnings when the pool water becomes corrosive or scale forming, need for superchlorination or neutralization.

- Full help information on pool chemicals, testing and treatment guidelines, glossary, and MSDS information.

- System Requirements

The Pool Professor softwares minimum requirements are an IBM PC or compatible, DOS 2.3 or higher, 1 MB of free disk space, and 1 MB of RAM.  A mouse or other pointing device is recommended, but not required.

- To create hard copies of suggested treatments, a parallel printer is required.  Most types of printers are supported.

- Installation

Before installing the Pool Professor read the licensing agreement and disclaimer included in this manual.  Also, make a backup copy prior to installation. Perform the following to install the Pool Professor:

1.  Install the included Pool Professor diskette in appropriate drive. (A or B dependent upon your system)

2.  Make appropriate drive the current drive.  Example A: then type install and press the enter key.

3.  The directory is POOL, to run from the hard drive (c:) type cd pool then press the enter key.

4.  At C:\>Pool type pool and press the enter key.

- File Menu and Pool Setup Menu

- File Menu and Pool Setup Menu Description

In the File Menu you can open the existing pool(s) to date, and also create a new site for additional facilities.  The File Menu can also backup and restore all the data that have been entered. The Pool Setup Menu is used to review that all the information entered about a particular pool or spa is correct.  The Pool Setup Menu allows the operator the ability to correct or change any of the defaults you entered for a facility.

When you press open in the file menu, you will see the existing pool(s) or a spa(s) already in the Pool Professor.  By pressing new you will be asked the following questions:

1.  Name of the pool or spa?

2.  Chemicals being used in the pool or spa? (Refer to the chemicals label of ingredients)

3.  Is this an indoor or outdoor facility?

4.  Is this a swimming pool or spa?

5.  Water volume? (This number is critical that it is accurate, all treatments are based on this number)

6.  The printer port you are using? (LPT1 or LPT2)

7.  The Target Levels you want the water test results to be for your facility?  The Pool Professor will allow                only values in accepted ranges for swimming pools and spas.

Note: Chemical selection, indoor or outdoor, and pool or spa selections are on the same screen.

The Pool Setup Menu is where changes or corrections can be made to previous selections for a particular pool or spa.  Note: Make sure your computers is reading the correct date and time, refer to computer owners manual.

- Help Menu

- Help Menu Description

The Help Menu will give the user a great deal of information about a wide assortment of swimming pool care and related topics.  The following is a list of some of the subjects that will e covered:

- Chemicals

Description of the chemical

Label of Ingredients

Advantages and Disadvantages of the chemical

How to add it safely to your water

Emergency Response Information for the chemical

Material Safety Data Sheets (MSDS) for the chemical

- How to properly test your swimming pool or spa water

- Treatment guidelines and steps to properly treat swimming pool or spa water

- A glossary of swimming pool and spa related terms

- Description of what is in each different menu windows

- Treatment Menu

- Treatment Menu Description

The Treatment Menu is one of the main working areas for the pool operator.  In this menu you find areas to enter and view information regarding proper treatment of your swimming pool or spa water.

Pool Test Enter the current water test results in this window by keying in the proper numbers and tab down to any other test result that needs to be changed.  When they are all correct press the OK selection button.

Step 1.	(Free Chlorine or Bromine) Display, the chemical you have entered and the amount of the chemical needed based on the number of gallons in your pool, to reach Target Level.

Step 2.	(Total Alkalinity) Display, the chemical you have entered and the amount of the chemical needed based on the number of gallons in your pool, to reach Target Level.

Step 3.	(pH) Display, the chemical you have entered and the amount of the chemical needed based on the number of gallons in your pool, to reach Target Level.

Step 4.	(Stabilizer) Display, the chemical you have entered and the amount of the chemical needed based on the number of gallons in your pool, to reach Target Level. (FOR OUTDOOR POOL USE ONLY)

Step 5.    (Calcium Hardness) Display, the chemical you have entered and the amount of the chemical needed based 	on the number of gallons in your pool, to reach Target Level.

- All the steps will display the amounts in oz., or fl. oz., lbs. or gallons, kg. or liters.

Pro Print	This step will allow you to print out one sheet with all the above, date, time, and recommendation for correcive steps.  It will also show input information of your test results.

- Specials Menu

- Special Menu Description

The Special Menu contains information that can be of great benefit to the pool operator.  The following is a description of what is contained in the different windows:

Saturation Index: The saturation index informs the pool operator if the swimming pool water is balanced, corrosive, or scale forming.  This is a complex calculation that is determined by the pool waters pH, total alkalinity, calcium hardness, and the waters temperature.  The Pool Professor will calculate the saturation index automatically when you enter all water test results.

SuperChlorination: If there is a difference between your free chlorine and total chlorine of greater than 0.2 ppm, the Pool Professor will recommend an amount of chlorine (calcium hypochlorite, sodium hypochlorite, or chlorine gas) to be used in order to superchlorinate the pool.  The printout from Pro Print will show that SuperChlorination is needed.

Neutralization: When you have a free chlorine reading above 5 ppm, neutralization will be recommended.  The display will show the amount of sodium thiosulfate needed to bring your water back to target level.

Pool Calculator: Performs a large number of valuable calculations related to a swimming pool.  Volume, surface area, water loss, filter sizing, bather load, flow rates, electrical costs, and conversions are some of the functions.

Archive: Contains a record of all your water test results.  The printout will show, on a single line, the date, time, free and total chlorine, pH, total alkalinity, calcium hardness, cyanuric acid, water temperature, and saturation index readings.  This may be viewed on the screen. You also delete the archive in this area, be sure to make a backup to disk and/or to hard copy before deleting.

- Report Menu

- Report Menu

The Report Menu is another working area for the pool operator.  Here you will find an easy to enter daily report form.  Use the arrow up and down key to change information in a cell, and the tab key to move around the screen.

General 	Information regarding the facility. Opening and closing, times, weather and air and water temperatures. You can type in the time (IE. 6:00 am), there is space for eight characters.

Chemicals	Enter the amount of chemical used in a certain group (IE. Chlorine 2 lbs.), there is space for eight characters.

Maintenance	These are yes and no questions to common daily routines around a swimming pool or spa.

Attendance	You may enter daily attendance figures in several categories, the total will appear at the bottom.

Operations	Deals with the flow rate on the water recirculation system and filter pressures.

Reports	Yes and no questions to the other reports that may have been filled out that day.

Print		A print copy of all information entered that day. Seven days of daily reports will fit on one sheet of paper.

- The Pool Professor

- Version 2.0 Menu Map

- License Agreement

The Pool Professor (this software) is licensed for the exclusive use of the original purchaser (you) for use on one computer only.

This software is not copy-protected.  Fielding Scuba Tours, Inc. authorizes you to make backup copies of the software for your archives only, for the sole purpose of protecting your investment from loss.

You are free to move this software from one computer location to another, as long as there is no possibility of it being used at two locations at one time.  This software should not be used in two different locations at the same time.  It is against the law to copy this software on any medium except as specifically allowed by Fielding Scuba Tours, Inc.

Hot Keys

Short Cuts

Pool Setup Menu						Alt-P

Treatment Menu						Alt-T

Specials Menu						Alt-S

Report Menu							Alt-R

Exit Pool Professor						Alt-X

Pool Tests							F2

Pro Print							F3

Treatment Step 1						Alt-1

Treatment Step 2						Alt-2

Treatment Step 3						Alt-3

Treatment Step 4						Alt-4

Treatment Step 5						Alt-5

Activate Menu Bar						F10

Close Active Window					Alt-F3

Moving Around a Screen					Tab

Moving in an Area						Arro