WGSL.COM v2.0, and this documentation file are

Copyright Wacom, Ltd. 1991.

The file WGSL.COM is a TSR driver for AT&T-GSL and Truevision compatible

products, which use an ATVISTA or TARGA video adapter.

USING WGSL DRIVER

Command line options may be in any order.  They are :

/1 for COM 1

/2 for COM 2

/A5  SD-510C tablet

/A4+ SD-420, SD-421, or SD-422 tablets

/A3  SD-311L or SD-312L tablets

/A3E SD-311E or SD-312E tablets

/A3+ SD-320L, SD-321, or SD-322 tablets

/A2  SD-210 tablet

/F for Full Tablet (Aspect ratio not maintained - without the /F flag

aspect ratio is maintained.)

/P for a pressure stylus

/U for un-install

The default settings are for COM 1, A4+, non-pressure stylus, maintain

aspect ration.  WGSL typed with no command line options is the same as

WGSL /1 /A4+.

EXAMPLES

WGSL /2 /A5

Would set the COM port to COM 2 and the tablet to SD-510C.

WGSL /1 /A2 /P /F

Would set the COM port to COM 1 and the tablet to SD-210, for use with a

pressure stylus, with full tablet (aspect ratio not maintained)..

WGSL /U

Will un-install (remove from memory) the WGSL GSL driver if the driver

was the last TSR loaded. (Last in and First out)

Wacom, Inc.

Technical Support

Park 80 West Plaza II

Saddle Brook, NJ 07662

201-791-4903 Voice

800-922-6635 Voice

201-265-4758   BBS

201-265-4722   Fax