Covox Voice Blaster TM

Version 4.0

Pre-Release

April 2, 1993

Copyright Covox, Inc. 1993

Section 1: Overview

Introduction

Welcome to the world of Voice Blaster for Windows! With this

program you can teach your computer to respond to human voice.

About this document.

This document is organized into two sections.

Part 1:   - Installation procedures.

- What you need to know about our Voice Recognition.

- The hardware and software required to start using Voice

Blaster.

- Windows conventions, hints on improving command

recognition, and installation guidelines.

Part 2:   - Reference describes Voice Blaster commands, menus,

screens and advanced functions in depth.

Installation Procedure

Hardware Connections:

- Connect the E/Q module (included with Voice Blaster) to your LPT

printer port. Note that your printer can be attached to the

other end of the E/Q module.

- There are three cables connected to the E/Q module, one male and

two females. Plug the male jack into the microphone input jack of

your sound card.

- Plug the red headset plug into the larger of the two female jacks

of the E/Q module. Your headset microphone should now be ready

for use. (The smaller female jack is for a foot switch that can

be used to activate recognition. Call Covox for details.)

Software Installation:

The installation program guides you through the process of

copying the Voice Blaster software to your hard drive. Insert the

disk 1 of 1 into the A or B disk drive of your computer.

If you have not yet started Windows, start it now by typing

win<ENTER> at the DOS prompt.

Go to the Program Manager window and click on File to display the

File Menu.

Select (click once) the Run menu option from the File menu

A dialogue box will then be displayed, click once on the browse

button.

A browse dialogue box will then be created. Click on the arrow in

the Drives menu to specify the disk drive where the installation

disk has been inserted.

Select the appropriate disk drive. The C: drive is always

designated as the hard drive but PCs vary  in their assignment of

the A: and B: drives. If the empty disk drive is selected from this

menu, a Browse Information  dialogue box will appear, alerting you

that it cannot read the selected drive.

When the proper drive has been selected you will see the vb40win

folder in the directories list. Double click on this entry, or

click once to select and the press the OK button.  Select

install.exe from the File menu on the left. Click on OK.

The Browse dialogue box will disappear and the file name

install.exe will be inserted into the command line field of the Run

dialogue box.

Leave the Run minimized box unchecked.

Click on the OK button.

Installation begins:

The screen will change to display a Covox graphic and a question

dialogue box prompting you to install Voice Blaster 4.0. Click on

the OK button to confirm and begin the installation. You will be

guided through a series of dialogue boxes, gathering information to

tailor the installation for your system.

If you have used Voice Blaster before, you will be asked whether

the Covox program group and items should be cleared. Click on yes.

You will then be prompted for the desired location of the Voice

Blaster files. You can click on OK to use the default location, or

enter a new directory.

As Voice Blaster is installed, a slider portraying the percentage

of completion will be displayed.

When complete, your windows session will need to be restarted for

the Covox group and Voice Blaster icon to be displayed. You will be

asked whether windows should be restarted for you.

Activating Voice Blaster

Check that your microphone is properly connected to the sound

hardware.

After Windows has been restarted, look in the Program Manager

window for the window with the Covox name in the titlebar.

Double click on the Voice Blaster icon.

System requirements

Hardware

To use Voice Blaster, you will need to have:

IBM AT-compatible - or higher computer.

Sound input hardware - installed in your PC, any PCM

sound card that has Windows 3.1 drivers. If your sound

input hardware has not yet been installed, refer to your

sound hardware instructions for guidance.

Parts supplied with Voice Blaster:

Microphone - headset with a 1/8" headphone sized plug.

E/Q module - for converting the microphone input signal

Software

Microsoft Windows 3.1.

Windows MME drivers (supplied with your sound card).

How Voice Blaster works

Voice Blaster utilizes the concepts of speaker-dependency, voice

macroing, and isolated word recognition. It has not been designed

for dictation (such a system would require enormous computing

resources), but it can save you time normally spent typing, making

your computer more efficient and easier to use.

Voice Blaster is a Terminate and Stay Resident program. Once

Voice Blaster is loaded, it remains in memory (even while other

software applications are running) until it is deactivated from DOS

or until your PC is turned off.

Speaker dependency

Voice Blaster is designed to operate with either gender and in

any language. Each user must train Voice Blaster in advance to

recognize his/her voice. Usually this means that different users

cannot activate one another's commands, but this is not always the

case.

Voice Blaster has not been designed for voice imprinting and is

not a reliable tool for determining the identity of the speaker.

Isolated Word

You must pause briefly between each voice command so that Voice

Blaster can determine the beginning and end of your command and

compare it to the trained templates.

Voice Macroing

The Voice Blaster system is set up to associate each voice

command with a series of keystrokes to be entered into the

computer. This sequence of keystrokes, called a macro, usually

takes the place of something you type frequently.

Voice Blaster compares the sound pattern of your voice command

with the trained templates in your computer's memory. When you say

a command (a word or short phrase), Voice Blaster translates the

digitized sound data into an algorithmic template representing the

frequency content of your voice. Voice Blaster recognizes your

commands according to the following procedure:

- Records your command.

- Removes extraneous sounds before and after the command.

- Compares the command to the trained templates using a complex,

pattern matching algorithm.

- Calculates a score that represents how close your command is to

each of the trained templates.

- Chooses the voice template with the closest score and determines

if that score is within the allowable tolerance.

- If recognition is successful, the macro associated with that

voice template is entered into the computer, otherwise, it does

nothing.

Improving recognition

Voice recognition is not completely accurate. After all, people

often do not understand speech perfectly either. You should expect

Voice Blaster to recognize between 75% and 95% of your commands

correctly. Voice recognition accuracy depends on:

- your experience using the system

- your consistency in pronunciation

- the number of commands trained in the current group

- the level of background noise

Position the Microphone Carefully

If you are using the Covox Headset, position it so that it fits

snugly on your head. Adjust the microphone so that the foam-covered

element is an inch or two away from the corner of your mouth. Do

not position the microphone directly in front of your lips,

moisture and air from your breath can interfere with recognition.

Voice Blaster works best when you use a similar microphone

position during training and recognition.

Choose Distinctive Sounds

Just like people, Voice Blaster can confuse similar sounding

commands. For example, we may easily confuse the words "choose" and

"shoes", or the sounds made by the letters "b", "d", and "v".

Choose commands that sound different from one another.

If you wish to enter individual letters by voice command, use the

Phonetic Alphabet:

Alpha

Bravo

Charley

Delta

Echo

Fox-trot

Golf

Hotel

India

Juliette

Kilo

Lima

Mike

November

Oscar

Papa

Quebec

Romeo

Sierra

Tango

Uniform

Victor

Whiskey

X-ray

Yankee

Zulu

When entering digits by voice, it may be helpful to use the

following operator's numbers:

Oh

One

Two

Thuh-ree

Fow-wer

Fie-yuv

Six

Seven

Eight

Nie-yun

If Voice Blaster frequently does not recognize a command, retrain

it with a different pronunciation.

Train Using a Natural Voice

Voice Blaster will recognize your commands more easily if you train

them the way you would normally say them.

Because your pronunciation and inflection can change throughout

a typical day, we recommend varying your tone and volume slightly

(within your normal speaking range) during training. Voice Blaster

can then produce an average voice profile and recognize your

commands more accurately. You may also need to create and train an

alternate macro/template file when your voice changes considerably,

(i.e. if you have a cold).

Pronounce Commands Consistently

Voice Blaster cannot recognize your voice if you alter your

speech dramatically. Speak naturally, both when training and using

voice commands. When using some applications this may be difficult.

For example, you will probably be excited while playing an action

game and speak voice commands differently. In these cases, train

each command according to the situation.

Adjust the Test Levels

The test command allows you to adjust how Voice Blaster listens

to your voice. Carefully adjusting the Sensitivity, Acceptance, and

Voice Input levels can improve accuracy tremendously.

Window conventions

Finding Help

Help will always be available through the system menu, click once

on the small button in the upper left hand corner. You can also

access help by pressing the F1 function key.

Ellipsis use...

Menu items that are followed by three dots ... will activate a

dialogue box if selected.

Access to non-active windows

Most dialogue boxes that are generated from another window (a child

of that window), require complete control of the keyboard until

they are closed. If you try to select a different window outside of

the dialogue box before the dialogue box is closed, you will hear

a bell reminding you to finish the current action first.

Standard buttons: OK, Cancel

Clicking on OK confirms any actions requested by you in a window,

while selecting Cancel will erase any changes you have made during

a dialogue.

Section 2: Reference

Main Window

Titlebar

System Menu

The first seven items in the system menu are standard for Windows

applications. You can double click on the system menu icon to close

Voice Blaster.

Help ~ Click on Help for guidelines on using Voice Blaster.

About ~ Select About to find the serial number and version number

for Voice Blaster.

Views ~ The Views option will open a dialogue box allowing you to

customize the look, size and font of the Voice Blaster main window.

Always on Top ~ If there is a check mark next to Always on Top, the

Voice Blaster windows will always be in front of other

application's windows. If not selected (no check mark), the active

application's window will be in front.

Title

Displays the application name and version number.

Voice recognition square

When voice recognition is enabled, this square in the title bar

gives a visual indication of how the sound is recognized.

Black ~ no sound input above the calibrated threshold level or

voice recognition is disabled.

Red ~ a command is being recognized.

Green ~ sound input above the calibrated threshold level.

White ~ Voice Blaster is echoing the captured sound.

Minimize Button

A shortcut to choosing minimize from the system menu, will turn the

application window into an icon.

Maximize Button

A shortcut to choosing maximize from the system menu, to expand the

application window to fit the screen.

Tool Bar

On/Off Switch

A toggle switch for turning voice recognition on and off.  By

setting the switch to off you will be able to use other programs

that record and playback .WAV sound files.

Up Arrow & Down Arrow

These arrows relate to the command list directly below them in the

window. The down arrow will be inaccessible and appear gray unless

a selection has been made from the command list. If a command is

highlighted (selected), you can click on the down arrow to display

the next level of commands available from the selected command.

Program attributes menu button ~ A

A menu of options for tailoring the user and command lists, and for

accessing the training window, where you will teach Voice Blaster

to respond to your spoken command.

Edit Commands...

Train Commands...

Edit User...

Sound configuration menu button ~ Swiss Army Knife

Menu items relating to sound management. This is where you tell

Voice Blaster how the sound should be monitored (continuously or on

demand), and how to acknowledge manual mode.  It is also where you

access testing for commands that have been trained, and settings

for sound input that can be customized for your voice and

environment.

Command List

Initially, this window will display the top level menu for an

application. You can navigate further into a program's list of

commands by selecting an item from this list and clicking on the

Down Arrow in the toolbar. The display will change to show the

subsidiary menu for your selection, you can think of it as

travelling down into an application to a more specific command.

The commands in this list that are gray, have not yet been trained.

The commands that are written in black have already been trained.

There are two shortcuts available to you using this command list.

First, by double clicking on a command entry, you are directly

selecting the train dialogue box for this one command. Second, by

double clicking on the new entry at the end of the list of commands

in the main window a dialogue box will prompt you for the command

to add, it's associated keyboard macro, and it's placement in the

command structure.

Recognized Command

When Voice Blaster recognizes a command it will insert the text of

the perceived selection into this window for verification.

Active Application

The name of the currently active program whose commands will be

executed is entered here by Voice Blaster.

Calibrate button & slider

The calibrate button can be pressed at any time to instruct Voice

Blaster to set the threshold for background noise. The slider bar

next to the calibrate button will reflect this level. You may also

adjust the slider to set an arbitrary threshold.

Voice recognition bar

Titlebar:System Menu:About

Voice Blaster TM

Software Version 4.0

Trademark 1993 Covox, Inc.

Titlebar:System Menu:Views

A walking menu off of the system menu that gives you the ability to

customize the look of the interface for Voice Blaster. You have the

option of removing titles for the various fields, changing the

typeface, size and style, or removing fields from the main window

altogether, either singly or en masse. The first six items, from

Menu Controls to Titles, will toggle on and off. A check mark will

appear next to the option if it is visible, and be missing if  the

selection is hidden in the main window.

Open All

Selecting this item will cause all of the fields to be displayed in

the main window.

Close All

When this item is selected, only the main window titlebar will be

displayed. This is convenient after all of your commands are

trained and the program is completely customized, at this point you

will only be concerned with the recognition of commands that you

give to your other applications. The voice recognition square

located in the titlebar, allows you to monitor the sound input.

Font

The font dialogue box lets you choose the look of the text in all

Voice Blaster windows.

Font ~ specify the typeface to be used in the display.

Font Style ~ regular, italic, bold, or bold italic.

Size ~ the point size of the type.

Sample ~ displays upper and lower case examples of the chosen

font.

Toolbar:Program attributes menu button ~ A

Edit Commands

Voice Blaster can capture the top level menu commands for all

applications. Due to the variation in software applications, Voice

Blaster cannot always detect the commands deeply embedded in a

program. Because of this, Voice Blaster allows you to supplement

the set of commands automatically captured by the program. You are

also given the ability to edit commands for maximum flexibility.

.

The Edit Command window can be accessed through the toolbar's

program attributes menu.

To use the Edit, New, Delete, and Train buttons in this window, a

command must be selected.

Command File:

The currently active vocabulary's application name is inserted in

this field automatically. If you wish to change the active focus to

a different program, you can select the down arrow to the right of

this field for a menu of known applications.

If you want to edit the vocabulary for an application that is not

on this list, close the Edit Commands window but leave Voice

Blaster running, start up the application of choice, then re-open

the Edit Commands window. Your new program will have been added to

the list.

Macro Definitions:

The keyboard sequences that are executed when a command is

recognized. This is what you would normally have to type if you

didn't have Voice Blaster.

Edit

Pressing this button when a command is highlighted will display the

Edit Vocabulary Item dialogue box. The vocabulary item will be

filled in for you, and the current attributes for this item. To

change the macro associated with a command you must press the clear

button to erase the old macro definition and enter the new keyboard

sequence into the Use Keyboard to Enter Definition Here box.

New

Pressing this button when a command is highlighted will display the

New Vocabulary Item dialogue box. The attributes for this command

must be defined.

Enter Definition Here

The keyboard sequence for command.

This definition valid after

The command (e.g. which menu option or button) that would cause

this selection to be displayed.

After recognizing item go to

Same group of items ~ back to where it was.

Top group of items ~ main program window.

Previous group of items ~ screen display before this item was

invoked.

Delete

Removes the selected item from the list.

Train

Displays the train dialogue box.

Capture

There must be a currently active application that is the focus for

the capture. If there is, the commands and macro definitions will

be overwritten by the commands that are retrieved programmatically.

This facility is a good safeguard if you have erroneously edited a

list of commands and wish to return to the standard set.

Train Commands

Command Selection

The selection from this list tells Voice Blaster which commands to

insert into the Command List.

Selected Commands ~ commands that have been selected and are

highlighted in the Command List. Click on a command(s) to select.

Click and drag to select more than one contiguous command.

Active Commands ~ this is a subset of the total vocabulary of an

application and indicates only the commands that are the present

focus of the currently active program listed in the main window.

Single Command File ~ the entire set of commands from the active

application.

All Command Files ~ all of the commands from all of the known

applications. It only takes running a program at the same time that

Voice Blaster is running to make it known to the Voice Blaster

software. The common commands are shared by all of the applications

that use them (i.e. Close, Save, etc.), so you only have to train

each command one time, even if it appears in many programs.

Untrained only

If this box is selected, the command list will reflect only the

commands that have not yet been trained by the current user, and

that match the vocabulary specifications.

Command List

This list is for display and individual command selection. The

contents are controlled by the Command Selection.

Train Button

Pressing this button will open the Train Dialogue Box.

Train Commands Dialogue Box

This is where you teach Voice Blaster to respond to your spoken

commands! Adjust your microphone so that it rests about an inch

away from your face, and at the corner of your mouth for best

recognition.

It is a good idea to have the settings dialogue box open while you

are training commands to watch the Recorded Command Length for

consistency in your training.

Press the Start button to begin training.

Because Voice Blaster memorizes rather than understands what you

say, you can say any easily remembered word or short phrase, even

if it has nothing to do with the command you typed. However, it

will be easier to remember commands if you repeat the command as it

is written on the list.

Your voice inflection, tone, and volume are important factors when

training. Speak naturally during the training process, voicing

slight variations between repetitions which Voice Blaster will

average for better results.

Each time a command is accepted, an x will replace the command name

in the training list. A command must be accepted three times before

it is trained.

Retrain will go back one iteration of one command at a time.

Edit User

All of the managerial functions for users can be performed in this

window. Each user's file contains the command templates that have

been trained by that user.

The currently active user's name will be inserted at the top of

this window.

Activate

To switch active users, select a name from the list of users on the

left and press the activate button. The new selection will then

become the active user, and the command templates for their voice

will be used to recognize and test commands. The edit user dialogue

box is automatically closed for you and the new user's name is

inserted into the toolbar on the main window.

Rename

Before selecting the Rename button, select a user from the list on

the left. Click on Rename and enter the new name in the dialogue

box provided.

Create

There is a maximum of eight characters that can be used in a user's

name before the .usr extension. You do not need to type the

extension, it will be appended automatically to the name.

Delete

Press on the delete button after selecting one of the user names

from the list. You will be asked for confirmation for a delete,

since removing a user causes the file containing all of the voice

templates that have been created to be erased.

Toolbar:Sound configuration menu button

Recognition Mode

Manual

There must be manual intervention through the keyboard, mouse or

foot switch to alert Voice Blaster to a voice command. Recognition

will be turned on for one command at a time.

Continuous

Voice input will be recognized automatically and continuously

without the need for manual intervention

.

Echo Commands

The sound input will be played back but the command will not be

executed.

Manual Mode..

.

Specification dialogue box for manual mode activation. This is

where you can tailor manual activation to your preferred style.

Keyboard

Mouse

Footswitch

Test Recognition

After training a command, you can verify that it was recorded

correctly and that Voice Blaster can recognize it. In addition, you

can set the levels that control the accuracy of recognition.

Voice Blaster calculates a score to represent how close your

command is to each of the templates in the current group, and

chooses the voice template with the lowest score. You can adjust

how the score is evaluated using the Covox Settings window (press

the Settings button to display).

Available Commands

The list of commands that have been trained.

Recognized Commands

When Voice Blaster recognizes a command during testing, it will

list the command at the top of this column and any other commands

that were within given tolerance levels. For a command to be

recognized during testing, it must be linked to a macro.

Test

This option activates the test procedure.

Train

Pressing this button will cause the Train Dialogue Box to be

displayed

.

Settings

Selecting this button will cause the Covox Settings Dialogue Box to

be displayed.

Covox Settings

Acceptance Level

With this slider, you can adjust the accuracy level for command

recognition. With the slider control at the left end of the scale,

you will have to be very exact in the repetition of each command

during training for the iteration to be accepted, and you will also

have to be very close to your original training voice when issuing

a command for it to be recognized. With the slider control at the

extreme right end of the scale, it will be easier for more than one

command to match the voice template.

You can use the test recognition function to help you adjust the

acceptance level for optimum performance for your voice.

Calibration Level

The calibration level can be set automatically for your environment

by pressing the calibration button found in the Settings window and

in the main Voice Blaster window.

The calibration level is designed to filter out the background

noise and separate it from the command word. You can test the

appropriateness of the calibration level by using the test

recognition function with echo on to hear a digitally compressed

version of the word you are testing. The recording should capture

the entire command, no more, no less. If the command sounds cut

off, move the calibration slider control to the right, if the

recording is too long, replace the slider control to the left.

Minimum Command Length

This is the shortest length a word can be to be recognized as a

command. To adjust, move the slider control to the right for a

longer minimum, and to the left for a shorter minimum.

Recognized Command Length

Voice Blaster will insert the recorded length of a recognized

command into this field.

Calibrate

Press this button to have Voice Blaster automatically calibrate the

background noise threshold level for your environment.