FEBRUARY 1993

FTP PC/TCP Installation

for

ETHER ETHER-16 Series Ethernet Card

=========================================

The following files will be used in this installation:

Files            Contents

-----------   -------------------------------------------------

METH16PD.SYS     Packet driver program for ETHER-16 series card

METH16PD.COM     Packet driver program for ETHER-16 series card

The ETHER packet driver programs above let you run FTP's PC/TCP

with the ETHER ETHER-16 Series Ethernet Card.  Two methods can be used:

METHOD 1:

If you use the METH16PD.SYS program, the following statement must

be included in your CONFIG.SYS file:

DEVICE = METH16PD.SYS <-d> <packet_int_no> <-w>

This statement may be placed anywhere in the CONFIG.SYS file.

METHOD 2:

If you use the METH16PD.COM program, follow the procedure below:

1. Execute the METH16PD.COM as follows:

>METH16PD <-d> <packet_int_no> <-w>

2. Execute the ETHDRV.EXE device driver program (provided by FTP).

You are now ready to use FTP's PC/TCP.

Notes:

<packet_int_no> is the software interrupt vector number used by the packet

driver program, which can be from 96 to 128.

<-d>            Delay Initial - The driver program will not initialize

the network interface card until the packet driver program

is accessed by its upper layer program first time.

<-w>            the -w parameter is for use with the Windows environment only.