Installation instructions for the packet driver collection

Document conventions

All numbers in this document are given in C-style

representation.  Decimal is expressed as 11, hexadecimal is

expressed as 0x0B, octal is expressed as 013.  All reference to

network hardware addresses (source, destination and multicast)

and demultiplexing information for the packet headers assumes

they are represented as they would be in a MAC-level packet

header being passed to the send_pkt() function.

Using the packet drivers

The packet driver must be installed prior to use.  Since

each packet driver takes only a few thousand bytes, this is best

done in your autoexec.bat.  Since the Ethernet boards typically

have jumpers on board, the packet driver must be informed of the

values of these jumpers (auto-configure is possible, but can

disturb other boards).  The first parameter is the software

interrupt used to communicate with the packet driver.  And

again, because each board is different, the rest of the

parameters will be different.

All parameters must be specified in C-style representation.

Decimal is expressed as 11, hexadecimal is expressed as 0x0B,

octal is expressed as 013.  Any numbers that the packet driver

prints will be in the same notation.

Before installing the packet driver, you must choose a software

interrupt number in the range between 0x60 and 0x80.  Some of

these interrupts are used for other purposes, so your first

choice may not work.  See Appendix A for the section of Ralf

Brown's interrupt list between 0x60 and 0x80.

Running a packet driver with no specifications will give a

usage message.  The parameters for each packet driver are

documented below.

The -n option is used to convert IPX packets between the

Ethernet II type 8137 encapsulation used by BYU's PDSHELL IPX

interface code and the 802.3-style encapsulation normally used

on Ethernet by Netware servers, shells and boot PROMs.  It will

also convert incoming type 8137 packets to type 8138 so that

Netware doesn't get confused.

The -d option is used to delay the initialization of the

Ethernet card until the packet driver is used for the first

time.  See PROMBOOT.NOT for how to use -d and -n in a PROM boot

environment.

The -w switch is used for Windows.  Install the packet driver

before running MS-Windows.  This switch does not prevent Windows

from swapping your network application out of memory, it simply

detects when that has happened, and drops the packets on the

floor.

3Com 3C501

usage: 3C501 [-n] [-d] [-w] packet_int_no [int_no

[io_addr]]

The 3c501 driver requires two additional parameters -- the

hardware interrupt number and the I/O address.  The defaults are

3 and 0x300.

3Com 3C503

usage: 3C503 [-n] [-d] [-w] packet_int_no [int_level(2-5)

[io_addr [cable_type]]]

The 3c503 driver requires three additional parameters --

the hardware interrupt number, the I/O address, and the cable

type.  The 3c503 can be attached to thick or thin Ethernet

cables, and the selection is made in software.  The cable type

parameter should be zero for thick, and one for thin.  The

defaults are 2, 0x300, and 1 (thin).  The 3c503 uses shared

memory whose address is set by jumpers, but the software can ask

the board what the address is.  You MUST set some shared memory

address.  Note that the card comes with the shared memory

jumper disabled as default.

3Com 3c505

usage: 3c505 [-n] [-d] [-w] packet_int_no [int_no [io_addr

[base_addr]]]

The 3c505 driver requires three additional parameters --

the hardware interrupt number, the I/O address, and the memory

base address.  The defaults are 2 and 0x300 and 0xd000.

3Com 3c507

usage: 3c507 [-n] [-d] [-w] packet_int_no [int_no [io_addr

[base_addr]]]

The 3c507 will determine its parameters by reading the

board.  The only time you would need to specify the parameters

is when you have multiple 3c507s in the same machine.

The 3c507 driver will use three additional parameters --

the hardware interrupt number, the I/O address, and the memory

base address.

3Com 3c523

usage: 3c523 [-n] [-d] [-w] packet_int_no [int_no [io_addr

[base_addr]]]

The 3c523 driver requires three additional parameters --

the hardware interrupt number, the I/O address, and the memory

base address.  The defaults are 3 and 0x300 and 0xc000.

AT&T

usage: at&t [-n] [-d] [-w] packet_int_no [int_no [io_addr

[base_addr]]]

The AT&T driver requires three additional parameters --

the hardware interrupt number, the I/O address, and the memory

base address.  The defaults are 2 and 0x360 and 0xd000.  This

driver supports the StarLAN 1, StarLAN 10 NAU, EN100 and StarLAN

Fiber NAU.

D-Link DE-600

usage: de600 [-n] [-d] [-w] packet_int_no

The D-Link Pocket Lan Adapter packet driver requires no

additional parameters.

Digital Equipment Corporation DEPCA

usage: depca [-n] [-d] [-w] <packet_int_no> [<int_no>

[<io_addr> [<mem_addr>]]]

The DEPCA packet driver requires three additional

parameters -- the hardware interrupt number, the I/O address,

and the memory base address.  The defaults are 5 and 0x300 and

0xd000.  The packet driver will resolve the io_addr

automatically if io_addr is set to '?', e.g.

depca 0x7e 5 ? 0xd000.

Mitel Express

usage: express [-n] [-d] [-w] <packet_int_no> [-n]

[<driver_class> [<int_no>]]

The Mitel Express packet driver has one optional switch,

and two optional parameters. The <driver_class> defaults to

SLIP, and the <int_no> defaults to 7.  The -n switch instructs

card to be an NT.  The <driver_class> should be SLIP or a

number.

HP EtherTwist/Starlan10/ThinLan

usage: hppclan [-n] [-d] [-w] packet_int_no [int_no

[io_addr]]

The hppclan driver supports 8 and 16 bit HP LAN cards

based on the National 8390 chip.  Earlier HP LAN cards should

use the 3Com 3C501 driver.  The hppclan driver requires two

additional parameters -- the hardware interrupt number and the

I/O address.  The defaults are 3 and 0x300.  On IBM PS/2

Microchannel machines, the defaults are read from the PS/2

configuration NOVRAM and additional parameters are not

normally required.

IBM Token Ring

usage: ibmtoken [-n] [-d] [-w] packet_int_no [adapter_no]

The IBM Token Ring packet driver requires one additional

parameters -- the adapter number.  The default is zero.  See

IBMTOKEN.DOC for more information.

LocalTalk

usage: localtlk [-n] [-d] [-w] <packetintno> [<IP address>]

The LocalTalk packet driver requires atalk.sys to be

installed.  Because it is not an Ethernet class driver, it

requires special code in the client.  See LOCALTLK.NOT for more

details.

NCR ET-105

usage: ncret105 [-n] [-d] [-w] <packet_int_no> <int_no>

<base_addr> <Ethernet_address>

The NCR ET-105 driver requires four additional parameters

-- the hardware interrupt number, the I/O address, the memory

base address, and the Ethernet address.  The Ethernet address

assigned to any particular board is printed on sticky labels

that come with the board.

Novell IPX

usage: ipxpkt [-n] [-d] [-w] packet_int_no

The ipxpkt packet driver simulates Ethernet on Novell IPX

protocol.

BICC Data Networks' ISOLAN 4110 ethernet

usage: isolan [-n] [-d] [-w] packet_int_no [int_no

[base_addr]]

The BICC Isolan requires three additional parameters --

the hardware interrupt number and the memory base address.  The

defaults are 2 and 0xb800h.

Netbios

usage: nb [-n] [-d] [-w] packet_int_no ip.ad.dr.ess

[receive queue size]

The netbios packet driver transports IP packets over

NetBIOS.

Novell ne1000

usage: ne1000 [-n] [-d] [-w] packet_int_no [int_no

[io_addr]]

The ne1000 driver requires two additional parameters --

the hardware interrupt number and the I/O address.  The defaults

are 3 and 0x300.

Novell ne/2

usage: ne2 [-n] [-d] [-w] <packet_int_no>

The ne/2 driver requires no additional parameters.

Novell ne2000

usage: ne2000 [-n] [-d] [-w] packet_int_no [int_no

[io_addr]]

The ne2000 driver requires two additional parameters --

the hardware interrupt number and the I/O address.  The defaults

are 2 and 0x300.

Racal-Interlan (Formerly Interlan) NI5010

usage: NI5010 [-n] [-d] [-w] packet_int_no [int_no

[io_addr]]

The NI5010 driver requires two additional parameters --

the hardware interrupt number and the I/O address.  The defaults

are 3 and 0x300.

Racal-Interlan (Formerly Micom-Interlan) NI5210

usage: ni5210 [-n] [-d] [-w] packet_int_no [int_no [io_addr

[base_addr]]]

The NI5210 driver requires three additional parameters --

the hardware interrupt number, the I/O address, and the memory

base address.  The defaults are 2 and 0x360 and 0xd000.  Note

that Interlan sets the default memory base to 0xa000, which is

brain-damaged, because that area of memory is specifically

reserved for video adapters, and in fact the EGA and VGA use

it.

Racal-Interlan NI6510

usage: ni6510 [-n] [-d] [-w] packet_int_no [int_no

[io_addr]]

The ni6510 driver has two additional parameters -- the

hardware interrupt number and the I/O address.  The defaults are

2 and auto-sense.  These parameters do not need to be set unless

the auto-sense routine fails, or otherwise disrupts operation of

your PC.

Racal-Interlan (Formerly Micom-Interlan) NI9210

usage: ni9210 [-n] [-d] [-w] packet_int_no [int_no [io_addr

[base_addr]]]

The ni9210 driver requires three additional parameters --

the hardware interrupt number, the I/O address, and the memory

base address.  The defaults are 2 and 0x360 and 0xd000.

NTI 16

usage: nti16 [-n] [-d] [-w] packet_int_no [int_no [io_addr

[base_addr]]]

The nti16 driver requires three additional parameters --

the hardware interrupt number, the I/O address, and the memory

base address.  The defaults are 3 and 0x338 and 0xd000.

Ungermann-Bass NIC-PC

usage: ubnicpc [-n] [-d] [-w] <packet_int_no> <int_no>

<base_addr>

The UB NIC-PC driver requires two additional parameters, the

hardware interrupt number, and the memory base address.

Ungermann-Bass NIC-PS/2

usage: ubnicps2 [-n] [-d] [-w] <packet_int_no> <int_no>

<io_addr> <base_addr>

The UB NIC-PS/2 requires three additional parameters -- the

hardware interrupt number, the I/O address, and the memory base

address.  The defaults are the contents of the POS registers, so

the only time you would need to use the parameters is if you're

using two NIC-PS/2 boards in one machine.

Tiara Lancard

usage: tiara [-n] [-d] [-w] packet_int_no [int_no

[io_addr]]

The Tiara driver runs the Tiara LANCARD/E cards, both

eight and sixteen bit cards.  The Tiara driver requires two

additional parameters, the hardware interrupt number, and the

I/O port.

Western Digital WD8003 E EBT EB ET/A and E/A

usage: WD8003E [-n] [-d] [-w] packet_int_no [-o]

[int_level [io_addr [mem_base]]]

The WD8003E driver runs the Western Digital E, EBT, EB,

ET/A, and E/A Ethernet cards.  The WD8003E requires three

additional parameters -- the hardware interrupt number, the I/O

address, and the memory base address.  The defaults are 2 and

0x280 and 0xd000.  The wd8003 cards do not enable their memory

until configuration time.  Some 386 memory mappers will map

memory into the area that the card intends to use.  You should

be able to configure your software to leave this area of memory

alone.  Also driver will refuse to map memory into occupied

memory.  The occupied memory test fails on some machines, so the

optional switch -o allows you to disable the check for occupied

memory.  Note:  You MUST use a 16 bit number for the shared

memory address.  An example would be 0xD000.  Do not use 0xD0000.

If you use the incorrect shared memory format you will see a

"Prom contents are invalid" error.

Errorlevels

Some of the packet drivers return error codes.  Some of

these error codes indicate fatal errors, and some are merely

warnings.  For the moment, you must consult the source to see

what the errorcodes mean.  For example, pktchk returns 0 if a

packet driver exists at a given address, and 1 if not.  You

might use it in a batch file that only installs a packet driver

if one is not found.

rem only install the packet driver if there isn't one

rem already.

pktchk 0x7e

if errorlevel 0 goto gotit

ni5210 0x7e

:gotit

The "errorlevel" test is true if the errorlevel is less

than or equal to the parameter.

Utility Programs

There are also several utility programs for packet

drivers:

PKTADDR

usage: pktaddr packet_int_no [ethernet_addr]

If the second argument is given, the Ethernet address of

the given packet driver is set.  The Ethernet address is printed

out.

PKTALL

usage: pktall packet_int_no

All packets are received and discarded from the given

packet driver.  This program is of most use with PKTMODE and

TRACE.

PKTCHK

usage: pktchk packet_int_no [packet_int_no]

Test for existance of a packet driver.  Returns with

errorlevel 0 if the specified interrupt has a packet driver.  If

the second argument is given, all interrupts in the range are

checked for a packet driver.  If no packet driver is found at

all, errorlevel 1 is returned.

PKTMODE

usage: pktmode packet_int_no [receive_mode]

If the second argument is given, the receive mode of the

given packet driver is set.  A decimal number from the list of

modes should be used.  All the possible modes are printed out.

Unimplemented modes are marked with "xx", and the current mode

is marked with "->".

PKTMULTI

usage: pktmulti packet_int_no [-f filename | address ...]

The specified addresses are set as allowed multicast

addresses.  If no list of addresses is given, then the current

list of addresses is printed.  The

addresses may either be specified on the command line, or in a

file using the -f option.  When a file is used, any whitespace

in the file is ignored.

PKTSTAT

usage: pktstat first_int_no [last_int_no]

The statistics for all packet drivers in the given range

are printed.  The default range is 0x60 through 0x80.  The

meanings of the columns are given below.

pkt_in  is the number of packets ever received by this driver.

pkt_out is the number of packets ever transmitted by this

driver.

byt_in  is the number of bytes ever received by this driver.

byt_out is the number of bytes ever transmitted by this driver.

pk_drop Packets dropped because there was no handler for that

Ethernet packet type.

err_in  Dependent upon the packet driver.

err_out Dependent upon the packet driver.

PKTSEND

usage: pktsend packet_int_no [-r] [-f filename | packet]

The specified packet is sent using the specified packet

driver.  The -r option says to repeat sending as fast as

possible.  You shouldn't use this option very often.  The packet

may either be specified on the command line, or in a file using

the -f option.  When a file is used, any whitespace in the file

is ignored.

PKTRING

usage: pktring [-m] dest_packet_int_no src_packet_int_no

Test two packet drivers loaded on two Ethernet cards in

the same machine.  Sends packets of every possible length using

the driver located at src_packet_int_no to the driver located at

dest_packet_int_no.  If the -m switch is used, then the various

receive modes are tested.  Note that the multicast tests (modes

four and five) do not work properly yet.

TERMIN

usage: termin [-s] packet_int_no

The specified packet driver is terminated, and its memory

recovered.

The s-option (stop) is used to prepare for termination.

The in-use flag for all handles are cleared. This prevents

upcalls to handlers that are to be removed and also makes it

possible to later terminate the packet driver even though

handles are not released.  Actually, doing termin -s after prom

boot is like cutting the branch you are sitting on.  Recipe for

removing packet driver, IPX and NET:

pktdrvr 0x7c ....

MARKNET C:IPX&NET3.MRK

PDIPX

NET3

. . .

NET3 u                ; unload netx to avoid

communication timeout

TERMIN -s 0x7c        ; pkt drvr no longer calls any

nonexistent rcvrs

RELNET ipxet3.mrk     ; IPX is "removed"

TERMIN 0x7c           ; It is now safe to terminate the

packet driver

TRACE

usage: trace packet_int_no [buffer_size]

Trace is very useful for debugging packet driver

troubles.  Trace lets you trace all transactions between a user

program and the packet driver.  The transactions are stored in a

memory buffer whose size is set with buffer_size.  The default

size is 10,000 bytes.

When you run trace, it sets itself up and then spawns

COMMAND.COM so that you can run a network program that uses the

packet driver.  After you quit your network session, you issue

an "EXIT" command.  This returns you to trace, which writes the

transaction log to "TRACE.OUT".  The following program, DUMP,

interprets TRACE.OUT.

DUMP

usage: dump

Interprets the contents of TRACE.OUT as written by TRACE.

Appendix A

Interrupt usage in the range 0x60 through 0x80, from Ralf

Brown's interrupt list.

60 -- -- reserved for user interrupt

60 -- -- FTP Driver - PC/TCP Packet Driver Specification

60 01 FF FTP Driver - DRIVER INFO

60 02 -- FTP Driver - ACCESS TYPE

60 03 -- FTP Driver - RELEASE TYPE

60 04 -- FTP Driver - SEND PACKET

60 05 -- FTP Driver - TERMINATE DRIVER FOR HANDLE

60 06 -- FTP Driver - GET ADDRESS

60 07 -- FTP Driver - RESET INTERFACE

60 11 -- 10-NET - LOCK AND WAIT

60 12 -- 10-NET - LOCK

60 13 -- 10-NET - UNLOCK

60 20 -- FTP Driver - SET RECEIVE MODE

60 21 -- FTP Driver - GET RECEIVE MODE

60 24 -- FTP Driver - GET STATISTICS

61 -- -- reserved for user interrupt

62 -- -- reserved for user interrupt

63 -- -- reserved for user interrupt

64 -- -- reserved for user interrupt

65 -- -- reserved for user interrupt

66 -- -- reserved for user interrupt

67 -- -- LIM EMS

...

67 DE 00 Virtual Control Program Interface - INSTALLATION CHECK

...

68 01 -- APPC/PC

...

69 -- -- unused

6A -- -- unused

6B -- -- unused

6C -- -- system resume vector (CONVERTIBLE)

6C -- -- DOS 3.2 Realtime Clock update

6D -- -- VGA - internal

6E -- -- unused

6F -- -- Novell NetWare - PCOX API (3270 PC terminal interface)

6F 00 -- 10-NET - LOGIN

...

70 -- -- IRQ8 - AT/XT286/PS50+ - REAL-TIME CLOCK

71 -- -- IRQ9 - AT/XT286/PS50+ - LAN ADAPTER 1

72 -- -- IRQ10 - AT/XT286/PS50+ - RESERVED

73 -- -- IRQ11 - AT/XT286/PS50+ - RESERVED

74 -- -- IRQ12 - PS50+ - MOUSE INTERRUPT

75 -- -- IRQ13 - AT/XT286/PS50+ - 80287 ERROR

76 -- -- IRQ14 - AT/XT286/PS50+ - FIXED DISK

77 -- -- IRQ15 - AT/XT286/PS50+ - RESERVED

78 -- -- not used

79 -- -- not used

7A -- -- Novell NetWare - LOW-LEVEL API

7A -- -- AutoCAD Device Interface

7B -- -- not used

7C -- -- not used

7D -- -- not used

7E -- -- not used

7F -- -- HDILOAD.EXE - 8514/A VIDEO CONTROLLER INTERFACE

7F -- -- HLLAPI (High-Level Language API)

80 -- -- reserved for BASIC