Please see READODI.ME for installation instructions using ODIPKT.COM

with ICE.TCP.

ODIPKT2.COM is a modified version of ODIPKT.COM.

James River Group

**********************************************************************

This software allows the use of networking applications written to the

Packet Driver specification over existing ODI drivers.

Please send any comments or bug reports via email to Dan Lanciani at

Harvard University using the following address:

ddl@das.harvard.edu

ODIPKT - packet driver over Open Data-Link Interface

Usage is:

odipkt [logical_card_number [packet_interrupt]]

where logical_card_number specifies which device to bind (default 0)

and packet_interrupt specifies the packet-driver interface interrupt

(default 105).  All parameters are decimal.

ODIPKT will attempt to adapt itself to 802.5, 802.3, and "ethernet II"

encapsulations (classes 3, 11, and 1).  However, only class 1/11 operation

has been tested.  Note that DOS ODI drivers allow multiple encapsulations

per card with each encapsulation assigned a different card number.  The

sample net.cfg configures all possible ethernet encapsulations.

The ODI spec no longer supports promiscuous mode operation, but

the code to set the receiver mode remains in this driver.

It is not possible to set the physical address of the card.