SNMP Network Management services

================================

This section explains the procedures for configuring the workstation

SNMP services provided with Novell DOS 7.

Through the SNMP agent provided with Novell DOS, workgroups can be managed

from industry-standard SNMP management consoles, from Novell NMS

(NetWare Management System), or from other third-party management systems.

To configure SNMP services on your computer, a MIB (Management Information

Base) must be installed on the computer. Novell DOS provides three MIBs.

HRMIB, the Host Resources MIB, collects information

about any computer running Novell DOS

PNWMIB, collects information about a desktop server

MIB-II, which is a standard SNMP MIB

Note that you require additional software running on a network management

console to monitor the information collected by the SNMP agent, and you

should refer to the documentation for that software to find out how to use

it with the Novell DOS MIBs.

Enabling Desktop SNMP services

------------------------------

If you chose the network option when you installed the operating system,

INSTALL copied the network software, including network management files,

to the directory C:\NWCLIENT, or to the directory you specified.

To enable desktop SNMP services

1.      At the system prompt, type SETUP

2.      Choose the Networking option

You can now page through the network configuration screens.

3.      Select the option Load SNMP Management Software, then save

the changes and exit SETUP.

SETUP automatically does the following:

Adds the command HRMIB to the file STARTNET.BAT on

your computer; this command enables the Host Resources MIB

Adds the commands VLM=MIB2IF.VLM and VLM=MIB2PROT.VLM to the

DOS Requester section in the NET.CFG file on your computer;

these commands enable support for the MIB-II interfaces group

and the TCP/IP groups

Checks whether the computer is configured as a desktop

server, and if it is, adds the commands VLM=PNWMIB.VLM

and VLM=PNWTRAP.VLM to NET.CFG; these commands

enable PNWMIB.

Novell DOS network management support also includes the files HOSTMIB.MIB

and PNW.MIB; these are needed on the network management console to enable

the console to view the data collected by the host resource and desktop

server MIBs respectively. HOSTMIB.MIB and PNW.MIB are held in the directory

C:\NWCLIENT. Refer to the documentation provided with the network

management console to find out how to integrate the MIBs with the software

on the console.

Sample NET.CFG File for Desktop SNMP Services

LINK DRIVER NE2000

PORT 300

INT 3

FRAME Ethernet_802.2

MEM D0000

LINK DRIVER COMX

COMX default parameters go here.

LINK DRIVER LPTX

LPTX default parameters go here.

NetWare DOS Requester

VLM = WSSNMP.VLM

VLM = WSTRAP.VLM

VLM = WSREG.VLM

VLM = WSASN1.VLM

VLM = MIB2IF.VLM

WORKGROUP NAME = PN_WORLD

WORKGROUPID HIGH = 23167

WORKGROUPID LOW = 27843

WORKGROUP NET = 01C22000:FFFFFFFFFFFF

PREFERRED SERVER = GED

FIRST NETWORK DRIVE = F

NAME CONTEXT = "NetwareServices"

WORKSTATION SNMP AGENT

sysName = ""

sysLocation = ""

sysContact =  ""

snmpEnableAuthenTraps =  ON

Enable Monitor Community = SPECIFIED

monitor community =  "public"

Enable Control Community = SPECIFIED

control community = "public"

Enable Trap Community =  SPECIFIED

trap community =  "public"

asynchronous timeout =  20

TRANSPORT PROVIDER IPX

trap target = 01c22000:00001b0ef8f8

trap target = 01c22f12:00001b1e1ef8

TRANSPORT PROVIDER UDP

trap target = 123.123.111.111

trap target = 222.121.111.122

Where To Go From Here

After you have completed the initial configuration of the desktop

SNMP agent, you can modify the SNMP options in the NET.CFG file

to meet your specific network requirements. You can configure the

following features:

Transport providers

MIB-II support

Community names

Asynchronous timeout

Conventional memory usage

The following sections describe each of these options.

Transport Providers

-------------------

The Desktop SNMP transport providers, STPIPX.COM and

STPUDP.COM, read the configuration file to discover trap targets.

You can specify a separate configuration file for each transport

provider when you load it.  The following sections describe trap

target configuration and configuration file specification.

Trap Target Addresses

Trap targets define the addresses to which SNMP sends SNMP

traps.  To receive traps sent by Desktop SNMP, make sure your

management station address is listed in the IPX (for IPX transport)

or UDP (for UDP/IP transport) section of the configuration file read

by the transport providers, as described in the next section,

Transport Provider Configuration File.

Note: By default, no trap targets are defined. If you do not configure at

least one trap target, Desktop SNMP does not send traps.

To add a trap target, add a line indented under the appropriate

transport provider section using the following format:

transport provider <IPX | UDP>

trap target = <IPXAddress | IPAddress>

A trap target configuration for a machine using both IPX and UDP/IP

transport is shown in the following example:

Transport provider IPX

trap target = ab123456:0123456789ab

trap target = cd654321:ba9876543210

Transport provider UDP

trap target = 999.88.77.66

trap target = 888.11.22.33

Transport Provider Configuration File

The transport providers read trap targets from the configuration file.

You can name a specific configuration file, other than NET.CFG, for

the transport providers, STPUDP.COM and STPIPX.COM.  (This

configuration file should match the one given in the VLM = command

line.)  To do this, load them with the /c option, as follows:

STPIPX [/c=[path]configurationFile.CFG]

STPUDP [/c=[path]configurationFile.CFG]

Note: The /c option is also available when you load VLM.EXE.

When you specify the configurationFile.CFG without the path, the

transport provider searches starting at the current directory.

When you do not use the /c option, the transport provider uses the

same configuration file used by the link support layer module,

LSL.COM.  This is usually NET.CFG, unless a different file is

specified using the /c option when LSL is loaded.

For example, to load STPIPX with the SNMPNET.CFG configuration

file, use the following command:

STPIPX /c=SNMPNET.CFG

MIB-II Support

--------------

Desktop SNMP provides support for SNMP MIB-II in three areas:

System and SNMP Groups

Interfaces Group

TCP/IP Groups (all other groups except the EGP and transmission

groups, which are not supported by Desktop SNMP)

System and SNMP Groups

Desktop SNMP automatically supports two MIB-II groups, system

and SNMP.  These groups provide SNMP management stations with

information about your workstation and about Desktop SNMP.

You can define some variables for your workstation in the NET.CFG

file (or other defined configuration file).  Modify the Desktop SNMP

section of NET.CFG to include lines in the following form:

Desktop SNMP

[sysName = "name"]

[sysLocation = "location"]

[sysContact = "contact"]

[snmpEnableAuthenTraps = <ON | OFF> ]

These lines are optional and you can include one without the others.

The first three parameters define information that can be retrieved

by SNMP management stations, or reported in SNMP traps.

Always enclose the Name, Location, or Contact information in

quotation marks.

Following is an example:

Desktop SNMP

sysName = "Suzanne Morley x893"

sysLocation = "Building 2"

sysContact = "suzanne@acompany.com"

snmpEnableAuthenTraps = ON

Interfaces Group

Desktop SNMP provides a separate VLM file, MIB2IF.VLM, that

supports the interfaces group.  This group provides information

about your network connection, such as the number of packets

transmitted and received and whether it is EtherNet or Token-Ring.

To load MIB2IF.VLM, add the following line to the NETWARE DOS

REQUESTER section of NET.CFG:

VLM = MIB2IF.VLM

Note: Always place the MIB2IF.VLM line after the lines that load the

Desktop SNMP VLM suite.

TCP/IP Groups

Desktop SNMP also provides a separate VLM file that supports the

remainder of MIB-II.  MIB2PROT.VLM provides MIB-II support for

the TCP/IP groups (all groups except interface, system, and SNMP,

which are supported by other VLM files, and EGP and transmission,

which are not supported).  Load this VLM when you are using the

TCP/IP stack from LAN Workplace.

To load MIB2PROT.VLM, add the following line to the NETWARE

DOS REQUESTER section of NET.CFG:

VLM = MIB2PROT.VLM

Note: Always place the MIB2PROT.VLM line after the lines that load the

Desktop SNMP VLM suite.

Example of NET.CFG File

On a workstation where the DOS Requester and comprehensive

MIB-II support are both loaded, the entire NETWARE DOS

REQUESTER and Desktop SNMP sections of NET.CFG might read

as follows:

NetWare DOS Requester

VLM = WSSNMP.VLM

VLM = WSTRAP.VLM

VLM = WSREG.VLM

VLM = WSASN1.VLM

VLM = MIB2IF.VLM

VLM = MIB2PROT.VLM

Desktop SNMP

sysName = "Suzanne Morley x893"

sysLocation = "Building 2"

sysContact = "suzanne@acompany.com"

snmpEnableAuthenTraps = ON

Communities

Desktop SNMP provides default community names for the monitor

(read-only) and control (read/write) communities, as well as a default

community used for traps.  Desktop SNMP and other SNMP entities

use these names for access control.  The community name

contained in a request message from an SNMP management station

must match the name expected by Desktop SNMP.

Note: If Desktop SNMP receives a request PDU whose community name

is not authorized, it does not respond to the request.  For example,

suppose the control community name is secret, and Desktop SNMP

receives a SETRequest PDU with a community name of public.

Desktop SNMP discards the SETRequest PDU and does not

respond to the PDU.  However, Desktop SNMP does send an

authentication trap to the trap targets if snmpEnableAuthenTraps is

ON.

Community types can also be disabled.  When a community type is

disabled, no management entity can access information for that

community.  For example, if you disable the control community, no

one can use Desktop SNMP to do SET operations against the data

it manages.

In the Desktop SNMP section of the NET.CFG file, you can define

three communities.

The syntax for defining a community in the NET.CFG file is as

follows:

Desktop SNMP

[Enable communityType Community = Off | Any | Specified ]

[communityType Community = "communityName"]

Examples of NET.CFG Files

To use the default trap community, allow any community name to be

used for read access, but set the read/write community name to

secret, place the following lines in NET.CFG:

Desktop SNMP

Enable Monitor Community = Any

Enable Control Community = Specified

Control Community = "secret"

To disable all traps and use the default values for the monitor and

control communities, place the following lines in NET.CFG:

Desktop SNMP

Enable Trap Community = Off

To specify a NULL string for the control community and use the

default values for the monitor and trap communities, place the

following lines in NET.CFG:

Desktop SNMP

Enable Control Community = Specified

Control Community = ""

Setting a NULL string is the same as setting the ENABLE

COMMUNITY line to OFF.

To allow only the private community to have access, and set the

community name for traps to AgentTrap, place the following lines in

NET.CFG:

Desktop SNMP

Enable Monitor Community = Specified

Monitor Community = "private"

Enable Control Community = Specified

Control Community = "private"

Enable Trap Community = Specified

Trap Community = "AgentTrap"

To temporarily allow any community name to have read-only access

in the previous example, modify the lines in NET.CFG as follows:

Desktop SNMP

Enable Monitor Community = Any

Monitor Community = "private"

Enable Control Community = Specified

Control Community = "private"

Enable Trap Community = Specified

Trap Community = "AgentTrap"

Desktop SNMP ignores the Monitor Community = "private" line until

you reset the Enable Monitor Community line to Specified. This

allows you to temporarily change the setting to Any or Off without

deleting the specific community name.

Asynchronous Timeout

By default, Desktop SNMP waits 20 system ticks, where each tick

is approximately 55 milliseconds (ms), before attempting to cancel

requests it has made against managed objects.  You can change

this time by placing a line of the following form in the NET.CFG file:

Desktop SNMP

asynchronous timeout = numberOfSystemTicks

For example, to have Desktop SNMP wait 35 system ticks

(approximately 2 seconds) before attempting to cancel a request,

add the following line to the NET.CFG file:

Desktop SNMP

asynchronous timeout = 35

Conventional Memory Usage

If you are willing to trade conventional memory for performance, you

can load one or more Desktop SNMP VLM files in conventional memory.

Note: To load *all* VLM files in conventional memory, use the VLM /mc

switch when running the VLM manager.

To load a Desktop SNMP VLM in conventional memory, place lines

of the following form in the NET.CFG file:

NetWare DOS Requester

VLM = VLMName.VLM

Load low VLMName = On

For example, to load Desktop SNMP while loading WSASN1.VLM

and WSREG.VLM in conventional memory, place the following lines

in NET.CFG:

NetWare DOS Requester

VLM = WSSNMP.VLM

VLM = WSTRAP.VLM

VLM = WSREG.VLM

VLM = WSASN1.VLM

Load low WSREG = On

Load low WSASN1 = On