===============================( README.DOC )===============================

README.DOC for Cake Pro

$Date:   29 April 1993 11:51:50  $

Cakewalk Professional is copyright (c) Greg Hendershott, 1988, 1989, 1990,

1993. All rights reserved.

Cakewalk Professional is a trademark of Twelve Tone Systems, Inc.

==============================( Release 5.0 )==============================

[] If you are installing Cake Pro for the first time, follow the normal

instructions in the manual, but also see information on "Drivers" below.

[] If you have already installed Cake Pro 4.0, you should install this 5.0

maintenance update, because it eliminates problems that some users had with

playback occasionally freezing.  Please follow the normal instructions in the

User's Guide.  NOTE: installing this will replace your existing MIDI driver

(.DRV) files with the new ones.  If you've run Setup to change the MIDI

interface settings to other than their factory default, you'll need to do so

again after installing.

-------

================================( DRIVERS )=================================

-------

Following is a list of Installable Port Drivers supplied with this

release:

MPU401.DRV	MPU-401 compatible (incl. Music Quest PC MIDI Card)

MQX16.DRV	Music Quest MQX-16

MQX16S.DRV	Music Quest MQX-16S

MQX32.DRV	Music Quest MQX-32 or MQX-32M

CMS101.DRV	Computer Music Supply CMS-101

CMS444.DRV	Computer Music Supply CMS-444

CMS444Z.DRV	Computer Music Supply CMS-444Z

IBMPS1.DRV	IBM PS/1 Audio Card and Joystick Option

KEY101.DRV	Key (KEE) MIDIator MS-101

KEY103.DRV	Key (KEE) MIDIator MS-103

OPTRON.DRV	Optronics Basic MIDI I/O card

YAMAHAC1.DRV	Yamaha C1

MQ32_MPU.DRV	1 Music Quest MQX-32(M) and 1 Roland MPU-401 compatible

_2MQ32.DRV	2 Music Quest MQX-32(M)'s

+-------------------------------------------------------------------------+

NOTE: These drivers are copyrighted material just like Cake Pro itself.

Your license does not include the right to copy them for other than

personal back-up purposes or the right or modify them.

+-------------------------------------------------------------------------+

REMEMBER to copy the appropriate driver file to a file named MIDIPORT.DRV.

Cakewalk needs to find MIDIPORT.DRV when it starts; otherwise you will get

an error message.  For example, if you are using an MPU-401 compatible,

you need to enter the following at the DOS Prompt:

COPY MPU401.DRV MIDIPORT.DRV

Then, if you are using a MIDI interface with other than its

factory-default settings -- for example, an MPU-401 compatible on other

than IRQ 2 and base port 330 -- you must run Setup.  Start Cake Pro by

entering:

CAKEPRO -S

Select choice (3), "MIDI Interface Settings", and enter the appropriate

information.

NOTE: IF YOU ARE UPGRADING FROM 3.0 AND ARE USING AN INTERFACE WITH OTHER

THAN FACTORY-DEFAULT SETTINGS, YOU MUST RUN SETUP AGAIN.  The settings you

previously made used to be stored in CAKEWALK.INI, but Cakewalk no longer

looks in CAKEWALK.INI for these values.  It looks in MIDIPORT.DRV.  When

you run Setup, the values you set are stored in MIDIPORT.DRV.  If you ever

recopy the original .DRV file to MIDIPORT.DRV, you will need to run Setup

again.

=====( Key MIDIator MS-101/103 Notes )======================================

Even if your serial port works fine for other software, the speed or

hardware timing may be off enough that it won't work with the Key

MIDIator, which requires exact conformance.  It's also critical to get a

properly wired serial cable that connects all the required pins; the

MIDIator uses some pins that most applications don't need.  If Cakewalk

can't find your Key MIDIator, first make sure you've selected the right

IRQ number and address for your serial port (COM1 = IRQ4 3F8, COM2 = IRQ3

2F8).  If that's ok, try a different serial cable.  If you notice

occasional or constant garbling of MIDI output, you should call Key

technical support at 817-560-9751.  They will provide you with a way to

test your serial port with the Key interface and get it working if

possible.  Please note that Key is much better equipped to help you with

this problem than we are, since this is not a Cakewalk-specific problem.

=====( MQX-32 Notes )=======================================================

If you have an MQX-32 or MQX-32M interface, and find that Cakewalk

Professional is sending all MIDI output to both ports, rather than the

ports you select in the Track View, then first verify that you're using

the MQX-32 driver, MQX32.DRV.  If so, then the problem is that you may

have an older MQX-32 that cannot separately address its MIDI ports when in

UART mode.  Contact Music Quest at (214) 881-7408 for information on how

to obtain a ROM upgrade for your interface card.

=====( Warning About SCSI Drives )==========================================

Some SCSI hard drive controller cards use IRQ 2.  Do not attempt to use a

MIDI interface set to IRQ 2 with such a hard drive: loss of data may

result.

If you are not sure what kind of hard drive controller you have, then you

*probably* don't have a SCSI controller.  They generally cost more than

ST-506 or ESDI controllers and usually don't come as standard equipment.

===( Systems with only 360K floppy drives )================================

NOTE: If you have a system with only 360K floppy drives (no hard drive),

you will not be able to use the help system.  There is barely room for

CAKEPRO.EXE on a single 360K disk.  As a result, the help file

(CW4PRO.HLP) will not fit on the same disk.  Similarly, the Dump Request

Macros stored in CAKEWALK.SXT will not be available to Cakewalk

Professional on start-up, because there is no room for CAKEWALK.SXT

either.

===( Do NOT rename CAKEPRO.EXE )===========================================

If you rename the executable file, Cakewalk Professional will not be able

to load overlays.  (If don't like to type in CAKEPRO, instead make a batch

file that calls the CAKEPRO.EXE file, and give the batch file the name

that you prefer.)

----------------------------

====================( MANUAL ADDITIONS/CORRECTIONS )=======================

----------------------------

===( Disabling MIDI Metronome )============================================

To disable the MIDI metronome, you put a zero in the *Velocity* field, NOT

the Duration field as the manual says.

===( SMPTE "Midnight roll-over" )==========================================

Cakewalk Professional 5.0 can sync to tapes striped beginning with SMPTE

times 23:59:59:00 or higher and rolling-over to 00:00:00:00.  Some studios

will provide tapes in this style, with the first action occurring at

00:00:00:00.  Rather than having the studio re-stripe the tape so that the

action begins at an offset like 00:00:02:00, you can specify an offset of

00:00:00:00 to Cakewalk Professional 5.0.

===( -D flag and Sysx Meta-Events )========================================

System Exclusive banks can be transmitted three ways in Cake Pro:

* You press the "Send" button in "View/System Exclusive".

* You set a bank as "Auto", so that whenever the file is

loaded, the bank is automatically sent.

* You insert a "Sysx" event in the Event View which triggers

transmission of the bank at that time during playback.

The third method has some restrictions.  As the manual notes, these

messages can't be very long, because MIDI doesn't allow much else to

happen while a sysx message is in progress.  For similar reasons, the "-D"

command line flag -- which slows down transmission of system exclusive

data -- has no effect on banks which are sent as "Sysx" meta-events during

playback.  Instead, Cake Pro sends the message as fast as possible so as

to have the least-disruptive effect on playback.  This means, however,

that if one of your instruments can't receive sysx at the full MIDI

bandwidth, you won't be able to send it sysx during playback.  You'll need

to use one of the first two methods.

=====( IBM PS/1 Owners Only:  Using the IBM PS/1's Thru/Out Port )==========

The IBM PS/1 Audio/Joystick Card has a MIDI Thru/Out port which can be set

to either Thru or Out.  When it's set to Thru, it outputs a copy of what

comes into the MIDI In port.  This is true even when Cake Pro is not

running.  When it's set to Out, it outputs a copy of what the MIDI Out port

is sending.  Cake Pro must be running to generate this output.

When the IBM PS/1 starts up, the MIDI Thru/Out port is set to Out.  We have

supplied two programs you can run to set this the way you prefer.  At the

DOS prompt, type PS1THRU to set it to Thru and PS1OUT to set it to Out.  You

may want to put one of these commands in your AUTOEXEC.BAT file, so it gets

set right automatically every time your PS/1 starts up.  Note that

Cake Pro sets it to Out while running, and restores the original state

when done.

Hint: You may want to put PS1THRU in your AUTOEXEC.BAT file, and then use

the MIDI Thru/Out Port instead of the MIDI Out Port.  This way, when

Cake Pro is running you get what the program sends, and when the program

is not running you get an echo of MIDI In.

===( Using Music Quest MQX Chase Lock Sync (CLS) )=========================

When using MQX Chase Lock Sync (CLS) you must run the Cake Pro Setup each

time you want to switch between two modes.  One mode is for striping a tape

(which you do infrequently).  The other mode is for playing in sync to a

striped tape (which is what you'll do most of the time).  So, you'll normally

leave Cake Pro configured to SYNC to CLS.  Whenver you need to stripe a new

tape, you need to run Setup to select STRIPING mode, stripe the tape, then

run Setup again to leave the mode set back to SYNC to CLS.  Following are

step-by-step instructions for each of these two procedures.

Note:| If you use SMPTE sync with Cakewalk Professional, none of this

| applies to you.

-----( Striping Chase Lock Sync )-----

When you wish to stripe a tape with CLS, you'll need to run the Setup

option to configure the driver and MQX card for striping mode:

(1) Start Cake Pro with the " -S" flag to run Setup.

(2) Pick the "MIDI Interface" choice.

(3) Set the sync choice "Stripe CLS", and press ENTER to go back to

the main setup menu.

(4) If you'll finish all your striping during this one Cakewalk session,

it will be easiest NOT to save this configuration.  Don't press "S" to save,

just press "C" to run Cake Pro with these new settings.

(5) Follow the normal procedure for striping a tape.  Here's a quick

summary of the information you should already be familiar with from the

Cake Pro and MQX Card manuals:

Connect an audio cable from the card's Tape Out jack to your tape

recorder's input.  Don't use noise reduction on the track with CLS.

Load your work file, and set Settings/Clock to "Internal".  Start the tape

recording, wait a few seconds, and then play your work from start to finish.

-----( Syncing to Chase Lock Sync )-----

Note:| Normally you'll need to do steps (1)-(5) only when you've just

| been striping a tape, and now want to switch the mode to

| syncing.  If you've already switched the mode to syncing to CLS

| and saved it, you won't need to run Setup each time.

(1) Start Cake Pro with the " -S" flag to run Setup.

(2) Pick the "MIDI Interface" choice.

(3) Set the sync choice "Sync to CLS", and press ENTER to go back

to the main Setup menu.

(4) Press "S" to save these settings. in the driver file.  You'll probably

want to do this because most of the time you'll be syncing to CLS, not

striping a tape with CLS.

(5) Press "C" to run Cake Pro now with these new settings.

(6) Follow the normal procedure for syncing to CLS.  Here's a quick summary

of the information you should already be familiar with from the Cake Pro

and MQX Card manuals:

Connect an audio cable from your tape recorder's sync output to the card's

Tape In jack.  Don't use noise reduction on the track with CLS.

Load your work file.  Set Settings/Clock to "MIDI Sync", if you haven't

already done so for this file.

Set Cake Pro to record or play (use Chase Mode if you like), and start

your tape playing wherever you like.  Cake Pro will chase to the right

location and begin record/playback.

[][][]===============================================================[][][]

Please remember to return your product Registration Card within 30 days of

purchase.  Doing so qualifies you for technical support, upgrades,

newsletters, offers on MIDI products available only to registered

users...and more!

****************************************************************************

End of README.DOC

****************************************************************************