*********************************************

*                                           *

*    SCSI01.SYS                             *

*    OS/2 Disk Driver with SCSI Support     *

*    Functional Overview                    *

*    OS/2 Version 1.1                       *

*                                           *

*********************************************

Introduction:

DISK01.SYS, the disk driver shipped with the OS/2 SDK and

BAK, provides support for standard floppy and hard drives on

AT machines.  SCSI01.SYS provides all the functionality of

DISK01.SYS, and in addition, supports the Adaptec AHA-154x

line of multitasking SCSI host adapters. As with DISK01.SYS,

OEMs wishing to use SCSI01.SYS and the AHA-154x may want to

modify the driver to best support their particular hardware

configurations.

The SCSI portion of SCSI01.SYS is made up of two different

sections:  The SCSI Manager, and the SCSI Disk Driver.

The SCSI Manager:

The SCSI manager is implemented as a character device which

loads before the disk portion of DISK01.  The SCSI manager

provides a generic multitasking interface to the AHA-154x

host adapter.  Any device driver can access the SM through

the ATTACH Device Help function in OS/2. For more

information on the specification for interfacing to the SCSI

manager, refer to the Adaptec OS/2 SCSI programming

interface specification.  Additional drivers for devices

such as Tape, etc.  will be available in the future

from Adaptec to take advantage of this interface.  Note that

the SCSI manager is also available as an installable driver

without the disk support.  If this driver is installed, the

SCSI manager in the disk module is de-installed and the

installable driver takes over the SCSI manager duties.

SCSI Disk Driver:

The SCSI Disk Driver uses the SM interface to provide

multitasking Disk I/O for SCSI Hard Drives. The

functionality of SCSI01.SYS for Hard  Drives is intended to

mirror DISK01.SYS.  Basically, SCSI01.SYS will support all

hard drive configurations supported by the AHA-154x BIOS.

These include:

(1) Both Drive C: and Drive D: are standard Hard drives.

(2) Drive C: is standard, Drive D: is SCSI Target 0

LUN_0.

(3) Drive C: is SCSI Target 0 LUN 0, Drive D:, if

installed, is  SCSI Target 0 LUN 1 or Target 1 LUN 0.

Like  DISK01, SCSI01 is presently limited to 2 Hard drives.

As future versions of OS/2 supply DISK01 and FDISK code

which supports more than 2 hard drives, SCSI01 will

obviously support as many SCSI drives as possible.

SCSI01.SYS does not use the ROM BIOS for Interrupt 13H

requests from the DOS compatibility box. Instead, interrupt

13H functionality is emulated by the SDD through the

multitasking SCSI manager.

SCSI01 Uses the OS/2 seek optomization routines when

queueing disk requests.

The SCSI Disk Driver presently assumes that all SCSI hard

drives on the system support the extended verify command.

In the future, SCSI01.SYS will be modified to work with

drives without this support as well as to take advantage of

drives which support the WRITE w/ VERIFY command.

At the present time, SCSI requests do not go through the

cache used for requests to standard hard drives.  It is

currently being studied how to handle caching for SCSI

drives inside the driver.

Support for LAN Manager 1.1 high performance disk caching and

Fault Tolerance is not yet implemented.

Although support for the AHA-154x High performance host

adapter is included in DISK01.SYS, the SCSI support routines

are in no way related to the single tasking state machine of

the standard AT controller support. The driver completely

supports multitasking between multiple drives and other

devices on the bus, and makes full use of the low overhead

provided by the host adapters BUS MASTER DMA and Intelligent

I/O processing.

The Only testing performed on the SCSI manager so far is

through the SCSI disk support in the same code module.

Although additional drivers should be able to ATTACH to this

version of code, this has not been tested.  In addition,

the ABORT function of the SCSI Programming interface is not

yet implemented.

SCSI01.SYS does not support the AHA-1640 Microchannel

adapter at this time.

To use SCSI01.SYS simply rename it to DISK01.SYS and copy it

to an OS/2 1.1 install diskette.