TDAT

TDAT is a program for collecting DAT straemer and tapes statistics,

specific it will translate the information for WangDAT and Sony streamers.

All DAT streameres support the LOG SENSE command, where some log pages

can be inquired. Some log page hold info about the streamer (over it

life time), fx. about total number of loaded tapes, and some log pages

are read from the current tape, fx. about total written groups.

The tape specific log pages are written to the tape before it's unloaded

(if it's not write protected!!!).

Some defintions:

Session = The period of time from tape mount to unmount

Track   = 5.184 bytes

Frame   = 2 * Track = 10.368 bytes

Group   = 23 * Frame = 238.464 bytes

Total count is since last format, and current/previous count

is for one session.

The syntax of TDAT is:

TDAT -u 4 log 2       ->     Get log page 2 from SCSI 4

Example output from ALLWANG.BAT:

--------------------------------------------------------------------------

Page 00 Size 9

Supported Log Pages : 00 01 02 03 30 31 32 33 34

Page 01 Size 8

Buffer Over/Under-Run

Under-Run Count  : 0

Page 02 Size 16

Write Error Count

Total Write Errors     : 0

Previous Write Errors  : 0

Current Write Errors   : 0

Page 03 Size 16

Read Error Count

Total Read Errors     : 0

Previous Read Errors  : 0

Current Read Errors   : 0

Page 30 Size 16

Groups Written

Total Write Groups     : 15762

Previous Write Groups  : 1979

Current Write Groups   : 0

Page 31 Size 16

Groups Read

Total Read Groups     : 1072

Previous Read Groups  : 40

Current Read Groups   : 0

Page 32 Size 8

Medium Mount

Mount Count  : 8

Page 33 Size 22

Drive Usage Log

Loaded Resets          : 12

Number of Loads        : 248

Number of Cleans       : 3

Maximum Cleaning Cycle : 44

Minimum Cleaning Cycle : 14

Last Cleaning Cycle    : 387

Drum Loaded Time       : 5348

--------------------------------------------------------------------------

Some comments about the output from a WangDAT (most also holds for a Sony):

Log pages 0+1+33 is read from streamer.

Log pages 2+3+30+32+32 is read from tape.

Page  1: Run count is per session, and tells the number of times

the streamer had to wait for data from the SCSI bus.

Page  2: No of write errors corrected by rewriting a frame.

Page  3: No of track read errors corrected with C3 ECC.

Page 32: Mount count is the cumulative number of times the tape

has been mounted in a streamer.

Page 33: Load Resets is the no of times the streamer has been reset

with a tape in place.

Cleaning Cycle is the largest/smallest time (in hours) between

cleaning cycles.

Last Cleaning Cycle is the elapsed time (in minutes) since the

last cleaning cycle.

Drum Loaded Time is the time (in minutes) that the drum has

been loaded against a tape.

TDAT (c) was programmed by Prins, and this doc written by ENUS.