SUPERVIEW : Notes for Release Version 3.00

------------------------------------------

Version 3.00 and subsequent releases of SuperView have many additional

features related to the Stress-to-Nodes method of analysis used by

Algor's stress and dynamic processors.  These features provide a more

practical and precise analysis of your model, making it easier to understand

and visualize how your model reacts under stress.  This in turn

simplifies the identification of problem areas in your model, pinpointing

areas where increasing mesh density will improve your analysis.

These release notes will eventually be incorporated into the documentation

for Algor's ViziCad Design and Visualization system.

List of New Features and Options - Overview

-------------------------------------------

Release 3.00 and subsequent releases of SuperView calculate stresses

at nodes rather than at Gaussian points or element surfaces.  This

provides stress analysis at practical points of interest where stresses

are likely to be highest.  Nodes shared by multiple elements receive

multiple estimates of the stress at that location.  These multiple

estimates of stresses at a single node are compared to determine the

precision of the analysis of your model at that point.

New post-processing and dithering options have been added to SuperView

to make use of this new information.  The Stress-Dithering ("Stress-di")

menu has been reorganized to group associated commands more intuitively.

The Post-Processing ("Post") menu now only contains dithering options.

An Auxiliary Post-Processing ("Aux post") menu has been added to

the Stress-Dithering menu to group all of the options shared between

post-processing styles.

Additional features added to SuperView include an easy way of obtaining

textual stress output from your model, and a method of specifying

a stress threshold used when dithering.  The threshold value enables

you to provide a lower limit for the range of stress values shown

in the dithered display and in the text output file.  Nodes and elements

with stress values below this threshold are not displayed.  This enables

you to instantly identify elements with excessive stress values.

The following is a list of the new features discussed in the remainder

of this document.  All new commands are found in SuperView's "Stress-di"

menu, or in its "Post", "Aux-post", or "General" submenus.  You can

access the "Stress-di" menu at any time by pressing the function key

labeled "F9", and selecting the "Stress-di" menu using the mouse or by

pressing the "S" key on your keyboard.

The topics discussed in this document are the following:

Overview of Changes to the "Stress-di" Menu

Overview of Changes to the "Post-DI" Menu

The "Aux Post" Menu

Displaying Stresses using a Threshold

ASCII Output of Stress Information

Precision Dithering

Gap and Boundary Element Dithering

The New "Max abs" and "Smoothed" Dithering Options

Contents of the "Aux post" Menu

Changes to the "General" Menu

New Files Used by SuperView

Overview of Changes to the "Stress-di" Menu

-------------------------------------------

The new "Stress-di" menu contains the options which affect the style,

format, and display of the dithered analysis of your model.  The following

menu items have been added to the "Stress-di" menu:

The "Aux post" Menu

This menu contains auxiliary actions associated with post-processing

dithering.  This menu is discussed later in this document.

The "Max abs" Dithering Display Option

This is an option you can set to cause the signed stress with the

largest absolute value to be used in the display of your dithered

model.  This option is discussed later in this document.

The "Contour" Option

This option has been moved to the new "Aux post" menu, and

has been renamed "L contour".  This option is discussed later in this

document.

The "Mesh" Option

The "Mesh" option has been included in the "Mesh col" command on the

"General" menu.  The use of the "Mesh col" option is explained later

in this document.

Overview of Changes to the "Post-Processing" Menu

-------------------------------------------------

The "Post" menu includes the new "Precision" dithering option.  "Precision"

dithering is explained later in this document.  A new option, "Gap+bdry",

has been added to enable you to view forces on gap and boundary elements.

This option is explained later in this document.  The "Get val", "Use

abs", and "Colors" options found on this menu prior to release

3.00 are now found on the "Stress-di" menu's "Aux post" submenu, discussed

elsewhere in this document.

The "Aux Post" Menu

-------------------

The new "Aux post" menu contains all of the auxiliary information and

options used in post-processing.  The options on this menu are grouped

separately because these items apply only to post-processing dithering

methods.  Items on the "General" menu apply to all three forms of

dithering (Hidden line, Light, and Post-Processing).  Items added to the

"General" bmenu are discussed later in this document.

The options you can set in the "Aux post" menu are the following:

Auto rng - This is the default ranging method

active when the "Aux post" menu is first opened.  If this option

is active, the dithering range is taken from the maximum and minimum

stress values found when analyzing your model.  When you select this

option to deactivate it, you are prompted for a range of values to

use when dithering.  This option was previously located on the "General"

menu.

Threshold - When you select this option, you are

prompted for a threshold value.  This value determines the lowest

stress magnitude displayed when your model is dithered.  The magnitude

is the absolute value of the stress at a given node or element.  Only

elements with stress magnitudes greater than this threshold value

are displayed.  See page <$R[P#,THRESH]6>  for more complete

information on using the threshold value.

Use abs - When you select this option, the absolute

value of the stress at a node determines the color assigned to that

node during dithering. This option is inactive by default.  This option

was previously located on the "Post" menu.

Backside - This option sets whether you want to view stresses from

the front or the back of Types 6, 15, and 16 elements.  This

option is inactive by default, meaning that stresses from the "front"

sides (the sides facing you) of these elements are displayed.  This

option was previously located on the "General" menu.

Type 6 sw - This option sets switches for the

display of Type 6 elements.  Selecting this menu option causes the

"Type 6 sw" menu to open.  This menu contains the same items

as in previous releases of SuperView.  This option was previously

located on the "General" menu.

Get val - This option enables you to graphically select an element

whose stress values you want to examine.  This option was previously

located on the "Post" menu.

File out - This option enables you to specify

the file to which stress information about the current dithering method

is written in tabular, text format. This information reflects the

current post-processing selection on the "Stress-di:Post" menu.

The "File out" option can be used with the "Threshold"

option to write information about nodes above a certain stress threshold

to the file.  This is explained later in this document.

L contour - This option enables you to display contour lines in dithered

stress output.  This option is inactive by default, meaning that contour

lines are not shown.  This option was previously located on the "Stress-di"

menu.

Colors - This option enables you to assign the

colors used to display dithered, post-processed output.  This option

was previously located on the "Post" menu.

Displaying Stresses Using a Threshold

-------------------------------------

When you select the "Threshold" option from the "Aux post" menu, the

the  "Enter Threshold" prompt is displayed in the dialog region at the

bottom of the screen. To display only those elements whose nodes have

stresses above a certain value, enter this value and press the return key.

The default setting for the threshold value is off when you first execute

SuperView, so that all stress information is displayed when your model is

dithered.  If you select the "Threshold" command, but do not want to change

the current threshold setting, press the return key when the prompt

is displayed.

When a model is dithered and the threshold value is set, only those

elements whose stress magnitudes are greater than that threshold are

re-drawn.  If you have already dithered your model, and then set a

threshold value, the previous dithered display is not erased.  To

clear a previous dithered display before dithering with a threshold

value, you can use the "F10:Redraw" command, or you can change

the background color to a color other than 0 (the default).  Using

the "F10:Redraw" command erases the screen and redraws the mesh

for your model.  Setting the background color to a color other than

0 causes only the elements above the threshold value to be displayed.

The mesh is not drawn.

ASCII Output of Stress Information

----------------------------------

The "File out" command enables you to write tabular information about

the stresses in your model to an ASCII text file.

Selecting the "File out" option from the "Aux post" menu

causes the following prompt to be displayed in the dialog region at

the bottom of the screen:

Enter name of output (.out) file: (model)

The default name for this file is the same as the name of your model.  To

write this information to the default file "model.out" (where

model is the name of your model), press return when the prompt

is displayed.  To specify another filename, press the <ESC> key to

clear the default setting, enter the name of that file, and press

return.  The default extension for these ASCII files is ".out".  If

you provide a filename with an extension, the extension is discarded

and the ".out" extension is appended to the name you specified.  You

may want to append a few letters to the specified name to indicate

the stress type and the number of the current load case.  If your

filename is six or fewer characters long, you could append two characters

and press return.  For example, the characters "2M" could be used

to indicate that the file contains information about von Mise's stresses

for load case number 2.

Information about the stresses in your model is written to the specified

file as soon as you press return.  This stress information is based

on the currently active post-processing options.  Only information

about elements which are currently displayed is written to the file.  No

information about hidden elements (if any) is written to the file.

If the "Aux post" menu's "Threshold" option is active, only

information about nodes whose stresses are higher than that threshold

value is written to the text file.

Precision Dithering

-------------------

Selecting "Precision" from SuperView's "Post" menu activates this option.

An asterisk appears beside the "Precision" option.

The "Precision" dithering option uses information provided by

the "Stress-to-Nodes" method of analysis.  (This information is only

produced when you have used v9.0.0 or greater of Algor's processors

to analyze your model.)  Adjoining elements with common nodes provide

independent predictions of the stress at these common nodes. When

displaying a dithered view of a model using the "Precision"

post-processing options, these multiple estimates of stresses at shared

nodes are compared to provide a precision index for these nodes.  Dithered

colors show the precision of the analysis at each node, based on this

comparison.  A precision index that is high in a critical or highly

stressed area indicates that increasing the density of the mesh in

that area may be necessary to obtain the most precise results possible.

Gap and Boundary Element Dithering

----------------------------------

The "Gap+bdry" option added to the "Post" menu enables you to dither

based on moments or reaction forces on Gap and Boundary elements.  When

this option is active (preceded by an asterisk), the scale displayed in

the drawing window's legend box indicates the range of forces on these

elements.  Each Gap and Boundary element is colored with a small square

indicating the degree of force at that element.

When you selecting this menu option, the "Gap+bdry" submenu

opens.  The options on this submenu have the following meanings:

Force - Dither and display the stress table based

on reaction forces.  Each boundary or gap element is marked with a

small, colored square which indicates the force on that element.

Moment - Dither and display the stress table based

on moments.  Each boundary element is marked with a small, colored

square which indicates the stress on that element.

Size - Enables you to set the size of the colored

square which indicates stresses on these elements.  Enter the new

size and press <CR>.  The default value is 3.

The New "Max abs" and "Smoothed" Dithering Options

--------------------------------------------------

The "Stress-di" menu also includes the new "Max abs" stress display option.

When active, this display option colors each element corresponding to the

highest stress, in absolute value, on that element.  This provides a

worst-case display, making it easy to recognize the areas in your model

with the highest stress.  You can use this option in conjunction with the

existing "Smoothed" stress display option to produce a dithered display

based on the largest stress predicted at each node.  Without the "Max abs"

option, the "Smoothed" option produces a smoothly dithered display using the

average of the predicted values at each node.

For elements with both front and back stresses, such as Types 6, 15,

and 16, using the "Max abs" option without the "Smoothed"

option causes stress values on both the front and back of each node

to be considered.  With the "Smoothed" and "Max abs" options

on, only the front stress (or back stress, if the "Backside"

option is on) of the highest magnitude is considered.

Changes to the "General" Menu

-----------------------------

The most significant change made to the "General" menu is the new use

of the "Mesh col" command.  This command now combines both the "Mesh col"

and "Mesh" commands from previous releases of SuperView.

The  new commands which appear on the general menu are the following:

"Inner 5" - This menu command replaces the "5) ty5 sw" command.

Previously, this  menu command controlled multiple display options for

Type 5 elements, and now simply toggles whether the inner faces of Type 5

elements are displayed.  This option is inactive by default, meaning that

the inner faces of these elements are not displayed during stress dithering.

When the "Threshold" option is on, inner faces are always drawn.  This

insures that inner bricks with stress magnitudes above the threshold are drawn.

"Mesh col" - This menu command controls whether the mesh is visible on your

dithered model and, if so, enables you to set the color of the mesh.  The

"Mesh col" option is active by default (preceded by an asterisk), meaning that

the mesh is shown when the dithered view of your model is displayed.

Selecting the "Mesh col" option when it is active disables the drawing the

mesh when your model is re-displayed.  Selecting "Mesh col" when it is

inactive turns on drawing the mesh and enables you to change the color of the

mesh by prompting you for a new color index number in the area below

the model display window.  Change the color by entering the index number of

the new color and pressing <CR>.  If you do not want to change the color,

press <ESC> rather than entering a new color index number.  This

toggles the mesh display without changing the mesh color.

New Files Used By SuperView

---------------------------

model.NSO - This file is a binary-format stress output file, used by

SuperView to display and interpret your model.  If this file is not

present, SuperView displays an error message, and can not display

stresses for your model.  Files with this extension replace the files

with the .TSO extension used by previous release of SuperView.  The

.TSO file did not contain stress-to-nodes information.