ALGOR Stress Processor - MKNSO.EXE Release Note

MKNSO Version 1.01 - Release Note 3/23/90

1) Starting from release 1.01, all Hyper (386 with 387/Weitek) versions

have been changed to use Phar Lap DOS-Eextender runtime environment.

This should provide a more compatible extended memory support on

386 machines.

2) The default formulation flag in the triangular plate element has been

changed such that no reduced shear integration scheme will be used.

This will make the stress results smoother and better.

MKNSO Version 1.006 - Release Note 1/02/90

For type 4 element, previous version of MKNSO will neglect the

reference temperature setting (reset to zero). This problem

has been fixed. (QAB report)

MKNSO Version 1.005 - Release Note 10/6/89

1) A check for older .DO files has been added. When calculating

stresses at nodal points based on a analysis done on a processor

preceeding the version 9.00 release, a warning message is given.

If the analysis included elements for which the default formulation

has changed, a choice is given as to whether older style or newer

style defaults are to be used. Be sure to choose the default that

applied to the processor that created the displacements.

2) An incorrect output format statement that halted processing of

a subclass of composite element problems has been fixed. Also, the

failure number for composite elements written to the .NSO file is

now an extrapolated nodal point value whereas the previous release

used a gaussian point value.

MKNSO Version 1.004 - Release Note 8/30/89

1) Enhancements have been implemented to avoid some real*4 to real*8

conversion problems.  Some diffculties had occurred when very small

coordinate numbers are used with extremely large element stiffnesses.

This numerical problem has been cured in the current version.

MKNSO Version 1.002/3 - Release Note 8/17/89

1) The previous version of MKNSOH.EXE, when accidentally loaded on

8086/80286 machines, will be executed from processors. The new version

will automatically detect the machine type and will not load the

386-version (MKNSOH) when running on such machines.

2) For plate element, the nodal stress calculations scheme has been

modified such that a smoother results will be calculated.

3) The composite element output from previous versions contain detailed

results at each integration point as well as those at node points.

However, the output are voluminous. The new version, by default, will

output only node point results. If results at Gaussian points are

desirable, the user can set NPAR(6) (see Reference Manual) to 1, and

more detailed results will be printed.

MKNSO Version 1.001 - Release Note 7/20/89

Algor is pleased to announce the release of MKNSO stress

processor.  The MKNSO processor will take the displacement output

file and compute node point stresses. The stresses will be first

computed within the element, and then extrapolated to the nodes

of the elements.  This will give the user more accurate and much

easier to use stress results.

MKNSO will recognize all elements which currently available in

the SUPERSAP FEA processors (types 1,2,3,4,5,6,7,13,14,15,16).

Internally, MKNSO will need a new .SST file to compute the

element stresses.  MKSV.exe can be used to produce the .SST file

for MKNSO.exe. However, this should be transparent to the user

since the processor will automatically call MKSV and MKNSO.

MKNSO will produce a nodal point stress output file (.S file). This

file will contain all stress results. The original processor output

file (.L file) should contain the input information along with

node point displacement results. MKNSO also produces a binary

.NSO output file to be used by SVIEW for graphic viewing process.

The detailed format of this .NSO file can be found in the Advanced

Programmers Kit.